Settlements and Billing

Configuration Guide: CG CC 1102 Black Start Capability Allocation v 5.0

2

Version 5.0

Table of Contents

1.	Purpose of Document	3

2.	Introduction	3

2.1	Background	3

2.2	Description	3

3.	Charge Code Requirements	4

3.1	Business Rules	4

3.2	Predecessor Charge Codes	4

3.3	Successor Charge Codes	4

3.4	Inputs – External Systems	4

3.5	Inputs - Predecessor Charge Codes or Pre-calculations	5

3.6	CAISO Formula	5

3.7	Outputs	5

4.	Charge Code Effective Dates	6

# Purpose of Document

The purpose of this document is to capture the requirements and design specification for a Charge Code in one document.

# Introduction

## Background

Black Start Energy is an Ancillary Service (A/S) that consists of dispatched Energy using Self-Started equipment following a system blackout.   Black Start equipment must be able to start without an external source of electricity and must be available to participate in system restoration under the coordination of California ISO.

The CAISO has the discretion to procure Black Start as needed and may establish agreements with eligible generators for the provision of Black Start services.   The CAISO is also responsible for ensuring that Black Start services are readily available in the event of a system blackout via periodic testing.

When CAISO contracts for Black Start Capability with owners of Reliability Must Run (RMR) or Black Start Generators, suppliers of Black Start Capability shall be paid under CC 3102 Black Start Capability Settlement, while costs are allocated under CC 1102 Black Start Capability Allocation.

## Description

On a monthly basis, for each PTO associated with active BSA resource(s), ISO settlements system shall allocate charges that include:

- the sum of all fixed monthly Black Start capability payments to resources identified as associated to the Participating Transmission Owner (PTO) per all active BSA contracts in MasterFile

- the sum of all PTB Black Start contractual cost recovery payments to resources identified as associated to the Participating Transmission Owner (PTO) per all active BSA contracts in MasterFile

.

# Charge Code Requirements

## Business Rules

| Bus Req ID | Business Rule |
| --- | --- |
| 1.0 | On a monthly basis, for each PTO associated with active BSA resource(s), ISO settlements system shall allocate charges that include: |
| 1.1 | the sum of all fixed monthly Black Start capability payments to resources identified as associated to the Participating Transmission Owner (PTO) per all active BSA contracts in MasterFile |
| 1.2 | the sum of all PTB Black Start contractual cost recovery payments to resources identified as associated to the Participating Transmission Owner (PTO) per all active BSA contracts in MasterFile |
| 1.0 | Granularity of charge is on a monthly basis. |
| 2.0 | Pass Through Bill Charge Adjustment Logic does not apply. |

## Predecessor Charge Codes

Charge Code/ Pre-calc Name

CC 3102 – Black Start Capability Settlement

## Successor Charge Codes

Charge Code/ Pre-calc Name

None

## Inputs – External Systems

| Row # | Variable Name | Description |
| --- | --- | --- |

## Inputs - Predecessor Charge Codes or Pre-calculations

| Row # | Variable Name | Predecessor Charge Code/ Pre-calc Configuration |
| --- | --- | --- |
| 1 | MonthlyBlackStartGOCapabilityPaymentAmount BrtPm | CC 3102 – Black Start Capability Settlement |

## CAISO Formula

    - MonthlyBlackStartCapabilityAllocationAmount Brm  =  (MonthlyBlackStartCapabilityPTOAllocationConversionAmount Brm)*(-1)

#### MonthlyBlackStartCapabilityPTOAllocationConversionAmount Brm = P t

ATTRIBUTESWAP(MonthlyBlackStartGOCapabilityPaymentAmount BrtPm)

Note: The attribute positions for BA_ID and PTO_ID will utilize the ATTRIBUTESWAP function in order to ensure that PTO’s are the business associates allocated Black Start charges.

## Outputs

| Output ID | Name | Description |
| --- | --- | --- |
| 1 | In addition to any outputs listed below, all inputs shall be included as outputs. |  |
| 2 | MonthlyBlackStartCapabilityAllocationAmount Brm | Monthly Black Start Capability Allocation for Business Associate B, Resource r, and Trading Month m. |
| 3 | MonthlyBlackStartCapabilityPTOAllocationConversionAmount Brm | This charge type solely exists to overwrite the BA_ID attribute of the input bill determinant (which itself reflects the Black Start Capability Payment to the GO) with the PTOs SC ID to ensure that PTOs receive the Black Start Capability Allocation charge |

- Charge Code Effective Dates

| Charge Code/ Pre-calc Name | Document Version | Effective Start Date | Effective End Date | Version Update Type |
| --- | --- | --- | --- | --- |
| CC 1102 – Black Start Capability Allocation | 5.0 | 5/1/2020 | Open | Documentation and Configuration Edits |