Settlements and Billing

Configuration Guide: CG CC 6482 Real Time Excess Cost for Instructed Energy Settlement v5.9

Version 5.9

Table of Contents

1	Purpose of Document	3

2	Introduction	3

2.1	Background	3

2.2	Description	4

3	Charge Code Requirements	4

3.1	Business Rules	4

3.2	Predecessor Charge Codes	6

3.3	Successor Charge Codes	7

3.4	Inputs – External Systems	7

3.5	Inputs – Predecessor Charge Codes or Pre-calculations	8

3.6	CAISO Formula	9

3.7	Outputs	14

4	Charge Code Effective Date	17

# Purpose of Document

The purpose of this document is to capture the requirements and design specification for a Charge Code in one document.

# Introduction

## Background

In Charge Code (CC) 6482 Real-Time Excess Cost for Instructed Energy Settlement the ISO settle Excess Costs for incremental FMM Exceptional Dispatch IIE and RTD Exceptional Dispatch IIE priced above the FMM Interval LMP or Settlement Interval Real-Time LMP, respectively. However, the Settlement of Excess Cost for decremental FMM Exceptional Dispatch IIE and RTD Exceptional Dispatch IIE priced below the FMM Interval LMP or Settlement Interval Real-Time LMP, respectively, is not required in CC 6482 since the entire Exceptional Dispatch price is settled completely (without any excess cost breakdown) in CC 6470. . CC 6482 settles the excess cost for Energy transactions where the Settlement type is one of the emergency “exceptional” Energy types (“SYSEMR”, “SYSEMR1” or “TEMR”) as defined in CAISO Tariff.  The SYSEMR Exceptional Dispatch Energy type represents Energy, including Energy from forced Start-Ups and Shut-Downs, that is delivered as a result of an Exceptional Dispatch for System Emergency conditions, to avoid intervention in market operations, mitigate over generation conditions, or to prevent or relieve an imminent System Emergency. TEMR represents Energy purchased by the CAISO from a utility in another Control Area as a result of a CAISO request for assistance in System Emergency conditions in order to avoid an intervention in market operations, or to prevent or relieve an imminent System Emergency. TEMR Energy excludes Energy from Exceptional Dispatches. CC 6482 does not cover Excess Costs for FMM Exceptional Dispatch IIE and RTD Exceptional Dispatch IIE associated with transmission and other various reasons not related to emergency conditions, as they are covered in other charge codes (for instance, CC 6488 – “Exceptional Dispatch Uplift Settlement”).

Emergency Energy for either SYSEMR or TEMR  from Fifteen Minute Market will be settled at the higher of the FMM Interval LMP, Energy Bid price or, if applicable, the Default Energy Bid price for Energy that does not have an Energy Bid price, or, as applicable to System Resources, the negotiated price. Emergency Energy for either SYSEMR or TEMR dispatched in RTD will be settled at the higher of the Settlement Interval LMP, Energy Bid price or, if applicable, the Default Energy Bid price for Energy that does not have an Energy Bid price, or, as applicable to System Resources, the negotiated price. The negotiated System Resource Energy price may exceed the Bid Ceiling.

These Excess Costs will be allocated in two-tier based total negative Uninstructed Imbalance Energy in CC 6486 Real Time Excess Cost for Instructed Energy Allocation and Measured Demand excluding Demand served through transmission rights in CC 6480 Excess Cost Neutrality Allocation.

## Description

The Excess Cost for Instructed Energy is the payment due to a Resource for incremental FMM Exceptional Dispatch IIE or RTD Exceptional Dispatch IIE (OOM or Out-of-Market Energy) for costs in excess of the FMM Interval LMP or  Settlement Interval RTD LMP, respectively.  Payments for decremental Energy are settled in CC 6470, Real Time Instructed Imbalance Energy Settlement.

In CC 6482 any Excess Cost for Instructed Energy is calculated for all Settlement Intervals in the Trading Hour that the resource has an incremental FMM Exceptional Dispatch IIE or RTD Exceptional Dispatch IIE with a SYSEMR, SYSEMR1, or TEMR exceptional type.  The Excess Cost amount due to a resource is calculated as the costs for the incremental IIE above the FMM Interval Real-Time LMP or Settlement Interval RTD LMP.  The “price difference” used to determine the Excess Cost Payment to the resource for each Settlement Interval is determined as the positive-valued difference by which the applicable incremental FMM IIE Exceptional Dispatch price exceeds the FMM Interval LMP or incremental IIE Exceptional Dispatch price exceeds the Settlement Interval RTD LMP. The total payment for Energy associated with Excess Cost is paid in three Charge Codes, in CC 6460 whereby the FMM Exceptional Dispatch Energy is paid at the FMM Settlement Interval LMP, the RTD Exceptional Dispatch Energy is paid at the Settlement Interval RTD LMP in CC 6470, and in the currently specified CC 6482 whereby the Energy is paid at the Excess Cost price difference. However, prior to the September 21, 2006 FERC Order, Energy Excess Costs are not paid in CC 6482, if the resource fails to operate within its relevant Tolerance Band. To be consistent with Paragraph #516 of the FERC Order, a resource will no longer be required to perform within its relevant Tolerance Band to receive Excess Cost Payments.  Additionally stated in Paragraph #516, resources that fall short of the Day Ahead and RT Dispatch Instructions shall only be guaranteed the recovery of cost associated with the Energy actually provided, and should not receive payment for deviations from DA Schedule or RT Dispatch Instructions; therefore Excess Cost Payments will be paid based on Energy actually provided and should not receive payment for deviations from RT Dispatch.

# Charge Code Requirements

## Business Rules

| Bus Req ID | Business Rule |
| --- | --- |

|  | FMM Exceptional Dispatch IIE quantities where the Exceptional Dispatch Type is SYSEMR or TEMR and that have costs above the FMM Interval LMP will receive Excess Cost for IIE associated with the Energy actually provided, and should not receive payment for deviations from RT Dispatch. |
| --- | --- |
|  | If the FMM Exceptional IIE Price is greater than the FMM Interval LMP, then the difference in Energy costs between settlement at the FMM Exceptional IIE Price and the FMM Interval LMP are paid as FMM Excess Cost for IIE Settlement Amount, if the Energy costs at the FMM Exceptional IIE Price are greater than the cost at the FMM Interval LMP. |
|  | Excess Cost Payments are paid for FMM Exceptional Dispatch Inc IIE and not for FMM Exceptional Dispatch Dec IIE. FMM Exceptional Dispatch Inc IIE is always a positive value. |
|  | RTD Exceptional Dispatch IIE quantities where the Exceptional Dispatch Type is SYSEMR or TEMR and that have costs above the Settlement Interval RTD LMP will receive Excess Cost for IIE associated with the Energy actually provided, and should not receive payment for deviations from RT Dispatch. |
|  | If the RTD Exceptional IIE Price is greater than the Settlement Interval RTD LMP, then the difference in Energy costs between settlement at the RTD Exceptional IIE Price and the Settlement Interval RTD LMP is paid as Excess Cost for IIE Settlement Amount, if the Energy costs at the RTD Exceptional IIE Price are greater than the cost at the Settlement Interval RTD LMP. |
|  | Excess Cost Payments are paid for RTD Exceptional Dispatch Inc IIE and not for RTD Exceptional Dispatch Dec IIE. RTD Exceptional Dispatch Inc IIE is always a positive value. |
|  | A resource that is committed or dispatched under Exceptional Dispatch shall be eligible for supplemental revenues only during such times that the capacity from the resource dispatched under Exceptional Dispatch is Eligible Capacity, the Eligible Capacity does not have an offer into the applicable CSP, and has declined an Exceptional Dispatch CPM designation. |
|  | If capacity that was not offered into the Intra-monthly CSP is offered an Exceptional Dispatch CPM designation and declines the designation, then the resource shall be compensated based on supplemental revenues. |
|  | Supplemental revenues shall not exceed within a 30-day period (this 30-day period begins on the day of the first Exceptional Dispatch of the resource and re-starts on the day of the first Exceptional Dispatch of the resource following the end of any prior 30-day period) the CPM Soft Offer Cap, for which revenues the resource would be eligible had its Eligible Capacity been designated as CPM Capacity. |
|  | For a resource that has declined an CPM Designation for Supplemental Revenue assessment, the FMM Exceptional Dispatch IIE Price or emergency Energy price for FMM Exceptional Dispatch for emergency Energy Incremental IIE with Exceptional Type of SYSEMR, SYSEMR1, or TEMR during the Supplemental Revenue designation period where resources supplemental Revenue does not exceed relevant CPM amount is the higher of the resource’s FMM LMP or Energy Bid Price or, if applicable, the Default Energy Bid price for Energy that does not have an Energy Bid Price, or, as applicable to System Resources providing emergency Energy, the pre-established or negotiated price as recorded by the CAISO operator at the time of Dispatch. |
|  | For a resource that has declined an CPM Designation for Supplemental Revenue assessment, the FMM Exceptional Dispatch IIE Price or emergency Energy price for FMM Exceptional Dispatch for emergency Energy Incremental IIE with Exceptional Type of SYSEMR, SYSEMR1, or TEMR during the Supplemental Revenue designation period where resources supplemental Revenue exceeds relevant CPM amount is the higher of the resource’s FMM LMP or the Default Energy Bid price for Energy. |
|  | For adjustments to the Charge Code that cannot be accomplished by correction of upstream data inputs, recalculation or operator override Pass Through Bill Charge (PTB) logic will be applied. |
|  | For RMR resources subject to new Tariff, variable energy cost opportunity cost adders shall reduce bid cost per MWh of exceptional dispatches. |
| 11.1 | For RMR resources subject to new Tariff, market revenues in excess of qualified costs shall be subject to a true up. This excess revenue will be subtracted from capacity payments. The qualified cost is the bid cost less variable energy costs opportunity cost adder. |
| 12.0 | When an eligible resource has an interval with a negative MWh meter, CAISO will not charge for the energy of those intervals. |

## Predecessor Charge Codes

Charge Code/ Pre-calc Name

Pre-calculation Real Time Price

Pre-calculation – Metered Demand TAC Area and CPM

Pre-Calculation Metered Energy Adjustment Factor

Real-Time Energy Pre-calculation

## Successor Charge Codes

Charge Code/ Pre-calc Name

Real Time Excess Cost for Instructed Energy Allocation (CC 6486)

Excess Cost Neutrality Allocation (CC 6480)

## Inputs – External Systems

| Row # | Variable Name | Description |
| --- | --- | --- |

|  | ExceptionalDispatchIIEPrice BrtObmdhcif | Price for Exceptional Dispatch IIE (in $/MWh) One of the following: (1) Bid, (2) the Default Energy Bid, (3) negotiated price, or (4) calculated price. |
| --- | --- | --- |
|  | ExceptionalDispatchIIE BrtuT’ObI’AA’Q’M’R’W’F’S’VL’Pmdhcif | RTD IIE (in MWh) produced or consumed by a resource in response to a manual Exceptional Dispatch instruction. |
|  | FMMExceptionalDispatchIIEPrice BrtObmdhcif | Price for Exceptional Dispatch IIE (in $/MWh) One of the following: (1) Bid, (2) the Default Energy Bid, (3) negotiated price, or (4) calculated price. |
|  | FMMExceptionalDispatchIIE BrtuT’ObI’Q’M’AA’R’W’F’S’VL’Pmdhcif | FMM IIE (in MWh) produced or consumed by a resource in response to a manual Exceptional Dispatch instruction. |
|  | BASettlementIntervalResourceRTExceptionalDispatchDEBPrc BrtObmdhcif | Bid price (in $ / MWh) of Exceptional Dispatch IIE Energy dispatched in real-time for the specified DEB segment and Settlement Interval. |
|  | BASettlementIntervalResourceRTExceptionalDispatchDEBQty BrtuT’ObI’AA’Q’M’R’W’F’S’VL’Pmdhcif | Exceptional Dispatch Energy (in MWh) dispatched through RTD for the specified DEB segment and Settlement Interval. |
|  | BASettlementIntervalResourceFMMExceptionalDispatchDEBPrc BrtObmdhcif | Bid price (in $ / MWh) of FMM Exceptional Dispatch IIE Energy dispatched in the RTM for the specified DEB bid segment and Settlement Interval. |
|  | BASettlementIntervalResourceFMMExceptionalDispatchDEBQty BrtuT’ObI’AA’Q’M’R’W’F’S’VL’Pmdhcif | Exceptional Dispatch Energy (in MWh) dispatched in the FMM for the specified DEB bid segment and Settlement Interval. |
|  | PTBChargeAdjustmentExcessCostIIESettlementAmount BJmdhcif | Excess Cost IIE Settlement Amount PTB Pay Charge Adjustment Amount due Business Associate B (in $). |

## Inputs – Predecessor Charge Codes or Pre-calculations

| Row # | Variable Name | Predecessor Charge Code/ Pre-calc Configuration |
| --- | --- | --- |

|  | SettlementIntervalRTDLMPPrice BrtuT’I’M’mdhcif | Pre-Calculation – Real Time Price |
| --- | --- | --- |
|  | FMMIntervalLMPPrice BrtuM’mdhc | Pre-Calculation – Real Time Price |
|  | FMMExceptionalDispatchIIECostAboveLMPPrice BrtObmdhcif | Pre-calculation – Real Time Price |
|  | RTDExceptionalDispatchIIECostAboveLMPPrice BrtObmdhcif | Pre-calculation – Real Time Price |
|  | ExceptionalDispatchMeteredEnergyAdjustmentFactor BrtuT’I’M’F’S’mdhcif | Pre-Calculation – Metered Energy Adjustment Factor |
|  | BASettlementIntervalResourceSurplusSupplementalRevenueFlag Brtmdhcif | Pre-calculation – Metered Demand TAC Area and CPM |
|  | ResourceWholesaleExemptionFlag rmdhcif | Pre-Calculation – Real Time Energy |

## CAISO Formula

### Excess Cost Settlement Amount and Quantity

#### ExcessCostIIESettlementAmount Brtmdhcif =

O IF

ResourceWholesaleExemptionFlag rmdhcif = 0

THEN

ExcessCostIIESettlementAmount Brtmdhcif = EDTypeExcessCostIIESettlementAmount BrtOmdhcif

ELSE

ExcessCostIIESettlementAmount Brtmdhcif = 0

#### EDTypeExcessCostIIESettlementAmount

EDTypeExcessCostIIESettlementAmount BrtOmdhcif =
uT'bI'AA'M'R'W'F'S'VL'(  ExcessCost5MFMMIIESettlementAmount BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif + ExcessCost5MIIESettlementAmount BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif + LimitedExcessCost5MFMMIIESettlementAmount BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif + LimitedExcessCost5MIIESettlementAmount BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif  )

#### SettlementIntervalEDTypeIIEExcessCostQuantity

SettlementIntervalEDTypeIIEExcessCostQuantity BrtOmdhcif =

uT'bI'AA'M'R'W'F'S'VL'(SettlementIntervalFMMIIEExcessCostQuantity BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif + SettlementIntervalRTDIIEExcessCostQuantity BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif + SettlementIntervalLimitedFMMIIEExcessCostQuantity BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif + SettlementIntervalLimitedRTDIIEExcessCostQuantity BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif )

#### SettlementIntervalTotalRTMIIEExcessCostQuantity

SettlementIntervalTotalRTMIIEExcessCostQuantity Brtmdhcif = OSettlementIntervalEDTypeIIEExcessCostQuantity BrtOmdhcif

### Non-Supplemental Revenue Or Supplemental Revenue Below CPM Soft Offer Cap Limit

#### ExcessCost5MIIESettlementAmount

ExcessCost5MIIESettlementAmount BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif =

(-1) * MAX (0, RTDExceptionalDispatchIIECostAboveLMPPrice BrtObmdhcif ) * (SettlementIntervalRTDIIEExcessCostQuantity BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif * ExceptionalDispatchMeteredEnergyAdjustmentFactor BrtuT’I’M’F’S’mdhcif)

Where

Exceptional Dispatch Type (O) in (SYSEMR, SYSEMR1, TEMR)

#### SettlementIntervalRTDIIEExcessCostQuantity

IF

( (1 – BASettlementIntervalResourceSurplusSupplementalRevenueFlag Brtmdhcif ) * 
RTDExceptionalDispatchIIECostAboveLMPPrice BrtObmdhcif   > 0 )

THEN

SettlementIntervalRTDIIEExcessCostQuantity BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif = PQ'MAX(0, ExceptionalDispatchIIE BrtuT’ObI’AA’Q’M’R’W’F’S’VL’Pmdhcif )

ELSE

SettlementIntervalRTDIIEExcessCostQuantity BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif =PQ'0

END IF

Where

Exceptional Dispatch Type (O) in (SYSEMR, SYSEMR1, TEMR)

And Where Exists

ExceptionalDispatchIIE BrtuT’ObI’AA’Q’M’R’W’F’S’VL’Pmdhcif

#### ExcessCost5MFMMIIESettlementAmount

ExcessCost5MFMMIIESettlementAmount BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif =

( (-1) * MAX (0, FMMExceptionalDispatchIIECostAboveLMPPrice BrtObmdhcif  ) *

(SettlementIntervalFMMIIEExcessCostQuantity BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif

* ExceptionalDispatchMeteredEnergyAdjustmentFactor BrtuT’I’M’F’S’mdhcif ) )

Where

Exceptional Dispatch Type (O) in (SYSEMR, SYSEMR1, TEMR)

#### SettlementIntervalFMMIIEExcessCostQuantity

IF

( (1 – BASettlementIntervalResourceSurplusSupplementalRevenueFlag Brtmdhcif ) * 
FMMExceptionalDispatchIIECostAboveLMPPrice BrtObmdhcif   > 0 )

THEN

SettlementIntervalFMMIIEExcessCostQuantity BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif

= PQ'MAX (0, FMMExceptionalDispatchIIE BrtuT’ObI’Q’M’AA’R’W’F’S’VL’Pmdhcif )

ELSE

SettlementIntervalFMMIIEExcessCostQuantity BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif

= PQ'0

END IF

Where

Exceptional Dispatch Type (O) in (SYSEMR, SYSEMR1, TEMR)

And Where Exists

FMMExceptionalDispatchIIE BrtuT’ObI’AA’Q’M’R’W’F’S’VL’Pmdhcif

### Supplemental Revenue At or Above CPM Soft Offer Cap limit Case Limit

#### LimitedExcessCost5MIIESettlementAmount

LimitedExcessCost5MIIESettlementAmount BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif =

(-1) * MAX (0, (BASettlementIntervalResourceRTExceptionalDispatchDEBPrc BrtObmdhcif – SettlementIntervalRTDLMPPrice BrtuT’I’M’mdhcif) * SettlementIntervalLimitedRTDIIEExcessCostQuantity BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif * ExceptionalDispatchMeteredEnergyAdjustmentFactor BrtuT’I’M’F’S’mdhcif) )

Where

Exceptional Dispatch Type (O) in (SYSEMR, SYSEMR1, TEMR)

#### SettlementIntervalLimitedRTDIIEExcessCostQuantity

IF

(BASettlementIntervalResourceSurplusSupplementalRevenueFlag Brtmdhcif * (BASettlementIntervalResourceRTExceptionalDispatchDEBPrc BrtObmdhcif – SettlementIntervalRTDLMPPrice BrtuT’I’M’mdhcif)  > 0	 )

THEN

SettlementIntervalLimitedRTDIIEExcessCostQuantity BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif =

PQ'MAX(0, BASettlementIntervalResourceRTExceptionalDispatchDEBQty BrtuT’ObI’AA’Q’M’R’W’F’S’VL’Pmdhcif )

ELSE

SettlementIntervalLimitedRTDIIEExcessCostQuantity BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif =

PQ'0

ENF IF

Where

Exceptional Dispatch Type (O) in (SYSEMR, SYSEMR1, TEMR)

And Where Exists

BASettlementIntervalResourceRTExceptionalDispatchDEBQty BrtuT’ObI’AA’Q’M’R’W’F’S’VL’Pmdhcif

#### LimitedExcessCost5MFMMIIESettlementAmount

LimitedExcessCost5MFMMIIESettlementAmount BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif =

( (-1) * MAX (0, (BASettlementIntervalResourceFMMExceptionalDispatchDEBPrc BrtObmdhcif – FMMIntervalLMPPrice BrtuM’mdhc ) *

(SettlementIntervalLimitedFMMIIEExcessCostQuantity BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif

* ExceptionalDispatchMeteredEnergyAdjustmentFactor BrtuT’I’M’F’S’mdhcif) ) )

Where

Exceptional Dispatch Type (O) in (SYSEMR, SYSEMR1, TEMR)

#### SettlementIntervalLimitedFMMIIEExcessCostQuantity

IF

(BASettlementIntervalResourceSurplusSupplementalRevenueFlag Brtmdhcif * (BASettlementIntervalResourceFMMExceptionalDispatchDEBPrc BrtObmdhcif – FMMIntervalLMPPrice BrtuM’mdhc  ) > 0 )

THEN

SettlementIntervalLimitedFMMIIEExcessCostQuantity BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif = PQ'MAX (0, BASettlementIntervalResourceFMMExceptionalDispatchDEBQty BrtuT’ObI’AA’Q’M’R’W’F’S’VL’Pmdhcif )

ELSE

SettlementIntervalLimitedFMMIIEExcessCostQuantity BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif = PQ'0

END IF

Where

Exceptional Dispatch Type (O) in (SYSEMR, SYSEMR1, TEMR)

And Where Exists

BASettlementIntervalResourceFMMExceptionalDispatchDEBQty BrtuT’ObI’AA’Q’M’R’W’F’S’VL’Pmdhcif

### RMR Related Calculations:

#### RMRExcessCost5MRTDIIETrueUpAmount BrtObmdhcif

= Sum over (u, T’, I’, A, A’, M’, R’, W’, F’, S’, V, L’)

{(-1) * MIN (0, RTDExceptionalDispatchIIECostAboveLMPPrice BrtObmdhcif )* SettlementIntervalRTDIIEExcessCostQuantity BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif * ExceptionalDispatchMeteredEnergyAdjustmentFactor BrtuT’I’M’F’S’mdhcif }

Where

Exceptional Dispatch Type (O) in (SYSEMR, SYSEMR1, TEMR)

#### RMRExcessCost5MFMMIIETrueUpAmount BrtObmdhcif

= Sum over (u, T’, I’, A, A’, M’, R’, W’, F’, S’, V, L’)

{ (-1) * MIN (0, FMMExceptionalDispatchIIECostAboveLMPPrice BrtObmdhcif )*

SettlementIntervalFMMIIEExcessCostQuantity BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif

* ExceptionalDispatchMeteredEnergyAdjustmentFactor BrtuT’I’M’F’S’mdhcif  }

Where

Exceptional Dispatch Type (O) in (SYSEMR, SYSEMR1, TEMR)

#### RMRDailyExcessCostIIETrueUpAmount Brmd

= Sum over (t, O, b, h, c, i, f)

(RMRExcessCost5MRTDIIETrueUpAmount BrtObmdhcif + RMRExcessCost5MFMMIIETrueUpAmount BrtObmdhcif)

## Outputs

| Row # | Name | Description |
| --- | --- | --- |
|  | In addition to any outputs listed below, all external inputs shall be included as outputs. |  |
|  | ExcessCostIIESettlementAmount Brtmdhcif | The Total Excess Cost Settlement amount (in $) for Exceptional Dispatch IIE for Resource r |
|  | EDTypeExcessCostIIESettlementAmount BrtOmdhcif | The total Excess Cost IIE Settlement Amount (in $) settled for a resource and Exceptional Dispatch Type. |
|  | SettlementIntervalEDTypeIIEExcessCostQuantity BrtOmdhcif | The Excess Cost IIE quantity (in MWh) per Exceptional Dispatch Type settled for resource r. |
|  | SettlementIntervalTotalRTMIIEExcessCostQuantity Brtmdhcif | The total Excess Cost IIE quantity (in MWh) for a resource r. |
|  | ExcessCost5MIIESettlementAmount BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif | RTD component of the total Excess Cost IIE Settlement Amount (in $) for a resource that (a) is not subject to Supplemental Revenue or (b) has selected the Supplemental Revenue option but its Supplemental Revenue, over a declined CPM designation period that includes the current Settlement Interval, does not equal the Supplemental Revenue CPM Soft Offer Cap limit. |
|  | SettlementIntervalRTDIIEExcessCostQuantity BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif | RTD component of the total Excess Cost IIE quantity (in MWh) settled for a resource that does not have Supplemental Revenue, over a declined CPM designation period that includes the current Settlement Interval, equaling the Supplemental Revenue CPM Soft Offer Cap limit. |
|  | ExcessCost5MFMMIIESettlementAmount BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif | FMM component of the total Excess Cost IIE Settlement Amount (in $) for a resource that (a) is not subject to Supplemental Revenue or (b) has selected the Supplemental Revenue option but its Supplemental Revenue, over a declined CPM designation period that includes the current Settlement Interval, does not equal the Supplemental Revenue CPM Soft Offer Cap limit. |
|  | SettlementIntervalFMMIIEExcessCostQuantity BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif | FMM component of the total Excess Cost IIE quantity (in MWh) settled for a resource that does not have Supplemental Revenue, over a declined CPM designation period that includes the current Settlement Interval, equaling the Supplemental Revenue CPM Soft Offer Cap limit. |
|  | LimitedExcessCost5MIIESettlementAmount BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif | RTD component of the total Excess Cost IIE Settlement Amount (in $) for a resource (a) that is subject to Supplemental Revenue and (b) for which its Supplemental Revenue, over a declined CPM designation period that includes the current Settlement Interval, equals the Supplemental Revenue CPM Soft Offer Cap limit. |

|  | SettlementIntervalLimitedRTDIIEExcessCostQuantity BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif | RTD component of the total Excess Cost IIE quantity (in MWh) settled for a resource (a) that is subject to Supplemental Revenue and (b) for which its Supplemental Revenue, over a declined CPM designation period that includes the current Settlement Interval, equals the Supplemental Revenue CPM Soft Offer Cap limit. |
| --- | --- | --- |
|  | LimitedExcessCost5MFMMIIESettlementAmount BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif | FMM component of the total Excess Cost IIE Settlement Amount (in $) for a resource (a) that is subject to Supplemental Revenue and (b) for which its Supplemental Revenue, over a declined CPM designation period that includes the current Settlement Interval, equals the Supplemental Revenue CPM Soft Offer Cap limit. |
|  | SettlementIntervalLimitedFMMIIEExcessCostQuantity BrtuT’ObI’AA’M’R’W’F’S’VL’mdhcif | FMM component of the total Excess Cost IIE quantity (in MWh) settled for a resource (a) that is subject to Supplemental Revenue and (b) for which its Supplemental Revenue, over a declined CPM designation period that includes the current Settlement Interval, equals the Supplemental Revenue CPM Soft Offer Cap limit. |
|  | PTBChargeAdjustmentExcessCostIIESettlementAmount Bjmdhcif | PTB Charge Adjustment Amount (in $) that is paid to Business Associate (BA) B as an adjustment to the BA’s overall Excess Cost IIE Settlement Amount. |
|  | RMRExcessCost5MRTDIIETrueUpAmount BrtObmdhcif | RMR revenue above its cost when compared to LMP for RTD for exceptional dispatch types SYSEMR, SYSEMR1, or TEMR. |
|  | RMRExcessCost5MFMMIIETrueUpAmount BrtObmdhcif | RMR revenue above its cost when compared to LMP for FMM for exceptional dispatch types SYSEMR, SYSEMR1, or TEMR. |
|  | RMRDailyExcessCostIIETrueUpAmount Brmd | RMR daily true up for exceptional dispatch of type SYSEMR, SYSEMR1, or TEMR. |

# Charge Code Effective Date

| Charge Code/ Pre-calc Name | Document Version | Effective Start Date | Effective End Date | Version Update Type |
| --- | --- | --- | --- | --- |

| CC 6482 – CG CC 6482 Real Time Excess Cost for Instructed Energy Settlement v5.9 | 5.0 | 04/01/09 | 08/31/09 | Documentation Edits Only |
| --- | --- | --- | --- | --- |
| CC 6482 – CG CC 6482 Real Time Excess Cost for Instructed Energy Settlement v5.9 | 5.1 | 08/01/10 | 07/31/10 | Documentation Edits and Configuration Impacted |
| CC 6482 – CG CC 6482 Real Time Excess Cost for Instructed Energy Settlement v5.9 | 5.2 | 09/01/09 | 4/30/14 | Documentation Edits and Configuration Impacted |
| CC 6482 – CG CC 6482 Real Time Excess Cost for Instructed Energy Settlement v5.9 | 5.3 | 5/1/14 | 9/30/14 | Configuration Impacted |
| CC 6482 – CG CC 6482 Real Time Excess Cost for Instructed Energy Settlement v5.9 | 5.4 | 10/1/14 | 11/03/15 | Configuration Impacted |
| CC 6482 – CG CC 6482 Real Time Excess Cost for Instructed Energy Settlement v5.9 | 5.5 | 11/04/15 | 10/31/16 | Documentation Edits and Configuration Impacted |
| CC 6482 – CG CC 6482 Real Time Excess Cost for Instructed Energy Settlement v5.9 | 5.6 | 11/01/16 | 10/31/16 | Documentation Edits and Configuration Impacted |
| CC 6482 – CG CC 6482 Real Time Excess Cost for Instructed Energy Settlement v5.9 | 5.7 | 11/01/16 | 12/31/19 | Documentation Edits and Configuration Impacted |
| CC 6482 – CG CC 6482 Real Time Excess Cost for Instructed Energy Settlement v5.9 | 5.8 | 1/1/20 | 9/30/20 | Configuration Impacted |
| CC 6482 – CG CC 6482 Real Time Excess Cost for Instructed Energy Settlement v5.9 | 5.9 | 10/1/20 | Open | Documentation Edits and Configuration Impacted |