# Settlements & Billing

# Configuration Guide:  Day Ahead Energy, Congestion, Loss Settlement

CC 6011

# Version 6.0.1

# Table of Contents

1.	Purpose of Document	3

2.	Introduction	3

2.1	Background	3

2.2	Description	3

3.	Charge Code Requirements	4

3.1	Business Rules	4

3.2	Predecessor Charge Codes	9

3.3	Successor Charge Codes	9

3.4	Inputs – External Systems	9

3.5	Inputs - Predecessor Charge Codes or Pre-calculations	12

3.6	CAISO Formula	13

3.7	Outputs	26

4.	Charge Code Effective Dates	35

# Purpose of Document

The purpose of this document is to capture the requirements and design specification for a Charge Code in one document.

# Introduction

## Background

The Integrated Forward Market (IFM) design allows CAISO to make efficient use of resources by simultaneously optimizing the different markets, through Locational Marginal Pricing (LMP), which is the industry’s preferred method of dealing with transmission traffic jams and determining the least cost method for meeting electricity Demand.

This Charge Code will deal only with the Energy portion of the IFM, specifically termed in this document as the Day Ahead Energy.

Business Associates with Day Ahead Schedules shall be paid or charged the LMP at the resource location. Since the Energy component of LMP is equal for all resource locations, the net of payments and charges for Day Ahead Schedules will comprise the net Congestion and loss revenues. The net Congestion revenues, when positive, will be given to CRR Holders through CC 6700 (CRR Hourly Settlement), and the loss surplus in Day Ahead Energy, Congestion, Loss Settlement (CC 6011) will be allocated to Measured Demand through CC 6947, the IFM Marginal Losses Surplus Credit Allocation.

As set in the Tariff, IFM Congestion Credits for ETC/TOR valid and balanced Self-Schedules as well as TOR loss credits to select TORs per CAISO agreements shall be provided. These credits shall be taken out of CC 6011, and thus these credits are also reflected in CC 6011’s successor Charge Codes.

Specific contracts can contain loss charging provision which can vary from one contract to another. This charge code shall provide the loss charge to such contracts in the Day-Ahead market, and the amount collected shall be included in the marginal loss revenue surplus (CC 6947) to be distributed to Measured Demand excluding demand quantity for which loss credits to contracts were provided in the Day-Ahead.

## Description

This Charge Code settles Day-Ahead Schedules of Business Associates by paying for Supply and charging for Demand based on the LMP at resource locations for each hour.

This Charge Code shall compute the Congestion credits to designated SCs of ETC/TOR and OATT contracts by making reversals of Congestion Charges and payments for the valid and balanced portion of the transmission contracts Self-Schedules. Losses credits to select TOR contracts shall also be computed and embedded in this charge code.

This charge code shall provide the specific loss charge to contracts in the Day-Ahead market, based on the contract-specific loss charge percentage, the Balancing Authority Area Marginal Energy Cost (MEC), and the Day-Ahead balanced capacity for the contract.

Per Business Associate per hour, the Settlement Amount shall be the net of its payments and charges for all its resources it used for the same hour less any contract Congestion and Loss credits.

The Charge Code also calculates the total net Congestion revenues per BAA for each Trading Hour, to be used for the hourly Settlement of CRRs (CC 6700) in the case of CISO BAA, and for the hourly Day-Ahead Congestion Precalc for non-CISO BAAs.

OATT contracts congestion credits are also computed in this charge code and only for those with DA financial rights. However, the actual settlement to the Billing SC for the respective contract and total values across all OATT contracts at the SC and BAA level are included in a successor charge code, CC 8704, Day Ahead Congestion Offset.

# Charge Code Requirements

## Business Rules

| Bus Req ID | Business Rule |
| --- | --- |
| 1.0 | The Settlement Amount per hour per SC for all its Day-Ahead Schedules is the net of its payments and charges for all the resources it scheduled for the same hour less any contract Congestion and loss credits and inclusive of any contract specific loss charge. |
| 1.0.1 | EDAM BAA SCs with Day-Ahead Schedules will be settled in this charge code. |
| 1.0.2 | DA Energy transfers at transfer system resources (TSR) will be calculated in this charge code. |
| 1.0.3 | OATT contracts congestion credits are also computed in this charge code and only for those with DA financial rights. However, the actual settlement to the Billing SC for the respective contract and total values across all OATT contracts at the SC and BAA level are included in a successor charge code, CC 8704. |
| 1.1 | This Charge Code shall be calculated daily on an hourly basis. |
| 1.2 | The Settlement for schedules for each MSS shall depend on the MSS’ elected Energy Settlement, which could be either “gross” or “net” Energy Settlement. |
| 1.2.1 | Gross or net energy election is determined through annual election by the MSS Operator. (Fact) |
| 1.2.2 | Each MSS entity with a unique set of elections is represented by the SC designated by the relevant MSS Operator, and CAISO settles with that SC. (Fact) |
| 1.2.3 | For an MSS Aggregation where all MSS entities have the same Energy Settlement elections, the CAISO shall settle at the MSS Aggregation level. (Fact) |
| 1.2.4 | For an MSS Aggregation where its MSS entities have different Energy Settlement elections, the CAISO shall settle with the unique SC representing the relevant MSS entity, not at the MSS Aggregation level. (Fact) |
| 1.2.5 | An MSS SC shall be able to schedule a set of non-MSS and MSS resources. (Fact) |
| 1.3 | The SC shall be charged for its Demand of Energy at a LAP and Participating Loads (when consuming Energy); and for exports of Energy at Scheduling Points. |
| 1.4 | The SC shall be paid for Supply of Energy from Generating Units, Participating Loads (when providing Energy to the CAISO through load reduction), and System Resources. |
| 1.5 | The Congestion credit for the valid and balanced portion of ETC/TOR and OATT Self-Schedules shall be calculated using the MCC at the relevant financial node (PNode, APNode, or PNode/APNode in combination with Intertie) associated with the ETC/TOR and OATT source or sink. |
| 1.5.1 | The Congestion credit for each CRN N shall be (1) the product of the DA computed valid and balanced self-schedules at a supply resource and the MCC of the contract’s financial source node associated with that resource, minus (2) the product of the DA computed valid and balanced self-schedules at a sink resource and the MCC of the contract’s financial sink node associated with that resource. |
| 1.6 | The reversal of Congestion Charges for valid and balanced portion of each ETC/TOR and OATT contract Self-Schedules shall all be credited only to the designated Billing SC for the contract. |
| 1.6.1 | For the case of a CRN chain, any congestion credits coming out of such would have been assigned to each Billing SC of the CRN segments, since the CRN chain self-schedule was segmented and assigned to the different CRN segments of the CRN chain, when determining the valid and balanced self-schedule quantities. (Fact) |
| 1.6.2 | The contribution to the total congestion credit amount of a CRN chain or individual CRN self-schedule at a resource by the original scheduler SC shall be calculated and shown for informational purposes only, as the actual congestion credit amount at the contract level is provided and settled only with the designated Billing SC for the contract. |
| 1.7 | The hourly DA Schedules include any valid and balanced portion of ETC/TOR and OATT Self-Schedules. (Fact) |
| 2.0 | For an MSS under GROSS Energy Settlement, the charge for Demand equals the product of: (1) the MWh quantity of Demand in the SC’s Day-Ahead Schedule (excluding ETC/TOR schedules) at the corresponding MSS LAP and (2) the Default LAP LMP Price for the MSS. OATT contracts are not applicable to any kind of Metered Subsystems (MSS), and thus do not have special treatment under this subsection. |
| 2.0.1 | The Default LAP LMP Price for the MSS shall be for the LAP within which the relevant MSS LAP is located. |
| 2.1 | For an MSS under GROSS Energy Settlement, the payment for Supply equals the product of: (1) the MWh quantity of Supply in the SC’s Day-Ahead Schedule (excluding ETC/TOR schedules) at the corresponding PNode and (2) the applicable resource-specific LMP at that PNode. |
| 2.2 | For an MSS under NET Energy Settlement, the charge for Demand equals the product of: (1) the net MSS Demand calculated from the Day-Ahead Schedules (excluding ETC/TOR schedules) and (2) the MSS LAP LMP Price. |
| 2.2.1 | The net MSS Demand is the quantity of MSS Demand that exceeds MSS Generation for the applicable MSS. |
| 2.3 | For an MSS under NET Energy Settlement, the payment for Supply equals the product of: (1) the net MSS Supply calculated from the Day-Ahead Schedules and (2) the Energy weighted average price of all IFM LMPs for all applicable PNodes within the relevant MSS. |
| 2.3.1 | The net MSS Supply is the quantity of MSS Generation that exceeds MSS Demand for the applicable MSS. |
| 2.3.2 | The weights used to compute the weighted average LMPs shall be equal to the Energy Schedule of MSS Generation included in the Day-Ahead Schedule (excluding ETC/TOR schedules). |
| 2.4 | The Settlement Amounts shall be summed for all scheduled resources of each SC. |
| 3.0 | The total CAISO net Congestion amount per hour of the Day Ahead market is the sum over all SCs and over all resources of the: (1) congestion revenues from non-contract schedules, plus (2) congestion revenues from contract schedules, plus (3) congestion credits to contract schedules for the same hour. |
| 3.1 | The computation of MCC price for an MSS resource shall parallel the computational business rules for LMP prices as provided in rules 2.0-2.3.2, that is, replace LMPs with MCCs and give different treatments for MSS Gross (generator or load) and MSS Net (either a net supplier or net consumer for the Trading Hour). |
| 4.0 | For adjustments to the Charge Code that cannot be accomplished by correction of upstream data inputs/recalculation or operator override, Pass Through Bill Charge adjustment shall be applied. |
| 4.1 | Import schedule which has been deemed to violate Scheduling Sourcing/Sinking in Same Balancing Authority Area provision will be settled at the lower of relevant LMP of the import Scheduling Point and the relevant LMP of the associated export Scheduling Point. |
| 4.2 | The difference in Settlement between the relevant LMP of the import Scheduling Point and the relevant LMP of the associated export Scheduling Point will be assessed as part of Pass Thru Charge Adjustment Mechanism |
| 4.3 | Import schedule which has been deemed to violate Scheduling Sourcing/Sinking in Same Balancing Authority Area provision will be settled at the lower of relevant MCC of the import Scheduling Point and the relevant MCC of the associated export Scheduling Point. |
| 4.4 | The difference in Settlement between the relevant MCC of the import Scheduling Point and the relevant MCC of the associated export Scheduling Point will be assessed through Pass Thru Charge MCC Adjustment Bill Determinant |
| 5.0 | Contract loss credit shall be provided only to select TOR contracts per CAISO agreements. |
| 5.1 | The entire contract loss credit amount shall be credited only to the designated Billing SC for the contract. |
| 5.2 | The Loss credit for the valid and balanced portion of TOR Self-Schedules shall be calculated using the MCL at the relevant financial node (PNode, APNode, or PNode/APNode in combination with Intertie) associated with the TOR source or sink. |
| 5.2.1 | The Loss credit for each CRN N of contract type TOR shall be (1) the product of the DA computed valid and balanced self-schedules at a supply resource and the MCL of the contract’s financial source node associated with that resource, minus (2) the product of the DA computed valid and balanced self-schedules at a sink resource and the MCL of the contract’s financial sink node associated with that resource. |
| 5.2.2 | For the case of a CRN chain, any loss credits coming out of such would have been assigned to each Billing SC of the CRN segments, since the CRN chain self-schedule was segmented and assigned to the different CRN segments of the CRN chain, when determining the valid and balanced self-schedule quantities. Only CRN segments of contract type TOR shall have contract loss credits. (Fact) |
| 5.3 | The contribution to the total loss credit amount of a CRN chain or individual CRN self-schedule at a resource by the original scheduler SC shall be calculated and shown for informational purposes only, as the actual loss credit amount at the contract level is provided and settled only with the designated Billing SC for the TOR contract. |
| 6.0 | Advisory settlement from NPM resources: DA Energy settlement for NPM resources, aggregated for each SC for each NPM BAA, will be computed in this charge code. |
| 7.0 | This charge code shall provide the specific loss charge to contracts in the Day-Ahead market, based on the contract-specific loss charge percentage, the Balancing Authority Area Marginal Energy Cost (MEC), and the Day-Ahead balanced capacity for the contract for the Trading Hour. |
| 8.0 | When an eligible resource has an interval with a negative MWh meter, CAISO will not charge for the energy of those intervals. |
| 8.0 | Balancing Authority Area (BAA) attribute shall be introduced where appropriate to allow for filtering of values, separation of settlements and related calculations across relevant BAAs. |
| 8.1 | This additional attribute is also intended to be used for upcoming initiatives. |
| 9.0 | The OATT contracts congestion contribution at the BAA level are calculated in this charge code, and are combined with TOR and ETC at the BAA level. This value then goes to successor charge code, DA Congestion Precalc. |

## Predecessor Charge Codes

Charge Code/ Pre-calc Name

ETC/TOR/CVR Quantity Pre-calculation

Real Time Energy Quantity Pre-calculation

CC 8411 – Day Ahead Energy Transfer Revenue Settlement

CC 8315 - Day Ahead Green House Gas Offset

## Successor Charge Codes

Charge Code/ Pre-calc Name

CC 6700 – CRR Hourly Settlement

CC 6947 – IFM Marginal Losses Surplus Credit Allocation

CC 6984 – RTM Net Marginal Loss Assessment per CAISO Agreement

Day Ahead Congestion Pre-calculation

CC 8704 – Day Ahead Congestion Offset

IFM Net Amount Pre-calculation

NPM Pre-calculation

## Inputs – External Systems

| Row # | Variable Name | Description |
| --- | --- | --- |
|  | BAHourlyResourceDayAheadLMP Brtmdh | LMP for resource r, resource type t, for Trading Hour h of the Day Ahead Market. ($/MWh) |
|  | BAHourlyResourceDayAheadMCC Brtmdh | Marginal Cost of Congestion component of the LMP for resource. ($/MWh) |
|  | HourlyDANodalMCCPrc M’Q’AA’Qpmdh | Hourly Interval Marginal Cost Of Congestion Price for Balancing Authority Area Q’. These are mapped from Pnode, APnode, or Pnode/APnode and Intertie combination MCC prices for Energy from the Day-Ahead Market, where the APnode is not of type “DEFAULT” or “CUSTOM”. ($/MWh) |
|  | HourlyDANodalMCLPrice AA’Qpmdh | Marginal Cost of Losses (MCL) for nodal location (Pnode p, APNode A) for Trading Hour h of the Day Ahead Market. This is mapped from Day-Ahead Energy MCL prices for nodal locations. ($/MWh) This input can include among others, APnodes of type A’ = ‘DEFAULT’ or ‘CUSTOM’. |
|  | DA_LAP_LMP AA’mdh | The LMP price for Day Ahead Energy for Trading Hour h for the LAP identified by APNode ID A, with APnode Type A’ = “DEFAULT” or “CUSTOM”. ($/MWh) |
|  | BAAHourlyDAMLAPMCCPrc M’Q’AA’Qpmdh | The MCC price for Day Ahead Energy for Trading Hour h for the LAP identified by BAA by APNode ID A, with APnode Type A’ = “DEFAULT” or “CUSTOM”. ($/MWh) |
|  | ContractBillingSCFactor BNz’Q’md | This has a value of 1 if BA ID B is the Billing SC (also called the responsible TO or SC) of contract N, contract type z’. This is specific for a Trading Day. The Billing SC for contract N designation comes from the Master File. |
|  | ContractDayAheadFinancialRightsFlag Nz’md | Indicates when value is 1 that the contract has financial rights in the Day-Ahead. |
|  | MSSResourceFlag rtmd | A flag with a value of 1 when resource r, resource type t, is an MSS resource. |
|  | MSSResourceInfo BrtuT’I’M’AA’VpLmd | A flag with a value of 1 when resource r is an MSS resource. This variable contains the information link between resource r and other MSS attributes. |
|  | PTBChargeAdjustmentBANetHourlyBAADAEnergyAmt BQ’Jmdh | PTB adjustment variable for this Charge Code per BA and per BAA. ($) |
|  | BAHourlyResourceDAEnergyCRNSchedulePercentage BrtQ’AA’Qpg’Nz’mdh | The percentage of accepted schedule to aggregated accepted schedule for self-schedules coming from each CRN chain or from individual CRNs, at resource r , for original scheduler SC. This value is provided in decimal terms. |
|  | HourlyDABAAMECPrc Q’mdh | The Marginal Energy Cost (MEC) for Balancing Authority Area Q’ in the Day Ahead market for Trading Hour h. ($/MWh) |
|  | ContractLossChargingPercentage Nz’md | Specific loss charging percentage contained in the contract for CRN N of contract type z’. This value is constant for Trading Day d, is applicable for any interval of the day. The value is provided in decimal terms. |
|  | ContractDailyTORLossCreditInclusionFlag Nz’md | This flag identifies that TOR contract N of contract type z’ is to be accorded the TOR loss credit for the Trading Day d, whenever the flag value is 1. The actual loss credit amount for the TOR contract shall eventually go the Billing SC for the contract. This value can change at most daily. |
|  | PTBHourlyResourceBAADAEnergyCongestionAdjustmentAmt BrtQ’Jmdh | PTB adjustment variable for congestion amount for this Charge Code. ($) If data relies on MCC prices, the basis would be MCC breakdown instead of total MCC price. That is, attribute Q’ is created due to the BAA from an MCC price BAA breakdown; and not the resource to BAA association in Masterfile. |
|  | NPMDALoadSchedule BrtuT’I’Q’M’F’S’mdh | The DA Energy Schedule for NPM load resources. |
|  | NPMDAScheduleEnergy BrtuT’I’Q’M’F’S’mdhcif | The DA Energy Schedule for NPM resources per settlement interval, exclusive of load and pumping energy. |
|  | NPMDAPumpingEnergy BrtuT’I’Q’M’F’S’mdhcif | The DA Pumping Energy for NPM resource per settlement interval. |
|  | NPMDATransferEnergy BrtuT’I’Q’M’F’S’mdh | The DA Energy for transfer NPM resources. |
|  | NPMBAAFlag Q’md | Identifies whether a BAA Q’ (or its NPM SC and NPM resources), participates as an NPM advisory settlement. Has value of 1 if it is, otherwise this variable is not created or is zero for the specific BAA Q’ value. |
|  | BAEDAMEntityFlag BQ’md | Flag indicating an EIM entity that specifically participates in EDAM. |

## Inputs - Predecessor Charge Codes or Pre-calculations

| Row # | Variable Name | Predecessor Charge Code/ Pre-calc Configuration |
| --- | --- | --- |
|  | HourlyResourceDABalancedContractScheduleEnergy BrtAA’QpNz’Q’mdh | ETC/TOR/CVR Quantity Pre-calculation This quantity is provided for both Supply and Demand resources utilizing contract self-schedules. For Demand resources r, that is, those of resource type t = “LOAD” or “ETIE”, this value is negative, thus conforming to the sign convention of this Charge Code. |
|  | DailyContractResourceFinancialNodeMap rtAA’QpNz’Q’md | ETC/TOR/CVR Quantity Pre-calculation |
|  | DABalanceCapacity Nz’Q’mdh | ETC/TOR/CVR Quantity Pre-calculation |
|  | HourlyResourceDABalancedContractAtScheduleEnergy BrtNQ’mdh | ETC/TOR/CVR Quantity Pre-calculation This quantity is provided for both Supply and Demand resources utilizing contract self-schedules. For Demand resources r, that is, those of resource type t = “LOAD” or “ETIE”, this value is negative, thus conforming to the sign convention of this Charge Code. |
|  | SettlementIntervalResouceDayAheadEnergy BrtuT’I’Q’M’F’S’mdhcif | Real Time Energy Quantity Pre-calculation |
|  | ResourceWholesaleExemptionFlag rmdhcif | Real Time Energy Quantity Pre-calculation |
|  | BABAATransferSystemResourceDAEnergyTransferToLMPAmount BrQ’AA’Qpr’d’Q’’Nz’mdh | CC 8411 – Day Ahead Energy Transfer Revenue Settlement |
|  | BABAATransferSystemResourceDAEnergyTransferFromLMPAmount BrQ’AA’Qpr’d’Q’’Nz’mdh | CC 8411 – Day Ahead Energy Transfer Revenue Settlement |
|  | DAMHourlyBAAGHGAreaEnergyQuantity BrtQ’M’AA’QpG’’mdh | CC 8315 – Day Ahead Green House Gas Offset Day Ahead Energy Schedule by BA, BAA, and GHG Regulation Area. |

## CAISO Formula

The formulas in this section use the following sign conventions:

- Demand Energy Schedule quantities for load, export, or participating load when consuming Energy are negative.

- Supply Energy Schedule quantities for generator, import, or participating load when providing Energy are positive.

- Pump storage can have either positive or negative Energy Schedule.

Day-Ahead Schedules Settlement Amount per BA

BANetHourlyDAEnergyAmt BQ’mdh  =

BAHourlyDAEnergyNetOfContractAmt BQ’mdh

+ BAHourlyDAEnergyContractAmt BQ’mdh

+ BAHourlyDAEnergyCongestionCredit BQ’mdh

+ BAHourlyDAEnergyTotalContractsLossCredit BQ’mdh

+ BAHourlyDAEnergyTotalContractSpecificLossChargeAmount BQ’mdh

+ BAHourlyBAADAEnergyChargeAdjustment BQ’mdh

Implementation Note: The business drivers will be BAHourlyDAEnergyNetOfContractAmt BQ’mdh

and BAHourlyBAADAEnergyChargeAdjustment BQ’mdh

Where BAHourlyDAEnergyNetOfContractAmt BQ’mdh =

Sum (r, t) {HourlyDAEnergyNetOfContractAmt BrtQ’mdh}

Where BAHourlyDAEnergyContractAmt BQ’mdh  =

Sum (r, t) {HourlyDAEnergyContractAmt BrtQ’mdh }

Where BAHourlyDAEnergyCongestionCredit BQ’mdh =

Sum (N, z’) {HourlyDAEnergyContractCongestionCredit BNz’Q’mdh }

Where BAHourlyDAEnergyTotalContractsLossCredit BQ’mdh =

Sum (N, z’) {HourlyDAEnergyContractLossCredit BNz’Q’mdh }

Where BAHourlyDAEnergyTotalContractSpecificLossChargeAmount BQ’mdh =

Sum (N, z’) {HourlyDAEnergyContractSpecificLossChargeAmount BNz’Q’mdh }

Where BAHourlyBAADAEnergyChargeAdjustment BQ’mdh =

Sum over (J) {PTBChargeAdjustmentBANetHourlyBAADAEnergyAmt BQ’Jmdh }

The sub-calculations are provided below.

HourlyDAEnergyNetOfContractAmt BrtQ’mdh

HourlyDAEnergyNetOfContractAmt BrtQ’mdh =

(-1) * (HourlyDAScheduleNetOfContract BrtQ’mdh* HourlyDAEnergyResourceLMP Brtmdh)

HourlyDAEnergyResourceLMP Brtmdh

- HourlyDAEnergyResourceLMP Brtmdh =

- NonMSSHourlyDAEnergyResourceLMP Brtmdh +

- MSSGrossGenHourlyDAEnergyResourceLMP Brtmdh +

- MSSGrossLoadHourlyDAEnergyResourceLMP Brtmdh +

- MSSNetHourlyDAEnergyResourceLMP Brtmdh

Note: Above implementation follows existing configuration, but uses appropriate description instead of using duplicate name. The component inputs are renamed respectively in the formulas below. Attribute B is now included.

HourlyMSSResourceDayAheadLMP rtmdh

- Sum (B)

- {IF MSSResourceFlag rtmd = 1 THEN

- BAHourlyResourceDayAheadLMP Brtmdh

- ELSE

- 0

- }

NonMSSHourlyDAEnergyResourceLMP Brtmdh =

- {IF MSSResourceFlag rtmd <> 1 THEN

BAHourlyResourceDayAheadLMP Brtmdh

- ELSE

- 0 }

IF MSSResourceFlag rtmd = 1 THEN

IF  I’ = “GROSS”  and t = “GEN” THEN

MSSGrossGenHourlyDAEnergyResourceLMP Brtmdh =

Average (u, T’, I’, M’, A, A’, V, p, L’)

MSSResourceFlag rtmd * MSSResourceInfo BrtuT’I’M’AA’VpLmd *  HourlyMSSResourceDayAheadLMP rtmdh )

###### IF   I’ = “GROSS”  and t = “LOAD” THEN

MSSGrossLoadHourlyDAEnergyResourceLMP Brtmdh =

Average (u, T’, I’, M’, A, A’, V, p, L’) (MSSResourceFlag rtmd * MSSResourceInfo BrtuT’I’M’AA’VpL’md * DA_LAP_LMP AA’mdh)

Where A’ = “DEFAULT”

IF  I’ = “NET”   THEN

###### IF    (DAEnergyMSSNetQty M’mdh >= 0)  THEN

MSSNetHourlyDAEnergyResourceLMP Brtmdh =

Average (u, T’, I’, M’, A, A’, V, p, L’) (MSSResourceFlag rtmd * MSSResourceInfo BrtuT’I’M’AA’VpL’md * DA_MSSNetSupplyLMP M’mdh)

###### ELSE {meaning DAEnergyMSSNetQty M’mdh < 0}  THEN

MSSNetHourlyDAEnergyResourceLMP Brtmdh =

Average (u, T’, I’, M’, A, A’, V, p, L’)

(MSSResourceFlag rtmd * MSSResourceInfo BrtuT’I’M’AA’VpL’md *

DA_MSSNetDemandLMP M’mdh )

Settlement of ETC/TOR/CVR Contract schedules at any resource r

HourlyDAEnergyContractAmt BrtQ’mdh =

(-1) * BAHourlyResourceDayAheadLMP Brtmdh * (BAHourlyResourceDABalancedTotalContractUsage BrtQ’mdh)

The IFM Congestion Credit to designated Billing SC B

HourlyDAEnergyContractCongestionCredit BNz’Q’mdh =

ContractBillingSCFactor BNz’Q’md * HourlyDAContractTotalCongestionCreditAmount Nz’Q’mdh

HourlyDAContractTotalCongestionCreditAmount Nz’Q’mdh =

Sum(B, A, A’, Q, p) HourlyDANodalCongestionCreditAmount BAA’QpNz’Q’mdh

HourlyDANodalCongestionCreditAmount BAA’QpNz’Q’mdh =

Sum (r, t) BAHourlyResourceDAEnergyContractCongestionCreditAmount BrtAA’QpNz’Q’mdh

BAHourlyResourceDAEnergyContractCongestionCreditAmount BrtAA’QpNz’Q’mdh =

BAHourlyResourceDAEnergyTORETCContractCongestionCreditAmount BrtAA’QpNz’Q’mdh

BAHourlyResourceDAEnergyTORETCContractCongestionCreditAmount BrtAA’QpNz’Q’mdh =

HourlyResourceDABalancedContractScheduleEnergy BrtAA’QpNz’Q’mdh*

HourlyDANodalTotalMCCPrice AA’Qpmdh

Where z’ in (‘TOR’, ‘ETC’)

BAHourlyResourceDAEnergyOATTContractCongestionCreditAmount BrtAA’QpNz’Q’mdh =

ContractDayAheadFinancialRightsFlag Nz’md *HourlyResourceDABalancedContractScheduleEnergy BrtAA’QpNz’Q’mdh*

HourlyDANodalTotalMCCPrice AA’Qpmdh

Where z’ in (‘OATT1’, ‘OATT2’)

Implementation Note: In the above formula, averaging of the price over the summed attributes (r, t) is intended to get the proper price for financial node or nodal location (AA’Qp).

HourlyDABAANodalMCCPrice Q’AA’Qpmdh = Average (M’) { HourlyDANodalMCCPrc M’Q’AA’Qpmdh + BAAHourlyDAMLAPMCCPrc M’Q’AA’Qpmdh}

HourlyDANodalTotalMCCPrice AA’Qpmdh = Sum (Q’) { HourlyDABAANodalMCCPrice Q’AA’Qpmdh }

BAHourlyResourceDAEnergyCRNScheduleCongestionCreditAmount BrtAA’Qpg’Nz’mdh =

BAHourlyResourceDAEnergyCRNSchedulePercentage BrtQ’AA’Qpg’Nz’mdh * BAHourlyResourceDAEnergyContractCongestionCreditAmount BrtAA’QpNz’Q’mdh

HourlyNodalDABalancedContractEnergy AA’Qpmdh =

HourlyNodalDAEnergyTORETCCongCreditQuantity AA’Qpmdh  + HourlyNodalDAEnergyOATTCongCreditQuantity AA’Qpmdh

BAAHourlyTotalDAEnergyNodalCongestionCreditAmount Q’AA’Qpmdh =

(-1)*HourlyNodalDABalancedContractEnergy AA’Qpmdh * HourlyDABAANodalMCCPrice Q’AA’Qpmdh

BAAHourlyTotalDAEnergyCongestionCreditAmount Q’mdh =

Sum (A, A’, Q, p) BAAHourlyTotalDAEnergyNodalCongestionCreditAmount Q’AA’Qpmdh

HourlyNodalDAEnergyTORETCCongCreditQuantity AA’Qpmdh =

Sum(B, r, t, N, z’, Q’) HourlyResourceDABalancedContractScheduleEnergy BrtAA’QpNz’Q’mdh

Where (z’) in (‘TOR’,’ETC’)

HourlyNodalDAEnergyOATTCongCreditQuantity AA’Qpmdh =

Sum(B, r, t, N, z’, Q’) {ContractDayAheadFinancialRightsFlag Nz’md *HourlyResourceDABalancedContractScheduleEnergy BrtAA’QpNz’Q’mdh }

Where (z’) in (‘OATT1’,’OATT2’)

CAISOBAATotalNetHourlyDAEnergyAmount mdh =

Sum over (Q’) {BAATotalNetHourlyDAEnergyAmount Q'mdh }

Where Q’ = ‘CISO’

The total Balancing Authority Area amount for Day-Ahead Energy per hour h

applying contract Congestion and contract Loss credits where applicable

BAATotalNetHourlyDAEnergyAmount Q'mdh =  BANetHourlyDAEnergyAmt BQ’mdh

Congestion and Congestion Credits for Day-Ahead Energy calculations:

BAANetHourlyDAEnergyCongestionNetOfCreditsAmount Q’mdh =

{ If NPMBAAFlag Q’md  <> 1

Then

BAAHourlyTotalDAEnergyMCCandCreditsAmount Q’mdh

Else

0

End If}

BAATotalHourlyNPMDAEnergyCongAmount Q’mdh =

{ If NPMBAAFlag Q’md  = 1

Then

BAAHourlyTotalDAEnergyMCCandCreditsAmount Q’mdh

Else

0

End If}

BAAHourlyTotalDAEnergyMCCAmount Q’mdh  =

( BAAHourlyDAEnergyNetofContractMCCAmount Q’mdh + BAAHourlyDAEnergyMSSNetMCCAmount Q’mdh  +

BAAHourlyDAEnergyContractMCCAmount Q’mdh +

BAAHourlyDAEnergyCongAdjAmount Q’mdh }

BAAHourlyTotalDAEnergyMCCandCreditsAmount Q’mdh  =

BAAHourlyTotalDAEnergyMCCAmount Q’mdh +

BAAHourlyTotalDAEnergyCongestionCreditAmount Q’mdh

BAAHourlyDAEnergyCongAdjAmount Q’mdh  =

Sum over (B, r, t, J) {PTBHourlyResourceBAADAEnergyCongestionAdjustmentAmt BrtQ’Jmdh }

Congestion Revenues for ETC/TOR/CVR Contract schedules at any resource r

BAAHourlyDAEnergyContractMCCAmount Q’mdh = Sum over (A,A’,p,Q) NodalHourlyBAADAEnergyContractMCCAmount Q’AA’pQmdh

NodalHourlyBAADAEnergyContractMCCAmount Q’AA’pQmdh =  (-1) * (NodalHourlyDAEnergyContractMCCQuantity AA’pQmdh  * HourlyDABAANodalMCCPrice Q’AA’Qpmdh AA’Qpmdh)

NodalHourlyDAEnergyContractMCCQuantity AA’pQmdh  = Sum over (B,r,t,Q’,M’,G’’)  (BAHourlyResourceDABalancedTotalContractUsage BrtQ’mdh)

- Where DAMHourlyBAAGHGAreaEnergyQuantity BrtQ’M’AA’QpG’’mdh exists for any interval of the Trading Day.

BAAHourlyDAEnergyNetofContractMCCAmount Q’mdh = Sum over (A,A’,p,Q) NodalHourlyDAEnergyNonMSSNetMCCAmount Q’AA’pQmdh

NodalHourlyDAEnergyNonMSSNetMCCAmount Q’AA’pQmdh =  (-1) * (NodalHourlyDAEnergyNetofContractMCCQuantity AA’pQmdh  * HourlyDABAANodalMCCPrice Q’AA’Qpmdh)

NodalHourlyDAEnergyNetofContractMCCQuantity AA’pQmdh  = Sum over (B,r,t,Q’,M’,G’’)  (HourlyDAScheduleNetOfContract BrtQ’mdh - BAHourlyResourceMSSNetDAEnergyQuantity BrtQ’mdh)

- Where DAMHourlyBAAGHGAreaEnergyQuantity BrtQ’M’AA’QpG’’mdh exists for any interval of the Trading Day.

MSS Net quantity and prices formulas

##### BAAHourlyDAEnergyMSSNetMCCAmount Q’mdh =

##### Sum over (M’)

##### {DAEnergyMSSNetQty M’mdh *(If DAEnergyMSSNetQty M’mdh > 0

Then

MSSHourlyDAEnergyMSSNetSupplyMCCPrice M’Q’mdh

ELSE

MSSHourlyDAEnergyMSSNetDemandMCCPrice M’Q’mdh)}

DAEnergyMSSNetQty M’mdh =

Sum over (B, r, t, Q’, u, T’, I’, F’, S’)

{ HourlyResourceDayAheadEnergy BrtuT’I’Q’M’F’S’mdh - BAHourlyResourceDABalancedTotalContractUsage BrtQ’mdh }

Where I’ = “NET”

Where

- IF (DAEnergyMSSNetTotalSupplyQty M’mdh <> 0)

- THEN

- DAEnergyMSSNetSupplyResourceWeight rtM’mdh =

(DAEnergyMSSNetSupplyResourceQty rtM’mdh /

DAEnergyMSSNetTotalSupplyQty M’mdh )

ELSE

- DAEnergyMSSNetSupplyResourceWeight rtM’mdh = 0

Where DAEnergyMSSNetSupplyResourceQty rtM’mdh =

Sum over (B, Q’, u, T’, I’, F’, S’)

{ HourlyResourceDayAheadEnergy BrtuT’I’Q’M’F’S’mdh  - BAHourlyResourceDABalancedTotalContractUsage BrtQ’mdh }

where I’ = “NET” and t = “GEN”

And Where DAEnergyMSSNetTotalSupplyQty M’mdh =

Sum (r, t) DAEnergyMSSNetSupplyResourceQty rtM’mdh

DA_MSSNetSupplyLMP M’mdh =

Sum (r, t) ( HourlyMSSResourceDayAheadLMP rtmdh * DAEnergyMSSNetSupplyResourceWeight rtM’mdh )

MSSHourlyDAEnergyMSSNetSupplyMCCPrice M’Q’mdh =

Average (A,A’,Q,p) (HourlyDANodalMCCPrc M’Q’AA’Qpmdh  * NodalHourlyDAEnergyMSSNetSupplyWeight M’AA’pmdh)

NodalHourlyDAEnergyMSSNetSupplyWeight M’AA’pmdh = Sum over (B,r,t,u,T’,I’,V,L’)

(MSSResourceInfo BrtuT’I’M’AA’VpL’md  * DAEnergyMSSNetSupplyResourceWeight rtM’mdh )

DA_MSSNetDemandLMP M’mdh =

Average (B, r, t, u, T’, I’, A, A’, V, p, L’) {MSSResourceInfo BrtuT’I’M’AA’VpL’md * DA_LAP_LMP AA’mdh }

Where I’ = “NET” and A’ = “CUSTOM”

MSSHourlyDAEnergyMSSNetDemandMCCPrice M’Q’mdh = Average over (A,A’,Q,p)

- (BAAHourlyDAMLAPMCCPrc M’Q’AA’Qpmdh * MSSDayNodalResourceInfo M’AA’md)

MSSDayNodalResourceInfo M’AA’md = Max  (B, r, t, u, T’, I’, p, V,  L’)

- (MSSResourceInfo BrtuT’I’M’AA’VpL’md)

Where I’ = “NET” and A’ = “CUSTOM”

- Note: This output is a flag, and as the input values are flags of 1, the max across the attributes are obtained. This charge is expected to have a value of 1.

Hourly DA Schedule mappings

HourlyDAScheduleNetOfContract BrtQ’mdh =

- (HourlyAllDASchedule BrtQ’mdh - BAHourlyResourceDABalancedTotalContractUsage BrtQ’mdh)

HourlyDASchedule BrtQ’mdh  = HourlyAllDASchedule BrtQ’mdh

HourlyAllDASchedule BrtQ’mdh  =

- Sum over (u, T’, I’, M’, F’, S’)

- {HourlyResourceDayAheadEnergy BrtuT’I’Q’M’F’S’mdh + HourlyResourceNPMDayAheadEnergy BrtuT’I’Q’M’F’S’mdh  }

BAHourlyResourceMSSNetDAEnergyQuantity BrtQ’mdh = Sum over (u, T’, I’, M’, F’, S’) HourlyResourceDayAheadEnergy BrtuT’I’Q’M’F’S’mdh - BAHourlyResourceDABalancedTotalContractUsage BrtQ’mdh)

- Where I’ = “NET”

HourlyResourceDayAheadEnergy BrtuT’I’Q’M’F’S’mdh  =

- Sum over (c, i, f)

- {(1-ResourceWholesaleExemptionFlag rmdhcif) * SettlementIntervalResouceDayAheadEnergy BrtuT’I’Q’M’F’S’mdhcif }

HourlyResourceNPMDayAheadEnergy BrtuT’I’Q’M’F’S’mdh  =

- Sum over (c, i, f) {(1-ResourceWholesaleExemptionFlag rmdhcif) *

- SettlementIntervalResNPMDayAheadEnergy BrtuT’I’Q’M’F’S’mdhcif}

SettlementIntervalResNPMDayAheadEnergy BrtuT’I’Q’M’F’S’mdhcif  =

SettlementIntervalResNPMGenAndTiesDAEnergy BrtuT’I’Q’M’F’S’mdhcif

- + SettlementIntervalResNPMLoadDAEnergy BrtuT’I’Q’M’F’S’mdhcif

SettlementIntervalResNPMGenAndTiesDAEnergy BrtuT’I’Q’M’F’S’mdhcif  =

(NPMDAScheduleEnergy BrtuT’I’Q’M’F’S’mdhcif + NPMDAPumpingEnergy BrtuT’I’Q’M’F’S’mdhcif + (NPMDATransferEnergy BrtuT’I’Q’M’F’S’mdh / 12))

- where Resource type (t) = GEN, ITIE, or ETIE

SettlementIntervalResNPMLoadDAEnergy BrtuT’I’Q’M’F’S’mdhcif  =

NPMDALoadSchedule BrtuT’I’Q’M’F’S’mdh / 12

- where Resource type (t) = Load

BAHourlyResourceDABalancedTotalContractUsage BrtQ’mdh  =

Sum (N) HourlyResourceDABalancedContractAtScheduleEnergy BrtQ’Nmdh

HourlyDAEnergyContractLossCredit BNz’Q’mdh =

TORContractBillingSCFactor BNz’Q’md * HourlyDAContractTotalLossCreditAmount Nz’Q’mdh

HourlyDAContractTotalLossCreditAmount Nz’Q’mdh =

Sum(B, A, A’, Q, p) HourlyDANodalLossCreditAmount BAA’QpNz’Q’mdh

HourlyDANodalLossCreditAmount BAA’QpNz’Q’mdh =

Sum (r, t) BAHourlyResourceDAEnergyContractLossCreditAmount BrtAA’QpNz’Q’mdh

BAHourlyResourceDAEnergyContractLossCreditAmount BrtAA’QpNz’Q’mdh =

HourlyResourceDABalancedContractScheduleEnergy BrtAA’QpNz’Q’mdh*

HourlyDAContractNodeMCL AA’QpNz’Q’mdh * ContractDailyTORLossCreditInclusionFlag Nz’md

where contract type z’ = “TOR”

HourlyDAContractNodeMCL AA’QpNz’Q’mdh =

- Average(r, t)

- {IF (contract type z’ = “TOR”) THEN

(DailyContractResourceFinancialNodeMap rtAA’QpNz’Q’md * HourlyDANodalMCLPrice AA’Qpmdh)

ELSE 0 }

Implementation Note: In the above formula, averaging of the price over the summed attributes (r, t) is intended to get the proper price for financial node or nodal location (AA’Qp).

BAHourlyResourceDAEnergyCRNScheduleLossCreditAmount BrtAA’Qpg’Nz’Q’mdh =

BAHourlyResourceDAEnergyCRNSchedulePercentage BrtAA’Qpg’Nz’Q’mdh * BAHourlyResourceDAEnergyContractLossCreditAmount BrtAA’QpNz’Q’mdh

where contract type z’ = “TOR”.

TORContractBillingSCFactor BNz’Q’md =

ContractBillingSCFactor BNz’Q’md

where contract type z’ = “TOR”.

HourlyDAEnergyContractSpecificLossChargeAmount BNz’Q’mdh =

(TORContractBillingSCFactor BNz’Q’md * ContractLossChargingPercentage Nz’Q’md *

HourlyDABAAMECPrc Q’mdh * DABalanceCapacity Nz’Q’mdh )

Reporting Quantity and Price (Estimated):

BAHourlyTotDAEnergyEstimatedQuantity BQ’mdh =

Sum over (r, t) { HourlyAllDASchedule BrtQ’mdh }

BAHourlyDAEnergyEstimatedPrice BQ’mdh =

BANetHourlyDAEnergyAmt BQ’mdh / BAHourlyTotDAEnergyEstimatedQuantity BQ’mdh

TSR Advisory Settlement Calculations:

BAHourlyTSRDAEnergyAdvisoryAmount BrQ’AA’Qpr’d’Q’’Nz’mdh =

(-1)* (BABAATransferSystemResourceDAEnergyTransferToLMPAmount BrQ’AA’Qpr’d’Q’’Nz’mdh

- BABAATransferSystemResourceDAEnergyTransferFromLMPAmount BrQ’AA’Qpr’d’Q’’Nz’mdh)

OATT Contract congestion credit:

HourlyDAEnergyOATTContractCongestionCreditAmount Nz’Q’mdh =

Sum (B, r, t, A, A’, Q, p) BAHourlyResourceDAEnergyOATTContractCongestionCreditAmount BrtAA’QpNz’Q’mdh

BAHourlyDAEnergyOATTContractCongestionCreditAmount BNz’Q’mdh =

ContractBillingSCFactor BNz’Q’md * HourlyDAEnergyOATTContractCongestionCreditAmount Nz’Q’mdh

## Outputs

| Output Req ID | Name | Description |
| --- | --- | --- |
|  | In addition to any outputs listed below, all inputs shall be included as outputs. |  |
|  | BANetHourlyDAEnergyAmt BQ’mdh | The Settlement Amount for Day Ahead Energy based on LMP less the (reversals of Congestion Charges and payments for valid and balanced schedules of ETC/TOR. ($) |
|  | BAHourlyDAEnergyContractAmt BQ’mdh | The portion of BANetHourlyDAEnergyAmt which corresponds to the charge or payment for valid and balanced Contract Self-Schedules of Business Associate B for Trading Hour h. ($) |
|  | BAHourlyDAEnergyNetOfContractAmt BQ’mdh | Portion of BANetHourlyDAEnergyAmt Bh that provides the net payment or charge for Day Ahead Energy schedule less the quantity for contracts using the relevant LMP price. This also excludes congestion credits. |
|  | BAHourlyDAEnergyCongestionCredit BQ’mdh | The IFM Congestion Credit to designated Billing SC B for valid and balanced portion of ETC/TOR/CVR Self-Schedule for Trading Hour h. ($) |
|  | BAHourlyDAEnergyTotalContractsLossCredit BQ’mdh | The BA total IFM Loss Credit to designated Billing SC B for valid and balanced portion of TOR Self-Schedule for Trading Hour h. ($) |
|  | BAHourlyDAEnergyTotalContractSpecificLossChargeAmount BQ’mdh | The sum of all Contract specific loss charging amounts assigned to BA ID B for Trading Hour h. ($) |
|  | BAHourlyBAADAEnergyChargeAdjustment BQ’mdh | Pass thru bill adjustment for this charge code by BA and by BAA. ($) |
|  | BAAHourlyDAEnergyCongAdjAmount Q’mdh | Total BAA PTB adjustment for congestion under this charge code. ($) |
|  | HourlyDAEnergyContractAmt BrtQ’mdh | Resource level charge or payment for valid and balanced Contract Self-Schedules. ($) |
|  | BAAHourlyDAEnergyContractMCCAmount Q’mdh | BAA level total congestion amount for DA energy priced based on MCC BAA breakdown. |
|  | NodalHourlyBAADAEnergyContractMCCAmount Q’AA’pQmdh | Nodal level congestion amount for DA energy priced based on MCC BAA breakdown. |
|  | NodalHourlyDAEnergyContractMCCQuantity Q’AA’pQmdh | Nodal level DA energy quantity. |
|  | BAAHourlyDAEnergyNetofContractMCCAmount Q’mdh | BAA level total congestion amount for DA energy excluding quantities from balanced contracts. |
|  | NodalHourlyDAEnergyNonMSSNetMCCAmount Q’AA’pQmdh | Nodal level total congestion amount for DA energy. Excludes MSS Net contribution. |
|  | NodalHourlyDAEnergyNetofContractMCCQuantity AA’pQmdh | Nodal level total congestion quantity for DA energy excluding quantities from balanced contracts. |
|  | HourlyDAEnergyNetOfContractAmt BrtQ’mdh | Resource level charge or payment based on the full LMP for schedules excluding valid and balanced Contract Self-Schedules. ($) |
|  | HourlyDASchedule BrtQ’mdh | The hourly Day Ahead Schedule for resource including Self-Schedules. (MWh) Applies only to CISO resources. |
|  | HourlyAllDASchedule BrtQ’mdh | The hourly Day Ahead Schedule for resource including Self-Schedules. (MWh) Applies to all resources, all BAA, and including NPM resources. |
|  | HourlyResourceDayAheadEnergy BrtuT’I’Q’M’F’S’mdh | The hourly Day Ahead Schedule for resource including Self-Schedules. (MWh). Excludes NPM resources. |
|  | HourlyResourceNPMDayAheadEnergy BrtuT’I’Q’M’F’S’mdh | The hourly Day Ahead Schedule for NPM resource including Self-Schedules. (MWh) |
|  | SettlementIntervalResNPMDayAheadEnergy BrtuT’I’Q’M’F’S’mdhcif | The settlement interval Day Ahead Schedule for NPM resource. (MWh) |
|  | SettlementIntervalResNPMGenAndTiesDAEnergy BrtuT’I’Q’M’F’S’mdhcif | The settlement interval Day Ahead Schedule for NPM resource of gen or import and export type. Also includes pumping energy and transfer energy. (MWh) |
|  | SettlementIntervalResNPMLoadDAEnergy BrtuT’I’Q’M’F’S’mdhcif | The settlement interval Day Ahead Schedule for NPM load resource. (MWh) |
|  | HourlyDAScheduleNetOfContract BrtQ’mdh | The hourly DA Energy Schedule excluding any valid and balanced Contract Self-Schedule .(MWh) |
|  | BAHourlyResourceDABalancedTotalContractUsage BrtQ’mdh | The hourly total for all contracts usage at a resource. (MWh) |
|  | HourlyDAEnergyResourceLMP Brtmdh | The applicable LMP for a resource for its Day Ahead Energy Schedules. ($/MWh) |
|  | HourlyDAEnergyContractCongestionCredit BNz’Q’mdh | The IFM Congestion Credit to designated Billing SC B for valid and balanced portion of contract Self-Schedule. ($) |
|  | HourlyDAContractTotalCongestionCreditAmount Nz’Q’mdh | The total congestion credit for a contract. This will be assigned to the designated Billing SC for the contract. ($) |
|  | HourlyDANodalCongestionCreditAmount BAA’QpNz’Q’mdh | The congestion credit at node AA’Qp of contract N. Attribute B here is still the original scheduler SC Business Associate B. ($) |
|  | BAHourlyResourceDAEnergyContractCongestionCreditAmount BrtAA’QpNz’Q’mdh | The congestion credit at a resource. Attribute B here is still the original scheduler SC Business Associate B. ($) |
|  | BAHourlyResourceDAEnergyTORETCContractCongestionCreditAmount BrtAA’QpNz’Q’mdh | The congestion credit at a resource for TOR or ETC. Attribute B here is still the original scheduler SC Business Associate B. ($) |
|  | BAHourlyResourceDAEnergyOATTContractCongestionCreditAmount BrtAA’QpNz’Q’mdh | The congestion credit at a resource for OATT1 or OATT2. Attribute B here is still the original scheduler SC Business Associate B. ($) |
|  | BAHourlyResourceDAEnergyCRNScheduleCongestionCreditAmount BrtAA’Qpg’Nz’Q’mdh | Shows scheduler SC Business Associate B how much its Chain CRN (attribute g’ value = the Chain_CRN_ID) or individual CRN (g’ is Null) self-schedule at a resource has contributed to the total congestion credit amount. However, this value is aggregated at the contract level and is assigned to the designated Billing SC for the contract. |
|  | HourlyNodalDABalancedContractEnergy AA’Qpmdh | Total balanced contract quantities at nodes. |
|  | BAAHourlyTotalDAEnergyNodalCongestionCreditAmount Q’AA’Qpmdh | Total congestion credit amounts for DA Energy at nodes. |
|  | BAAHourlyTotalDAEnergyCongestionCreditAmount Q’mdh | Total congestion credit amounts for DA Energy per BAA. |
|  | HourlyNodalDAEnergyBalancedTORETCCongCreditQuantity AA’Qpmdh | Total balanced TOR/ETC contract quantities at nodes eligible for cong credit. |
|  | HourlyNodalDAEnergyBalancedOATTCongCreditQuantity AA’Qpmdh | Total balanced OATT1/OATT2 contract quantities at nodes eligible for cong credit. |
|  | HourlyDAEnergyContractLossCredit BNz’Q’mdh | The IFM Loss Credit to designated Billing SC B for valid and balanced portion of Self-Schedule. ($) |
|  | HourlyDAContractTotalLossCreditAmount Nz’Q’mdh | The total loss credit for contract N of contract type z’ = “TOR”, for Trading Hour h. This will be assigned to the designated Billing SC for the contract. ($) |
|  | HourlyDANodalLossCreditAmount BAA’QpNz’Q’mdh | The TOR loss credit at financial node AA’Qp of contract N, contract type z’ for Trading Hour h. Attribute B here is still the original scheduler SC B. ($) |
|  | BAHourlyResourceDAEnergyContractLossCreditAmount BrtAA’QpNz’Q’mdh | The TOR loss credit at a resource. Attribute B here is still the original scheduler SC B. ($) |
|  | HourlyDAContractNodeMCL AA’QpNz’Q’mdh | The marginal cost of losses (MCL) at financial node. ($/MWh) |
|  | BAHourlyResourceDAEnergyCRNScheduleLossCreditAmount BrtAA’Qpg’Nz’Q’mdh | Shows scheduler SC Business Associate B how much its Chain CRN (attribute g’ value = the Chain_CRN_ID) or individual CRN (g’ is Null) self-schedule at a resource has contributed to the total loss credit amount for the contract. However, this value is aggregated at the contract level and is assigned to the designated Billing SC for the contract. |
|  | TORContractBillingSCFactor BNz’Q’md | This has a value of 1, if B is the Billing SC (also called the responsible TO or SC) of contract N of contract type z’ = TOR. |
|  | DAEnergyMSSNetQty M’mdh | The Day Ahead hourly net Demand or net Supply quantity for MSS Subgroup M’ that elected Net Energy Settlement. (MWh) |
|  | DA_MSSNetSupplyLMP M’mdh | The Day Ahead hourly average price for MSS Subgroup M’ that elected Net Energy settlement, when such MSS is a net supplier of Energy for Trading Hour h. ($/MWh) |
|  | DA_MSSNetDemandLMP M’mdh | The Day Ahead hourly average price for MSS Subgroup M’ that elected Net Energy settlement, when such MSS is a net consumer of Energy for the Trading Hour h. ($/MWh) |
|  | DAEnergyMSSNetSupplyResourceWeight rtM’mdh | MSS Net supply resource’s weight or contribution to the total MSS Net price. The weight is applicable to both LMP and MCC prices and is based on the Energy Schedule net of valid and balanced contract Self-Schedules. This pertains to resource r of resource type t inside MSS Subgroup M’ for Trading Hour h. |
|  | DAEnergyMSSNetSupplyResourceQty rtM’mdh | Energy Schedule net of valid and balanced contract Self-Schedules for a resource of an MSS that elected Net energy settlement. |
|  | DAEnergyMSSNetTotalSupplyQty M’mdh | The total of DAEnergyMSSNetSupplyResourceQty across all supply resources of an MSS Subgroup M’ entity which elected Net energy settlement. |
|  | CAISOBAATotalNetHourlyDAEnergyAmount mdh | CAISO BAA total settlement for DA Energy. |
|  | BAATotalNetHourlyDAEnergyAmount Q’mdh | The total Balancing Authority Area hourly net amount for Day Ahead Energy for Trading Hour h. This also corresponds to the system-wide net Congestion revenues plus net loss overcollection plus net Congestion credits. ($) |
|  | BAANetHourlyDAEnergyCongestionNetOfCreditsAmount Q’mdh | The total net Congestion amount for Day Ahead Energy for Trading Hour h. This already excludes Congestion credit for transmission contracts. ($) |
|  | BAATotalHourlyNPMDAEnergyCongAmount Q’mdh | The total NPM Congestion amount for Day Ahead Energy for Trading Hour h per BAA. ($) |
|  | BAAHourlyTotalDAEnergyMCCAmount Q’mdh | The total Congestion amount for Day Ahead Energy for Trading Hour h. Value before factoring in Congestion credit for transmission contracts. ($) |
|  | BAAHourlyTotalDAEnergyMCCandCreditsAmount Q’mdh | The total net Congestion amount for Day Ahead Energy for Trading Hour h. This already excludes Congestion credit for transmission contracts. ($) |
|  | HourlyDAEnergyContractSpecificLossChargeAmount BNz’Q’mdh | The Contract specific loss charging amount assigned to BA ID B for contract N of contract type z’ for Trading Hour h. ($) |
|  | HourlyMSSResourceDayAheadLMP rtmdh | DA LMP for an MSS resource, further subject to MSS pricing rules |
|  | NonMSSHourlyDAEnergyResourceLMP Brtmdh | The applicable LMP for a Non-MSS resource for its Day Ahead Energy Schedules. ($/MWh) |
|  | MSSGrossGenHourlyDAEnergyResourceLMP Brtmdh | The applicable LMP for an MSS Generator resource for its Day Ahead Energy Schedules. ($/MWh) |
|  | MSSGrossLoadHourlyDAEnergyResourceLMP Brtmdh | The applicable LMP for an MSS Load resource for its Day Ahead Energy Schedules. ($/MWh) |
|  | MSSNetHourlyDAEnergyResourceLMP Brtmdh | The applicable LMP for an MSS Net resource for its Day Ahead Energy Schedules. ($/MWh) |
|  | BAAHourlyDAEnergyMSSNetMCCAmount Q’mdh | BAA level total congestion amount for DA energy coming from MSSNet contribution. |
|  | MSSHourlyDAEnergyMSSNetSupplyMCCPrice M’Q’mdh | Average MCC price applicable to supply resources within an MSSNet when the entity is supplying. |
|  | NodalHourlyDAEnergyMSSNetSupplyWeight M’AA’pmdh | Nodal weights at MSS when the entity is supplying. |
|  | MSSHourlyDAEnergyMSSNetDemandMCCPrice M’Q’mdh | MSSNet MCC Price when the entity is consuming. |
|  | MSSDayNodalResourceInfo M’AA’pmd | MSSNet Nodal locations info |
|  | BAHourlyResourceMSSNetDAEnergyQuantity BrtQ’mdh | MSSNet resource level DA energy quantities. |
|  | BAHourlyTotDAEnergyEstimatedQuantity BQ’mdh | Estimated total quantity for DA Energy per BA_ID per BAA_ID. (MWh) |
|  | BAHourlyDAEnergyEstimatedPrice BQ’mdh | Estimated price for DA Energy per BA_ID per BAA_ID. ($/MWh) |
|  | BAHourlyTSRDAEnergyAdvisoryAmount BrQ’AA’Qpr’d’Q’’Nz’mdh | DA Energy Advisory Settlement for Transfer system resources per Hour as a product of nodal LMP and the difference between Transfer To and Transfer From quantities. |
|  | HourlyDAEnergyOATTContractCongestionCreditAmount Nz’Q’mdh | OATT contract congestion credit value. |
|  | BAHourlyDAEnergyOATTContractCongestionCreditAmount BNz’Q’mdh | OATT contract congestion credit value assigned to contract Billing SC. |

# Charge Code Effective Dates

| Charge Code/ Pre-calc Name | Document Version | Effective Start Date | Effective End Date | Version Update Type |
| --- | --- | --- | --- | --- |
| CC 6011 – Day Ahead Energy, Congestion, Loss Settlement | 5.0 | 04/01/09 | 03/31/09 | Documentation Edits Only |
| CC 6011 – Day Ahead Energy, Congestion, Loss Settlement | 5.1 | 04/01/09 | 1/31/13 | Configuration Impacted |
| CC 6011 – Day Ahead Energy, Congestion, Loss Settlement | 5.2 | 02/01/13 | 4/30/14 | Configuration Impacted |
| CC 6011 – Day Ahead Energy, Congestion, Loss Settlement | 5.2a | 5/1/14 | 9/30/14 | Documentation Edits Only |
| CC 6011 – Day Ahead Energy, Congestion, Loss Settlement | 5.3 | 10/1/14 | 10/31/16 | Configuration Impacted |
| CC 6011 – Day Ahead Energy, Congestion, Loss Settlement | 5.3a | 11/1/16 | 9/30/20 | Documentation Edits Only |
| CC 6011 – Day Ahead Energy, Congestion, Loss Settlement | 5.4 | 10/1/21 | 12/31/20 | Configuration Impacted |
| CC 6011 – Day Ahead Energy, Congestion, Loss Settlement | 5.5 | 1/1/21 | 4/30/21 | Configuration Impacted |
| CC 6011 – Day Ahead Energy, Congestion, Loss Settlement | 5.6 | 5/1/21 | 4/30/26 | Configuration Impacted |
| CC 6011 – Day Ahead Energy, Congestion, Loss Settlement | 6.0 | 5/1/26 | 4/30/26 | Configuration Impacted |
| CC 6011 – Day Ahead Energy, Congestion, Loss Settlement | 6.0.1 | 5/1/26 | Open | Configuration Impacted |