Settlements and Billing

Configuration Guide: CG CC 1001 Black Start Energy Payment v 5.33

Version 5.3

Table of Contents

1.	Purpose of Document	3

2.	Introduction	3

2.1	Background	3

2.2	Description	3

3.	Charge Code Requirements	4

3.1	Business Rules	4

3.2	Predecessor Charge Codes	5

3.3	Successor Charge Codes	5

3.4	Inputs – External Systems	5

3.5	Inputs - Predecessor Charge Codes or Pre-calculations	6

3.6	CAISO Formula	7

3.7	Outputs	7

4.	Charge Code Effective Date	8

- Purpose of Document

The purpose of this document is to capture the requirements and design specification for a charge calculation in one document.

- Introduction

## Background

Black Start Energy is an Ancillary Service (A/S) that consists of dispatched Energy using self-started equipment following a system blackout.  Black Start equipment must be able to start without an external source of electricity and must be available to participate in system restoration under the coordination of California ISO.

The CAISO has the discretion to procure Black Start as needed and may establish agreements with eligible generators for the provision of Black Start services.  The CAISO is also responsible for ensuring that Black Start services are readily available in the event of a system blackout via periodic testing.

Whenever a Black Start unit provides Black Start in accordance with the CAISO’s instructions, the CAISO will pay the Energy from the unit based upon the negotiated rate per established agreement.  The cost of Black Start Energy is allocated to market participants based upon measured demand.

## Description

Black start Energy is settled hourly.  The Energy is comprised of both start-up and minimum run components for which the Settlements System receives 5-minute quantity values and their associated Energy prices.  Not considered normal market Energy, the Energy is considered exceptional Energy, and is settled at a contracted Energy price and not the Locational Marginal Price (LMP) of the Generator providing the Energy.  The CAISO determines the Settlement payment for Black Start Energy from a Generator resource by multiplying each 5-minute FMM or RTD Exceptional Dispatch Energy quantity by its applicable FMM or RTD Energy price, and then summing the resulting 5-minute amounts to form the resource’s Energy payment amount for the Trading Hour.  The calculation described herein determines the Energy payment amount for each Black Start unit and Trading Hour.

The costs of providing Black Start Energy service (payments) are recovered from Business Associates (BAs) (as charges) through the Black Start Energy Allocation (CC 1353) allocation based on Measured Demand.

- Charge Code Requirements

## Business Rules

| Bus Req ID | Business Rule |
| --- | --- |
| 1.0 | The Black Start Energy Payment Pre-calculation software applies to BAs with units that were instructed for Black Start Energy during a Trading Hour. |
| 2.0 | A Black Start Energy Payment amount and quantity shall be calculated for each resource “r” of its Business Associate “B” and for each Trading Hour “h” and Trading Date “d” for which Black Start Energy has been dispatched. |
| 3.0 | A Black Start Energy Payment amount and quantity shall be calculated by Business Associate “B” for each Trading Hour “h” and Trading Date “d” for which Black Start Energy has been dispatched. |
| 4.0 | The expected Black Start Energy quantity (in MWh) from a unit dispatched during a Trading Hour shall be computed as the sum of the 5-minute quantities for each of the unit’s instructions (Black Start Energy components) over all the 5-minute Dispatch Intervals of the relevant Trading Hour. |
| 4.1 | The quantity of each bid segment (“b”) of Black Start Energy for resource “r” during Dispatch Interval “f” shall be provided as an input to the Black Start Energy Payment calculation. |
| 4.1.1 | A resource’s instructed Black Start Energy components (for which the current calculation will receive both a 5-minute quantity and Energy price value) represent “start-up” and “minimum run” Black Start Energy. (Fact) |
| 5.0 | The expected Black Start Energy payment amount (in $) for a unit dispatched during a Trading Hour shall be computed as the sum of the 5-minute payment amounts for each of the unit’s instructions (Black Start Energy components) over the all 5-minute Dispatch Intervals of the relevant Trading Hour. |
| 5.1 | The 5-minute payment amount for a Black Start Energy component in a 5-minute interval shall be computed at the product of the Energy’s quantity and its price for the 5-minute interval. |
| 5.1.1 | The 5-minute payment amount will usually be a negative value when Exceptional Dispatch IIE (quantity) > 0. (Fact) |
| 5.1.2 | The Energy price in a 5-minute interval will usually be a positive value when Exceptional Dispatch IIE (quantity) > 0. (Fact) |
| 6.0 | A Pass-Through-Bill capability shall provide for an adjustment to the Black Start Energy Payment Amount by BA output value (to provide for the case when adjustment of the latter value cannot be accomplished by correction of upstream data inputs or cannot be performed by Operator override). |
| 7.0 | For RMR resources subject to new Tariff, variable energy cost opportunity cost adders shall reduce bid cost per MWh of exceptional dispatches. |

## Predecessor Charge Codes

Charge Code/ Pre-calc Name

Pre-calculation – Real Time Price

## Successor Charge Codes

Charge Code/ Pre-calc Name

Black Start Energy Allocation (CC 1353)

## Inputs – External Systems

| Row # | Variable Name | Description |
| --- | --- | --- |
| 1 | ExceptionalDispatchIIE BrtT’ObuI’Q’M’AA’R’W’S’F’VL’Phcif | ExceptionalDispatchIIE BrtT’ObuI’M’AA’R’W’S’F’VL’Phif represents the Expected Energy (provided by MQS) from Resource ID r as a result of Bid Segment b, where the Exceptional Dispatch type (O) = “Black Start”. |
| 2 | ExceptionalDispatchIIEPrice BrtObhcif | ExceptionalDispatchIIEPrice BrtObhif represents the 5-minute bid price (provided by MQS) for Black Start Energy from resource ID r as a result of Bid Segment b, where the Exceptional Dispatch type (O) = “Black Start”. |
| 3 | PTBChargeAdjustmentBlackStartEnergyPayment BJdh | PTB Charge Adjustment Black Start Energy Payment associated with Business Associate “B”, PTB ID J, Trading Day d, Trading Month m. |
| 4 | FMMExceptionalDispatchIIE BrtuT’ObI’Q’M’AA’R’W’F’S’PVL’mdhcif | IIE due to an FMM Exceptional Dispatch (MWh) |
| 5 | FMMExceptionalDispatchIIEPrice BrtObmdhcif | Price for Exceptional Dispatch IIE ($/MWh) one of the following: (1) Bid, (2) the Default Energy Bid, (3) negotiated price, or (4) calculated price. |

## Inputs - Predecessor Charge Codes or Pre-calculations

| Row # | Variable Name | Predecessor Charge Code/ Pre-calc Configuration |
| --- | --- | --- |
| 1 | FMMExceptionalDispatchIIELessVECPrice BrtObmdhcif | Pre-calculation – Real Time Price |
| 2 | RTDExceptionalDispatchIIELessVECPrice BrtObmdhcif | Pre-calculation – Real Time Price |

## CAISO Formula

    - BlackStartEnergyPaymentAmount Brtmdh =

- cf∈c BlackStart5MinuteEnergyPaymentAmount Brtmdhcif

    - BlackStart5MinuteEnergyPaymentAmount Brtmdhcif  =

- T'uI'Q'M'S'F'AOA'bR' W' V L'P (-1) *

### (max(0, ExceptionalDispatchIIE BrtT’ObuI’Q’M’AA’R’W’S’F’VL’Phcif) * RTDExceptionalDispatchIIELessVECPrice BrtObmdhcif

### ExceptionalDispatchIIEPrice 	BrtObhcif) + (max (0, FMMExceptionalDispatchIIE BrtuT’ObI’Q’M’AA’R’W’F’S’PVL’mdhcif) * FMMExceptionalDispatchIIELessVECPrice BrtObmdhcif

- FMMExceptionalDispatchIIEPrice BrtObmdhcif))

- And EXCEPTIONAL DISPATCH TYPE (O) = ‘BS’ (Black Start)

    - BlackStartEnergyPaymentQuantity Brtmdh =   cf∈c BlackStart5MinuteEnergyPaymentQuantity Brtmdhcif

    - BlackStart5MinuteEnergyPaymentQuantity Brtmdhcif =

- T'uI'Q'M'F'S'AOA'b R'W'VL' P (max(0, ExceptionalDispatchIIE BrtT’ObuI’Q’M’AA’R’W’S’F’VL’Phcif) + max (0,FMMExceptionalDispatchIIE BrtuT’ObI’Q’M’AA’R’W’F’S’PVL’mdhcif))

- And EXCEPTIONAL DISPATCH TYPE O = ’BS’ (Black Start)

## Outputs

| Output ID | Name | Description |
| --- | --- | --- |
| 1 | BlackStartEnergyPaymentQuantity Brtmdh | The hourly Black Start Energy Payment Quantity output (in MW/h) for a Resource ID r, Resource Type t attributed to Business Associate B and for Trading Day d and Trading Hour h. |
| 2 | BlackStart5MinuteEnergyPaymentAmount Brtmdhcif | The hourly Black Start Energy Payment Amount output (in $) for a Resource ID r, Resource Type t attributed to Business Associate B and for Trading Day “d” and Trading Hour “h” and Settlement Interval f. |
| 3 | - BlackStartEnergyPaymentAmount Brtmdh | The hourly Black Start Energy Payment Amount output (in $) for a Resource ID r, Resource Type t attributed to Business Associate B and for Trading Day d and Trading Hour h. |
| 4 | - BlackStart5MinuteEnergyPaymentQuantity Brtmdhcif | The hourly Black Start Energy Payment Quantity output (in MW/h) for a Resource ID r, Resource Type t attributed to Business Associate B and for Trading Day d and Trading Hour h and for Settlement Interval f. |

# Charge Code Effective Date

| Charge Code/ Pre-calc Name | Document Version | Effective Start Date | Effective End Date | Version Update Type |
| --- | --- | --- | --- | --- |
| CG CC 1001 Black Start Energy Payment v 5.33 | 5.0 | 04/01/09 | 4/30/14 | Documentation Edits Only |
| CG CC 1001 Black Start Energy Payment v 5.33 | 5.1 | 5/1/14 | 9/30/14 | Configuration Impacted |
| CG CC 1001 Black Start Energy Payment v 5.33 | 5.2 | 10/1/14 | 12/31/19 | Configuration Impacted |
| CG CC 1001 Black Start Energy Payment v 5.33 | 5.3 | 1/1/20 | Open | Configuration Impacted |