Settlements and Billing

Configuration Guide: CG CC 3303 Supplemental Reactive Energy Settlement v 5.5

Version 5.5

Table of Contents

1.	Purpose of Document	3

2.	Introduction	3

2.1	Background	3

2.2	Description	3

3.	Charge Code Requirements	3

3.1	Business Rules	3

3.2	Predecessor Charge Codes	4

3.3	Successor Charge Codes	4

3.4	Inputs – External Systems Requirements	4

3.5	Inputs - Predecessor Charge Codes or Pre-calculations	5

3.6	CAISO Formula	6

3.7	Outputs	7

4.	Charge Code Effective Date	8

# Purpose of Document

The purpose of this document is to capture the requirements and design specification for a SaMC Charge Code in one document.

# Introduction

## Background

Voltage Support services can be provided by Generating Units or other equipment such as shunt capacitors, static var compensators, or synchronous condensers.  All Generating Units are required by the CAISO Tariff to provide reactive Energy by operating within a power factor range of 0.90 lag and 0.95 lead.  To the extent that CAISO needs additional reactive Energy in Real-Time that requires a Generating Unit to operate outside of this range, CAISO can call on Generators to provide additional short-term Voltage Support referred to as Supplemental Reactive Energy.

Payment for Supplemental Reactive Energy provided is settled in this charge.  .  Voltage Support costs incurred by the CAISO are recovered from all BAs based on measured demand in CC 1303 Supplemental Reactive Energy Allocation.

## Description

Payments for Supplemental Reactive Energy are essentially lost opportunity cost payments to Business Associates for providing the service.

The CAISO selects Generating Units based on their real-time Energy Bids.  Subject to any locational requirements the CAISO will select the Generating Unit with the highest decremental Real-Time Energy Bid to reduce MW output by such amount as is necessary to achieve the instructed MVar reactive energy production.  The CAISO will pay the Business Associate for that Generating Unit the lost opportunity cost resulting from the reduction of MW output.

# Charge Code Requirements

## Business Rules

| Bus Req ID | Business Rule |
| --- | --- |
| 1.0 | RTD Negative Exceptional Dispatch IIE quantities where the Exceptional Dispatch Type is Voltage Support will receive Supplemental Reactive Energy payments for the lost opportunity cost, which is the difference between the Settlement Interval RTD LMP and the Bid Price for that Exceptional Dispatch IIE. |
| 2.0 | If the Settlement Interval RTD LMP is lower than or equal to the Bid Price, there is no lost opportunity cost and no payment is calculated. |
| 3.0 | FMM Negative Exceptional Dispatch IIE quantities where the Exceptional Dispatch Type is Voltage Support will receive Supplemental Reactive Energy payments for the lost opportunity cost, which is the difference between the FMM Interval LMP and the Bid Price for that Exceptional Dispatch IIE. |
| 3.1 | For RMR resources subject to new Tariff, variable energy cost opportunity cost adders shall also reduce bid cost (or price) for exceptional dispatches. |
| 3.2 | For RMR resources subject to new Tariff, market revenues in excess of qualified costs shall be subject to a true up. This excess revenue will be subtracted from capacity payments. The qualified cost is the bid cost less variable energy costs opportunity cost adder. |
| 4.0 | If the FMM Interval LMP is lower than or equal to the Bid Price, there is no lost opportunity cost and no payment is calculated. |
| 5.0 | This Charge Code must be computed daily on a Settlement Interval basis as the sum of the RTD Supplemental Reactive Energy Settlement Amount and the FMM Supplemental Reactive Energy Settlement Amount. |
| 6.0 | For adjustments to the Charge Code that cannot be accomplished by correction of upstream data inputs, recalculation or operator override Pass Through Bill Charge (PTB) logic will be applied. |
| 7.0 | The formulas adopt the convention that payments made by CAISO to BAs will be negative, while payments received by CAISO from BAs (charges to BAs) will be positive. |

## Predecessor Charge Codes

Charge Code/ Pre-calc Name

Pre-calculation - Real Time Price

## Successor Charge Codes

Charge Code/ Pre-calc Name

CC 1303 – Supplemental Reactive Energy Allocation

## Inputs – External Systems Requirements

| Row # | Variable Name | Description |
| --- | --- | --- |
| 1 | ExceptionalDispatchIIEprice BrtObmdhcif | Exceptional Dispatch. IIE Price represents the 5-minute real-time energy price for Supplemental Reactive Energy bid segment “b” that is associated with Business Associate B, and resource id r where the Dispatch type “O” = “VS” (“Voltage Support”). |
| 2 | ExceptionalDispatchIIE BrtuT’ObI’Q’M’AA’R’W’F’S’VLP’mdhcif | Exceptional Dispatch IIE represents the expected Supplemental Reactive Energy where the Exceptional Dispatch type “O” = “Voltage Support”. |
| 3 | PTBChargeAdjustmentSupplementalReactiveEnergySettlementAmount BJmdhcif | PTB Charge Adjustment Supplemental Reactive Energy Settlement Amount for Business Associate B and PTB ID J. |
| 4 | FMMExceptionalDispatchIIE BrtuT’ObI’Q’M’AA’R’W’F’S’PVL’mdhcif | IIE due to an FMM Exceptional Dispatch (MWh) |
| 5 | FMMExceptionalDispatchIIEPrice BrtObmdhcif | Price for Exceptional Dispatch IIE ($/MWh) one of the following: (1) Bid, (2) the Default Energy Bid, (3) negotiated price, or (4) calculated price. |

## Inputs - Predecessor Charge Codes or Pre-calculations

| Row # | Variable Name | Predecessor Charge Code/ Pre-calc Configuration |
| --- | --- | --- |
|  | ### RTDExceptionalDispatchIIECostAboveLMPPrice BrtObmdhcif | Pre-calculation – Real Time Price |
|  | ### FMMExceptionalDispatchIIECostAboveLMPPrice BrtObmdhcif | Pre-calculation – Real Time Price |

## CAISO Formula

    - SupplementalReactiveEnergySettlementAmount Brtmdhcif =

- M'AA'ObuT'I'F'S'R'W'VL'

- (FMMSupplementalReactiveEnergySettlementAmount BrtuT’ObI’M’AA’R’W’F’S’VL’mdhcif  + RTDSupplementalReactiveEnergySettlementAmount BrtuT’ObI’M’AA’R’W’F’S’VL’mdhcif )

    - RTDSupplementalReactiveEnergySettlementAmount BrtuT’ObI’M’AA’R’W’F’S’VL’mdhcif  =

### PQ'(-1)*(min (0, RTDExceptionalDispatchIIECostAboveLMPPrice BrtObmdhcif) * min(0, ExceptionalDispatchIIE BrtuT’ObI’Q’M’AA’R’W’F’S’VLP’mdhcif))

Where Exceptional Dispatch Type O =”VS” (Voltage Support)

    - FMMSupplementalReactiveEnergySettlementAmount BrtuT’ObI’M’AA’R’W’F’S’VL’mdhcif  =

### PQ'(-1)*(min (0, FMMExceptionalDispatchIIECostAboveLMPPrice BrtObmdhcif

- )* min(0, FMMExceptionalDispatchIIE BrtuT’ObI’Q’M’AA’R’W’F’S’PVL’mdhcif))

where Exceptional Dispatch Type O =”VS” (Voltage Support)

    - RTDRMR5minSuppReactiveEnergyTrueUpAmount BrtObmdhcif

- =Sum over (u, T’, I’, Q’, M’, A, A’, R’, W’, F’, S’, V, L’, P)

- (-1)*max{0, RTDExceptionalDispatchIIECostAboveLMPPrice BrtObmdhcif }* min(0, ExceptionalDispatchIIE BrtuT’ObI’Q’M’AA’R’W’F’S’PVL’mdhcif))

Where Exceptional Dispatch Type O =”VS” (Voltage Support)

    - FMMRMR5minSuppReactiveEnergyTrueUpAmount BrtObmdhcif

- = Sum over (u, T’, I’, Q’, M’, A, A’, R’, W’, F’, S’, V, L’, P)

- (-1)*max {0, FMMExceptionalDispatchIIECostAboveLMPPrice BrtObmdhcif  }* min(0, FMM

- ExceptionalDispatchIIE BrtuT’ObI’Q’M’AA’R’W’F’S’VLP’mdhcif))

Where Exceptional Dispatch Type O =”VS” (Voltage Support)

    - RMRDailySuppReactiveEnergyTrueUpAmount Brmd

- =Sum over (t, O, b, h, c, i, f)

- RTDRMR5minSuppReactiveEnergyTrueUpAmount BrtObmdhcif + FMMRMR5minSuppReactiveEnergyTrueUpAmount BrtObmdhcif

- Outputs

| Output Req ID | Name | Description |
| --- | --- | --- |
|  | In addition to any outputs listed below, all inputs shall be included as outputs. |  |
|  | RTDSupplementalReactiveEnergySettlementAmount BrtuT’ObI’M’AA’R’W’F’S’VL’mdhcif | 5Min Supplemental Reactive Energy Settlement Amount |
|  | SupplementalReactiveEnergySettlementAmount Brtmdhcif | The settlement amount paid to Business Associate B and Resource ID r for its lost opportunity cost for providing short-term Voltage Support. |
|  | FMMSupplementalReactiveEnergySettlementAmount BrtuT’ObI’M’AA’R’W’F’S’VL’mdhcif | The FMM Supplemental Reactive Energy Settlement Amount |
|  | RTDRMR5minSuppReactiveEnergyTrueUpAmount BrtObmdhcif | RMR excess revenue in RTD from exceptional dispatch of type voltage support (supplemental reactive energy). |
|  | FMMRMR5minSuppReactiveEnergyTrueUpAmount BrtObmdhcif | RMR excess revenue in FMM from exceptional dispatch of type voltage support (supplemental reactive energy). |
|  | RMRDailySuppReactiveEnergyTrueUpAmount Brmd | Total daily RMR excess revenue from exceptional dispatch of type voltage support (supplemental reactive energy). |

# Charge Code Effective Date

| Charge Code/ Pre-calc Name | Document Version | Effective Start Date | Effective End Date | Version Update Type |
| --- | --- | --- | --- | --- |
| CC 3303 – Supplemental Reactive Energy Settlement | 5.0 | 04/01/09 | 4/30/14 | Documentation Edits Only |
| CC 3303 – Supplemental Reactive Energy Settlement | 5.1 | 05/1/14 | 9/30/14 | Configuration Change |
| CC 3303 – Supplemental Reactive Energy Settlement | 5.2 | 10/1/14 | 11/3/15 | Configuration Change |
| CC 3303 – Supplemental Reactive Energy Settlement | 5.3 | 11/4/15 | 10/31/16 | Configuration Change |
| CC 3303 – Supplemental Reactive Energy Settlement | 5.4 | 11/1/16 | 12/31/19 | Configuration Change |
| CC 3303 – Supplemental Reactive Energy Settlement | 5.5 | 1/1/20 | Open | Configuration Change |