Settlements & Billing

Configuration Guide:  CG CC 6294 Non Spinning Reserve Obligation Settlement v5.2a

CC 6294

Version 5.2a

Table of Contents

1.	Purpose of Document	3

2.	Introduction	3

2.1	Background	3

2.2	Description	4

3.	Charge Code Requirements	4

3.1	Business Rules	4

3.2	Predecessor Charge Codes	5

3.3	Successor Charge Codes	5

3.4	Inputs – External Systems	6

3.5	Inputs - Predecessor Charge Codes or Pre-calculations	6

3.6	CAISO Formula	8

3.7	Outputs	9

4.	Charge Code Effective Date	10

# Purpose of Document

The purpose of this document is to capture the business and functional requirements for the MRTU SaMC Non-Spinning Reserve Obligation Settlement, Charge Code 6294.

# Introduction

## Background

The CAISO will procure the Ancillary Services, Regulation Up, Regulation Down, Spinning Reserve, and Non-Spinning Reserve in the Day Ahead Integrated Forward Market (IFM) and procure incrementally as needed in the Real-Time Market (RTM).  Ancillary Services (AS) are procured simultaneously with Energy bids to meet Regulation and Operating Reserve requirements, using submitted Ancillary Service bids.  IFM is performed for each hour of the next trading day. The Fifteen Minute Market (FMM) performs unit commitment and AS procurement, if needed, at 15-minutes intervals for the current hour and next Trading Hour.  The AS awards published for the first 15-minute interval of the time horizon are binding, the rest are advisory.  The AS Pricing and Settlement will be based on Ancillary Service Marginal Prices (ASMP), which are calculated for each AS region for each market time interval for each market.

The AS procurement cost is the payment for AS Awarded bids in the Day Ahead IFM and RTM.  The Day Ahead and Real-Time Ancillary Services Capacity Settlement Charge Codes are a family of Charge Codes for payment to Scheduling Coordinators (SCs) for Awarded Ancillary Services Capacity bids: (1) Regulation Up, (2) Regulation Down, (3) Spinning Reserve, and (4) Non-Spinning Reserve.

The fundamental concepts of the settlement methodology for allocation of AS procurement cost to scheduling coordinators are as follows:

- The AS procurement cost allocation for all AS commodity types is hourly, system-wide, and across the IFM and Real-Time Markets

- The cost of procuring the AS by the CAISO on behalf of the demand will be allocated to the demand using a system-wide user rate.  The user rate is the average cost of procuring a type of AS in both the forward and Real-Time Market for the whole CAISO system

- The rate for each AS incorporates the No Pay/Non Compliance Capacity and the No Pay/Non Compliance Charge to reflect the ultimate average AS cost

- The rate for each AS reflects an average AS substitution to capture the cascaded AS procurement as it is performed optimally in each AS market.  For example, Settlements reflects that multiple service types are procured and substituted simultaneously during the IFM optimization

- The difference between total net AS Requirements and total AS Obligations results in a neutrality adjustment for each Scheduling Coordinator for each of the Regulation Up, Regulation Down, Spinning Reserve, and Non-Spinning Reserve AS types.

- The difference between total AS Procurement and total AS Requirements over all Spinning, Non-Spinning and Regulation Up Ancillary Services results in a single Upward neutrality adjustment for all these services.

- Ancillary Services awards from Intertie Resources are charged explicitly for the Marginal Cost of Congestion on the relevant inter-tie interface at the relevant Shadow Price.  The cost of AS Congestion Charges is not recovered through the AS cost allocation, but is settled in the RT Congestion Offset, CC 6774.

By design, the AS Settlement methodology has the following property: If the total AS Procurement matches the total AS Requirements, and if the AS Requirement matches the total AS Obligation for each AS, the AS Cost Allocation is neutral.

By reflecting AS substitution in the AS Rates, this AS settlement methodology eliminates any neutrality loss due to AS substitution and results in an equitable AS Cost Allocation to Scheduling Coordinators’ that Self-Provide AS, since there is no AS substitution among Self-Provided AS.

This Charge Code deals with Non-Spinning Reserve Obligation Settlement.

## Description

The Non-Spinning Reserve Obligation Settlement charges Scheduling Coordinators for the cost of its Non-Spinning Reserve Obligation that was not self-provided by the Scheduling Coordinator in the Day Ahead and Real-Time markets.

The Settlement System calculates Non-Spinning Reserve Obligation charge amount by hour by Scheduling Coordinator.  Charges are calculated as the product of the calculated Non-Spinning Reserve rate and the Net Non-Spinning Reserve Obligation.  The Net Non-Spinning Reserve Obligation is calculated as the difference between the Non-Spinning Reserve Obligation and the Effective Qualified Self-Provision.  The Non-Spinning Reserve rate is calculated based on the cost of Non-Spinning reserve procured to meet the Non-Spinning reserve requirements and cost of any Regulation Up substitution and Spinning Reserve Substitution procured to meet Non-Spinning Reserve requirements.

# Charge Code Requirements

## Business Rules

| Bus Req ID | Business Rule |
| --- | --- |
| 1.0 | The Non-Spinning Reserve Obligation charge amount shall be calculated by hour by Scheduling Coordinator. |
| 2.0 | Non-Spinning Reserve Obligation charges shall be calculated as the product of Non-Spinning Reserve rate and the Non-Spinning Reserve Net Obligation. |
| 2.1 | The Non-Spinning Reserve Net Obligation shall be calculated as the difference between the Non-Spinning Reserve Obligation and the Effective Qualified Self-Provision where the Effective Qualified Self-Provision does not exceed Non-Spinning Reserve Obligation. |
| 2.2 | The Non-Spinning Reserve rate shall be calculated as the ratio of the cost of Non-Spinning Reserve procured to meet the Non-Spinning Reserve requirements and cost of any Regulation Up substitution and Spinning Reserve substitution procured to meet Non-Spinning Reserve requirements over Non-Spinning Reserve procured to meet the Non-Spinning Reserve requirements and any Regulation Up substitution and Spinning Reserve substitution procured to meet Non-Spinning Reserve requirements |
| 2.3 | The CAISO Hourly Total Non-Spinning Reserve Procurement cost shall be calculated as sum of the Non-Spinning Reserve Procurement costs in Day Ahead and Real-Time markets, as well as the Non-Compliance Costs associated with these markets. |
| 3.0 | For adjustments to the Charge Code that cannot be accomplished by correction of upstream data inputs, recalculation or operator override Pass Through Bill (PTB) Charge logic will be applied. |

## Predecessor Charge Codes

Charge Code/ Pre-calc Name

Ancillary Services Pre-calculation

Day Ahead Non-Spinning Reserve Capacity Settlement (CC 6200)

Real Time Non-Spinning Reserve Capacity Settlement (CC 6270)

No Pay Non-Spinning Reserve Settlement (CC 6224)

Regulation Up Obligation Settlement (CC 6594)

Spinning Reserve Obligation Settlement (CC 6194)

## Successor Charge Codes

Charge Code/ Pre-calc Name

Upward Ancillary Services Neutrality Allocation (CC 6090)

Non-Spinning Reserve Neutrality Allocation (CC 6296)

## Inputs – External Systems

| Input Req ID | Variable Name | Description |
| --- | --- | --- |
| 1 | PTBChargeAdjustmentObligationNonSpin BJmdh | Non-Spinning Reserve Obligation PTB Charge Adjustment Amount (in $) for a given Business Associate and Trading Hour. |

## Inputs - Predecessor Charge Codes or Pre-calculations

| Input Req ID | Variable Name | Predecessor Charge Code/ Pre-calc Configuration |
| --- | --- | --- |
| 1 | CAISOHourlyTotalSpinNetProc mdh | Ancillary Services Pre-calculation |
| 2 | BAHourlyTotalNonSpinEQSP Bmdh | Ancillary Services Pre-calculation |
| 3 | ScaledHourlyTotalSpinNetReq mdh | Ancillary Services Pre-calculation |
| 4 | CAISOHourlyTotalRegUpNetProc mdh | Ancillary Services Pre-calculation |
| 5 | ScaledHourlyTotalRegUpNetReq mdh | Ancillary Services Pre-calculation |
| 6 | NonSpinObligMW Bmdh | Ancillary Services Pre-calculation |
| 7 | CAISOHourlyTotalNonSpinNetProc mdh | Ancillary Services Pre-calculation |
| 8 | CAISOHourlyTotalDANonSpinSettlementAmount mdh | Day Ahead Non-Spinning Reserve Capacity Settlement (CC 6200) |
| 9 | CAISOHourlyTotalRTNonSpinSettlementAmount mdh | Real Time Non-Spinning Reserve Capacity Settlement (CC 6270) |
| 10 | CAISOHourlyTotalNoPayNonSpinSettlementAmount mdh | No Pay Non-Spinning Reserve Settlement (CC 6224) |
| 11 | RegUpRate mdh | Regulation Up Obligation Settlement (CC 6594) |
| 12 | Not Used |  |
| 13 | SpinRateSpin mdh | Spinning Reserve Obligation Settlement (CC 6194) |

## CAISO Formula

### NonSpinObligAmount Bmdh

NonSpinObligAmount Bmdh  = NonSpinObligQuantity Bmdh * NonSpinRate mdh

      - Where

- NonSpinObligQuantity Bmdh = Min(NonSpinObligMW Bmdh, Max(0, NonSpinObligMW Bmdh  - BAHourlyTotalNonSpinEQSP Bmdh))

      - And where NonSpinRate mdh  is defined via

- IF   NonSpinCascadeProc mdh    >  0

- THEN

NonSpinRate mdh  =  ( RegUpRate mdh  * RegUpSubsNonSpinProc mdh  +

SpinRateSpin mdh  * SpinSubsNonSpinProc mdh + CAISOHourlyTotalNonSpinCost mdh)  /  NonSpinCascadeProc mdh

- Else

NonSpinRate mdh  = 0

        - Where  NonSpinCascadeProc  mdh  is defined as

NonSpinCascadeProc mdh    =  RegUpSubsNonSpinProc mdh  +  SpinSubsNonSpinProc mdh  +  CAISOHourlyTotalNonSpinNetProc mdh

        - In which RegUpSubsNonSpinProc  mdh  is defined as

RegUpSubsNonSpinProc mdh = Max (0, CAISOHourlyTotalRegUpNetProc mdh – ScaledHourlyTotalRegUpNetReq mdh – ScaledHourlyTotalSpinNetReq mdh )

        - And SpinSubsNonSpinProc mdh  is defined as

SpinSubsNonSpinProc mdh  = Max (0,  CAISOHourlyTotalSpinNetProc mdh  - Max (0, ScaledHourlyTotalSpinNetReq  mdh  - Max ( 0, CAISOHourlyTotalRegUpNetProc mdh  - ScaledHourlyTotalRegUpNetReq mdh) ) )

      - And where CAISOHourlyTotalNonSpinCost mdh  is defined as

CAISOHourlyTotalNonSpinCost mdh  = (-1) * (CAISOHourlyTotalDANonSpinSettlementAmount mdh  + CAISOHourlyTotalRTNonSpinSettlementAmount mdh   + CAISOHourlyTotalNoPayNonSpinSettlementAmount mdh  )

NOTE: CAISOHourlyTotalNonSpinObligSettlementAmount mdh is calculated as part of the reporting structure and will not be configured as an individual charge type.

## Outputs

.

| Output Req ID | Name | Description |
| --- | --- | --- |
|  | In addition to the outputs listed below, all inputs shall be included as outputs. |  |
| 1 | NonSpinObligAmount Bmdh | Non-Spinning Reserve Obligation charge amount (in $) due ISO for a given Business Associate and Trading Hour. |
| 2 | NonSpinObligQuantity Bmdh | Total Non-Spinning Obligation Quantity (in MW) for a given Business Associate and Trading Hour. |
| 3 | NonSpinRate mdh | Non-Spinning Reserve charge rate (in $/MW) for a given Trading Day. |
| 4 | NonSpinCascadeProc mdh | Non-Spinning Reserve (in MW) cascaded procurement for a given Trading Hour. |
| 5 | RegUpSubsNonSpinProc mdh | Regulation Up capacity (in MW) substituted for Non-Spinning Reserve procurement for a given Trading Hour. |
| 6 | SpinSubsNonSpinProc mdh | Spinning Reserve Substitution (in MW) procured for Non-Spinning Reserve for a given Trading Hour. |
| 7 | CAISOHourlyTotalNonSpinCost mdh | CAISO Total Spinning Reserve cost (in $) for a given Trading Hour. |

# Charge Code Effective Date

| Charge Code/ Pre-calc Name | Document Version | Effective Start Date | Effective End Date | Version Update Type |
| --- | --- | --- | --- | --- |
| Non-Spinning Reserve Obligation Settlement (CC 6294) | 5.0 | 04/01/09 | 4/30/14 | Documentation Edits Only |
| Non-Spinning Reserve Obligation Settlement (CC 6294) | 5.1 | 5/1/14 | 9/30/14 | Configuration Impacted |
| Non-Spinning Reserve Obligation Settlement (CC 6294) | 5.2 | 10/1/14 | 10/31/18 | Configuration Impacted |
| Non-Spinning Reserve Obligation Settlement (CC 6294) | 5.2a | 11/1/18 | Open | Documentation Edits Only |