Settlements and Billling

Configuration Guide: CG CC 6296 Non Spinning Reserve Neutrality Allocation v5.0b

CC 6296

Version 5.0b

Table of Contents

1.	Purpose of Document	3

2.	Introduction	3

2.1	Background	3

2.2	Description	4

3.	Charge Code Requirements	4

3.1	Business Rules	4

3.2	Predecessor Charge Codes	5

3.3	Successor Charge Codes	5

3.4	Inputs – External Systems	5

3.5	Inputs - Predecessor Charge Codes or Outputs of Pre-calculations	5

3.6	CAISO Formula	6

3.7	Outputs	6

4.	Charge Code Effective Date	7

# Purpose of Document

The purpose of this document is to capture the business and functional requirements for the MRTU SaMC Non-Spinning Reserve Neutrality Allocation, Charge Code 6296.

# Introduction

## Background

The CAISO will procure the Ancillary Services, Regulation Up, Regulation Down, Spinning Reserve, and Non-Spinning Reserve in the Day Ahead Integrated Forward Market (IFM) and procure incrementally as needed in the Real-Time Market (RTM).  Ancillary Services (AS) are procured simultaneously with Energy bids to meet Regulation and Operating Reserve requirements, using submitted Ancillary Service bids.  IFM is performed for each hour of the next Trading Day.  The Fifteen Minute Market performs AS procurement, if needed, at 15-minutes intervals for the current hour and next Trading Hour.  The AS awards published for the first 15-minutes interval of the time horizon are binding, the rest are advisory.  The AS pricing and Settlement will be based on Ancillary Service Marginal Price (ASMP), which are calculated for each AS region for each market time interval for each market.

The AS procurement cost is the payment for AS Awarded bids in the Day Ahead IFM and RTM.  This Charge Code is part of the family of Charge Codes for payment to Scheduling Coordinators (SCs) for Awarded Ancillary Services Capacity bids: (1) Regulation Up, (2) Regulation Down, (3) Spinning Reserve, and (4) Non-Spinning Reserve.

The fundamental concepts of Settlement methodology for allocation of AS procurement cost to scheduling coordinators are as follows:

- The AS procurement cost allocation for all AS commodity types is hourly, system-wide, and across IFM and Real-Time markets

- The cost of procuring the AS by the CAISO on behalf of the demand will be allocated to the demand using a system wide user rate.  The user rate is the average cost of procuring a type of AS in both the Forward and Real-Time Market for the whole CAISO system

- The rate for each AS incorporates the No Pay/Non Compliance Capacity and the No Pay/Non Compliance Charge to reflect the ultimate average AS cost

- The rate for each AS reflects an average AS substitution to capture the cascaded AS procurement as it is performed optimally in each AS market.  For example, Settlements reflects that multiple service types are procured and substituted simultaneously during IFM optimization

- A difference between total net AS Requirements and total AS Obligations results in a neutrality adjustment for each Scheduling Coordinator for each of the Regulation Up, Regulation Down, Spinning Reserve, and Non-Spinning Reserve AS types.

- The difference between total AS Procurement and total AS Requirements over all Spinning, Non-Spinning and Regulation Up Ancillary Services results in a single Upward neutrality adjustment for all these services.

- Ancillary Services awards from Intertie Resources are charged explicitly for the Marginal Cost of Congestion on the relevant inter-tie interface at the relevant Shadow Price.  The cost of AS Congestion Charges is not recovered through the AS cost allocation, but is settled in the RT Congestion Offset, CC 6774.

By design, the AS settlement methodology has the following property: If the total AS Procurement matches the total AS Requirements, and if the AS Requirement matches the total AS Obligation for each AS, the AS Cost Allocation is neutral.

By reflecting AS substitution in the AS Rates, this AS Settlement methodology eliminates any neutrality loss due to AS substitution and results in an equitable AS Cost Allocation to Scheduling Coordinators’ that self-provide AS, since there is no AS substitution among self-provided AS.

This Charge Code deals with Non-Spinning Reserve Neutrality Allocation.

## Description

The Non-Spinning Reserve Neutrality Allocation recovers from all Scheduling Coordinators the total Non-Spinning Reserve Neutrality amount, in proportion to their positive Non-Spinning reserve Obligation.  The total Non-Spinning Reserve Neutrality amount is calculated as the difference between the Non-Spinning reserve Net Requirement at the Non-Spinning reserve rate and the total revenue from the Non-Spinning reserve charge to all the Scheduling Coordinators.

# Charge Code Requirements

## Business Rules

| Bus Req ID | Business Rule |
| --- | --- |
| 1.0 | The Non-Spinning Reserve Neutrality Allocation amount shall be calculated by hour by Scheduling Coordinator. |
| 1.1 | For each Scheduling Coordinator, the Non-Spinning Reserve Neutrality Allocation amount shall be calculated by allocating the total Non-Spinning reserve Neutrality amount between Scheduling Coordinators in proportion to the positive Non-Spinning reserve Obligation for each Scheduling Coordinator. |
| 1.2 | The total Non-Spinning Reserve Neutrality amount shall be calculated as the difference between the Non-Spinning reserve Net Requirement at the Non-Spinning reserve rate and the total revenue from the Non-Spinning reserve charge to all the Scheduling Coordinators. |
| 2.0 | The PTB logic will not be directly applied in CC6296. Instead, PTB amounts will be included in and propagated from their predecessor Charge Codes. |

## Predecessor Charge Codes

Charge Code/ Pre-calc Name

Ancillary Services Pre-calculation

Non-Spinning Reserve Obligation Settlement (CC 6294)

## Successor Charge Codes

Charge Code/ Pre-calc Name

Ancillary Service Upward Neutrality Allocation (CC 6090)

## Inputs – External Systems

| Row # | Variable Name | Description |
| --- | --- | --- |
| 1 | TotalRTNonSpinReq mdh | Total Real-Time Non-Spinning Reserve Requirement for Trading Day d and Trading Hour h for the CAISO. (MW) |

## Inputs - Predecessor Charge Codes or Outputs of Pre-calculations

| Row # | Variable Name | Predecessor Charge Code/ Pre-calc Configuration |
| --- | --- | --- |
| 1 | CAISOHourlyTotalNonSpinEQSP mdh | Ancillary Services Pre-calculation |
| 2 | NonSpinObligNoTradeMW Bmdh | Ancillary Services Pre-calculation |
| 3 | NonSpinRate mdh | Non-Spinning Reserve Obligation Settlement (CC 6294) |

## CAISO Formula

### NonSpinNeutralityAmount Bmdh

NonSpinNeutralityAmount Bmdh  =

CAISOHourlyTotalNonSpinNeutralityAmount mdh  *

Max (0, NonSpinObligNoTradeMW Bmdh) / CAISOHourlyTotalPosNonSpinObligNoTradeQty mdh

      - Where CAISOHourlyTotalNonSpinNeutralityAmount mdh  =

NonSpinRate mdh * [Max (0, TotalRTNonSpinReq mdh – CAISOHourlyTotalNonSpinEQSP mdh)

–  (CAISOHourlyNonSpinObligNoTradeMW mdh - CAISOHourlyTotalNonSpinEQSP mdh )]

      - Where CAISOHourlyTotalPosNonSpinObligNoTradeQty mdh =

BMax (0, NonSpinObligNoTradeMW Bmdh )

      - Where CAISOHourlyNonSpinObligNoTradeMW mdh =

BNonSpinObligNoTradeMW Bmdh

NOTE: CAISOHourlyTotalNonSpinNeutralitySettlementAmount mdh is calculated as part of the reporting structure and will not be configured as an individual charge type.

Note: The above changes reflect what is in Configuration. New output variables were simply created to contain existing formulas.

## Outputs

.

| Row # | Name | Description |
| --- | --- | --- |
|  | In addition, all inputs are required to be accessible for review by analysts and report on Settlement Statements. | N/A |
| 1 | NonSpinNeutralityAmount Bmdh | Non-Spinning Reserve Neutrality amount due ISO for Business Associate B for Trading Day d and Trading Hour h ($) |
| 2 | CAISOHourlyTotalNonSpinNeutralityAmount mdh | CAISO Hourly total Non-Spinning Reserve Neutrality amount for Trading Day d and Trading Hour h ($) Intermediate computation value required to be accessible for review by analysts |
| 3 | CAISOHourlyTotalPosNonSpinObligNoTradeQty mdh | CAISO Total Positive Non-Spinning Obligation No Trade Quantity for Trading Hour h |
| 4 | CAISOHourlyNonSpinObligNoTradeMW mdh | CAISO Total Non-Spinning Obligation No Trade Quantity for Trading hour h. |

# Charge Code Effective Date

| Charge Code/ Pre-calc Name | Document Version | Effective Start Date | Effective End Date | Version Update Type |
| --- | --- | --- | --- | --- |
| Non-Spinning Reserve Neutrality Allocation (CC 6296) | 5.0 | 04/01/09 | 4/30/14 | Documentation Edits Only |
| Non-Spinning Reserve Neutrality Allocation (CC 6296) | 5.0a | 5/1/14 | 10/31/18 | Documentation Edits Only |
| Non-Spinning Reserve Neutrality Allocation (CC 6296) | 5.0b | 11/1/18 | Open | Documentation Edits Only |