Settlements & Billing

Configuration Guide: CG CC 6460 FMM Instructed Imbalance Energy Settlement v5.7

CC 6460

Version 5.7

Table of Contents

1.	Purpose of Document	3

2.	Introduction	3

2.1	Background	3

2.2	Description	3

3.	Charge Code Requirements	3

3.1	Business Rules	3

3.2	Predecessor Charge Codes	7

3.3	Successor Charge Codes	7

3.4	Inputs – External Systems	7

3.5	Inputs - Predecessor Charge Codes or Pre-calculations	8

3.6	CAISO Formula	9

3.7	Outputs	17

4.	Charge Code Effective Dates	19

# Purpose of Document

The purpose of this document is to capture the requirements and design specification for a Charge Code in one document.

# Introduction

## Background

The CAISO calculates and accounts for Imbalance Energy for each Dispatch Interval and settles Imbalance Energy for each Settlement Interval for each resource within the CAISO Control Area and all System Resources Dispatched in Real-Time.

Imbalance Energy consists of following:

- IIE –  Instructed Imbalance Energy

  - FMM Instructed Imbalance Energy Settlement (CC 6460)

  - RTD Instructed Imbalance Energy Settlement (CC 6470)

- UIE – Real Time Uninstructed Imbalance Energy Settlement (CC 6475)

- UFE – Real Time Unaccounted for Energy Settlement (CC 6474)

To the extent that the sum of the Settlement Amounts for IIE, UIE, and UFE does not equal zero, the CAISO will assess Charges or make Payments in Real Time Imbalance Energy Offset (CC 6477) for the resulting differences to all Scheduling Coordinators based on a pro rata share of their Measured Demand for the relevant Settlement Interval.

In the Real-Time Market, the negative and positive Congestion Charges associated with a valid post-Day-Ahead TOR and ETC schedule change (including changes submitted to the Fifteen Minute Market  and changes submitted closer to Real-Time where allowed by the contract) will be reversed in CC 6774 RT Congestion Offset.  Because Congestion Charges are implicitly collected by the CAISO in the Real-Time settlement and there are no holders of rights to receive Real-Time Congestion revenues, all charges for Real-Time Congestion will be accumulated in a special and separate neutrality account to be distributed back to non-ETC Control Area metered Demand and exports on a per-MWh basis in Real Time Congestion Offset (CC 6774).

## Description

CC 6460 FMM Instructed Imbalance Energy Settlement will perform the calculations necessary to implement the business rules identified in the Business Rules section below.

# Charge Code Requirements

## Business Rules

| Bus Req ID | Business Rule |
| --- | --- |
| 1.0 | For each Settlement Interval, FMM IIE consists of the following types of Energy: (1) FMM Optimal Energy; (2) FMM Minimum Load Energy; (3) FMM Exceptional Dispatch Energy; (4) FMM Derate Energy; and (5) FMM Pumping Energy. This applies to non-Load resources. |
| 1.1 | A positive Energy value indicates Incremental Energy. |
| 1.2 | A negative Energy value indicates Decremental Energy. |
| 1.3 | This Charge Code shall be calculated daily on a Settlement Interval basis. |
| 1.4 | Payments and charges for FMM IIE attributable to each resource in each Settlement Interval shall be settled by debiting or crediting, as appropriate, the specific Scheduling Coordinator’s FMM IIE Settlement Amount. |
| 2.0 | The IIE Settlement Amounts for FMM Optimal Energy, FMM Minimum Load Energy, FMM Derate Energy, and FMM Pumping Energy, and shall be calculated as the product of the sum of all of these types of Energy and the FMM LMP. |
| 2.1 | For MSS Operators that have elected gross Settlement, regardless of whether that entity has elected to follow its Load or to participate in RUC, the FMM IIE for such entities is settled similarly to non-MSS entities as described in a previous business rule above. |
| 2.2 | For MSS Operators that have elected net Settlement, the FMM IIE Settlement Amounts for Energy dispatched through the FMM, FMM Minimum Load Energy from System Units dispatched in FMM, FMM Derate Energy, and FMM Pumping Energy shall be calculated as the product of the sum of all of these types of Energy and the FMM MSS Price. |
| 2.2.1 | The FMM MSS Price for an MSS which elected Net settlement is the FMM Interval Real Time MSS Price which is - the Hourly Interval Real-Time Market LAP price for the MSS LAP, if the MSS internal metered Demand exceeds the MSS internal measured Generation; or - the weighted average of the FMM Interval LMPs for all applicable Pnodes, PODs, or AGENs within the relevant MSS; where the weighting factors for computing the weighted average are the Metered Energy of all Generation at the corresponding Pnodes, if MSS internal metered Generation exceeds MSS internal metered Demand. |
| 3.0 | The remaining FMM IIE Settlement Amounts for Exceptional Dispatches are settled pursuant to Section 11.5.6, and further described in following business rules. |
| 3.1 | Settlement Amount for each non-MSS and MSS resource regardless of any MSS elections for each Settlement Interval for Exceptional Dispatch Incremental/ Decremental Energy shall be calculated as the sum of the product of the FMM Exceptional Dispatch Incremental/Decremental Energy and the relevant price. |
| 3.2 | The Exceptional Dispatch IIE Price or emergency Energy price for Exceptional Dispatch or emergency Energy Incremental or Decremental IIE with Exceptional Type of SYSEMR, TEMR, Tmodel, or NonTModel is the higher of the resource’s Resource-Specific FMM LMP, Energy Bid Price or, if applicable, the Default Energy Bid price for Energy that does not have an Energy Bid Price, or, as applicable to System Resources providing emergency Energy, the pre-established or negotiated price as recorded by the CAISO operator at the time of Dispatch. |
| 3.2.1 | For resource who have declined an CPM Designation for Supplemental Revenue assessment, the FMM Exceptional Dispatch IIE Price or emergency Energy price for FMM Exceptional Dispatch for emergency Energy Incremental IIE with Exceptional Type of SYSEMR, TEMR, TModel, or Non-TModel during the Supplemental Revenue designation period where resources supplemental Revenue does not exceed relevant CPM amount is the higher of the resource’s FMM LMP or Energy Bid Price or, if applicable, the Default Energy Bid price for Energy that does not have an Energy Bid Price, or, as applicable to System Resources providing emergency Energy, the pre-established or negotiated price as recorded by the CAISO operator at the time of Dispatch. |
| 3.2.2 | For resource who have declined an CPM Designation for Supplemental Revenue assessment, the FMM Exceptional Dispatch IIE Price or emergency Energy price for FMM Exceptional Dispatch for emergency Energy Incremental IIE with Exceptional Type of SYSEMR, TEMR, TModel, or Non-TModel during the Supplemental Revenue designation period where resources supplemental Revenue exceeds relevant CPM amount is the higher of the resource’s FMM LMP or the Default Energy Bid price for Energy. |
| 3.3 | The Exceptional Dispatch IIE Price for Exceptional Dispatch Incremental or Decremental IIE with Exceptional Dispatch type of ASTEST or TEST is the resource’s FMM LMP except in the case when the resource has a Bid (that is higher than the FMM LMP for incremental IIE or lower than the FMM LMP for decremental IIE). |
| 3.4 | The Exceptional Dispatch Incremental IIE Price for RMRS, RMR and RMR Condition 1 and 2 units is a Resource-Specific FMM LMP and the difference between Contract Price and Resource-Specific FMM LMP is paid on the RMR Invoice |
| 3.5 | The Exceptional Dispatch Incremental or Decremental IIE Price for RMR Condition 2 units is the Contract Price on the RMR Invoice |
| 5.0 | The CAISO will take the following actions (through this charge code) regarding Schedules that clear the Day-Ahead Market (or the RUC Schedule if lower than DA Schedule) at the Interties and that a Scheduling Coordinator wholly or partially reverses prior to HASP solution availability. This is identified as the HASP reversal settlement rule and it applies to any import or export that clear the Day-Ahead Market (or the RUC Schedule if lower than DA Schedule) and is reduced prior to the HASP solution availability for which the Scheduling Coordinator has failed to submit an E-Tag consistent with the Scheduling Coordinator’s Day-Ahead Schedule (or the RUC Schedule if lower than DA Schedule). |
| 5.1 | For both imports and exports, the HASP reversal settlement rule shall not apply to Schedules that clear the Day-Ahead Market (or the RUC Schedule if lower than DA Schedule) at the Interties and that a Scheduling Coordinator wholly or partially reverses in the HASP to the extent such Schedules are valid and balanced ETC and TOR Self-Schedules in the Day-Ahead Market. |
| 5.2 | The quantity of any imports or exports that clear the Day-Ahead Market (or the RUC Schedule if lower than DA Schedule) that are reduced in the HASP for which the Scheduling Coordinator has failed to submit an E-Tag consistent with the Scheduling Coordinator’s Day-Ahead Schedule (or the RUC Schedule if lower than DA Schedule) and WECC scheduling requirements is the “un-tagged” MW, with the exception identified in the previous business rule. |
| 5.3 | For Import, the “un-tagged” MW up to FMM Scheduled Energy quantity will be charged the positive price difference between the Day Ahead LMP and the FMM LMP for the import resource. |
| 5.4 | For Export, the “un-tagged” MW up to FMM Scheduled Energy quantity will be charged the positive price difference between the FMM LMP and the Day Ahead LMP for the export resource. |
| 5.5 | The amount from the HASP reversal settlement rule shall always be a charge to an SC, never a payment to an SC. |
| 5.6 | The tagged MW value per resource ID as of the time when HASP solution is available must be captured for implementation of the HASP reversal settlement rule. |
| 5.7 | HASP reversal settlement shall not apply to Pseudo Tie Dynamic resources as these are not considered imports. There are no intertie locations associated with these resources. These resources install ISO meter. |
| 6.0 | Import schedule which has been deemed to violate Scheduling Sourcing/Sinking in Same Balancing Authority Area provision will be settled at the lower of relevant LMP of the import Scheduling Point and the relevant LMP of the associated export Scheduling Point. |
| 6.1 | The difference in Settlement between the relevant LMP of the import Scheduling Point and the relevant LMP of the associated export Scheduling Point will be assessed as part of Pass Thru Charge Adjustment Mechanism |
| 7.0 | For adjustments to the Charge Code that cannot be accomplished by correction of upstream data inputs/recalculation or operator override Pass Through Bill Charge logic will be applied. |

## Predecessor Charge Codes

Charge Code/ Pre-calc Name

Real Time Energy Pre-calculation

Real Time Price Pre-calculation

CC 6011 – Day-Ahead Energy, Congestion, Loss Settlement

Pre-calculation – Metered Demand TAC Area and CPM

## Successor Charge Codes

Charge Code/ Pre-calc Name

CC 6477 – Real Time Imbalance Energy Offset

CC 6774 – Real Time Congestion Offset

CC 6788 – Real Time Congestion Credit Settlement

## Inputs – External Systems

| Row # | Variable Name | Description |
| --- | --- | --- |
|  | FMMExceptionalDispatchIIE BrtuT’ObI’Q’M’AA’R’W’F’S’PVL’mdhcif | IIE due to an FMM Exceptional Dispatch (MWh) |
|  | FMMExceptionalDispatchIIEPrice BrtObmdhcif | Price for Exceptional Dispatch IIE ($/MWh) one of the following: (1) Bid, (2) the Default Energy Bid, (3) negotiated price, or (4) calculated price. |
|  | BAHourlyResourceCASTaggedDAEnergyMW Brtmdh | Tagged MW for DA Energy for system resource as of the time when HASP solution is available to CAISO check out system Data will be positive for ITIEs and ETIEs. |
|  | PTBChargeAdjustmentBA5MFMMEnergyAmt BJmdhcif | PTB settlement adjustment amount for this Charge Code |
|  | ResourceRUCCapacityTotalIncludingDayAheadSchedule BrtT’uI’M’R’W’F’S’VL’mdh | Resource RUC Capacity Total Including Day Ahead Schedule Data will be positive for ITIEs and ETIEs |
|  | BASettlementIntervalResourceFMMExceptionalDispatchDEBQty BrtuT’ObI’AA’Q’M’R’W’F’S’VL’Pmdhcif | Exceptional Dispatch Energy (in MWh) dispatched through FMM for the specified DEB segment and Settlement Interval. |
|  | BASettlementIntervalResourceFMMExceptionalDispatchDEBPrc BrtObmdhcif | Bid price (in $ / MWh) of FMM Exceptional Dispatch IIE Energy dispatched in the RTM for the specified DEB bid segment and Settlement Interval. |
|  | BADayResourcePseudoTieDynamicFlag Brtmd | A flag with a value of 1 created only for pseudo tie dynamic resources. |

## Inputs - Predecessor Charge Codes or Pre-calculations

| Row # | Variable Name | Predecessor Charge Code/ Pre-calc Configuration |
| --- | --- | --- |
|  | SettlementIntervalTotalFMMPart1Qty BrtuT’I’Q’M’F’S’mdhcif | RT Energy Pre-calculation |
|  | FMMIntervalMSSPrice uM’mdhc | RT Price Pre-Calculation |
|  | HourlyDASchedule Brtmdh | CC 6011 – Day-Ahead Energy, Congestion, Loss Settlement Data will be positive for ITIEs and negative for ETIEs |
|  | HourlyDAEnergyResourceLMP rtmdh | CC 6011 – Day-Ahead Energy, Congestion, Loss Settlement |
|  | BAHourlyResourceDABalancedTotalContractUsage Brtmdh | CC 6011 – Day-Ahead Energy, Congestion, Loss Settlement Data will be positive for ITIEs and negative for ETIEs |
|  | FMMIntervalLMPPrice BrtuM’mdhc | RT Price Pre-Calculation |
|  | BASettlementIntervalResourceSurplusSupplementalRevenueFlag Brtmdhcif | Pre-calculation – Metered Demand TAC Area and CPM |

## CAISO Formula

    - BA5MResourceFMMIIESettlementAmount BrtuT’I’M’F’S’mdhcif =

BA5MResourceFMMIIEAssessmentAmount BrtuT’I’M’F’S’mdhcif

+ SettlementIntervalFMMEDEIncAmount BrtuT’I’M’F’S’mdhcif

+ SettlementIntervalFMMEDEDecAmount BrtuT’I’M’F’S’mdhcif

+ BAHourlyResourceImportHASPReversalAmount BrtuT’I’M’F’S’mdh /12

+ BAHourlyResourceExportHASPReversalAmount BrtuT’I’M’F’S’mdh  /12

    - BASettlementIntervalFMMEnergyPrice BrtuT’I’Q’M’F’S’mdhcif  =

IF

SettlementIntervalTotalFMMPart1Qty BrtuT’I’Q’M’F’S’mdhcif  attribute (T’) = MSS and attribute (I’) = Net Settlement Election

THEN

BASettlementIntervalFMMEnergyPrice BrtuT’I’Q’M’F’S’mdhcif =

FMMIntervalMSSPrice uM’mdhc

ELSE

BASettlementIntervalFMMEnergyPrice BrtuT’I’Q’M’F’S’mdhcif =

FMMIntervalLMPPrice BrtuM’mdhc

Note: The SettlementIntervalTotalFMMPart1Qty BrtuT’I’Q’M’F’S’mdhcif is the business driver and its attributes will be inherited by the output charge type. Both of the 15-minute inputs will be replicated in each corresponding three 5-minute intervals of the output charge type.

    - BA5MResourceFMMIIEAssessmentAmount BrtuT’I’M’F’S’mdhcif  =

Q ' ( (-1) * BASettlementIntervalFMMEnergyPrice BrtuT’I’Q’M’F’S’mdhcif *

SettlementIntervalTotalFMMPart1Qty BrtuT’I’Q’M’F’S’mdhcif )

Where Q’ = ‘CISO’

    - BASettlementIntervalFMMIIEAmount Bmdhcif  =

rtuT 'I 'M 'F 'S ' BA5MResourceFMMIIESettlementAmount BrtuT’I’M’F’S’mdhcif

Note: This is provided as part of reporting structure and is not configured as an individual charge type. This is shown as a reporting BD in the BD matrix file.

    - CAISOSettlementIntervalTotalFMMIIEAmount mdhcif  =

B BASettlementIntervalFMMIIEAmount Bmdhcif

Note: This is provided as part of reporting structure and is not configured as an individual charge type. This is shown as a reporting BD in the BD matrix file.

FMM Exceptional Dispatch calculations:

    - SettlementIntervalFMMEDE1IncAmount BrtOuT’I’M’F’S’mdhcif =

Where

FMMExceptionalDispatchIIE BrtuT’ObI’Q’M’AA’R’W’F’S’PVL’mdhcif attribute Exceptional Dispatch Type (O) is in  (SYSEMR, SYSEMR1, TEMR, TMODEL, TMODEL1, TMODEL2, TMODEL3, TMODEL4, TMODEL5, TMODEL6, TMODEL7,TORETC, TORETC1, RMRR, RMRS, RMRT, SLIC, and OTHER)

SettlementIntervalFMMEDE1IncAmount BrtOuT’I’M’F’S’mdhcif =

R'bW'AA'VL'PQ ' (-1) * (Max(FMMExceptionalDispatchIIE BrtuT’ObI’Q’M’AA’R’W’F’S’PVL’mdhcif, 0) *  FMMIntervalLMPPrice BrtuM’mdhc

NOTE: For implementation purpose the following Exceptional Dispatch Types will be excluded: Exceptional Dispatch Type O NOT in (NONTMOD, ASTEST, TEST, BS, VS, RMRRC2)

    - SettlementIntervalFMMEDE2IncAmount BrtOuT’I’M’F’S’mdhcif =

Where

FMMExceptionalDispatchIIE BrtuT’ObI’Q’M’AA’R’W’F’S’PVL’mdhcif attribute Exceptional Dispatch Type (O) is in  ( NONTMOD, ASTEST, TEST)

IF

BASettlementIntervalResourceSurplusSupplementalRevenueFlag Brtmdhcif = 1

THEN

SettlementIntervalFMMEDE2IncAmount BrtOuT’I’M’F’S’mdhcif =

b R' W'AA'  V L' PQ ' (-1) * (Max (BASettlementIntervalResourceFMMExceptionalDispatchDEBQty BrtuT’ObI’AA’Q’M’R’W’F’S’VL’Pmdhcif, 0) *  Max (FMMIntervalLMPPrice BrtuM’mdhc , BASettlementIntervalResourceFMMExceptionalDispatchDEBPrc BrtObmdhcif)

ELSE

SettlementIntervalFMMEDE2IncAmount BrtOuT’I’M’F’S’mdhcif =

b R' W'AA'  V L' PQ ' (-1) * (Max(FMMExceptionalDispatchIIE BrtuT’ObI’Q’M’AA’R’W’F’S’PVL’mdhcif  , 0) *  Max (FMMIntervalLMPPrice BrtuM’mdhc, FMMExceptionalDispatchIIEPrice BrtObmdhcif )

    - SettlementIntervalFMMEDE3IncAmount BrtOuT’I’M’F’S’mdhcif =

Where

FMMExceptionalDispatchIIE BrtuT’ObI’Q’M’AA’R’W’F’S’PVL’mdhcif attribute Exceptional Dispatch Type (O) is in  ( RMRRC2)

SettlementIntervalFMMEDE3IncAmount BrtOuT’I’M’F’S’mdhcif =

b R' W'AA'  V L' P Q '

(-1) * (Max(FMMExceptionalDispatchIIE BrtuT’ObI’Q’M’AA’R’W’F’S’PVL’mdhcif  , 0) *

FMMExceptionalDispatchIIEPrice BrtObmdhcif )

    - SettlementIntervalFMMEDEIncAmount BrtuT’I’M’F’S’mdhcif =

O(SettlementIntervalFMMEDE1IncAmount BrtOuT’I’M’F’S’mdhcif  + SettlementIntervalFMMEDE2IncAmount BrtOuT’I’M’F’S’mdhcif +

SettlementIntervalFMMEDE3IncAmount BrtOuT’I’M’F’S’mdhcif )

    - SettlementIntervalFMMEDE1DecAmount BrtOuT’I’M’F’S’mdhcif =

Where

FMMExceptionalDispatchIIE BrtuT’ObI’Q’M’AA’R’W’F’S’PVL’mdhcif attribute Exceptional Dispatch Type (O) is in  (TEMR, TMODEL, TMODEL1, TMODEL2, TMODEL3, TMODEL4, TMODEL5, TMODEL6, TMODEL7, TORETC, TORETC1, RMRR, RMRS, RMRT, SLIC, and OTHER)

SettlementIntervalFMMEDE1DecAmount BrtOuT’I’M’F’S’mdhcif =

bAA'R'W'VL'PQ '(-1) * (Min(FMMExceptionalDispatchIIE BrtuT’ObI’Q’M’AA’R’W’F’S’PVL’mdhcif ,0) *  FMMIntervalLMPPrice BrtuM’mdhc )

NOTE: For implementation purpose the following Exceptional Dispatch Types will be excluded: Exceptional Dispatch Type O NOT in (NONTMOD, ASTEST, TEST,  BS, VS, RMRRC2, SYSEMR, SYSEMR1)

    - SettlementIntervalFMMEDE2DecAmount BrtOuT’I’M’F’S’mdhcif =

Where

FMMExceptionalDispatchIIE BrtuT’ObI’Q’M’AA’R’W’F’S’PVL’mdhcif attribute Exceptional Dispatch Type (O) is in (NONTMOD, ASTEST, TEST, SYSEMR, SYSEMR1)

SettlementIntervalFMMEDE2DecAmount BrtOuT’I’M’F’S’mdhcif =

bAA'R'W'VL'PQ '(-1) *  (Min(FMMExceptionalDispatchIIE BrtuT’ObI’Q’M’AA’R’W’F’S’PVL’mdhcif ,0) *  Min (FMMIntervalLMPPrice BrtuM’mdhc , FMMExceptionalDispatchIIEPrice BrtObmdhcif  ))

    - SettlementIntervalFMMEDE3DecAmount BrtOuT’I’M’F’S’mdhcif =

Where

FMMExceptionalDispatchIIE BrtuT’ObI’Q’M’AA’R’W’F’S’PVL’mdhcif attribute Exceptional Dispatch Type (O) is in (RMRRC2)

SettlementIntervalFMMEDE3DecAmount BrtOuT’I’M’F’S’mdhcif =

bAA'R'W'VL'PQ '(-1) *  (Min(FMMExceptionalDispatchIIE BrtuT’ObI’Q’M’AA’R’W’F’S’PVL’mdhcif ,0) *  FMMExceptionalDispatchIIEPrice BrtObmdhcif  ))

    - SettlementIntervalFMMEDEDecAmount BrtuT’I’M’F’S’mdhcif =

O (SettlementIntervalFMMEDE1DecAmount BrtOuT’I’M’F’S’mdhcif  +

SettlementIntervalFMMEDE2DecAmount BrtOuT’I’M’F’S’mdhcif +

SettlementIntervalFMMEDE3DecAmount BrtOuT’I’M’F’S’mdhcif )

    - BAASettlementIntervalTotalFMMEDEQuantity BrtuT’I’Q’M’F’S’mdhcif =

ObAA'R'W'VL'PFMMExceptionalDispatchIIE BrtuT’ObI’Q’M’AA’R’W’F’S’PVL’mdhcif

    - SettlementIntervalTotalFMMEDEQuantity BrtuT’I’M’F’S’mdhcif =

Q 'BAASettlementIntervalTotalFMMEDEQuantity BrtuT’I’Q’M’F’S’mdhcif

HASP reversal settlement calculations:

    - HourlyTotalHASPPart1Quantity BrtuT’I’M’F’S’mdh =

Q 'cc ∈ fSettlementIntervalTotalFMMPart1Qty BrtuT’I’Q’M’F’S’mdhcif

where resource type (t) in ( “ITIE”, “ETIE”)

and where Q’ = ‘CISO’

    - BAResourceRUCCapacityTotalIncludingDayAheadSchedule Brtmdh =

T 'uI'M'R'W'F'S'VL'ResourceRUCCapacityTotalIncludingDayAheadSchedule BrtT’uI’M’R’W’F’S’VL’mdh

where resource type (t) in ( “ITIE”, “ETIE”)

    - BAHourlyResourceImportHASPUntaggedMW BrtuT’I’M’F’S’mdh

IF

HourlyTotalHASPPart1Quantity BrtuT’I’M’F’S’mdh < 0

THEN

BAHourlyResourceImportHASPUntaggedMW BrtuT’I’M’F’S’mdh =

max(0,(min(HourlyDASchedule Brtmdh , BAResourceRUCCapacityTotalIncludingDayAheadSchedule Brtmdh ) - BAHourlyResourceCASTaggedDAEnergyMW Brtmdh ))

ELSE

BAHourlyResourceImportHASPUntaggedMW BrtuT’I’M’F’S’mdh  = 0

where resource type (t) = “ITIE”

Note: HourlyTotalHASPPart1Quantity BrtuT’I’M’F’S’mdh is the business driver.

    - BAHourlyResourceImportHASPReductionMW BrtuT’I’M’F’S’mdh

IF

HourlyTotalHASPPart1Quantity BrtuT’I’M’F’S’mdh < 0

THEN

BAHourlyResourceImportHASPReductionMW BrtuT’I’M’F’S’mdh =

min (max(0, min(HourlyDASchedule Brtmdh , BAResourceRUCCapacityTotalIncludingDayAheadSchedule Brtmdh ) - BAHourlyResourceDABalancedTotalContractUsage Brtmdh ) , (-1) * HourlyTotalHASPPart1Quantity BrtuT’I’M’F’S’mdh)

ELSE

BAHourlyResourceImportHASPReductionMW BrtuT’I’M’F’S’mdh = 0

where resource type (t) = “ITIE”

Note: HourlyTotalHASPPart1Quantity BrtuT’I’M’F’S’mdh is the business driver.

    - BAHourlyResourceImportHASPReversalMW BrtuT’I’M’F’S’mdh =

- min(BAHourlyResourceImportHASPReductionMW BrtuT’I’M’F’S’mdh, BAHourlyResourceImportHASPUntaggedMW BrtuT’I’M’F’S’mdh)

    - BAFMMIntervalResourceImportHASPReversalPrice BrtuT’I’M’F’S’mdhc  =

max (HourlyDAEnergyResourceLMP rtmdh – FMMIntervalLMPPrice BrtuM’mdhc, 0  )

Note: BAHourlyResourceImportHASPReversalMW BrtuT’I’M’F’S’mdh is the business driver. Hourly value will be replicated in each of the relevant 15-minute intervals.

    - BAHourlyResourceImportHASPReversalAmount BrtuT’I’M’F’S’mdh  =

(1- BADayResourcePseudoTieDynamicFlag Brtmd)*(BAHourlyResourceImportHASPReversalMW BrtuT’I’M’F’S’mdh *

(cBAFMMIntervalResourceImportHASPReversalPrice BrtuT’I’M’F’S’mdhc  /4))

    - BAHourlyResourceExportHASPUntaggedMW BrtuT’I’M’F’S’mdh

IF 	HourlyTotalHASPPart1Quantity BrtuT’I’M’F’S’mdh > 0

THEN

BAHourlyResourceExportHASPUntaggedMW BrtuT’I’M’F’S’mdh =

min(0, max(HourlyDASchedule Brtmdh , (-1)* BAResourceRUCCapacityTotalIncludingDayAheadSchedule Brtmdh )  + BAHourlyResourceCASTaggedDAEnergyMW Brtmdh)

ELSE

BAHourlyResourceExportHASPUntaggedMW BrtuT’I’M’F’S’mdh  = 0

where resource type (t) = “ETIE”

Note: HourlyTotalHASPPart1Quantity BrtuT’I’M’F’S’mdh is the business driver.

    - BAHourlyResExportHASPReductionMW BrtuT’I’M’F’S’mdh

IF 	HourlyTotalHASPPart1Quantity BrtuT’I’M’F’S’mdh > 0

THEN

BAHourlyResExportHASPReductionMW BrtuT’I’M’F’S’mdh =

min ((-1)* min(0, max(HourlyDASchedule Brtmdh , (-1)* BAResourceRUCCapacityTotalIncludingDayAheadSchedule Brtmdh ) - BAHourlyResourceDABalancedTotalContractUsage Brtmdh ) ,

HourlyTotalHASPPart1Quantity BrtuT’I’M’F’S’mdh )

ELSE

BAHourlyResExportHASPReductionMW BrtuT’I’M’F’S’mdh =0

where resource type (t) = “ETIE”

Note: HourlyTotalHASPPart1Quantity BrtuT’I’M’F’S’mdh is the business driver.

    - BAHourlyResourceExportHASPReversalMW BrtuT’I’M’F’S’mdh =

- min(BAHourlyResExportHASPReductionMW BrtuT’I’M’F’S’mdh , (-1) * BAHourlyResourceExportHASPUntaggedMW BrtuT’I’M’F’S’mdh)

    - BAFMMIntervalResourceExportHASPReversalPrice BrtuT’I’M’F’S’mdhc  =

Max (FMMIntervalLMPPrice BrtuM’mdhc – HourlyDAEnergyResourceLMP rtmdh, 0  )

Note: BAHourlyResourceExportHASPReversalMW BrtuT’I’M’F’S’mdh is the business driver. Hourly value will be replicated in each of the relevant 15-minute intervals.

    - BAHourlyResourceExportHASPReversalAmount BrtuT’I’M’F’S’mdh  =

(1- BADayResourcePseudoTieDynamicFlag Brtmd)*(BAHourlyResourceExportHASPReversalMW BrtuT’I’M’F’S’mdh  *

(c BAFMMIntervalResourceExportHASPReversalPrice BrtuT’I’M’F’S’mdhc  /4))

## Outputs

| Output Req ID | Name | Description |
| --- | --- | --- |
|  | In addition to any outputs listed below, all inputs shall be included as outputs. |  |
|  | BA5MResourceFMMIIESettlementAmount BrtuT’I’M’F’S’mdhcif | FMM IIE settlement amount ($) |
|  | BASettlementIntervalFMMEnergyPrice BrtuT’I’Q’M’F’S’mdhcif | Applicable LMP price for FMM IIE. |
|  | BA5MResourceFMMIIEAssessmentAmount BrtuT’I’M’F’S’mdhcif | LMP Assessment amount for FMM IIE, intermediate calculation. |
|  | BASettlementIntervalFMMIIEAmount Bmdhcif | The BA total FMM IIE Settlement Amount ($) |
|  | CAISOSettlementIntervalTotalFMMIIEAmount mdhcif | The CAISO Total FMM IIE Settlement Amount ($) |
|  | SettlementIntervalFMMEDE1IncAmount BrtOuT’I’M’F’S’mdhcif | Intermediate amount for specific exceptional dispatch type |
|  | SettlementIntervalFMMEDE2IncAmount BrtOuT’I’M’F’S’mdhcif | Intermediate amount for specific exceptional dispatch type |
|  | SettlementIntervalFMMEDE3IncAmount BrtOuT’I’M’F’S’mdhcif | Intermediate amount for specific exceptional dispatch type |
|  | SettlementIntervalFMMEDEIncAmount BrtuT’I’M’F’S’mdhcif | Total incremental exceptional dispatch assessment amount at Settlement interval |
|  | SettlementIntervalFMMEDE1DecAmount BrtOuT’I’M’F’S’mdhcif | Intermediate amount for specific exceptional dispatch type |
|  | SettlementIntervalFMMEDE2DecAmount BrtOuT’I’M’F’S’mdhcif | Intermediate amount for specific exceptional dispatch type |
|  | SettlementIntervalFMMEDE3DecAmount BrtOuT’I’M’F’S’mdhcif | Intermediate amount for specific exceptional dispatch type |
|  | SettlementIntervalFMMEDEDecAmount BrtuT’I’M’F’S’mdhcif | Total decremental exceptional dispatch assessment amount at Settlement interval |
|  | SettlementIntervalTotalFMMEDEQuantity BrtuT’I’M’F’S’mdhcif | Total FMM EDE quantity. (MW) |
|  | HourlyTotalHASPPart1Quantity BrtuT’I’M’F’S’mdh | Total hourly quantity for ITIE and ETIE. (MW) |
|  | BAHourlyResourceImportHASPUntaggedMW BrtuT’I’M’F’S’mdh | The quantity of import that clear the Day-Ahead Market for which the Scheduling Coordinator has failed to submit an E-Tag prior to HASP solution availability. This is for a resource of type t = “ITIE”.(MW) |
|  | BAHourlyResourceImportHASPReductionMW BrtuT’I’M’F’S’mdh | The HASP reduction MW from Day Ahead Schedules net of Contracts MW for a resource of type t = “ITIE”. (MW) |
|  | BAHourlyResourceImportHASPReversalMW BrtuT’I’M’F’S’mdh | The MW basis for the FMM reversal settlement rule for a resource of type t = “ITIE”. (MW) |
|  | BAFMMIntervalResourceImportHASPReversalPrice BrtuT’I’M’F’S’mdhc | The price basis for the FMM reversal settlement rule. This is resource of type t = “ITIE”. (MW) |
|  | BAHourlyResourceImportHASPReversalAmount BrtuT’I’M’F’S’mdh | The amount assessed for the FMM reversal settlement rule. This is for a resource of type t = “ITIE”. ($) This will always be a charge to the SC. |
|  | BAHourlyResourceExportHASPUntaggedMW BrtuT’I’M’F’S’mdh | The quantity of import that clear the Day-Ahead Market for which the Scheduling Coordinator has failed to submit an E-Tag prior to HASP solution availability. This is for a resource of type t = “ETIE”.(MW) |
|  | BAHourlyResExportHASPReductionMW BrtuT’I’M’F’S’mdh | The HASP Reduction MW from Day Ahead Schedules net of Contracts MW for a resource of type t = “ETIE”.(MW) |
|  | BAHourlyResourceExportHASPReversalMW BrtuT’I’M’F’S’mdh | The MW basis for the HASP reversal settlement rule for a resource of type t = “ETIE”. (MW) |
|  | BAFMMIntervalResourceExportHASPReversalPrice BrtuT’I’M’F’S’mdhc | The price basis for the HASP reversal settlement rule. This is for a resource of type t = “ETIE”.. (MW) |
|  | BAHourlyResourceExportHASPReversalAmount BrtuT’I’M’F’S’mdh | The amount assessed for the HASP reversal settlement rule. This is for a resource of type t = “ETIE”. ($) This will always be a charge to the SC. |
|  | BAASettlementIntervalTotalFMMEDEQuantity BrtuT’I’Q’M’F’S’mdhcif | Total FMM EDE per BAA. |
|  | BAResourceRUCCapacityTotalIncludingDayAheadSchedule Brtmdh | Resource RUC Capacity Total Including Day Ahead Schedule |

# Charge Code Effective Dates

| Charge Code/ Pre-calc Name | Document Version | Effective Start Date | Effective End Date | Update Version Type |
| --- | --- | --- | --- | --- |
| CC 6460 – FMM Instructed Imbalance Energy Settlement | 5.0 | 05/01/14 | 4/30/14 | Configuration Impacted |
| CC 6460 – FMM Instructed Imbalance Energy Settlement | 5.1 | 5/01/14 | 9/30/14 | Configuration Impacted |
| CC 6460 – FMM Instructed Imbalance Energy Settlement | 5.2 | 10/01/14 | 2/09/15 | Configuration Impacted |
| CC 6460 – FMM Instructed Imbalance Energy Settlement | 5.3 | 11/4/15 | 11/3/15 | Configuration Impacted |
| CC 6460 – FMM Instructed Imbalance Energy Settlement | 5.4 | 11/1/16 | 10/31/16 | Configuration Impacted |
| CC 6460 – FMM Instructed Imbalance Energy Settlement | 5.5 | 2/10/15 | 11/3/15 | Configuration Impacted |
| CC 6460 – FMM Instructed Imbalance Energy Settlement | 5.6 | 11/4/15 | 10/31/16 | Configuration Impacted |
| CC 6460 – FMM Instructed Imbalance Energy Settlement | 5.7 | 11/1/16 | Open | Configuration Impacted |