Settlements and Billing

Configuration Guide:  Real Time Uninstructed Imbalance Energy

Version 5.5

Table of Contents

1.	Purpose of Document	3

2.	Introduction	3

2.1	Background	3

2.2	Description	3

3.	Charge Code Requirements	4

3.1	Business Rules	4

3.2	Predecessor Charge Codes	5

3.3	Successor Charge Codes	5

3.4	Inputs – External Systems	6

3.5	Inputs - Predecessor Charge Codes or Pre-calculations	6

3.6	CAISO Formula	7

3.7	Outputs	12

4.	Charge Code Effective Dates	14

# Purpose of Document

The purpose of this document is to capture the business and functional requirements for the MRTU Real Time Uninstructed Imbalance Energy Settlement, Charge Code 6475.

# Introduction

## Background

The CAISO calculates and accounts for Imbalance Energy for each Dispatch Interval and settles Imbalance Energy for each Settlement Interval for each resource within the CAISO Control Area and all System Resources Dispatched in Real-Time.

Imbalance Energy consists of following:

- IIE –  Instructed Imbalance Energy

  - FMM Instructed Imbalance Energy Settlement (CC 6460)

  - RTD Instructed Imbalance Energy Settlement (CC 6470)

- UIE – Real Time Uninstructed Imbalance Energy Settlement (CC 6475)

- UFE – Real Time Unaccounted for Energy Settlement (CC 6474)

To the extent that the sum of the Settlement Amounts for IIE, UIE, and UFE does not equal zero, the CAISO will assess Charges or make Payments in Real Time Imbalance Energy Offset (CC 6477) for the resulting differences to all Scheduling Coordinators based on a pro rata share of their Measured Demand for the relevant Settlement Interval.

In the Real-Time Market, the negative and positive Congestion Charges associated with a valid post-Day-Ahead TOR and ETC schedule change (including changes submitted to the Hour-Ahead Scheduling Process and changes submitted closer to Real-Time where allowed by the contract) will be reversed in CC 6474 RT Instructed Imbalance Energy Settlement on the standard Real-Time 10-minute interval basis.  Because Congestion Charges are implicitly collected by the CAISO in the Real-Time settlement and there are no holders of rights to receive Real-Time Congestion revenues under the MRTU design, all charges for Real-Time Congestion will be accumulated in a special and separate neutrality account to be distributed back to non-ETC Control Area metered Demand and exports on a per-MWh basis in Real Time Congestion Offset (CC 6774).

## Description

The Real Time Uninstructed Imbalance Energy (UIE) Settlement Amount is the payment or charge due to or from a resource for its UIE.  UIE quantities are calculated for each resource that has a Day-Ahead Schedule, Dispatch Instruction, Real-Time Interchange Export Schedule or Metered Quantity. UIE shall be calculated every five minutes based upon the resource’s Dispatch Instructions.

For all resources, including Generating Units, System Units of MSS Operators that have elected gross Settlement, Physical Scheduling Plants, System Resources and all Participating Load and Proxy Demand Resources, the UIE Settlement Amount is calculated for each Settlement Interval as the product of its UIE quantity (MWh) and the applicable RTD LMP.

The UIE Settlement Amount for non-Participating Load and MSS Demand under gross Settlement shall be calculated for each Settlement Interval as the product of its UIE quantity (MWh) and the applicable Hourly Real-Time LAP Price.

For eligible PIR Resources, the CAISO Tariff allows for PIRP Protective Measures Settlement of Charge Code 711 Intermittent Resources Net Deviation Settlement.  As such, this Charge Code shall initially calculate a daily Real Time UIE Settlement Amount for eligible PIR Resources.  At the end of the month, the daily Real Time UIE Settlement Amount for the eligible PIR Resources shall be reversed in Charge Code 722 Intermittent Resource Net Deviation Reversal and a monthly protective measured UIE Settlement Amount in CC 711.  In addition, the difference between the monthly netted UIE Settlement Amount calculated in CC 711 and the sum of PIR Real Time UIE Settlement Amount shall be settled in CC 721, Intermittent Resources Net Deviation Allocation.

# Charge Code Requirements

## Business Rules

| Bus Req ID | Business Rule |
| --- | --- |
| 1.0 | A positive Energy value indicates “Incremental” Energy. |
| 1.1 | A negative Energy value indicates “Decremental” Energy. |
| 1.2 | Instructed Imbalance Energy (IIE) is the calculated Energy expected to be produced or consumed as a result of responding to FMM or RTD Dispatch Instructions |
| 1.3 | UIE quantities shall be calculated for each resource that has a Day-Ahead Schedule, FMM Instructed Imbalance Energy, RTD Dispatch Instruction, Real-Time Interchange schedule or Metered Quantity. |
| 1.4 | The PIR resources with PIRProtectiveMeasuresFlag = ‘Y’ (resources eligible for monthly PIR Net Deviation settlement) shall be calculated daily as PIR Real Time UIE Settlement Accrual Amount, but settled monthly in Charge Code 711 and CC 721. |
| 2.0 | The Settlement Interval UIE Settlement Amount for Generating Unit, System Unit, System Resource, Participating Load and Pumping Load shall be calculated as the product of Settlement Interval UIE quantity and the applicable RTD Locational Marginal Price. |
| 3.0 | For Generating Units, MSS Operators that have elected gross Settlement, System Resources, Participating Load and Pumping Load, the UIE Settlement Amount is calculated for each Settlement Interval as the product of its UIE quantity and its’ relevant RTD Price. |
| 4.0 | The Settlement Interval UIE Settlement Amount for non-Participating Demand resources shall be calculated as the sum of: - UIE LAP Settlement Amount - UIE Neutrality Amount attributable |
| 4.1 | UIE LAP Settlement Amount for non-Participating Demand resources shall be calculated as the product of Hourly Real-Time LAP Price and Settlement Interval Real-Time UIE. |
| 4.2 | UIE Neutrality Amount for non-Participating Demand resources attributable to change in Day Ahead Load Distribution Factors and Real Time Load Distribution Factor. |
| 4.3 | Settlement Amount for Demand and net MSS Demand resources shall be calculated at hourly level by summing up the Settlement Interval values. |
| 5.0 | For adjustments to the Charge Code that cannot be accomplished by correction of upstream data inputs/recalculation or operator override Pass through Bill Charge logic will be applied. |
| 6.0 | For Generating Units, MSS Operators that have elected net Settlement, the UIE Settlement Amount is calculated for each Settlement Interval as the product of its UIE quantity and the relevant MSS Price, as calculated in Real Time Price Pre-calculation. |

## Predecessor Charge Codes

Charge Code/ Pre-calc Name

Real Time Energy Quantity Pre-calculation

Real Time Price Pre-calculation

Real Time Instructed Imbalance Energy Settlement (CC 6470)

MSS Netting Pre-calculation

Measured Demand Over Control Area

## Successor Charge Codes

Charge Code/ Pre-calc Name

Real Time Imbalance Energy Offset (CC 6477)

Intermittent Resources Net Deviation Allocation (CC 721)

Intermittent Resources Net Deviation Reversal (CC 722)

## Inputs – External Systems

| Input Req ID | Variable Name |  | Description |
| --- | --- | --- | --- |
| 1 |  | PTBChargeAdjustmentSettlementIntervalUIEAmount Bjmdhcif | Real Time Uninstructed Imbalance Energy Settlement Amount PTB Charge Adjustment Amount for Business Associate B.($) |
| 2 |  | PIRProtectiveMeasuresFlag Brtmd | PIR Protective Measures Flag for PIRP Resource r. ‘Y’ identifies a PIR unit has applied for and grant PIR Protective Measure Treatment and receive separate Settlement calculation of FMM IIE, RTD IIE and UIE. |
| 3 |  | DALoadSchedule BrtuT’I’Q’M’AA’R’pW’F’S’VvL’mdh | DA Load Schedule for Resource r, Resource type t, and Trading Hour h as provided by MQS where Entity Type T’, UDC Index u, MSS Gross/Net Energy Settlement Type I’, and MSS Subgroup M’ are mapped to the Master File and v is TAC AREA ID. |
| 4 |  | HourlyRTMLAPPrice AA’mdh | Hourly Real Time Market LAP Price for Apnode A’. |

## Inputs - Predecessor Charge Codes or Pre-calculations

| Input Req ID | Variable Name | Predecessor Charge Code/ Pre-calc Configuration |
| --- | --- | --- |
| 1 | SettlementIntervalRealTimeUIE BrtuT’I’Q’M’F’S’mdhcif | Real-Time Energy Pre-calculation |
| 2 | HourlyRealTimeLMP pmdh | Real Time Price Pre-calculation |
| 3 | SettlementIntervalNodalMeteredCAISODemandQuantity_MDOverCA AA’mdhcif | Measured Demand over Control Area Pre-calculation |
| 4 | SettlementIntervalRealTimeMSSPrice uM’mdhcif | Real Time Price Precalc |
| 5 | BAResEntitySettlementIntervalMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’VvHn’L’mdhcif | MSS Netting Precalc |
| 6 | SettlementIntervalRealTimeLMP BrtuM’mdhcif | Real Time Price Pre-calculation |
| 7 | HourlyDANodalLDF uM’AA’pmdh | Real Time Price Pre-calculation |
| 8 | HourlyRTNodalLDF uM’AA’pmdh | Real Time Price Pre-calculation |

## CAISO Formula

    - SettlementIntervalUIESettlementAmount

- SettlementIntervalUIESettlementAmount BrtuT’I’M’mdhcif =

SettlementIntervalLAPUIESettlementAmount BrtuT’I’M’mdhcif +

SettlementIntervalMSSNETUIESettlementAmount BrtuT’I’M’mdhcif +

- SettlementIntervalTIEGENUIESettlementAmount BrtuT’I’M’mdhcif +

SettlementIntervalPLOADUIESettlementAmount BrtuT’I’M’mdhcif +

SettlementIntervalMSSGROSSGENUIESettlementAmount BrtuT’I’M’mdhcif  +

SettlementIntervalGENUIESettlementAmount BrtuT’I’M’mdhcif

+   PIRRealTimeUIESettlementAccrualAmount BrtuT’I’M’mdhcif

    - SettlementIntervalGENUIESettlementAmount

- Where Resource type (t) = Generating Unit, and T’ = ‘UDC’,

- IF

- PIRProtectiveMeasuresFlag Brtmd <> ‘Y’

- THEN

- SettlementIntervalGENUIESettlementAmount BrtuT’I’M’mdhcif =

F'S' SettlementIntervalGenerationUIEAmount BrtuT’I’M’F’S’mdhcif

ELSE

- SettlementIntervalGENUIESettlementAmount BrtuT’I’M’mdhcif = 0

    - SettlementIntervalTIEGENUIESettlementAmount

Where Resource type (t) = ITIE or ETIE, and ENTITY_COMPONENT_TYPE = ‘TG’

IF

PIRProtectiveMeasuresFlag Brtmd  <> 1

THEN

SettlementIntervalTIEGENUIESettlementAmount BrtuT’I’M’mdhcif  =

F'S' SettlementIntervalGenerationUIEAmount BrtuT’I’M’F’S’mdhcif

ELSE

SettlementIntervalTIEGENUIESettlementAmount BrtuT’I’M’mdhcif  = 0

    - SettlementIntervalPLOADUIESettlementAmount

### SettlementIntervalPLOADUIESettlementAmount BrtuT’I’M’mdhcif =

F'S' SettlementIntervalPMPSTPLUIEAmount BrtuT’I’M’F’S’mdhcif  +

F'S' SettlementIntervalUIEPLOADLAPAmount BrtuT’I’M’F’S’mdhcif

    - SettlementIntervalPMPSTPLUIEAmount

SettlementIntervalPMPSTPLUIEAmount BrtuT’I’M’F’S’mdhcif = Q' (-1) * (SettlementIntervalRealTimeUIE BrtuT’I’Q’M’F’S’mdhcif * SettlementIntervalRealTimeLMP BrtuM’mdhcif)

Where Entity Component Type (F’) = ‘PMPST’ and Entity Component SubType (S’) = ‘PL’ and Balancing Authority Area (Q’) = ‘CISO’

    - SettlementIntervalUIEPLOADLAPAmount

#### SettlementIntervalUIEPLOADLAPAmount BrtuT’I’M’F’S’mdhcif   = Q'R'pW'vVL'

- ((-1) * (AA'HourlyRTMLAPPrice AA’mdh * SettlementIntervalUIEPLLAPLoadQuantity BrtuT’I’M’F’S’mdhcif))

Where DALoadSchedule BrtuT’I’Q’M’AA’R’pW’F’S’vVL’mdh  and SettlementIntervalFilteredDemandQuantity BrtuT’I’Q’M’AA’F’R’pW’S’VvL’mdhcif

Entity Component Subtype (S’) = ‘PL’ and Apnode Type = ‘Custom’ and Balancing Authority Area (Q’) = ‘CISO’

#### SettlementIntervalUIEPLLAPLoadQuantity

SettlementIntervalUIEPLLAPLoadQuantity BrtuT’I’M’F’S’mdhcif = Q'SettlementIntervalRealTimeUIE BrtuT’I’Q’M’F’S’mdhcif

Where Entity Component Type (F’) = ‘PUMP’ or ‘PMPP’ and Entity Component SubType (S’) = ‘PL’ and Balancing Authority Area (Q’) = ‘CISO’

    - SettlementIntervalMSSGROSSGENUIESettlementAmount

IF

I’ = GROSS, T’ = MSS, and Resource Type = GEN and PIRProtectiveMeasuresFlag Brtmd  <> ‘Y’

THEN

SettlementIntervalMSSGROSSGENUIESettlementAmount BrtuT’I’M’mdhcif =

F'S' SettlementIntervalGenerationUIEAmount BrtuT’I’M’F’S’mdhcif

ELSE

SettlementIntervalMSSGROSSGENUIESettlementAmount BrtuT’I’M’mdhcif = 0

### PIRRealTimeUIESettlementAccrualAmount

IF

PIRProtectiveMeasuresFlag Brtmd = ‘Y’

THEN

PIRRealTimeUIESettlementAccrualAmount BrtuT’I’M’mdhcif =

F'S' SettlementIntervalGenerationUIEAmount BrtuT’I’M’F’S’mdhcif

ELSE

PIRRealTimeUIESettlementAccrualAmount BrtuT’I’M’mdhcif  = 0

      - BASettlementIntervalPIRPUIEAccrualAmount

- BASettlementIntervalPIRPUIEAccrualAmount Bmdhcif = rtuT'I'M' PIRRealTimeUIESettlementAccrualAmount BrtuT’I’M’mdhcif

      - BADailyPIRPUIEAccrualAmount

- BADailyPIRPUIEAccrualAmount Bmd =hcc∈f BASettlementIntervalPIRPUIEAccrualAmount Bmdhcif

      - CAISOPIRRealTimeUIESettlementAccrualAmount

- CAISOPIRRealTimeUIESettlementAccrualAmount mdhcif =

rtuT'I'M' PIRRealTimeUIESettlementAccrualAmount BrtuT’I’M’mdhcif

      - CAISODailyPIRRealTimeUIESettlementAccrualAmount

- CAISODailyPIRRealTimeUIESettlementAccrualAmount md =         hcc∈fCAISOPIRRealTimeUIESettlementAccrualAmount mdhcif

    - SettlementIntervalGenerationUIEAmount

SettlementIntervalGenerationUIEAmount BrtuT’I’M’F’S’mdhcif = Q' (-1) * (SettlementIntervalRealTimeUIE BrtuT’I’Q’M’F’S’mdhcif * SettlementIntervalRealTimeLMP BrtuM’mdhcif)

- Where Resource Type (t) = ‘GEN’ or ‘ITIE’ and Balancing Authority Area (Q’) = ‘CISO’ and SettlementIntervalRealTimeUIE attribute MSS GROSS OR NET FLAG (I’) <> ‘NET’

    - SettlementIntervalLAPUIESettlementAmount

SettlementIntervalLAPUIESettlementAmount BrtuT’I’M’mdhcif =

F'S' SettlementIntervalUIELAPAmount BrtuT’I’M’F’S’mdhcif +

F'S' SettlementIntervalUIENeutralityAmount BrtuT’I’M’F’S’mdhcif

    - SettlementIntervalUIELAPAmount

SettlementIntervalUIELAPAmount BrtuT’I’M’F’S’mdhcif = AA'Q'R'pW'vVL'(-1) *  (HourlyRTMLAPPrice AA’mdh * SettlementIntervalUIENPLLAPLoadQuantity BrtuT’I’M’F’S’mdhcif)

- Where DALoadSchedule BrtuT’I’Q’M’AA’R’pW’F’S’VvL’mdh and SettlementIntervalFilteredDemandQuantity BrtuT’I’Q’M’AA’F’R’pW’S’VvL’mdhcif attribute MSS_GROSS_OR_NET FLAG <> ‘NET’ and Balancing Authority Area (Q’) = ‘CISO’

#### SettlementIntervalUIENPLLAPLoadQuantity brtuT’I’M’F’S’mdhcif

SettlementIntervalUIENPLLAPLoadQuantity BrtuT’I’M’F’S’mdhcif = Q'SettlementIntervalRealTimeUIE BrtuT’I’Q’M’F’S’mdhcif

Where Resource Type (t) = ‘LOAD’ and Entity Component SubType (S’) = ‘NPL’ or ‘GL’ and Balancing Authority Area (Q’) = ‘CISO’

#### SettlementIntervalFilteredDemandQuantity

SettlementIntervalFilteredDemandQuantity BrtuT’I’Q’M’AA’F’R’pW’S’VvL’mdhcif = Qdn'Nz'HPBAResEntitySettlementIntervalMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’VvHn’L’mdhcif

- Where Resource type (t) = ‘LOAD’

    - SettlementIntervalUIENeutralityAmount

SettlementIntervalUIENeutralityAmount BrtuT’I’M’F’S’mdhcif =

Q'  R'p  AA'P  W'  Q  d'  N  z'  d'  v  H  n'  L'   (SettlementIntervalNeutralityAllocation uM’AA’mdhcif *

(BAResEntitySettlementIntervalMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’VvHn’L’mdhcif / SettlementIntervalNodalMeteredCAISODemandQuantity_MDOverCA AA’mdhcif))

- Where Resource type (t) = ‘LOAD’

- and ENTITY_COMPONENT_SUBTYPE (S’) = ‘NPL’ OR ’GL’

      - SettlementIntervalNeutralityAllocation

- SettlementIntervalNeutralityAllocation uM’AA’mdhcif = (-1) * [1/12 * (BrtT'I'R'W'F'S'VL'pvQ' DALoadSchedule BrtuT’I’Q’M’AA’R’pW’F’S’VvL’mdh) * HourlyLapNeutralityPrice AA’mdh )]

- Where ENTITY_COMPONENT_SUBTYPE (S’) = ‘NPL’ OR ‘GL’

    - HourlyLapNeutralityPrice

- HourlyLapNeutralityPrice AA’mdh = uM'p(HourlyRealTimeLMP pmdh  * HourlyNodalLDFChangeDAtoRT uM’AA’pmdh)

    - HourlyNodalLDFChangeDAtoRT

- HourlyNodalLDFChangeDAtoRT uM’AA’pmdh = (HourlyRTNodalLDF uM’AA’pmdh - HourlyDANodalLDF uM’AA’pmdh

    - SettlementIntervalMSSNETUIESettlementAmount

- SettlementIntervalMSSNETUIESettlementAmount BrtuT’I’M’mdhcif = F'S'Q' ((-1) SettlementIntervalRealTimeUIE BrtuT’I’Q’M’F’S’mdhcif * SettlementIntervalRealTimeMSSPrice uM’mdhcif)

- And Where MSS election (I’) = Net T’ = MSS and Balancing Authority Area (Q’) = ‘CISO’

## Outputs

| Output ID | Name | Description |
| --- | --- | --- |
| 1 | In addition, all inputs are required to be accessible for review by analysts and report on Settlement statements. |  |
| 2 | SettlementIntervalUIESettlementAmount BrtuT’I’M’mdhcif | Settlement Interval UIE Settlement Amount for resource r ($) |
| 3 | SettlementIntervalGenerationUIEAmount BrtuT’I’M’F’S’mdhcif | Settlement Interval Generation UIE Settlement Amount for resource r ($). Where Resource type (t) = ‘GEN’ or ‘ITIE’ |
| 4 | SettlementIntervalUIENeutralityAmount BrtuT’I’M’F’S’mdhcif | Settlement Interval UIE Settlement Amount attributable to Hourly Real-Time LMP Price and Settlement Interval LDF Change Amount for (Load, Aggregated Point) resource r ($) |
| 5 | PTBChargeAdjustmentSettlementIntervalUIEAmount BJmdhcif | Real Time Uninstructed Imbalance Energy Settlement Amount PTB Charge Adjustment Amount for Business Associate B ($) |
| 6 | SettlementIntervalNeutralityAllocation uM’AA’mdhcif | Settlement Interval Neutrality Allocation for Aggregated Price Node A. ($) |
| 7 | PIRRealTimeUIESettlementAccrualAmount BrtuT’I’M’mdhcif | Settlement Interval PIR UIE Accrual Amount for resource r which qualifies for PIRP Protective Measures. ($) |
| 8 | SettlementIntervalMSSGROSSGENUIESettlementAmount BrtuT’I’M’mdhcif | Settlement Interval UIE Settlement amount for MSS GROSS Generators ($) |
| 9 | SettlementIntervalMSSNETUIESettlementAmount BrtuT’I’M’mdhcif | Settlement Interval UIE Settlement amount for MSS NET Generators and Load. ($) |
| 10 | SettlementIntervalGENUIESettlementAmount BrtuT’I’M’mdhcif | Settlement Interval UIE Settlement amount for Generators ($) |
| 11 | SettlementIntervalLAPUIESettlementAmount BrtuT’I’M’mdhcif | Settlement Interval UIE Settlement amount for resource r. ($) |
| 12 | - SettlementIntervalTIEGENUIESettlementAmount BrtuT’I’M’mdhcif | Settlement Interval UIE Settlement amount for Tie Generators. ($) |
| 13 | SettlementIntervalPLOADUIESettlementAmount BrtuT’I’M’mdhcif | Settlement Interval UIE amount for Participating Pump and Pump Storage Load. ($) |
| 14 | #### CAISOPIRRealTimeUIESettlementAccrualAmount mdhcif | Total Settlement Interval ISO PIR UIE Accrual amount. ($) |
| 15 | CAISODailyPIRRealTimeUIESettlementAccrualAmount md | Total Daily ISO PIR UIE Accrual amount. ($) |
| 16 | SettlementIntervalUIEPLOADLAPAmount BrtuT’I’M’F’S’mdhcif | Settlement Interval Participating Load UIE Settlement amount attributable to Hourly Real-Time LAP Price. ($) |
| 17 | SettlementIntervalFilteredDemandQuantity BrtuT’I’Q’M’AA’F’R’pW’S’VvL’mdhcif | Settlement Interval Metered Demand Quantity for resource r. (MWh) |
| 18 | BASettlementIntervalPIRPUIEAccrualAmount Bmdhcif | Settlement Interval PIRP UIE Accrual Amount for Business Associate B. ($) |
| 19 | BADailyPIRPUIEAccrualAmount Bmd | Daily PIRP UIE Accrual Amount for Business Associate B. ($) |
| 20 | SettlementIntervalPMPSTPLUIEAmount BrtuT’I’M’F’S’mdhcif | Settlement Interval Pump Storage Settlement Amount for resource r ($) |
| 21 | SettlementIntervalUIEPLLAPLoadQuantity BrtuT’I’M’F’S’mdhcif | Settlement Interval Participating Load LAP Quantity for resource r. (MWh) Where Entity Component Type F’ equals ‘PUMP’ or ‘PMPP’ and Entity Component Type S’ equals ‘PL’ |
| 22 | SettlementIntervalUIELAPAmount BrtuT’I’M’F’S’mdhcif | Settlement Interval UIE LAP Amount for Non-participating Load Resources. ($) |
| 23 | SettlementIntervalUIENPLLAPLoadQuantity BrtuT’I’M’F’S’mdhcif | Settlement Interval UIE LAP Load Quantity for Non-participating Load Resources. ($) Where resource type t equals ‘Load’ and Entity Component Subtype S’ equals “NPL’ or “GL’ |
| 24 | HourlyLapNeutralityPrice AA’mdh | Hourly LAP Neutrality Price for Aggregated Price Node A. ($/MWh) |
| 25 | HourlyNodalLDFChangeDAtoRT uM’AA’pmdh | Hourly Nodal Load Distribution Factor (LDF) change between Day Ahead Market and Real Time Market for Aggregated Pricing Node A |

# Charge Code Effective Dates

| Charge Code/ Pre-calc Name | Document Version | Effective Start Date | Effective End Date | Version Update Type |
| --- | --- | --- | --- | --- |
| Real Time Uninstructed Imbalance Energy Settlement (CC 6475) | 5.0 | 04/01/09 | 03/31/09 | Documentation Edits Only |
| Real Time Uninstructed Imbalance Energy Settlement (CC 6475) | 5.1 | 04/01/09 | 10/31/09 | Configuration Impacted |
| Real Time Uninstructed Imbalance Energy Settlement (CC 6475) | 5.2 | 11/01/09 | 4/30/14 | Configuration Impacted |
| Real Time Uninstructed Imbalance Energy Settlement (CC 6475) | 5.3 | 5/1/14 | 9/30/14 | Configuration Impacted |
| Real Time Uninstructed Imbalance Energy Settlement (CC 6475) | 5.4 | 10/1/14 | 3/31/18 | Configuration Impacted |
| Real Time Uninstructed Imbalance Energy Settlement (CC 6475) | 5.5 | 4/1/18 | Open | Configuration Impacted |