Settlements and Billling

Configuration Guide: CG CC 64770 Real Time Imbalance Energy Offset EIM v 5.2 EIM

CC 64770

Version 5.2

Table of Contents

1.	Purpose of Document	3

2.	Introduction	3

2.1	Background	3

2.2	Description	4

3.	Charge Code Requirements	4

3.1	Business Rules	4

3.2	Predecessor Charge Codes	5

3.3	Successor Charge Codes	5

3.4	Inputs – External Systems	5

3.5	Inputs - Predecessor Charge Codes or Pre-calculations	6

3.6	CAISO Formula	7

3.8	Outputs	12

4.	Charge Code Effective Dates	13

# Purpose of Document

The purpose of this document is to capture the requirements and design specification for a Charge Code in one document.

# Introduction

## Background

The CAISO calculates and accounts for Imbalance Energy for each Dispatch Interval and settles Imbalance Energy for each Settlement Interval for each resource within the EIM Area and all System Resources Dispatched in Real-Time.

Imbalance Energy consists of following:

- IIE –  instructed imbalance energy

  - FMM Instructed Imbalance Energy Settlement (CC 6460)

  - FMM Instructed Imbalance Energy EIM Settlement (CC 64600)

  - RTD Instructed Imbalance Energy Settlement (CC 6470)

  - RTD Instructed Imbalance Energy EIM Settlement (CC 64700)

- UIE – Uninstructed Imbalance Energy

  - Real Time Uninstructed Imbalance Energy Settlement (CC 6475)

  - Real Time Uninstructed Imbalance Energy EIM Settlement (CC 64750)

- UFE – Unaccounted for Energy

  - Real Time Unaccounted for Energy Settlement (CC 6474)

  - Real Time Unaccounted for Energy EIM Settlement (CC 64740)

- GHG - Greenhouse Gas Emission Cost Revenue (CC 491)

To the extent that the sum of the Settlement Amounts for IIE, UIE, and UFE does not equal zero within the CAISO Balancing Authority Area, the CAISO will assess Charges or make Payments in Real Time Imbalance Energy Offset (CC 6477) and in Real Time Imbalance Energy Offset EIM (CC 64770)for the resulting differences to all Scheduling Coordinators based on a pro rata share of their Measured Demand for the relevant Settlement Interval. To the extent that the sum of the Settlement Amounts for IIE, UIE, UFE, and GHG does not equal zero within the EIM Balancing Authority Area, the CAISO will assess Charges or make Payments in Real Time Imbalance Energy Offset EIM (CC 64770)for the resulting differences to EIM Entity Scheduling Coordinator ID, respectively.

In the Real-Time Market, the negative and positive Congestion Charges associated with a valid post-Day-Ahead TOR and ETC schedule change (including changes submitted to the Hour-Ahead Scheduling Process and changes submitted closer to Real-Time where allowed by the contract) will be reversed in CC 6788 RTM Congestion Credit Settlement. Because Congestion Charges are implicitly collected by the CAISO in the Real-Time settlement and there are no holders of rights to receive Real-Time Congestion revenues, all charges for Real-Time Congestion will be accumulated in special and separate Balancing Authority Area neutrality accounts.  The CAISO Real-Time Congestion Charges less Virtual Bid Adjustment shall be distributed back to non-ETC Control Area metered Demand and exports in Real Time Congestion Offset (CC 6774).  The EIM Balancing Authority Area Real-Time Congestion Charges shall be distributed to the applicable EIM Entity Scheduling Coordinator in Real Time Congestion Offset EIM (CC 67740).

## Description

The calculation of Real-Time Imbalance Energy Offset EIM also includes the settlement of Greenhouse Gas Compensation. Additional adjustments are calculated to subtract congestion and transmission losses, and adjust for participation in the Energy Imbalance Market.

To the extent that the sum of the Settlement Amounts for EIM Financial Transfer, Greenhouse Gas Compensation, IIE, UIE, and UFE,  less the RT Energy Congestion revenues computed within Real-Time Congestion Offset (from CC 67740) less the Real-Time Marginal Cost of Losses Offset (from CC 69850) and EIM Transfer Adjustment does not equal zero, the CAISO will assess Charges or make Payments in Real Time Imbalance Energy Offset (CC 64770) for the resulting differences to the EIM Entity Scheduling Coordinator.

# Charge Code Requirements

## Business Rules

| Bus Req ID | Business Rule |
| --- | --- |
| 1.0 | This Charge Code shall be calculated and output on a 5-minute Settlement Interval basis. |
| 2.0 | The Initial Real-Time Imbalance Energy Offset for each EIM Balancing Authority Area in the EIM Area as the sum of the financial value of EIM Transfers and the Settlement amounts for FMM Instructed Imbalance Energy, RTD Instructed Imbalance Energy, Uninstructed Imbalance Energy, and Unaccounted for Energy, less Real-Time Congestion Offset and less the Real-Time Marginal Cost of Losses Offset. |
| 3.0 | The Financial Value of EIM Transfers shall be the product of the MWh, and the System Marginal Economic Cost. (Fact) |
| 3.1 | EIM Transfers from Base EIM Transfer System Resources that elected settlement, for both FMM and RTD (inclusive of updates after RTM schedules through e-tags) deviations with respect to their base schedules, will not be included in the computation of financial value of EIM Transfers as those are already settled RT Imbalance Energy charge code(s). (Fact) |
| 3.2 | A Master File – resident flag (Yes / No) shall indicate whether (Yes) or not (No) an ETSR has elected to participate in imbalance energy settlement. (Fact) |
| 4.0 | The Initial Real Time Imbalance Energy Offset Amount shall be adjusted by the BAA Total Financial Value Credit Amount |
| 4.1 | The BAA Total Financial Value Credit Amount is calculated as the sum of BAA FMM GHG Credit Amount, BAA RTD GHG Credit Amount, and BAA ETSR Transfer Operation Adjustment Credit Amount |
| 4.2 | FMM GHG Credit Amount is calculated is the product FMM GHG Credit Quantity and BAA FMM Greenhouse Gas Marginal Costs |
| 4.3 | FMM GHG Credit Quantity is calculated is the FMM Net EIM Transfer Quantity less BAA FMM Total Greenhouse Gas Obligation |
| 4.4 | RTD GHG Credit Amount is calculated is the product RTD GHG Credit Quantity and BAA RTD Greenhouse Gas Marginal Costs |
| 4.5 | RTD GHG Credit Quantity is calculated is the RTD Net EIM Transfer Quantity less FMM Net EIM Trasnfer Quantity less the difference of BAA RTD Total Greenhouse Gas Obligation and BAA FMM Greenhouse Gas Obligation |
| 4.6 | BAA ETSR Transfer Operation Adjustment Credit Amount is calculated is the product BAA Net ETSR Operation Adjustment Quantity and BAA RTD Greenhouse Gas Marginal Costs |
| 5.0 | The allocation of the Real-Time Imbalance Energy Offset for EIM Entity Balancing Authority shall be to the EIM Entity Scheduling Coordinator. |

## Predecessor Charge Codes

Charge Code/ Pre-calc Name

RT Energy Pre-calculation

CC 491 – Greenhouse Gas Emission Cost Revenue

CC 64600 - FMM Instructed Imbalance Energy EIM (FMM IIE)

CC 64700 - Real Time Instructed Imbalance Energy EIM Settlement (RTM IIE)

CC 64740 – Real Time Unaccounted for Energy EIM Settlement (UFE)

CC 64750 - Real Time Uninstructed Imbalance Energy EIM Settlement (UIE)

CC 67740 – Real Time Congestion Offset EIM

CC 69850 – Real Time Losses Offset EIM

## Successor Charge Codes

Charge Code/ Pre-calc Name

None

## Inputs – External Systems

| Row # | Variable Name | Description |
| --- | --- | --- |
|  | EIMEntitySCFlag BQ’ | A flag input that, when = 1, relates an EIM Balancing Authority Area (EIM BAA) with the associated Business Associate of the Scheduling Coordinator ID. |
|  | EIMFMMGHGBidAdderPrice BrtQ’mdhc | The fourth component in the (LMPs) of EIM Entity BAAs (-). Real Time Pre-Dispatch |
|  | EIMRTDGHGBidAdderPrice BrtQ’mdhcif | The fourth component in the (LMPs) of EIM Entity BAAs (-). Real Time Dispatch |
|  | BAResourceEIMFMMGHGQuantity BrtQ’F’S’mdhc | Net export (MW) to the ISO BAA for each EIM resource for GHG payment (+).Real Time Pre-Dispatch |

## Inputs - Predecessor Charge Codes or Pre-calculations

| Row # | Variable Name | Predecessor Charge Code/ Pre-calc Configuration |
| --- | --- | --- |
|  | - RTBAACongestionRevenueAmount Q’mdhcif | Real Time Congestion PC |
|  | - BA_EIMBAA_SettlementInterval_UnaccountedforEnergy_SettlementAmount BuQ’mdhcif | CC 64740 - Real Time Unaccounted for EIM Energy Settlement (UFE). |
|  | - EIMSettlementIntervalUIESettlementAmount BrtuT’I’Q’M’mdhcif | CC 64750 - Real Time Uninstructed Imbalance Energy EIM Settlement (UIE) |
|  | - EIMBA5MResourceFMMIIESettlementAmount BrtuQ’M’mdhcif | CC 64600 – FMM Instructed Imbalance Energy EIM Settlement (FMM IIE) |
|  | - EIMSettlementIntervalIIEAmount BrtQ’mdhcif | CC 64700 - Real Time Instructed Imbalance Energy Settlement (IIE) |
|  | - BAResourceEIMGHGPaymentAmount BrtQ’F’S’mdhcif | CC 491 – Greenhouse Gas Emission Cost Revenue |
|  | - BAAFMMFinancialValueTransfer Q’mdhcif | CC 6477 – Real-Time Imbalance Energy Offset |
|  | - BAARTDFinancialValueTransfer Q’mdhcif | CC 6477 – Real-Time Imbalance Energy Offset |
|  | - EIMBAARTMarginalLossesOffsetAmount Q’mdhcif | CC 69850 - Real Time Loss Offset EIM. |
|  | - BAResourceEIMRTDGHGObligationQuantity BrtQ’F’S’mdhcif | CC 491 – Greenhouse Gas Emission Cost Revenue |
|  | - BAAFMMETSRFinancialValueFromQuantity Q’mdhcif | CC 6477 – Real-Time Imbalance Energy Offset |
|  | - BAAFMMETSRFinancialValueToQuantity Q’mdhcif | CC 6477 – Real-Time Imbalance Energy Offset |
|  | - BAAResourceRTDScheduleTransferFromQuantity rQ’AA’Qpmdhcif | Real Time Energy PC |
|  | - BAAResourceRTDScheduleTransferToQuantity rQ’AA’Qpmdhcif | Real Time Energy PC |
|  | - BAAResourceSettlementIntervalRTDTransferDevFromQuantity rQ’AA’Qpmdhcif | Real Time Energy PC |
|  | - BAAResourceSettlementIntervalRTDTransferDevToQuantity rQ’AA’Qpmdhcif | Real Time Energy PC |

## CAISO Formula

    - EIMEntityRealTimeImbalanceEnergyOffsetAllocationAmount BQ’mdhcif

EIMEntityRealTimeImbalanceEnergyOffsetAllocationAmount BQ’mdhcif =

(-1) * EIMBAATotalRTIEOSettlementAmount Q’mdhcif * EIMEntitySCFlag BQ’

### EIMBAATotalRTIEOSettlementAmount Q’mdhcif

EIMBAATotalRTIEOSettlementAmount Q’mdhcif =

EIMBAAInitialRealTimeImbalanceEnergyOffsetSettlementAmount Q’mdhcif

Where Q’ <> ‘CISO’

### EIMBAAInitialRealTimeImbalanceEnergyOffsetSettlementAmount Q’mdhcif

EIMBAAInitialRealTimeImbalanceEnergyOffsetSettlementAmount Q’mdhcif =

EIMBAATotalFinancialValueTransfer Q’mdhcif +

EIMBAATotalGHGCompensation Q’mdhcif +

EIMBAATotalRealTimeIIESettlementAmount Q’mdhcif +

EIMBAATotalFMMIIEAmount Q’mdhcif +

EIMBAATotalRealTimeUIESettlementAmount Q’mdhcif +

EIMBAATotalUFESettlementAmount Q’mdhcif -

EIMBAATotalRTEnergyCongestionAmt Q’mdhcif -

EIMBAARTLossOffsetAmount Q’mdhcif

### EIMBAATotalFinancialValueTransfer Q’mdhcif

EIMBAATotalFinancialValueTransfer Q’mdhcif =

(BAAFMMFinancialValueTransfer Q’mdhcif + BAARTDFinancialValueTransfer Q’mdhcif + BAATotalFinancialValueCreditAmount Q’mdhcif)

Where Q’ <> ‘CISO’

    - BAATotalFinancialValueCreditAmount

- BAATotalFinancialValueCreditAmount Q’mdhcif = (BAAFMMGHGCreditAmount Q’mdhcif + BAARTDGHGCreditAmount Q’mdhcif + BAAETSRTransferDevCreditAmount Q’mdhcif)

    - BAAFMMGHGCreditAmount

- BAAFMMGHGCreditAmount Q’mdhcif = (BAAFMMETSRGHGCreditQuantity Q’mdhcif  * BAA15MAverageFMMGHGPrice Q’mdhcif)

    - BAAFMMETSRGHGCreditQuantity

- BAAFMMETSRGHGCreditQuantity Q’mdhcif  = ((BAAFMMETSRFinancialValueFromQuantity Q’mdhcif - BAA5MTotalFMMGHGQuantity Q’mdhcif) - BAAFMMETSRFinancialValueToQuantity Q’mdhcif)

    - BAA5MTotalFMMGHGQuantity

- BAA5MTotalFMMGHGQuantity Q’mdhcif = sum over (B,r,t,F’S’) IntDuplicate(BAResourceEIMFMMGHGQuantity BrtQ’F’S’mdhc)/12

    - BAA15MAverageFMMGHGPrice

BAA15MAverageFMMGHGPrice Q’mdhcif = Average (sum over (B,r,t) EIMFMMGHGBidAdderPrice BrtQ’mdhc)

    - BAARTDGHGCreditAmount

- BAARTDGHGCreditAmount Q’mdhcif = (BAARTDETSRGHGCreditQuantity Q’mdhcif)* BAA5MAverageRTDGHGPrice Q’mdhcif

    - BAARTDETSRGHGCreditQuantity

- BAARTDETSRGHGCreditQuantity Q’mdhcif  = ((BAARTDETSRTransferFromQuantity Q’mdhcif - BAA5MTotalRTDGHGQuantity Q’mdhcif) - BAARTDETSRTransferToQuantity Q’mdhcif)

    - BAARTDETSRTransferFromQuantity

- BAARTDETSRTransferFromQuantity Q’mdhcif = Sum over (r,A,A’,Q,p) (BAAResourceRTDScheduleTransferFromQuantity rQ’AA’Qpmdhcif * (1 - ResourceETSRElectSettlementFlag rmd))

    - BAARTDETSRTransferToQuantity

- BAARTDETSRTransferToQuantity Q’mdhcif = Sum over (r,A,A’,Q,p) (BAAResourceRTDScheduleTransferToQuantity rQ’AA’Qpmdhcif * (1 - ResourceETSRElectSettlementFlag rmd))

    - BAA5MTotalRTDGHGQuantity

- BAA5MTotalRTDGHGQuantity Q’mdhcif =

- sum over (B,r,t,F’S’) BAResourceEIMRTDGHGObligationQuantity BrtQ’F’S’mdhcif

    - BAA5MAverageRTDGHGPrice

- BAA5MAverageRTDGHGPrice Q’mdhcif = Average (sum over (B,r,t) EIMRTDGHGBidAdderPrice BrtQ’mdhcif)

    - BAAETSRTransferDevCreditAmount

- BAAETSRTransferDevCreditAmount Q’mdhcif = (BAARTDETSRTransferDevQuantity Q’mdhcif * EIMAreaRTDMarginalGHGCreditPrice mdhcif)

    - EIMAreaRTDMarginalGHGCreditPrice

- EIMAreaRTDMarginalGHGCreditPrice mdhcif = Average over (Q’) BAA5MAverageRTDMarginalGHGPrice Q’mdhcif

    - BAARTDETSRTransferDevQuantity

- BAARTDETSRTransferDevQuantity Q’mdhcif = Sum over (r,A,A’,Q,p) ((BAAResourceSettlementIntervalRTDTransferDevFromQuantity rQ’AA’Qpmdhcif - BAAResourceSettlementIntervalRTDTransferDevToQuantity rQ’AA’Qpmdhcif) * (1 - ResourceETSRElectSettlementFlag rmd))

### EIMBAATotalGHGCompensation Q’mdhcif

EIMBAATotalGHGCompensation Q’mdhcif =

BrtF'S' BAResourceEIMGHGPaymentAmount BrtQ’F’S’mdhcif

Note: This calculation is being performed in the hierarchy of Charge Code 491

### EIMBAATotalRealTimeIIESettlementAmount Q’mdhcif

EIMBAATotalRealTimeIIESettlementAmount Q’mdhcif =

Brt EIMSettlementIntervalIIEAmount BrtQ’mdhcif

Note: This calculation is being performed in the hierarchy of Charge Code 64700

### EIMBAATotalFMMIIEAmount Q’mdhcif

EIMBAATotalFMMIIEAmount Q’mdhcif =

BrtuM'EIMBA5MResourceFMMIIESettlementAmount BrtuQ’M’mdhcif

Note: This calculation is being performed in the hierarchy of Charge Code 64600

### EIMBAATotalRealTimeUIESettlementAmount Q’mdhcif

EIMBAATotalRealTimeUIESettlementAmount Q’mdhcif =

BrtuT 'I 'M ' EIMSettlementIntervalUIESettlementAmount BrtuT’I’Q’M’mdhcif

Note: This calculation is being performed in the hierarchy of Charge Code 64750

### EIMBAATotalUFESettlementAmount Q’mdhcif

EIMBAATotalUFESettlementAmount Q’mdhcif = BuBA_EIMBAA_SettlementInterval_UnaccountedforEnergy_SettlementAmount BuQ’mdhcif

Note: This calculation is being performed in the hierarchy of Charge Code 64740

### EIMBAATotalRTEnergyCongestionAmt Q’mdhcif

EIMBAATotalRTEnergyCongestionAmt Q’mdhcif =

RTBAACongestionRevenueAmount Q’mdhcif

Where Balancing Authority Area <> ‘CISO’

### EIMBAATotalRTLossOffsetAmt Q’mdhcif

EIMBAATotalRTLossOffsetAmt Q’mdhcif =

EIMBAARTMarginalLossesOffsetAmount Q’mdhcif

## Outputs

| Output ID | Name | Description |
| --- | --- | --- |
|  | In addition to any outputs listed below, all inputs shall be included as outputs. |  |
|  | EIMEntityRealTimeImbalanceEnergyOffsetAllocationAmount BQ’mdhcif | Total Real Time Imbalance Energy Offset Settlement Amount for an EIM Entity Scheduling Coordinator by Balancing Authority Area. |
|  | EIMBAATotalRTIEOSettlementAmount Q’mdhcif | Total Real Time Imbalance Energy Offset Amount for EIM Balancing Authority Area. |
|  | EIMBAAInitialRealTimeImbalanceEnergyOffsetSettlementAmount Q’mdhcif | Initial Calculation of EIM Balancing Authority Area’s Real Time Imbalance Energy Offset Amount. |
|  | EIMBAATotalFinancialValueTransfer Q’mdhcif | Balancing Authority Area’s Financial EIM transfer from the fifteen minute and real time market. |
|  | EIMBAATotalGHGCompensation Q’mdhcif | EIM Balancing Authority Area Greenhouse Gas Compensation. |
|  | EIMBAATotalRealTimeIIESettlementAmount Q’mdhcif | EIM Balancing Authority Area Real Time Instructed Imbalance Energy. |
|  | EIMBAATotalFMMIIEAmount Q’mdhcif | EIM Balancing Authority Area Fifteen Minute Market Instructed Imbalance Energy. |
|  | EIMBAATotalRealTimeUIESettlementAmount Q’mdhcif | EIM Balancing Authority Area Uninstructed Imbalance Energy. |
|  | EIMBAATotalUFESettlementAmount Q’mdhcif | EIM Balancing Authority Area Unaccounted for Energy. |
|  | EIMBAATotalRTEnergyCongestionAmt Q’mdhcif | EIM Balancing Authority Area Real-Time Energy Congestion. |
|  | EIMBAATotalRTLossOffsetAmt Q’mdhcif | EIM Balancing Authority Area Real-Time Energy Loss. |
|  | BAATotalFinancialValueCreditAmount Q’mdhcif | Balancing Authority Area Total Financial Value Credit Amount ($) |
|  | BAAFMMGHGCreditAmount Q’mdhcif | Balancing Authority Area FMM Greenhouse Gas Credit Amount ($) |
|  | BAAFMMETSRGHGCreditQuantity Q’mdhcif | Balancing Authority Area FMM Greenhouse Gas Credit Quantity (MWh) |
|  | BAA5MTotalFMMGHGQuantity Q’mdhcif | Balancing Authority Area Total FMM Greenhouse Gas Obligation Quantity (MWh) |
|  | BAA15MAverageFMMGHGPrice Q’mdhcif | Balancing Authority Area Average FMM Marginal Greenhouse Gas Price |
|  | BAARTDGHGCreditAmount Q’mdhcif | Balancing Authority Area RTD Greenhouse Gas Credit Amount ($) |
|  | BAARTDETSRGHGCreditQuantity Q’mdhcif | Balancing Authority Area RTD Greenhouse Gas Credit Quantity (MWh) |
|  | BAARTDETSRTransferFromQuantity Q’mdhcif | Balancing Authority Area RTD ETSR Transfer From Quantity |
|  | BAARTDETSRTransferToQuantity Q’mdhcif | Balancing Authority Area RTD ETSR Transfer To Quantity |
|  | BAA5MTotalRTDGHGQuantity Q’mdhcif | Balancing Authority Area Total FMM Greenhouse Gas Obligation Quantity (MWh) |
|  | BAA15MAverageFMMGHGPrice Q’mdhcif | Balancing Authority Area Average FMM Marginal Greenhouse Gas Price |
|  | BAAETSRTransferDevCreditAmount Q’mdhcif | Balancing Authority Area ETSR Transfer Deviation Credit Amount |
|  | BAAETSRTransferDevCreditQuantity Q’mdhcif | Balancing Authority Area ETSR Transfer Deviation Credit Quantity |
|  | EIMAreaRTDMarginalGHGCreditPrice mdhcif | EIM Area RTD Marginal GHG Credit Price |

# Charge Code Effective Dates

| Charge Code/ Pre-calc Name | Document Version | Effective Start Date | Effective End Date | Version Update Type |
| --- | --- | --- | --- | --- |
| CC 64770 – Real Time Imbalance Energy Offset EIM | 5.0 | 10/01/14 | 11/3/15 | Initial Version |
| CC 64770 – Real Time Imbalance Energy Offset EIM | 5.1 | 11/4/15 | 4/3/18 | Configuration Impacted |
| CC 64770 – Real Time Imbalance Energy Offset EIM | 5.1a | 4/4/18 | 7/31/19 | Documentation Only |
| CC 64770 – Real Time Imbalance Energy Offset EIM | 5.2 | 8/1/19 | Open | Configuration Impacted |