Settlements & Billing

Configuration Guide: CG CC 7070 Flexible Ramp Forecasted Movement Settlement v5.1

Version 5.1

Table of Contents

1.	Purpose of Document	3

2.	Introduction	3

2.1	Background	3

2.2	Description	3

3.	Charge Code Requirements	3

3.1	Business Rules	3

3.2	Predecessor Charge Codes	5

3.3	Successor Charge Codes	5

3.4	Inputs – External Systems	5

3.5	Inputs - Predecessor Charge Codes or Pre-calculations	5

3.6	CAISO Formula	6

3.7	Outputs	8

4.	Charge Code Effective Dates	9

# Purpose of Document

The purpose of this document is to capture the business and functional requirements of  a charge code in one document.

# Introduction

## Background

The Flexible Ramping product (FRP) is designed to ensure that there is sufficient ramping capability available in the financially binding five-minute interval to meet the forecasted net load for interval t+5 and cover upwards and downwards forecast error uncertainty.

FRP will help the system to maintain and use dispatchable capacity, as well as provide the market more transparent and less volatile price signals when undergoing forecasted ramp-constrained conditions.  It will be procured and dispatched to meet five minute to five minute net forecast (load forecast minus VER forecast) changes plus uncertainty and will be modeled as a ramping capability constraint.

The ISO will financially settle FRP in the fifteen-minute market and the five-minute market, with rescission of payments applied to resources with UIE (uninstructed imbalance energy) or OA (operational adjustment) amounts, positive or negative, which are imposed on reserved FRP capacity awards. Settlement and allocation of FRP costs will happen on a daily basis for forecasted movement portion and uncertainty award portion.  At the end of the month, the uncertainty award allocation will be reversed and will be re-allocated based on the month’s net UIE or OA values.

As no economic bids are applied to FRP, FRU/FRD awards will be exempt from grid management charges (GMC).  Additionally, dispatchable resources will have their FRP awards and forecasted movement assessments - ignoring rescission settlement - included as part of daily RTM bid cost recovery calculations.

## Description

For each Settlement Interval, this charge code will generate the Flexible Ramp  forecasted movement assessment that goes into RTM Net Amount pre-calculation for BCR contribution as well as the flexible ramp forecasted movement assessment inclusive of rescission that gets allocated in CC 7076 – Flexible Ramp Forecast Movement Allocation.

# Charge Code Requirements

## Business Rules

| Bus Req ID | Business Rule |
| --- | --- |
| 1.0 | For each Settlement Interval, this charge code will settle the Flexible Ramp forecasted movement, and apply rescission, if any, to avoid double payment for UIE. |
| 2.0 | Forecasted movement MW shall be converted into MWh values for both FMM and RTD. |
| 2.1 | FMM forecasted movement shall settle at the difference between (FMM flexible ramp up price (FRUP) and FMM flexible ramp down price (FRDP). Upward (or positive) FMM forecasted movement shall be paid at this price difference, FMM FRUP – FMM FRDP; while downward (represented as negative) FMM forecasted movement shall be charged at this price difference, FMM FRUP – FMM FRDP. |
| 2.2 | RTD forecasted movement per 5-minute interval shall settle as incremental changes between FMM and RTD forecasted movements at the price difference, RTD FRUP – RTD FRDP. RTD incremental forecasted movement, is calculated as the RTD forecasted movement less FMM forecasted movement. Upward (or positive) RTD incremental forecasted movement shall be paid at the RTD FRU the price difference, RTD FRUP – RTD FRDP; while downward (represented as negative) RTD forecasted movement shall be charged at the price difference, RTD FRUP – RTD FRDP. |
| 3.0 | The resource UIE (at interval t) for the case of supply and load, and resource OA (operational adjustments; also at interval t) for the case of interties, shall be compared with the resource’s RTD FRP award (at interval t) plus RTD forecasted movement (at interval t) to determine the rescission quantities. Any overlap between the UIE (or OA) and the FRP award (FRU or FRD) plus the forecasted movement will be rescinded. The overlapped quantity with the uncertainty portion will be rescinded first and this becomes the uncertainty award rescission quantity; while any remaining overlap quantity will be rescinded for the forecasted movement, and this becomes the forecasted movement rescission quantity. In other words, the uncertainty award is rescinded first and the forecasted movement rescission follows for the remainder. For FRU rescission process, positive UIE for generation including dynamic system resource, positive OA for non-dynamic import resources, and negative OA for export resources shall be compared with the FRU award and positive (for generation, including dynamic system resource, or imports) or negative (for exports) forecasted movement. (Fact) |
| 3.1 | Any forecasted movement rescission quantities shall settle at the price difference, RTD FRUP – RTD FRDP. |
| 4.0 | For adjustments to the Charge Code that cannot be accomplished by correction of upstream data inputs/recalculation or operator override Pass Through Bill Charge logic will be applied. |
| 5.0 | When an eligible resource has an interval with a negative MWh meter, CAISO will not charge for the energy of those intervals. |

## Predecessor Charge Codes

Charge Code/ Pre-calc Name

CC 7071 – Flexible Ramp Up Uncertainty Capacity Settlement

CC 7081 – Flexible Ramp Down Uncertainty Capacity Settlement

Real Time Energy Pre Calculation

## Successor Charge Codes

Charge Code/ Pre-calc Name

CC 7076 - Flexible Ramp Forecast Movement Allocation

PC RTM Net Amount

## Inputs – External Systems

| Input Req ID | Variable Name | Description |
| --- | --- | --- |
|  | BA15mResourceFMMFlexRampForecastedMovementMWQty BrtQ’uT’I’M’L’F’S’mdhc | FMM flex ramp forecasted movement quantity (in MW) |
|  | BA5mResourceRTDFlexRampForecastedMovementMWQty BrtQ’uT’I’M’L’F’S’mdhcif | RTD flex ramp forecasted movement quantity (in MW) |
|  | BA15mResourceFMMFlexRampUpTotalPrice Brmdhc | FMM FRUP, flex ramp up price for FMM (in $/MWh) |
|  | BA5mResourceRTDFlexRampUpTotalPrice Brmdhcif | RTD FRUP, flex ramp up price for RTD (in $/MWh) |
|  | BA15mResourceFMMFlexRampDownTotalPrice Brmdhc | FMM FRDP, flex ramp down price for FMM (in $/MWh) |
|  | BA5mResourceRTDFlexRampDownTotalPrice Brmdhcif | RTD FRDP, flex ramp down price for RTD (in $/MWh) |
|  | PTB_BAFRForecastedMovementChargeAdjustmentAmount BJmdhcif | PTB charge adjustment for forecasted movement settlement (in $) |

## Inputs - Predecessor Charge Codes or Pre-calculations

| Input Req ID | Variable Name | Predecessor Charge Code/ Pre-calc Configuration |
| --- | --- | --- |
|  | BA5mResFRUForecastedMovementRescissionQuantity BrtQ’uT’I’M’L’F’S’mdhcif | CC 7071 – Flexible Ramp Up Uncertainty Capacity Settlement |
|  | BA5mResFRDForecastedMovementRescissionQuantity BrtQ’uT’I’M’L’F’S’mdhcif | CC 7081 – Flexible Ramp Down Uncertainty Capacity Settlement This quantity is coming in as non-negative value. |
|  | ResourceWholesaleExemptionFlag rmdhcif | Real Time Energy PC |

## CAISO Formula

### BA5mResFRForecastedMovementSettlementAmount BrtQ’uT’I’M’L’F’S’mdhcif =

If

ResourceWholesaleExemptionFlag rmdhcif = 0

THEN

BA5mResTotalFRForecastedMovementAssessmentAmount BrtQ’uT’I’M’L’F’S’mdhcif + BA5mResFRForecastedMovementRescissionAmount BrtQ’uT’I’M’L’F’S’mdhcif

ELSE

0

### BA5mResFMMFlexRampForecastedMovementMWhQuantity BrtQ’uT’I’M’L’F’S’mdhcif =

(1/12)*(INTDUPLICATE(BA15mResourceFMMFlexRampForecastedMovementMWQty BrtQ’uT’I’M’L’F’S’mdhc))

Implementation Note: Each hourly value of   BA15mResourceFMMFlexRampForecastedMovementMWQty BrtQ’uT’I’M’L’F’S’mdhc that comes every fifteen minutes will be duplicated for each of the three relevant five-minute intervals of such fifteen minute interval. All these values are then divided by 12 to convert the MW into MWh.

### BA5mResRTDFlexRampForecastedMovementMWhQuantity BrtQ’uT’I’M’L’F’S’mdhcif =

(1/12)*BA5mResourceRTDFlexRampForecastedMovementMWQty BrtQ’uT’I’M’L’F’S’mdhcif

### BA5mResRTDIncFlexRampForecastedMovementMWhQuantity BrtQ’uT’I’M’L’F’S’mdhcif =

BA5mResRTDFlexRampForecastedMovementMWhQuantity BrtQ’uT’I’M’L’F’S’mdhcif  - BA5mResFMMFlexRampForecastedMovementMWhQuantity BrtQ’uT’I’M’L’F’S’mdhcif

### BA5mResFMMFlexRampForecastedMovementAssessmentAmount BrtQ’uT’I’M’L’F’S’mdhcif =

(-1)*BA5mResFMMFlexRampForecastedMovementMWhQuantity BrtQ’uT’I’M’L’F’S’mdhcif * (BA15mResourceFMMFlexRampUpTotalPrice Brmdhc - BA15mResourceFMMFlexRampDownTotalPrice Brmdhc)

Implementation Note: Both 15-minute prices for flex ramp up and down above will be duplicated in each of the corresponding three five-minute intervals of the 15-minute interval.

### BA5mResRTDFlexRampForecastedMovementAssessmentAmount BrtQ’uT’I’M’L’F’S’mdhcif =

(-1)* BA5mResRTDIncFlexRampForecastedMovementMWhQuantity BrtQ’uT’I’M’L’F’S’mdhcif *  (BA5mResourceRTDFlexRampUpTotalPrice Brmdhcif - BA5mResourceRTDFlexRampDownTotalPrice Brmdhcif )

### BA5mResTotalFRForecastedMovementAssessmentAmount BrtQ’uT’I’M’L’F’S’mdhcif =

BA5mResFMMFlexRampForecastedMovementAssessmentAmount BrtQ’uT’I’M’L’F’S’mdhcif + BA5mResRTDFlexRampForecastedMovementAssessmentAmount BrtQ’uT’I’M’L’F’S’mdhcif

### BA5mResFRForecastedMovementRescissionAmount BrtQ’uT’I’M’L’F’S’mdhcif =

(BA5mResFRUForecastedMovementRescissionQuantity BrtQ’uT’I’M’L’F’S’mdhcif - BA5mResFRDForecastedMovementRescissionQuantity BrtQ’uT’I’M’L’F’S’mdhcif) * (BA5mResourceRTDFlexRampUpTotalPrice Brmdhcif - BA5mResourceRTDFlexRampDownTotalPrice Brmdhcif )

### Total5mFRForecastedMovementSettlementAmount mdhcif =

BrtQ 'uT 'I 'M 'L 'F 'S 'BA5mResFRForecastedMovementSettlementAmount BrtQ’uT’I’M’L’F’S’mdhcif

Note: This will be calculated within the hierarchy of this charge code.

## Outputs

| Output ID | Name | Description |
| --- | --- | --- |
|  | In addition to the outputs below, all inputs are required to be accessible for review by analysts and report on Settlement statements. |  |
|  | BA5mResFRForecastedMovementSettlementAmount BrtQ’uT’I’M’L’F’S’mdhcif | Flex Ramp settlement amount for forecasted movement ($) |
|  | BA5mResFMMFlexRampForecastedMovementMWhQuantity BrtQ’uT’I’M’L’F’S’mdhcif | FMM Flex Ramp forecasted movement MWh quantity (MWh) |
|  | BA5mResRTDFlexRampForecastedMovementMWhQuantity BrtQ’uT’I’M’L’F’S’mdhcif | RTD Flex Ramp forecasted movement MWh quantity (MWh) |
|  | BA5mResRTDIncFlexRampForecastedMovementMWhQuantity BrtQ’uT’I’M’L’F’S’mdhcif | Incremental RTD Flex Ramp forecasted movement MWh quantity (MWh) |
|  | BA5mResFMMFlexRampForecastedMovementAssessmentAmount BrtQ’uT’I’M’L’F’S’mdhcif | FMM Flex Ramp forecasted movement assessment amount ($) |
|  | BA5mResRTDFlexRampForecastedMovementAssessmentAmount BrtQ’uT’I’M’L’F’S’mdhcif | RTD Flex Ramp forecasted movement assessment amount ($) |
|  | BA5mResTotalFRForecastedMovementAssessmentAmount BrtQ’uT’I’M’L’F’S’mdhcif | Total FMM and RTD flex ramp forecasted movement assessment amount ($) This value does not include any rescission amounts. |
|  | BA5mResFRForecastedMovementRescissionAmount BrtQ’uT’I’M’L’F’S’mdhcif | Flex ramp forecasted movement rescission amount ($) |
|  | Total5mFRForecastedMovementSettlementAmount mdhcif | Total Flex Ramp settlement amount for forecasted movement for the EIM Area ($) |

# Charge Code Effective Dates

| Charge Code/ Pre-calc Name | Document Version | Effective Start Date | Effective End Date | Version update Type |
| --- | --- | --- | --- | --- |
| Flexible Ramp Forecasted Movement Settlement (CC 7070) | 5.0 | 11/01/16 | 9/30/2020 | Configuration Impacted |
| Flexible Ramp Forecasted Movement Settlement (CC 7070) | 5.1 | 10/1/2020 | Open | Configuration Impacted |