Settlements & Billing

CG CC 7076 Flexible Ramp Forecast Movement Allocation v5.1

Version 5.1

Table of Contents

1.	Purpose of Document	3

2.	Introduction	3

2.1	Background	3

2.2	Description	3

3.	Charge Code Requirements	3

3.1	Business Rules	3

3.2	Predecessor Charge Codes	4

3.3	Successor Charge Codes	4

3.4	Inputs – External Systems	4

3.5	Inputs - Predecessor Charge Codes or Pre-calculations	5

3.6	CAISO Formula	5

3.7	Outputs	7

4.	Charge Code Effective Dates	8

# Purpose of Document

The purpose of this document is to capture the business and functional requirements of  a charge code in one document.

# Introduction

## Background

The Flexible Ramping product (FRP) is designed to ensure that there is sufficient ramping capability available in the financially binding five-minute interval to meet the forecasted net load for interval t+5 and cover upwards and downwards forecast error uncertainty.

FRP will help the system to maintain and use dispatchable capacity, as well as provide the market more transparent and less volatile price signals when undergoing forecasted ramp-constrained conditions.  It will be procured and dispatched to meet five minute to five minute net forecast (load forecast minus VER forecast) changes plus uncertainty and will be modeled as a ramping capability constraint.

The ISO will financially settle FRP in the fifteen-minute market and the five-minute market, with rescission of payments applied to resources with UIE (uninstructed imbalance energy) or OA (operational adjustment) amounts, positive or negative, which are imposed on reserved FRP capacity awards. Settlement and allocation of FRP costs will happen on a daily basis for forecasted movement portion and uncertainty award portion.  At the end of the month, the uncertainty award allocation will be reversed and will be re-allocated based on the month’s net UIE or OA values.

As no economic bids are applied to FRP, FRU/FRD awards will be exempt from grid management charges (GMC).  Additionally, dispatchable resources will have their FRP awards and forecasted movement assessments - ignoring rescission settlement - included as part of daily RTM bid cost recovery calculations.

## Description

For each Settlement Interval, this charge code, CC 7076 Flexible Ramp Movement Allocation will assess the Residual Forecasted Movement Settlement Amount calculated in CC 7070 Flexible Ramp Up Forecasted Movement Settlement to Scheduling Coordinator with meter EIM Demand or metered CAISO Demand in propotion to its share of the total metered EIM Demand and metered CAISO Demand.

# Charge Code Requirements

## Business Rules

| Bus Req ID | Business Rule |
| --- | --- |
| 1.0 | For each Settlement Interval, this charge code will allocate the Residual Forecast Movement Amount to the BAA based upon the host control area ID. |
| 2.0 | For the EIM_area host control area ID, Settlements shall allocate the costs to the BAAs that pass the sufficiency test. |
| 3.0 | For the forecasted movement cost associated with the BAA that did not pass the sufficiency test, those costs will allocated to the meter demand of that BAA, however if metered demand is not available and there are BAA specific costs, those will be allocated to the entity. |
| 3.0 | The Flex Ramp Forecast Movement Allocation Amount is calculated as the Product of the Scheduling Coordinators metered EIM Demand or metered CAISO Demand and the Flex Ramp Forecast Movement Allocation Price |
| 4.0 | The Flex Ramp Forecast Movement Allocation Price is calculated as the Product of the Residual Forecast Movement Amount divided by the total EIM Area metered Demand |
| 5.0 | For adjustments to the Charge Code that cannot be accomplished by correction of upstream data inputs/recalculation or operator override Pass Through Bill Charge logic will be applied. |

## Predecessor Charge Codes

Charge Code/ Pre-calc Name

CC 7070 – Flexible Ramp Forecasted Movement Settlement

PC – Flexible Ramp Product

## Successor Charge Codes

Charge Code/ Pre-calc Name

CC 4989 - Daily Rounding Adjustment Allocation

## Inputs – External Systems

| Input Req ID | Variable Name | Description |
| --- | --- | --- |
|  | PTBBAFRForecastedMovementAllocAdjustmentAmount BJmdhcif | PTB adjustment for forecasted movement allocation (in $) |
|  | BADayGenOnlyBAAFlag BQ'md | Flag indicating a Gen-Only BAA with 1. |

## Inputs - Predecessor Charge Codes or Pre-calculations

| Input Req ID | Variable Name | Predecessor Charge Code/ Pre-calc Configuration |
| --- | --- | --- |
|  | ### BAA5mFRDForecastedMovementByHostControlAreaSettlementAmount Q’Q’’mdhcif | CC 7070 |
|  | ### BAA5mFRUForecastedMovementByHostControlAreaSettlementAmount Q’Q’’mdhcif | CC 7070 |
|  | EIMArea5mFRDPassGroupMeteredDemandAllocationQuantity mdhcif | PC- Flex Ramp Product |
|  | EIMArea5mFRUPassGroupMeteredDemandAllocationQuantity mdhcif | PC- Flex Ramp Product |
|  | BAA5mBAASpecificFRDMeteredDemandAllocationQuantity Q’mdhcif | PC- Flex Ramp Product |
|  | BAA5mBAASpecificFRUMeteredDemandAllocationQuantity Q’mdhcif | PC- Flex Ramp Product |
|  | BAA5mBABAASpecificFRUMeteredDemandAllocationQuantity BQ’mdhcif | PC- Flex Ramp Product |
|  | BAA5mBABAASpecificFRDMeteredDemandAllocationQuantity BQ’mdhcif | PC- Flex Ramp Product |
|  | BAA5mBAPassGroupFRUMeteredDemandAllocationQuantity BQ’mdhcif | PC- Flex Ramp Product |
|  | BAA5mBAPassGroupFRDMeteredDemandAllocationQuantity BQ’mdhcif | PC- Flex Ramp Product |

## CAISO Formula

### BA5mFlexRampForecastMvmtAllocationAmount Bmdhcif =

BA5mFRDForecastMvmtBAASpecificAllocationAmount Bmdhcif + 
BA5mFRUForecastMvmtBAASpecificAllocationAmount Bmdhcif + BA5mFRDForecastMvmtPassGroupAllocationAmount Bmdhcif + 
BA5mFRUForecastMvmtPassGroupAllocationAmount Bmdhcif

### BA5mFRDForecastMvmtBAASpecificAllocationAmount Bmdhcif =

Sum(Q’)

If BADayGenOnlyBAAFlag BQ'md =1

Then

BADayGenOnlyBAAFlag BQ'md * BAA5mFRDForecastedMovementBAASpecificAllocationAmount Q’mdhcif

Else

(BAA5mBABAASpecificFRDMeteredDemandAllocationQuantity BQ’mdhcif / BAA5mBAASpecificFRDMeteredDemandAllocationQuantity Q’mdhcif) * 
BAA5mFRDForecastedMovementBAASpecificAllocationAmount Q’mdhcif

### BA5mFRUForecastMvmtBAASpecificAllocationAmount Bmdhcif =

Sum(Q’)

If BADayGenOnlyBAAFlag BQ'md =1

Then

BADayGenOnlyBAAFlag BQ'md * BAA5mFRUForecastedMovementBAASpecificAllocationAmount Q’mdhcif

Else

(BAA5mBABAASpecificFRUMeteredDemandAllocationQuantity BQ’mdhcif / BAA5mBAASpecificFRUMeteredDemandAllocationQuantity Q’mdhcif) * 
BAA5mFRUForecastedMovementBAASpecificAllocationAmount Q’mdhcif

### BA5mFRDForecastMvmtPassGroupAllocationAmount Bmdhcif =

Sum(Q’)

(BAA5mBAPassGroupFRDMeteredDemandAllocationQuantity BQ’mdhcif / EIMArea5mFRDPassGroupMeteredDemandAllocationQuantity mdhcif) * 
EIMArea5mFRDForecastedMovementPassGroupAllocationAmount mdhcif

### BA5mFRUForecastMvmtPassGroupAllocationAmount Bmdhcif =

Sum(Q’)

(BAA5mBAPassGroupFRUMeteredDemandAllocationQuantity BQ’mdhcif / EIMArea5mFRUPassGroupMeteredDemandAllocationQuantity mdhcif) * 
EIMArea5mFRUForecastedMovementPassGroupAllocationAmount mdhcif

### BAA5mFRDForecastedMovementBAASpecificAllocationAmount Q’mdhcif = 
Sum (Q’’)
BAA5mFRDForecastedMovementByHostControlAreaSettlementAmount Q’Q’’mdhcif

Where Q’’ <> FRD_PASS_GRP

### BAA5mFRUForecastedMovementBAASpecificAllocationAmount Q’mdhcif =

Sum (Q’’)
BAA5mFRUForecastedMovementByHostControlAreaSettlementAmount Q’Q’’mdhcif

Where Q’’ <> FRU_PASS_GRP

### EIMArea5mFRDForecastedMovementPassGroupAllocationAmount mdhcif =

Sum (Q’Q’’)
BAA5mFRDForecastedMovementByHostControlAreaSettlementAmount Q’Q’’mdhcif

Where Q’’ = FRD_PASS_GRP

### EIMArea5mFRUForecastedMovementPassGroupAllocationAmount mdhcif =

Sum (Q’Q’’)
BAA5mFRUForecastedMovementByHostControlAreaSettlementAmount Q’Q’’mdhcif

Where Q’’ = FRU_PASS_GRP

## Outputs

| Output ID | Name | Description |
| --- | --- | --- |
|  | In addition to the outputs below, all inputs are required to be accessible for review by analysts and report on Settlement statements. |  |
|  | BA5mFlexRampForecastMvmtAllocationAmount Bmdhcif | Flex Ramp Forecast Movement Allocation Amount by Business Associate B for Settlement Interval f |
|  | BA5mFRDForecastMvmtBAASpecificAllocationAmount Bmdhcif | Flex Ramp Down Forecast Movement Allocation Amount by Business Associate B for Settlement Interval f for BAAs that failed their Flex Ramp Down Balancing Test. |
|  | BA5mFRUForecastMvmtBAASpecificAllocationAmount Bmdhcif | Flex Ramp Up Forecast Movement Allocation Amount by Business Associate B for Settlement Interval f for BAAs that failed their Flex Ramp Up Balancing Test. |
|  | BA5mFRDForecastMvmtPassGroupAllocationAmount Bmdhcif | Flex Ramp Down Forecast Movement Allocation Amount by Business Associate B for Settlement Interval f for BAAs that passed their Flex Ramp Down Balancing Test. |
|  | BA5mFRUForecastMvmtPassGroupAllocationAmount Bmdhcif | Flex Ramp Up Forecast Movement Allocation Amount by Business Associate B for Settlement Interval f for BAAs that passed their Flex Ramp Up Balancing Test. |
|  | BAA5mFRDForecastedMovementBAASpecificAllocationAmount Q’mdhcif | Flex Ramp Down Forecast Movement Allocation Amount by BAA Q’ for Settlement Interval f for BAAs that failed their Flex Ramp Down Balancing Test. |
|  | BAA5mFRUForecastedMovementBAASpecificAllocationAmount Q’mdhcif | Flex Ramp Up Forecast Movement Allocation Amount by BAA Q’ for Settlement Interval f for BAAs that failed their Flex Ramp Up Balancing Test. |
|  | EIMArea5mFRDForecastedMovementPassGroupAllocationAmount mdhcif | Flex Ramp Down Forecast Movement Allocation Amount by BAA Q’ for Settlement Interval f for BAAs that passed their Flex Ramp Down Balancing Test. |
|  | EIMArea5mFRUForecastedMovementPassGroupAllocationAmount mdhcif | Flex Ramp Up Forecast Movement Allocation Amount by BAA Q’ for Settlement Interval f for BAAs that passed their Flex Ramp Up Balancing Test. |

# Charge Code Effective Dates

| Charge Code/ Pre-calc Name | Document Version | Effective Start Date | Effective End Date | Version update Type |
| --- | --- | --- | --- | --- |
| Flexible Ramp Forecast Movement Allocation (CC 7076) | 5.0 | 11/01/16 | 9/30/2022 | Configuration Impacted |
| Flexible Ramp Forecasted Movement Settlement (CC 7070) | 5.3 | 10/1/2022 | Open | Configuration Impacted |