Configuration Guide: Daily Flexible Ramp Up Uncertainty Award Allocation

Version 5.2

Table of Contents

1.	Purpose of Document	3

2.	Introduction	3

2.1	Background	3

2.2	Description	3

3.	Charge Code Requirements	3

3.1	Business Rules	3

3.2	Predecessor Charge Codes	7

3.3	Successor Charge Codes	7

3.4	Inputs - External Systems	7

3.5	Inputs - Predecessor Charge Codes or Pre-calculations	8

3.6	CAISO Formula	10

3.7	Outputs	19

4.	Charge Code Effective Dates	25

# Purpose of Document

The purpose of this document is to capture the requirements and design specification for a Charge Code in one document.

# Introduction

## Background

The Flexible Ramping product (FRP) is designed to ensure that there is sufficient ramping capability available in the financially binding five-minute interval to meet the forecasted net load for interval t+5 and cover upwards and downwards forecast error uncertainty.

FRP will help the system to maintain and use dispatchable capacity, as well as provide the market more transparent and less volatile price signals when undergoing forecasted ramp-constrained conditions.  It will be procured and dispatched to meet five minute to five minute net forecast (load forecast minus VER forecast) changes plus uncertainty and will be modeled as a ramping capability constraint.

The ISO will financially settle FRP in the fifteen-minute market and the five-minute market, with rescission of payments applied to resources with UIE (uninstructed imbalance energy) or OA (operational adjustment) amounts, positive or negative, which are imposed on reserved FRP capacity awards. Settlement and allocation of FRP costs will happen on a daily basis for forecasted movement portion and uncertainty award portion.  At the end of the month, the uncertainty award allocation will be reversed and will be re-allocated based on the month’s net UIE or OA values.

As no economic bids are applied to FRP, FRU/FRD awards will be exempt from grid management charges (GMC).  Additionally, dispatchable resources will have their FRP awards and forecasted movement assessments - ignoring rescission settlement - included as part of daily RTM bid cost recovery calculations.

## Description

For each Balancing Authority Area (BAA) and Trading Day this charge code configuration on a daily basis shall allocate the charges associated with the total Flexible Ramp Up Uncertainty capacity award settlement amounts that are paid in association with charge code CC 7071 (Daily Flexible Ramp Up Uncertainty Capacity Settlement) to the BAA in accord with the business rules and attendant allocation criteria stipulated in Business Rules section 3.1 below.

# Charge Code Requirements

## Business Rules

| Bus Req ID | Business Rule |
| --- | --- |
|  | FRU and FRD uncertainty movement payments and charges shall apply daily with monthly resettlement of the FRU and FRD uncertainty movement charges. |
|  | The CAISO will allocate: |
|  | - the cost of the Uncertainty Award within each Balancing Authority Area in the EIM Area and within the EIM Area on a daily basis according to resource categories; |
|  | - the daily amounts to Scheduling Coordinators. |
|  | For each interval, the CAISO will calculate the net Uncertainty Movement of each resource as follow: |
|  | - for Supply resources other than non-Dynamic System Resources as the difference between the Dispatch Instruction of the binding interval in the next RTD run and the first advisory RTD interval in the current run. |
|  | - for non-Dynamic System Resource and export schedule as the difference between the schedule used in the RTD (accounting for ramp) for the binding interval in the next RTD run and the scheduled use for the first advisory interval in the current RTD run. |
|  | The CAISO will determine the total net RTD Uncertainty Movement for each category separately for each Balancing Authority Area in the EIM Area and by EIM Area as follows: |
|  | - for the category of Supply resources, which shall not include non-Dynamic System Resources, as the net sum of the five-minute Uncertainty Movement of all the Supply resources in the category; |
|  | - for the category of Intertie resources, which shall comprise non-Dynamic System Resources and exports, as the net sum of the five-minute Uncertainty Movement determined of all the non-Dynamic System resources and export schedules; |
|  | - for the non-Participating Load category, as the difference between- - the CAISO Forecast of CAISO Demand, the CAISO forecast of Balancing Authority Area EIM Demand, or the CAISO forecast of EIM Area EIM Demand, as applicable, of the binding interval in the next RTD run; and - the CAISO Forecast of CAISO Demand, the CAISO forecast of Balancing Authority Area EIM Demand, or the CAISO forecast of EIM Area EIM Demand, for the first advisory interval in the current RTD run. |
|  | The CAISO will allocate the total upward Uncertainty Award cost to each of the three (3) resource categories based on — |
|  | - for upward Uncertainty Award cost, the ratio of such category’s positive Uncertainty Movement to the sum of the positive Uncertainty Movements of all categories with positive Uncertainty Movement for - each Balancing Authority Area in the EIM Area and - the EIM Area; and, |
|  | - for downward Uncertainty Award cost, the ratio of such category’s positive Uncertainty Movement to the sum of the negative Uncertainty Movements of all categories with negative Uncertainty Movement for - each Balancing Authority Area in the EIM Area and - the EIM Area. |
|  | The CAISO will allocate the Uncertainty Awards costs of the non-Participating Load category to Scheduling Coordinators as follows: |
|  | - for upward Uncertainty Award cost in proportion to the Scheduling Coordinator’s negative non-Participating Load UIE, excluding the non-Participating Load of an MSS that has elected to load-follow according to an MSS Agreement, without netting that UIE across Settlement Intervals, to the total of such negative non-Participating Load UIE, without netting that UIE across Settlement Intervals, in the Balancing Authority Area or EIM Area as applicable; |
|  | The allocated upward Uncertainty Award costs shall signify an increase in the Scheduling Coordinator’s Demand (i.e., more Demand than expected). |
|  | - for downward Uncertainty Award cost in proportion to the Scheduling Coordinator’s daily positive non-Participating Load UIE, excluding the non-Participating Load of an MSS that has elected to load-follow according to an MSS Agreement, without netting that UIE across Settlement Intervals, to the total of such positive non-Participating Load UIE, without netting that UIE across Settlement Intervals, in the BAA or EIM Area as applicable. |
|  | The allocated upward Uncertainty Award costs shall signify a decrease in the Scheduling Coordinator’s Demand (i.e., less Demand than expected). |
|  | The CAISO will allocate the Uncertainty Awards costs of the Supply category to Scheduling Coordinators for each resource in the Supply category based on the sum of the resource’s Uncertainty Movement and UIE as follows: |
|  | - for upward Uncertainty Award cost in proportion to the Scheduling Coordinator’s negative sum of the resource’s Uncertainty Movement and UIE, without netting that sum across Settlement Intervals, to the total negative sum of all resources’ Uncertainty Movement and UIE, without netting that sum across Settlement Intervals, in the Balancing Authority Area or EIM Area as applicable, except as stipulated in 3) below; |
|  | The allocated upward Uncertainty Award costs shall signify a decrease in the Scheduling Coordinator’s Generation (i.e., less Generation than expected). |
|  | - for downward Uncertainty Award cost in proportion to the Scheduling Coordinator’s positive sum of the resource’s Uncertainty Movement and UIE, without netting that sum across Settlement Intervals, to the total positive sum of all resources’ Uncertainty Movement and UIE, without netting that sum across Settlement Intervals, in the BAA or EIM Area as applicable, except as stipulated in 3) below; |
|  | The allocated upward Uncertainty Award costs shall signify an increase in the Scheduling Coordinator’s Generation (i.e., more Generation than expected). |
|  | - for the MSS that have elected to load follow pursuant to an MSS Agreement, the CAISO will calculate the positive and negative sums specified above for each Settlement Interval as the sum of MSS non-Participating Load UIE, Supply resources within the MSS UIE, MSS Load Following Energy, MSS Load Following Operational Adjustments, and Uncertainty Movement of resources within the MSS Aggregation. |
|  | The CAISO will allocate the Uncertainty Awards costs of the Intertie category to Scheduling Coordinators for each non-Dynamic System Resource and export based on the sum of the resource’s Uncertainty Movement and Operational Adjustment as follows: |
|  | - for upward Uncertainty Award cost in proportion to the magnitude of the Scheduling Coordinator’s negative Operational Adjustment for non-Dynamic System Resources, or positive Operational Adjustment for export resources (where export Energy is considered positive) relative to the sum of magnitude of such Operational Adjustments in the Balancing Authority Area or EIM Area, as applicable, without netting that sum across Settlement Intervals; |
|  | The allocated upward Uncertainty Award costs shall signify a decrease in the Scheduling Coordinator’s import Generation (i.e., imported Energy less than expected) or an increase in the Scheduling Coordinator’s export Generation (i.e., exported Energy more than expected). |
|  | - for downward Uncertainty Award cost in in proportion to the magnitude of the Scheduling Coordinator’s positive Operational Adjustment for non-Dynamic System Resources, or magnitude of negative Operational Adjustment for export resources (where export Energy is considered positive) relative to the sum of the magnitude of such Operational Adjustments in the Balancing Authority Area or EIM Area, as applicable, without netting that sum across Settlement Intervals; |
|  | The allocated upward Uncertainty Award costs shall signify an increase in the Scheduling Coordinator’s import Generation (i.e., imported Energy more than expected) or a decrease in the Scheduling Coordinator’s export Generation (i.e., exported Energy less than expected). |
|  | - for the purposes of the allocations specified above, the MSS Load Following Operational Adjustment is excluded. |
|  | If the sum of the settlement of Uncertainty Awards and the charges to Scheduling Coordinators for Uncertainty Award costs is nonzero, the CAISO will allocate such amounts to Scheduling Coordinators based on the ratio of its metered CAISO Demand and metered EIM Demand to the total EIM area metered demand. |
|  | For adjustments to the Charge Code that cannot be accomplished by correction of upstream data inputs/recalculation or operator override, Pass Through Bill Charge logic will be applied. |
|  | When an eligible resource has an interval with a negative MWh meter, CAISO will not charge for the energy of those intervals. |

## Predecessor Charge Codes

Charge Code/ Pre-calc Name

Pre-calculation MSS Netting

Pre-calculation Real Time Energy Quantity

CC 7071 Daily Flexible Ramp Up Uncertainty Capacity Settlement

## Successor Charge Codes

Charge Code/ Pre-calc Name

CC 4989 Daily Rounding Adjustment Allocation

CC 7078 Monthly Flexible Ramp Up Uncertainty Award Allocation

## Inputs - External Systems

| Row # | Variable Name | Description |
| --- | --- | --- |
|  | BA5mResourceRTDFlexRampUncertaintyMovementQty BrtQ’uT’I’M’L’F’S’mdhcif | The RTD-specific Flexible Ramping Uncertainty movement (in MW) that is identified for a specified supply or intertie resource. |
|  | MSSResourceInfo BrtuT'I'M'AA'VpL'md | A flag (0/1), that, when = 1, indicates that the identified MSS resource is load-following. |
|  | BAA5mTotalLoadUncertaintyQty Q’’mdhcif | The total of Flexible Ramping Uncertainty (in MW) from the Load Uncertainty category of resources in association with a BAA constraint. A positive value represents FRU Uncertainty, while a negative value represents FRD Uncertainty. |
|  | BAA5mTotalIntertieUncertaintyQty Q’’mdhcif | The total of Flexible Ramping Uncertainty (in MW) from the intertie Uncertainty category of resources in association with a BAA constraint. A positive value represents FRU Uncertainty, while a negative value represents FRD Uncertainty. |
|  | BAA5mTotalSupplyUncertaintyQty Q’’mdhcif | The total of Flexible Ramping Uncertainty (in MW) from the Supply Uncertainty category of resources in association with a BAA constraint. A positive value represents FRU Uncertainty, while a negative value represents FRD Uncertainty. |
|  | PTBBAADayFRUUncertaintyAllocAmt BJmd | PTB charge adjustment for FRU Uncertainty Allocation (in $). |

## Inputs - Predecessor Charge Codes or Pre-calculations

| Row # | Variable Name | Predecessor Charge Code/ Pre-calc Configuration |
| --- | --- | --- |
|  | EIMAreaSettlementIntervalTotalMeteredDemandQuantity mdhcif | Pre-calculation MSS Netting |
|  | BASettlementIntervalMeteredDemandQuantity BuT’I’Q’M’AA’W’VL’mdhcif | Pre-calculation MSS Netting |
|  | SettlementIntervalRealTimeUIE BrtuT’I’Q’M’F’S’mdhcif | Pre-calculation Real Time Energy Quantity |
|  | SettlementIntervalOAEnergy BrtuT’I’Q’M’F’S’mdhcif | Pre-calculation Real Time Energy Quantity |
|  | SettlementIntervalMSSIIE BrtuT’I’Q’M’F’S’mdhcif | Pre-calculation Real Time Energy Quantity |
|  | SettlementIntervalMSSLFOAEnergy BrtuT’I’M’F’S’mdhcif | Pre-calculation Real Time Energy Quantity |
|  | SettlementIntervalMSSLFMeteredEnergyDifference BrtuT’I’Q’M’F’S’mdhcif | Pre-calculation Real Time Energy Quantity |
|  | BAAConstraint5mFlexRampUpUncertaintyAmount Q’Q’’mdhcif | CC 7071 Daily Flexible Ramp Up Uncertainty Capacity Settlement |
|  | ResourceWholesaleExemptionFlag rmdhcif | Pre-calculation Real Time Energy Quantity |

## CAISO Formula

The CAISO formula for Daily Flexible Ramp Up Uncertainty charge allocation by BA ID (B) is as follows:

    - BADailyCompleteFRUUncertaintyAllocationAmount BQ’md =

hcf∈c (BA5mCompleteFRUUncertaintyAllocationAmount BQ’mdhcif )

    - BA5mCompleteFRUUncertaintyAllocationAmount BQ’mdhcif =

(BA5mFRUEIMAreaMeteredDemandAllocatedUncertaintyAmount BQ’mdhcif + BA5mFRUCategorySpecificAllocatedUncertaintyAmount BQ’mdhcif )

    - BA5mFRUEIMAreaMeteredDemandAllocatedUncertaintyAmount BQ’mdhcif =

(BAA5mBAMeteredDemandAllocationQuantity BQ’mdhcif / EIMAreaSettlementIntervalTotalMeteredDemandQuantity mdhcif ) * EIMArea5mFRUNeutralityMeteredDemandAllocatedAmount mdhcif

    - EIMArea5mFRUNeutralityMeteredDemandAllocatedAmount mdhcif =

EIMArea5mFRUUncertaintyAllocationAmount mdhcif –

EIMArea5mFRUAllocatedUncertaintyAmount mdhcif

    - EIMArea5mFRUUncertaintyAllocationAmount mdhcif =

Q'Q'' (-1) * BAAConstraint5mFlexRampUpUncertaintyAmount Q’Q’’mdhcif

    - EIMArea5mFRUAllocatedUncertaintyAmount mdhcif =

BQ'BA5mFRUCategorySpecificAllocatedUncertaintyAmount BQ’mdhcif

    - BA5mFRUCategorySpecificAllocatedUncertaintyAmount BQ’mdhcif =

rtF'S'(BA5mResourceBAALoadFRUUncertaintyAllocationAmount BrtF’S’Q’mdhcif +

BA5mResourceBAAIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’mdhcif +

BA5mResourceBAASupplyFRUUncertaintyAllocationAmount BrtF’S’Q’mdhcif )

    - BA5mResourceBAALoadFRUUncertaintyAllocationAmount BrtF’S’Q’mdhcif =

IF

BAA5mTotalLoadFRUUncertaintyAllocationQuantity Q’mdhcif <> 0

THEN

BA5mResourceBAALoadFRUUncertaintyAllocationAmount BrtF’S’Q’mdhcif =

(BA5mResourceBAALoadFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif / BAA5mTotalLoadFRUUncertaintyAllocationQuantity Q’mdhcif ) * BAA5mLoadCategoryFRUUncertaintyAllocationAmount Q’mdhcif

ELSE

BA5mResourceBAALoadFRUUncertaintyAllocationAmount BrtF’S’Q’mdhcif =

0

END IF

    - EIMArea5mTotalLoadFRUUncertaintyAllocationQuantity mdhcif =

Q'BAA5mTotalLoadFRUUncertaintyAllocationQuantity Q’mdhcif

    - BAA5mTotalLoadFRUUncertaintyAllocationQuantity Q’mdhcif =

BrtF'S' BA5mResourceBAALoadFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - BA5mResourceBAALoadFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif =

uT 'I 'M 'Min(0, (1 – MSSLoadFollowingResourceFlag BrtuT'I'M'md ) * SettlementIntervalRealTimeUIE BrtuT’I’Q’M’F’S’mdhcif)

Where

t = ‘LOAD’

### BA5mResourceBAAIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’mdhcif =

IF

BAA5mTotalIntertieFRUUncertaintyAllocationQuantity Q’mdhcif <> 0

THEN

BA5mResourceBAAIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’mdhcif =

(BA5mResourceBAAIntertieFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif / BAA5mTotalIntertieFRUUncertaintyAllocationQuantity Q’mdhcif ) * BAA5mIntertieCategoryFRUUncertaintyAllocationAmount Q’mdhcif

ELSE

BA5mResourceBAAIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’mdhcif =

0

END IF

    - EIMArea5mTotalIntertieFRUUncertaintyAllocationQuantity mdhcif =

Q'BAA5mTotalIntertieFRUUncertaintyAllocationQuantity Q’mdhcif

    - BAA5mTotalIntertieFRUUncertaintyAllocationQuantity Q’mdhcif =

BrtF'S'BA5mResourceBAAIntertieFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - BA5mResourceBAAIntertieFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif =

uT 'I 'M ' Min(0, SettlementIntervalOAEnergy BrtuT’I’Q’M’F’S’mdhcif – SettlementIntervalMSSLFOAEnergy BrtuT’I’M’F’S’mdhcif )

Where

Resource Type (t) IN { ‘ITIE’, ‘ETIE’ } And Entity Component Type (F’) <> ‘TG’

    - BA5mResourceBAASupplyFRUUncertaintyAllocationAmount BrtF’S’Q’mdhcif =

IF

BAA5mTotalSupplyFRUUncertaintyAllocationQuantity Q’mdhcif <> 0

THEN

BA5mResourceBAASupplyFRUUncertaintyAllocationAmount BrtF’S’Q’mdhcif =

(BA5mResourceBAASupplyFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif / BAA5mTotalSupplyFRUUncertaintyAllocationQuantity Q’mdhcif ) * BAA5mSupplyCategoryFRUUncertaintyAllocationAmount Q’mdhcif

ELSE

BA5mResourceBAASupplyFRUUncertaintyAllocationAmount BrtF’S’Q’mdhcif =

0

END IF

    - EIMArea5mTotalSupplyFRUUncertaintyAllocationQuantity mdhcif =

Q'BAA5mTotalSupplyFRUUncertaintyAllocationQuantity Q’mdhcif

    - BAA5mTotalSupplyFRUUncertaintyAllocationQuantity Q’mdhcif =

BrtF'S' BA5mResourceBAASupplyFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - BA5mResourceBAASupplyFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif =

BA5mResourceBAAGenerationSupplyFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif + BA5mResourceBAAMSSLFSupplyFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - BA5mResourceBAAGenerationSupplyFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif =

uT 'I 'M '

Min(0, (1 – MSSLoadFollowingResourceFlag BrtuT'I'M'md ) * (BA5mResourceRTDFlexRampActualUncertaintyMovementQuantity BrtuT’I’Q’M’F’S’mdhcif + (1- ResourceWholesaleExemptionFlag rmdhcif)*SettlementIntervalRealTimeUIE BrtuT’I’Q’M’F’S’mdhcif ) )

Where

Resource Type (t) = ‘GEN’ Or Entity Component Type (F’) = ‘TG’

    - BA5mResourceBAAMSSLFSupplyFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

IF

BA5mBAAMSSLoadFollowingSupplyFRUUncertaintyAllocationQuantity BQ’mdhcif = 0

THEN

BA5mResourceBAAMSSLFSupplyFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif = 0

ELSE

BA5mResourceBAAMSSLFSupplyFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif =

BA5mBAAMSSLoadFollowingSupplyFRUUncertaintyAllocationQuantity BQ’mdhcif * (BA5mResBAAMSSLFFRURatioAllocationQuantity BrtF’S’Q’mdhcif/ BA5mBAAMSSLFFRUAggregationRatioAllocationQuantity BQ’mdhcif)

END IF

    - BA5mBAAMSSLFFRUAggregationRatioAllocationQuantity BQ’mdhcif =

rtF'S'( BA5mResBAAMSSLFFRURatioAllocationQuantity BrtF’S’Q’mdhcif)

    - BA5mResBAAMSSLFFRURatioAllocationQuantity BrtF’S’Q’mdhcif =

Min (0, BA5mRSRCBAAMSSLoadFollowingFRUncertaintyAllocationQuantity BrtF’S’Q’mdhcif )

    - BA5mBAAMSSLoadFollowingSupplyFRUUncertaintyAllocationQuantity BQ’mdhcif =

Min (0, BA5mBAAMSSLoadFollowingFRUncertaintyAllocationQuantity BQ’mdhcif)

    - BA5mBAAMSSLoadFollowingFRUncertaintyAllocationQuantity BQ’mdhcif =

rtF'S'( BA5mRSRCBAAMSSLoadFollowingFRUncertaintyAllocationQuantity BrtF’S’Q’mdhcif )

    - BA5mRSRCBAAMSSLoadFollowingFRUncertaintyAllocationQuantity BrtF’S’Q’mdhcif =

uT 'I 'M '(MSSLoadFollowingResourceFlag BrtuT'I'M'md * (SettlementIntervalRealTimeUIE BrtuT’I’Q’M’F’S’mdhcif + SettlementIntervalMSSIIE BrtuT’I’Q’M’F’S’mdhcif ) +BA5mResourceRTDFlexRampLFActualUncertaintyMovementQuantity BrtuT’I’Q’M’F’S’mdhcif + SettlementIntervalMSSLFOAEnergy BrtuT’I’M’F’S’mdhcif)

Where Exists

SettlementIntervalRealTimeUIE BrtuT’I’Q’M’F’S’mdhcif, SettlementIntervalMSSIIE BrtuT’I’Q’M’F’S’mdhcif, SettlementIntervalMSSLFMeteredEnergyDifference BrtuT’I’Q’M’F’S’mdhcif

    - BA5mResourceRTDFlexRampActualUncertaintyMovementQuantity BrtuT’I’Q’M’F’S’mdhcif =

L'(BA5mResourceRTDFlexRampUncertaintyMovementQty BrtQ’uT’I’M’L’F’S’mdhcif / 12)

    - BA5mResourceRTDFlexRampLFActualUncertaintyMovementQuantity BrtuT’I’Q’M’F’S’mdhcif =

L'(BA5mResourceRTDFlexRampUncertaintyMovementQty BrtQ’uT’I’M’L’F’S’mdhcif / 12)

Where

Load Following Resource (L’) = YES

    - MSSLoadFollowingResourceFlag BrtuT'I'M'md =

AA''VpL' MSSResourceInfo BrtuT'I'M'AA'VpL'md

Where

Load Following Resource (L’) = YES

    - BAA5mLoadCategoryFRUUncertaintyAllocationAmount Q’mdhcif =

BAA5mLoadCategoryBAAConstraintFRUUncertaintyAllocationAmount Q’mdhcif + BAA5mLoadCategoryEIMAreaConstraintFRUUncertaintyAllocationAmount Q’mdhcif

    - BAA5mLoadCategoryEIMAreaConstraintFRUUncertaintyAllocationAmount Q’mdhcif =

IF

EIMArea5mTotalLoadFRUUncertaintyAllocationQuantity mdhcif <> 0

THEN

BAA5mLoadCategoryEIMAreaConstraintFRUUncertaintyAllocationAmount Q’mdhcif =

EIMArea5mLoadCategoryFRUUncertaintyAllocationAmount mdhcif * (BAA5mTotalLoadFRUUncertaintyAllocationQuantity Q’mdhcif / EIMArea5mTotalLoadFRUUncertaintyAllocationQuantity mdhcif )

ELSE

BAA5mLoadCategoryEIMAreaConstraintFRUUncertaintyAllocationAmount Q’mdhcif = 0

END IF

    - EIMArea5mLoadCategoryFRUUncertaintyAllocationAmount mdhcif =

Q'BAA5mLoadCategoryConstraintFRUUncertaintyAllocationAmount Q’mdhcif

Where

Q’ = ‘EIM_AREA’

    - BAA5mLoadCategoryBAAConstraintFRUUncertaintyAllocationAmount Q’mdhcif =

BAA5mLoadCategoryConstraintFRUUncertaintyAllocationAmount Q’mdhcif

Where

Q’ <> ‘EIM_AREA’

    - BAA5mLoadCategoryConstraintFRUUncertaintyAllocationAmount Q’mdhcif =

Q''AttributeSwap (Q’, Q’’) On BAA5mConstraintLoadCategoryFRUUncertaintyAllocationAmount Q’Q’’mdhcif

    - BAA5mConstraintLoadCategoryFRUUncertaintyAllocationAmount Q’Q’’mdhcif =

IF

BAA5mAllCategoriesFRUUncertaintyQuantity Q’’mdhcif <> 0

THEN

BAA5mConstraintLoadCategoryFRUUncertaintyAllocationAmount Q’Q’’mdhcif =

(-1) * BAAConstraint5mFlexRampUpUncertaintyAmount Q’Q’’mdhcif * (BAA5mLoadFRUUncertaintyQuantity Q’’mdhcif / BAA5mAllCategoriesFRUUncertaintyQuantity Q’’mdhcif )

ELSE

BAA5mConstraintLoadCategoryFRUUncertaintyAllocationAmount Q’Q’’mdhcif = 0

END IF

Note:

BAAConstraint5mFlexRampUpUncertaintyAmount Q’Q’’mdhcif represents the total FRU uncertainty movement settlement amount for a particular BAA (Q’) and flexible ramping constraint ID (Q’’), as the sum of:

- FMM FRU uncertainty settlement amount,

- RTD FRU uncertainty settlement amount and

- FRU rescission settlement amount.

The amount shall include the effect of any PTB adjustment to the FRU uncertainty movement settlement amount.

    - BAA5mIntertieCategoryFRUUncertaintyAllocationAmount Q’mdhcif =

BAA5mIntertieCategoryBAAConstraintFRUUncertaintyAllocationAmount Q’mdhcif + BAA5mIntertieCategoryEIMAreaConstraintFRUUncertaintyAllocationAmount Q’mdhcif

    - BAA5mIntertieCategoryEIMAreaConstraintFRUUncertaintyAllocationAmount Q’mdhcif =

IF

EIMArea5mTotalIntertieFRUUncertaintyAllocationQuantity mdhcif <> 0

THEN

BAA5mIntertieCategoryEIMAreaConstraintFRUUncertaintyAllocationAmount Q’mdhcif =

EIMArea5mIntertieCategoryFRUUncertaintyAllocationAmount mdhcif * (BAA5mTotalIntertieFRUUncertaintyAllocationQuantity Q’mdhcif / EIMArea5mTotalIntertieFRUUncertaintyAllocationQuantity mdhcif )

ELSE

BAA5mIntertieCategoryEIMAreaConstraintFRUUncertaintyAllocationAmount Q’mdhcif = 0

END IF

    - EIMArea5mIntertieCategoryFRUUncertaintyAllocationAmount mdhcif =

Q'BAA5mIntertieCategoryConstraintFRUUncertaintyAllocationAmount Q’mdhcif

Where

Q’ = ‘EIM_AREA’

    - BAA5mIntertieCategoryBAAConstraintFRUUncertaintyAllocationAmount Q’mdhcif =

BAA5mIntertieCategoryConstraintFRUUncertaintyAllocationAmount Q’mdhcif

Where

Q’ <> ‘EIM_AREA’

    - BAA5mIntertieCategoryConstraintFRUUncertaintyAllocationAmount Q’mdhcif =

Q''AttributeSwap (Q’, Q’’) On BAA5mConstraintIntertieCategoryFRUUncertaintyAllocationAmount Q’Q’’mdhcif

    - BAA5mConstraintIntertieCategoryFRUUncertaintyAllocationAmount Q’Q’’mdhcif =

IF

BAA5mAllCategoriesFRUUncertaintyQuantity Q’Q’’mdhcif <> 0

THEN

BAA5mConstraintIntertieCategoryFRUUncertaintyAllocationAmount Q’Q’’mdhcif =

(-1) * BAAConstraint5mFlexRampUpUncertaintyAmount Q’Q’’mdhcif * (BAA5mIntertieFRUUncertaintyQuantity Q’’mdhcif / BAA5mAllCategoriesFRUUncertaintyQuantity Q’’mdhcif )

ELSE

BAA5mConstraintIntertieCategoryFRUUncertaintyAllocationAmount Q’Q’’mdhcif = 0

END IF

    - BAA5mSupplyCategoryFRUUncertaintyAllocationAmount Q’mdhcif =

BAA5mSupplyCategoryBAAConstraintFRUUncertaintyAllocationAmount Q’mdhcif + BAA5mSupplyCategoryEIMAreaConstraintFRUUncertaintyAllocationAmount Q’mdhcif

    - BAA5mSupplyCategoryEIMAreaConstraintFRUUncertaintyAllocationAmount Q’mdhcif =

IF

EIMArea5mTotalSupplyFRUUncertaintyAllocationQuantity mdhcif <> 0

THEN

BAA5mSupplyCategoryEIMAreaConstraintFRUUncertaintyAllocationAmount Q’mdhcif =

EIMArea5mSupplyCategoryFRUUncertaintyAllocationAmount mdhcif * (BAA5mTotalSupplyFRUUncertaintyAllocationQuantity Q’mdhcif / EIMArea5mTotalSupplyFRUUncertaintyAllocationQuantity mdhcif )

ELSE

BAA5mSupplyCategoryEIMAreaConstraintFRUUncertaintyAllocationAmount Q’mdhcif = 0

END IF

    - EIMArea5mSupplyCategoryFRUUncertaintyAllocationAmount mdhcif =

Q'BAA5mSupplyCategoryConstraintFRUUncertaintyAllocationAmount Q’mdhcif

Where

Q’ = ‘EIM_AREA’

    - BAA5mSupplyCategoryBAAConstraintFRUUncertaintyAllocationAmount Q’mdhcif =

BAA5mSupplyCategoryConstraintFRUUncertaintyAllocationAmount Q’mdhcif

Where

Q’ <> ‘EIM_AREA’

    - BAA5mSupplyCategoryConstraintFRUUncertaintyAllocationAmount Q’mdhcif =

Q''AttributeSwap (Q’, Q’’) On BAA5mConstraintSupplyCategoryFRUUncertaintyAllocationAmount Q’Q’’mdhcif

    - BAA5mConstraintSupplyCategoryFRUUncertaintyAllocationAmount Q’Q’’mdhcif =

IF

BAA5mAllCategoriesFRUUncertaintyQuantity Q’’mdhcif <> 0

THEN

BAA5mConstraintSupplyCategoryFRUUncertaintyAllocationAmount Q’Q’’mdhcif =

(-1) * BAAConstraint5mFlexRampUpUncertaintyAmount Q’Q’’mdhcif * (BAA5mSupplyFRUUncertaintyQuantity Q’’mdhcif / BAA5mAllCategoriesFRUUncertaintyQuantity Q’’mdhcif )

ELSE

BAA5mSupplyCategoryFRUUncertaintyAllocationAmount Q’Q’’mdhcif = 0

END IF

    - BAA5mAllCategoriesFRUUncertaintyQuantity Q’’mdhcif =

BAA5mLoadFRUUncertaintyQuantity Q’’mdhcif + 
BAA5mIntertieFRUUncertaintyQuantity Q’’mdhcif + 
BAA5mSupplyFRUUncertaintyQuantity Q’’mdhcif

    - BAA5mLoadFRUUncertaintyQuantity Q’’mdhcif =

Max(0, BAA5mTotalLoadUncertaintyQty Q’’mdhcif )

    - BAA5mIntertieFRUUncertaintyQuantity Q’’mdhcif =

Max(0, BAA5mTotalIntertieUncertaintyQty Q’’mdhcif )

    - BAA5mSupplyFRUUncertaintyQuantity Q’’mdhcif =

Max(0, BAA5mTotalSupplyUncertaintyQty Q’’mdhcif )

Metered Demand Allocation Calculations

    - BAA5mBAMeteredDemandAllocationQuantity BQ’mdhcif =

uT'I'M'AA'W'VL'BASettlementIntervalMeteredDemandQuantity BuT’I’Q’M’AA’W’VL’mdhcif

## Outputs

| Row # | Name | Description |
| --- | --- | --- |
| -- | In addition to any outputs listed below, all inputs shall be included as outputs. |  |
|  | BADailyCompleteFRUUncertaintyAllocationAmount BQ’md | FRU Uncertainty Charge (in $) allocated to a BA of resources throughout the EIM Area as a share of the cost of the FRU award settlement amount for the Trading Day. |
|  | BA5mCompleteFRUUncertaintyAllocationAmount BQ’mdhcif | FRU Uncertainty Charge (in $) allocated to the scheduling BA of resources throughout the EIM Area, as a share of the cost of the FRU award settlement amount for the Trading Day. |
|  | BA5mFRUEIMAreaMeteredDemandAllocatedUncertaintyAmount BQ’mdhcif | FRU Uncertainty Charge (in $) allocated to a BA as a share of the FRU award settlement amount for uncertainty movement that was not identified with any particular BAA. |
|  | EIMArea5mFRUNeutralityMeteredDemandAllocatedAmount mdhcif | The FRU Uncertainty Charge (in $) resulting from the FRU award settlement amount (in $) for uncertainty movement that was not identified with any particular BAA. |
|  | EIMArea5mFRUUncertaintyAllocationAmount mdhcif | The amount (in $) representing the total charge allocation for FRU uncertainty movement over the EIM Area, as the product of minus 1 (-1) times the sum of FMM FRU uncertainty settlement amount, RTD FRU uncertainty settlement amount and FRU rescission settlement amount. The amount includes the effect of any PTB adjustment to the FRU uncertainty movement settlement amount. The amount should be positive to reflect an actual charge. |
|  | EIMArea5mFRUAllocatedUncertaintyAmount mdhcif | The sum of FRU Uncertainty Charges (in $) associated with the FRU uncertainty award settlement amounts relating to the uncertainty movement of each BAA. |
|  | BA5mFRUCategorySpecificAllocatedUncertaintyAmount BQ’mdhcif | The FRU Uncertainty charge (in $) that reflects the total of FRU award settlement amounts allocated to resources of a specified BAA and scheduling BA based on the resources’ relative share of FRU uncertainty movement within the BAA. |
|  | BA5mResourceBAALoadFRUUncertaintyAllocationAmount BrtF’S’Q’mdhcif | The FRU Uncertainty charge component (in $) that reflects the total of a BAA-specific FRU award settlement amount that is allocated to a Load resource based on its relative share of FRU uncertainty movement within the BAA. |
|  | EIMArea5mTotalLoadFRUUncertaintyAllocationQuantity mdhcif | The total FRU uncertainty (in MWh) that is attributable to all of the Load resources over the entire EIM Area. |
|  | BAA5mTotalLoadFRUUncertaintyAllocationQuantity Q’mdhcif | The total FRU uncertainty (in MWh) that is attributable to all of the Load resources over a BAA. |
|  | BA5mResourceBAALoadFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif | The total FRU uncertainty (in MWh) that is attributable to a Load resource associated with the specified BAA. |
|  | BA5mResourceBAAIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’mdhcif | The FRU Uncertainty charge component (in $) that reflects the total of a BAA-specific FRU award settlement amount that is allocated to an intertie resource based on its relative share of FRU uncertainty movement within the BAA. |
|  | EIMArea5mTotalIntertieFRUUncertaintyAllocationQuantity mdhcif | The total FRU uncertainty (in MWh) that is attributable to all of the intertie resources over the entire EIM Area. |
|  | BAA5mTotalIntertieFRUUncertaintyAllocationQuantity Q’mdhcif | The total FRU uncertainty (in MWh) that is attributable to all of the intertie resources over a BAA. |
|  | BA5mResourceBAAIntertieFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif | The total FRU uncertainty (in MWh) that is attributable to an intertie resource associated with the specified BAA. |
|  | BA5mResourceBAASupplyFRUUncertaintyAllocationAmount BrtF’S’Q’mdhcif | The FRU Uncertainty charge component (in $) that reflects the total of a BAA-specific FRU award settlement amount that is allocated to an supply resource based on its relative share of FRU uncertainty movement within the BAA. |
|  | EIMArea5mTotalSupplyFRUUncertaintyAllocationQuantity mdhcif | The total FRU uncertainty (in MWh) that is attributable to all of the supply resources over the entire EIM Area. |
|  | BAA5mTotalSupplyFRUUncertaintyAllocationQuantity Q’mdhcif | The total FRU uncertainty (in MWh) that is attributable to all of the supply resources over a BAA. |
|  | BA5mResourceBAASupplyFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif | The total FRU uncertainty (in MWh) that is attributable to a supply resource associated with the specified BAA. |
|  | BA5mResourceBAAGenerationSupplyFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif | The total FRU uncertainty (in MWh) that is attributable to a BAA resource that is a generating unit or dynamic system resource that is not a MSS load-following resource. |
|  | BA5mResourceBAAMSSLFSupplyFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif | MSS Load Following Supply Flexible Ramp Up Uncertainty Allocation Quantity by Business Associate ID and resource |
|  | BA5mBAAMSSLFFRUAggregationRatioAllocationQuantity BQ’mdhcif | The MSS Load Following Allocation Quantity by Business Associate ID. This Quantity is used to distribute the BA5mBAAMSSLoadFollowingFRUncertaintyAllocationQuantity to the resource which contribute to the FRU Uncertainty need. |
|  | BA5mResBAAMSSLFFRURatioAllocationQuantity BrtF’S’Q’mdhcif | The MSS Load Following Allocation Quantity by Resource. This Quantity is used to distribute the BA5mBAAMSSLoadFollowingFRUncertaintyAllocationQuantity to the resource which contribute to the FRU Uncertainty need. |
|  | BA5mBAAMSSLoadFollowingSupplyFRUUncertaintyAllocationQuantity BQ’mdhcif | The total FRU uncertainty (in MWh) that is attributable to the BA of a MSS within the specified BAA, for the case where the uncertainty is associated with MSS load-following resources. |
|  | BA5mBAAMSSLoadFollowingFRUncertaintyAllocationQuantity BQ’mdhcif | The total uncertainty (in MWh) that is attributable to the BA of a MSS within the specified BAA, for the case where the uncertainty is associated with MSS load-following resources. |
|  | BA5mRSRCBAAMSSLoadFollowingFRUncertaintyAllocationQuantity BrtF’S’Q’mdhcif | The MSS Load Following Flexible Ramp Uncertainty Allocation Quantity by Resource |
|  | BA5mResourceRTDFlexRampActualUncertaintyMovementQuantity BrtuT’I’Q’M’F’S’mdhcif | The RTD-specific Flexible Ramping Uncertainty Movement (in MWh) that is identified for a specified supply or intertie resource. |
|  | BA5mResourceRTDFlexRampLFActualUncertaintyMovementQuantity BrtuT’I’Q’M’F’S’mdhcif | The RTD-specific Flexible Ramping Uncertainty Movement (in MWh) that is identified for a specified supply or intertie load-following resource. |
|  | MSSLoadFollowingResourceFlag BrtuT'I'M'md | A flag (0/1), that, when = 1, indicates that the identified MSS resource is load-following. |
|  | BAA5mLoadCategoryFRUUncertaintyAllocationAmount Q’mdhcif | The FRU Uncertainty charge component (in $) for a BAA that is attributable to the BAA’s Load resources. |
|  | BAA5mLoadCategoryEIMAreaConstraintFRUUncertaintyAllocationAmount Q’mdhcif | The BAA FRU Uncertainty charge component (in $) associated with FRU Uncertainty awards that serve to satisfy the part of an EIM Area FRU Uncertainty constraint requirement for a BAA that pertains to Load uncertainty. |
|  | EIMArea5mLoadCategoryFRUUncertaintyAllocationAmount mdhcif | The EIM Area FRU Uncertainty charge component (in $) associated with FRU Uncertainty awards that serve to satisfy the part of an EIM Area FRU Uncertainty constraint requirement that pertains to Load uncertainty. |
|  | BAA5mLoadCategoryBAAConstraintFRUUncertaintyAllocationAmount Q’mdhcif | The BAA FRU Uncertainty charge component (in $) associated with FRU Uncertainty awards that serve to satisfy the part of a BAA-specific FRU Uncertainty constraint requirement for a BAA that pertains to Load uncertainty. |
|  | BAA5mLoadCategoryConstraintFRUUncertaintyAllocationAmount Q’mdhcif | The FRU Uncertainty charge component (in $) associated with FRU Uncertainty awards that serve to satisfy the Load uncertainty related to a FRU Uncertainty constraint requirement. |
|  | BAA5mConstraintLoadCategoryFRUUncertaintyAllocationAmount Q’Q’’mdhcif | The FRU Uncertainty charge component (in $) associated with FRU Uncertainty awards for a BAA that serve to satisfy the Load uncertainty related to a FRU Uncertainty constraint requirement. |
|  | BAA5mIntertieCategoryFRUUncertaintyAllocationAmount Q’mdhcif | The FRU Uncertainty charge component (in $) for a BAA that is attributable to the BAA’s intertie resources. |
|  | BAA5mIntertieCategoryEIMAreaConstraintFRUUncertaintyAllocationAmount Q’mdhcif | The BAA FRU Uncertainty charge component (in $) associated with FRU Uncertainty awards that serve to satisfy the part of an EIM Area FRU Uncertainty constraint requirement for a BAA that pertains to intertie uncertainty. |
|  | EIMArea5mIntertieCategoryFRUUncertaintyAllocationAmount mdhcif | The EIM Area FRU Uncertainty charge component (in $) associated with FRU Uncertainty awards that serve to satisfy the part of an EIM Area FRU Uncertainty constraint requirement that pertains to intertie uncertainty. |
|  | BAA5mIntertieCategoryBAAConstraintFRUUncertaintyAllocationAmount Q’mdhcif | The BAA FRU Uncertainty charge component (in $) associated with FRU Uncertainty awards that serve to satisfy the part of a BAA-specific FRU Uncertainty constraint requirement for a BAA that pertains to intertie uncertainty. |
|  | BAA5mIntertieCategoryConstraintFRUUncertaintyAllocationAmount Q’mdhcif | The FRU Uncertainty charge component (in $) associated with FRU Uncertainty awards that serve to satisfy the intertie uncertainty related to a FRU Uncertainty constraint requirement. |
|  | BAA5mConstraintIntertieCategoryFRUUncertaintyAllocationAmount Q’Q’’mdhcif | The FRU Uncertainty charge component (in $) associated with FRU Uncertainty awards for a BAA that serve to satisfy the intertie uncertainty related to a FRU Uncertainty constraint requirement. |
|  | BAA5mSupplyCategoryFRUUncertaintyAllocationAmount Q’mdhcif | The FRU Uncertainty charge component (in $) for a BAA that is attributable to the BAA’s supply resources. |
|  | BAA5mSupplyCategoryEIMAreaConstraintFRUUncertaintyAllocationAmount Q’mdhcif | The BAA FRU Uncertainty charge component (in $) associated with FRU Uncertainty awards that serve to satisfy the part of an EIM Area FRU Uncertainty constraint requirement for a BAA that pertains to supply uncertainty. |
|  | EIMArea5mSupplyCategoryFRUUncertaintyAllocationAmount mdhcif | The EIM Area FRU Uncertainty charge component (in $) associated with FRU Uncertainty awards that serve to satisfy the part of an EIM Area FRU Uncertainty constraint requirement that pertains to supply uncertainty. |
|  | BAA5mSupplyCategoryBAAConstraintFRUUncertaintyAllocationAmount Q’mdhcif | The BAA FRU Uncertainty charge component (in $) associated with FRU Uncertainty awards that serve to satisfy the part of a BAA-specific FRU Uncertainty constraint requirement for a BAA that pertains to supply uncertainty. |
|  | BAA5mSupplyCategoryConstraintFRUUncertaintyAllocationAmount Q’mdhcif | The FRU Uncertainty charge component (in $) associated with FRU Uncertainty awards that serve to satisfy the supply uncertainty related to a FRU Uncertainty constraint requirement. |
|  | BAA5mConstraintSupplyCategoryFRUUncertaintyAllocationAmount Q’Q’’mdhcif | The FRU Uncertainty charge component (in $) associated with FRU Uncertainty awards for a BAA that serve to satisfy the supply uncertainty related to a FRU Uncertainty constraint requirement. |
|  | BAA5mAllCategoriesFRUUncertaintyQuantity Q’’mdhcif | The total of FRU Uncertainty (in MW) from all three (3) of the FRU Uncertainty categories (supply, intertie, load) of resources that is associated with a BAA constraint. |
|  | BAA5mLoadFRUUncertaintyQuantity Q’’mdhcif | The total of FRU Uncertainty (in MW) from the Load FRU Uncertainty category of resources in association with a BAA constraint. |
|  | BAA5mIntertieFRUUncertaintyQuantity Q’’mdhcif | The total of FRU Uncertainty (in MW) from the intertie FRU Uncertainty category of resources in association with a BAA constraint. |
|  | BAA5mSupplyFRUUncertaintyQuantity Q’’mdhcif | The total of FRU Uncertainty (in MW) from the supply FRU Uncertainty category of resources in association with a BAA constraint. |
|  | BAA5mBAMeteredDemandAllocationQuantity BQ’mdhcif | The metered Demand (in MWh) of a BAA that is attributable to a particular BA. |

# Charge Code Effective Dates

| Charge Code / Pre-calc Name | Document Version | Effective Start Date | Effective End Date | Version Update Type |
| --- | --- | --- | --- | --- |
| CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation | 5.0 | 11/01/16 | 10/31/16 | Initial Implementation |
| CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation | 5.1 | 11/01/16 | 9/30/20 | Documentation Edits and Configuration Impacted |
| CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation | 5.2 | 10/1/20 | Open | Documentation Edits and Configuration Impacted |