Monthly Flexible Ramp Up Uncertainty Award Allocation

Version 5.1

Table of Contents

1.	Purpose of Document	3

2.	Introduction	3

2.1	Background	3

2.2	Description	3

3.	Charge Code Requirements	3

3.1	Business Rules	4

3.2	Predecessor Charge Codes	8

3.3	Successor Charge Codes	8

3.4	Inputs - External Systems	8

3.5	Inputs - Predecessor Charge Codes or Pre-calculations	8

3.6	CAISO Formula	12

3.7	Outputs	27

4.	Charge Code Effective Dates	43

# Purpose of Document

The purpose of this document is to capture the requirements and design specification for a Charge Code in one document.

# Introduction

## Background

The Flexible Ramping product (FRP) is designed to ensure that there is sufficient ramping capability available in the financially binding five-minute interval to meet the forecasted net load for interval t+5 and cover upwards and downwards forecast error uncertainty.

FRP will help the system to maintain and use dispatchable capacity, as well as provide the market more transparent and less volatile price signals when undergoing forecasted ramp-constrained conditions.  It will be procured and dispatched to meet five minute to five minute net forecast (load forecast minus VER forecast) changes plus uncertainty and will be modeled as a ramping capability constraint.

The ISO will financially settle FRP in the fifteen-minute market and the five-minute market, with rescission of payments applied to resources with UIE (uninstructed imbalance energy) or OA (operational adjustment) amounts, positive or negative, which are imposed on reserved FRP capacity awards. Settlement and allocation of FRP costs will happen on a daily basis for forecasted movement portion and uncertainty award portion.  At the end of the month, the uncertainty award allocation will be reversed and will be re-allocated based on the month’s net UIE or OA values.

As no economic bids are applied to FRP, FRU/FRD awards will be exempt from grid management charges (GMC).  Additionally, dispatchable resources will have their FRP awards and forecasted movement assessments - ignoring rescission settlement - included as part of daily RTM bid cost recovery calculations.

## Description

For each Balancing Authority Area (BAA) and Trading Day this charge code configuration shall allocate the charges associated with the total Flexible Ramp Up Uncertainty capacity award settlement amounts that are paid in association with charge code CC 7071 (Daily Flexible Ramp Up Uncertainty Capacity Settlement) to the BAA. The allocation shall be performed in accord with the business rules and attendant allocation criteria stipulated in Business Rules section 3.1 below. The allocation shall be on a monthly basis and result in the resettlement of the daily charges already calculated with charge code CC 7077 (Daily Flexible Ramp Up Uncertainty Award Allocation), with separate monthly allocation amounts determined for the Peak Flexible Ramp Hours and Off-Peak Flexible Ramp Hours of the Trading Month.

# Charge Code Requirements

## Business Rules

| Bus Req ID | Business Rule |
| --- | --- |
|  | FRU and FRD uncertainty movement payments and charges shall apply daily with monthly resettlement of the FRU and FRD uncertainty movement charges. |
|  | The CAISO will allocate: |
|  | - the cost of the Uncertainty Award within each Balancing Authority Area in the EIM Area and within the EIM Area on a daily basis according to resource categories; |
|  | - the daily amounts to Scheduling Coordinators. |
|  | - Uncertainty awards to the BAA based upon the pass group. For the EIM_area host control area ID, Settlements shall allocate the costs to the BAAs that pass the sufficiency test based on their categories. - For the uncertainty award cost associated with the BAA that did not pass the sufficiency test, those costs will be allocated to the BAA based on its categories and any residual unallocated balance to the metered demand of that BAA, however if metered demand is not available and there are BAA specific costs, those will be allocated to the entity.. |
|  | For monthly allocation of costs of Flexible Ramping Uncertaintly Movement procured, the CAISO will resettle the costs of the Uncertainty Awards by — |
|  | - reversing the daily allocation; |
|  | - assigning the monthly costs of the Uncertainty Awards to Peak Flexible Ramp Hours and Off-Peak Flexible Ramp Hours; |
|  | - separately allocating the monthly Peak Flexible Ramp Hours amounts and Off-Peak Flexible Ramp Hours amounts to the resource categories within each Balancing Authority Area in the EIM Area and within the EIM Area, as described below in subsequent business rules; and |
|  | - allocating the monthly amounts in each category to Scheduling Coordinators. |
|  | For each interval, the CAISO will calculate the net Uncertainty Movement of each resource as follow: |
|  | - for Supply resources other than non-Dynamic System Resources as the difference between the Dispatch Instruction of the binding interval in the next RTD run and the first advisory RTD interval in the current run. |
|  | - for non-Dynamic System Resource and export schedule as the difference between the schedule used in the RTD (accounting for ramp) for the binding interval in the next RTD run and the scheduled use for the first advisory interval in the current RTD run. |
|  | The CAISO will determine the total net RTD Uncertainty Movement for each category separately for each Balancing Authority Area in the EIM Area and by EIM Area as follows: |
|  | - for the category of Supply resources, which shall not include non-Dynamic System Resources, as the net sum of the five-minute Uncertainty Movement of all the Supply resources in the category; |
|  | - for the category of Intertie resources, which shall comprise non-Dynamic System Resources and exports, as the net sum of the five-minute Uncertainty Movement determined of all the non-Dynamic System resources and export schedules; |
|  | - for the non-Participating Load category, as the difference between- - the CAISO Forecast of CAISO Demand, the CAISO forecast of Balancing Authority Area EIM Demand, or the CAISO forecast of EIM Area EIM Demand, as applicable, of the binding interval in the next RTD run; and - the CAISO Forecast of CAISO Demand, the CAISO forecast of Balancing Authority Area EIM Demand, or the CAISO forecast of EIM Area EIM Demand, for the first advisory interval in the current RTD run. |
|  | The CAISO will allocate the total upward Uncertainty Award cost to each of the three (3) resource categories based on — |
|  | - for upward Uncertainty Award cost, the ratio of such category’s positive Uncertainty Movement to the sum of the positive Uncertainty Movements of all categories with positive Uncertainty Movement for - each Balancing Authority Area in the EIM Area that failed their Flex Ramp Up Balancing Test - the EIM Area for EIM entities that passed their Flex Ramp Up Balancing Test. |
|  | - for downward Uncertainty Award cost, the ratio of such category’s positive Uncertainty Movement to the sum of the negative Uncertainty Movements of all categories with negative Uncertainty Movement for - each Balancing Authority Area in the EIM Area that failed their Flex Ramp Up Balancing Test - the EIM Area for EIM entities that passed their Flex Ramp Up Balancing Test.. |
|  | The CAISO will allocate the Uncertainty Awards costs of the non-Participating Load category to Scheduling Coordinators as follows: |
|  | - for upward Uncertainty Award cost in proportion to the Scheduling Coordinator’s negative non-Participating Load UIE, excluding the non-Participating Load of an MSS that has elected to load-follow according to an MSS Agreement, without netting that UIE across Settlement Intervals, to the total of such negative non-Participating Load UIE, without netting that UIE across Settlement Intervals, in the Balancing Authority Area or EIM Area as applicable; |
|  | The allocated upward Uncertainty Award costs shall signify an increase in the Scheduling Coordinator’s Demand (i.e., more Demand than expected). |
|  | - for downward Uncertainty Award cost in proportion to the Scheduling Coordinator’s daily positive non-Participating Load UIE, excluding the non-Participating Load of an MSS that has elected to load-follow according to an MSS Agreement, without netting that UIE across Settlement Intervals, to the total of such positive non-Participating Load UIE, without netting that UIE across Settlement Intervals, in the BAA or EIM Area as applicable. |
|  | The allocated upward Uncertainty Award costs shall signify a decrease in the Scheduling Coordinator’s Demand (i.e., less Demand than expected). |
|  | The CAISO will allocate the Uncertainty Awards costs of the Supply category to Scheduling Coordinators for each resource in the Supply category based on the sum of the resource’s Uncertainty Movement and UIE as follows: |
|  | - for upward Uncertainty Award cost in proportion to the Scheduling Coordinator’s negative sum of the resource’s Uncertainty Movement and UIE, without netting that sum across Settlement Intervals, to the total negative sum of all resources’ Uncertainty Movement and UIE, without netting that sum across Settlement Intervals, in the Balancing Authority Area or EIM Area as applicable, except as stipulated in 3) below; |
|  | The allocated upward Uncertainty Award costs shall signify a decrease in the Scheduling Coordinator’s Generation (i.e., less Generation than expected). |
|  | - for downward Uncertainty Award cost in proportion to the Scheduling Coordinator’s positive sum of the resource’s Uncertainty Movement and UIE, without netting that sum across Settlement Intervals, to the total positive sum of all resources’ Uncertainty Movement and UIE, without netting that sum across Settlement Intervals, in the BAA or EIM Area as applicable, except as stipulated in 3) below; |
|  | The allocated upward Uncertainty Award costs shall signify an increase in the Scheduling Coordinator’s Generation (i.e., more Generation than expected). |
|  | - for the MSS that have elected to load follow pursuant to an MSS Agreement, the CAISO will calculate the positive and negative sums specified above for each Settlement Interval as the sum of MSS non-Participating Load UIE, Supply resources within the MSS UIE, MSS Load Following Energy, MSS Load Following Operational Adjustments, and Uncertainty Movement of resources within the MSS Aggregation. |
|  | The CAISO will allocate the Uncertainty Awards costs of the Intertie category to Scheduling Coordinators for each non-Dynamic System Resource and export based on the sum of the resource’s Uncertainty Movement and Operational Adjustment as follows: |
|  | - for upward Uncertainty Award cost in proportion to the magnitude of the Scheduling Coordinator’s negative Operational Adjustment for non-Dynamic System Resources, or positive Operational Adjustment for export resources (where export Energy is considered positive) relative to the sum of magnitude of such Operational Adjustments in the Balancing Authority Area or EIM Area, as applicable, without netting that sum across Settlement Intervals; |
|  | The allocated upward Uncertainty Award costs shall signify a decrease in the Scheduling Coordinator’s import Generation (i.e., imported Energy less than expected) or an increase in the Scheduling Coordinator’s export Generation (i.e., exported Energy more than expected). |
|  | - for downward Uncertainty Award cost in in proportion to the magnitude of the Scheduling Coordinator’s positive Operational Adjustment for non-Dynamic System Resources, or magnitude of negative Operational Adjustment for export resources (where export Energy is considered positive) relative to the sum of the magnitude of such Operational Adjustments in the Balancing Authority Area or EIM Area, as applicable, without netting that sum across Settlement Intervals; |
|  | The allocated upward Uncertainty Award costs shall signify an increase in the Scheduling Coordinator’s import Generation (i.e., imported Energy more than expected) or a decrease in the Scheduling Coordinator’s export Generation (i.e., exported Energy less than expected). |
|  | - for the purposes of the allocations specified above, the MSS Load Following Operational Adjustment is excluded. |
|  | If the sum of the settlement of Uncertainty Awards and the charges to Scheduling Coordinators for Uncertainty Award costs is nonzero, the CAISO will allocate such amounts to Scheduling Coordinators based on the ratio of its metered CAISO Demand and metered EIM Demand to the total EIM area metered demand. |
|  | For adjustments to the Charge Code that cannot be accomplished by correction of upstream data inputs/recalculation or operator override, Pass Through Bill Charge logic will be applied. |
|  | The categories for the EIM Area Pass Group will be derived as the total quantity of each category for BAAs that belong to the Pass Group. |

## Predecessor Charge Codes

Charge Code/ Pre-calc Name

CC 7071 Daily Flexible Ramp Up Uncertainty Capacity Settlement

CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation

Pre-calculation MSS Netting

Pre-calculation Flexible Ramp Product

## Successor Charge Codes

Charge Code/ Pre-calc Name

CC 4999 Monthly Rounding Adjustment Allocation

## Inputs - External Systems

| Row # | Variable Name | Description |
| --- | --- | --- |
|  | PeakHourFlag mdh | A flag (1/0/NULL) that, when equal to 1, identifies an associated Trading Hour as being a Peak Flexible Ramp Hour. When the flag is equal to 0 or is NULL (missing), the Trading Hour is deemed to be an Off Peak Flexible Ramp Hour. Peak Flexible Ramp Hours are defined to be Trading Hours 7 through 22, while Off Peak Flexible Ramp Hours consist of Trading Hours 1 through 6 and 23 through 25. |
|  | PTBBAAMonthFRUUncertaintyAllocationAmount BJm | PTB charge adjustment for FRU Uncertainty Allocation (in $) |
|  | BADayGenOnlyBAAFlag BQ'md | Flag indicating a Gen-Only BAA with 1. |

## Inputs - Predecessor Charge Codes or Pre-calculations

| Row # | Variable Name | Predecessor Charge Code/ Pre-calc Configuration |
| --- | --- | --- |
| 1 | BAAMonthPeakBAPassGroupFRUMeteredDemandAllocationQuantity BQ’m | PC - Flexible Ramp Product |
| 2 | BAAMonthPeakBAASpecificFRUMeteredDemandAllocationQuantity Q’m | PC - Flexible Ramp Product |
| 3 | EIMAreaMonthPeakFRUPassGroupMeteredDemandAllocationQuantity m | PC - Flexible Ramp Product |
| 4 | BAAMonthPeakBABAASpecificFRUMeteredDemandAllocationQuantity BQ’m | PC - Flexible Ramp Product |
| 5 | BAAMonthOffPeakBAPassGroupFRUMeteredDemandAllocationQuantity BQ’m | PC - Flexible Ramp Product |
| 6 | EIMAreaMonthOffPeakFRUPassGroupMeteredDemandAllocationQuantity m | PC - Flexible Ramp Product |
| 7 | BAAMonthOffPeakBAASpecificFRUMeteredDemandAllocationQuantity Q’m | PC - Flexible Ramp Product |
| 8 | BAAMonthOffPeakBABAASpecificFRUMeteredDemandAllocationQuantity BQ’m | PC - Flexible Ramp Product |
| 9 | BAA5mTotalBAASpecificIntertieFRUUncertaintyAllocationQuantity Q’mdhcif | CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation |
| 10 | BAA5mTotalBAASpecificSupplyFRUUncertaintyAllocationQuantity Q’mdhcif | CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation |
| 11 | BA5mResourceBAASpecificLoadFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif | CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation |
| 12 | BA5mResourceBAASpecificIntertieFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif | CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation |
| 13 | BA5mResourcePassGroupLoadFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif | CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation |
| 14 | BAA5mBAASpecificIntertieFRUUncertaintyQuantity Q’mdhcif | CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation |
| 15 | BAA5mTotalBAASpecificLoadFRUUncertaintyAllocationQuantity Q’mdhcif | CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation |
| 16 | BADailyCompleteFRUUncertaintyAllocationAmount BQ’md | CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation |
| 17 | EIMArea5mPassGroupFRUUncertaintyAllocationAmount mdhcif | CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation |
| 18 | BA5mResourcePassGroupIntertieFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif | CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation |
| 19 | EIMArea5mTotalPassGroupIntertieFRUUncertaintyAllocationQuantity mdhcif | CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation |
| 20 | BAA5mBAASpecificAllCategoriesFRUUncertaintyQuantity Q’mdhcif | CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation |
| 21 | EIMArea5mPassGroupIntertieFRUUncertaintyQuantity mdhcif | CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation |
| 22 | BAA5mBAASpecificLoadFRUUncertaintyQuantity Q’mdhcif | CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation |
| 23 | BAA5mBAASpecificSupplyFRUUncertaintyQuantity Q’mdhcif | CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation |
| 24 | EIMArea5mTotalPassGroupLoadFRUUncertaintyAllocationQuantity mdhcif | CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation |
| 25 | EIMArea5mTotalPassGroupSupplyFRUUncertaintyAllocationQuantity mdhcif | CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation |
| 26 | EIMArea5mPassGroupAllCategoriesFRUUncertaintyQuantity mdhcif | CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation |
| 27 | EIMArea5mPassGroupLoadFRUUncertaintyQuantity mdhcif | CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation |
| 28 | BAA5mBAASpecificFRUUncertaintyAllocationAmount Q’mdhcif | CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation |
| 29 | BA5mResourceBAASpecificSupplyFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif | CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation |
| 30 | BA5mResourcePassGroupSupplyFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif | CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation |
| 31 | EIMArea5mPassGroupSupplyFRUUncertaintyQuantity mdhcif | CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation |
| 32 | BAAConstraint5mFlexRampUpUncertaintyAmount Q’Q’’mdhcif | CC 7071 Daily Flexible Ramp Up Uncertainty Award Allocation |

## CAISO Formula

The CAISO formula for Monthly Flexible Ramp Up Uncertainty charge allocation by BA ID (B) is as follows:

    - BAMonthlyCompleteFRUUncertaintyAllocationAmount BQ’m =

(– BAMonthlyTotalDailyFRUUncertaintyAllocationAmount BQ’m) + BAMonthAllHourCompleteFRUUncertaintyAllocationAmount BQ’m

    - BAMonthlyTotalDailyFRUUncertaintyAllocationAmount BQ’m =

dBADailyCompleteFRUUncertaintyAllocationAmount BQ’md

    - BAMonthAllHourCompleteFRUUncertaintyAllocationAmount BQ’m =

BAMonthPeakPassGroupFRUMeteredDemandAllocatedUncertaintyAmount BQ’m +

BAMonthOffPeakPassGroupFRUMeteredDemandAllocatedUncertaintyAmount BQ’m + BAMonthPeakBAASpecificFRUMeteredDemandAllocatedUncertaintyAmount BQ’m +

BAMonthOffPeakBAASpecificFRUMeteredDemandAllocatedUncertaintyAmount BQ’m +

BAMonthPeakFRUBAACategorySpecificAllocatedUncertaintyAmount BQ’m +

BAMonthOffPeakFRUBAACategorySpecificAllocatedUncertaintyAmount BQ’m +

BAMonthPeakFRUPassGroupCategorySpecificAllocatedUncertaintyAmount BQ’m +

BAMonthOffPeakFRUPassGroupCategorySpecificAllocatedUncertaintyAmount BQ’m +

    - BAMonthPeakPassGroupFRUMeteredDemandAllocatedUncertaintyAmount BQ’m =

(BAAMonthPeakBAPassGroupFRUMeteredDemandAllocationQuantity BQ’m / EIMAreaMonthPeakFRUPassGroupMeteredDemandAllocationQuantity m) * EIMAreaMonthPeakPassGroupFRUNeutralityMeteredDemandAllocatedAmount m

    - BAMonthOffPeakPassGroupFRUMeteredDemandAllocatedUncertaintyAmount BQ’m =

(BAAMonthOffPeakBAPassGroupFRUMeteredDemandAllocationQuantity BQ’m / EIMAreaMonthOffPeakFRUPassGroupMeteredDemandAllocationQuantity m) * EIMAreaMonthOffPeakPassGroupFRUNeutralityMeteredDemandAllocatedAmount m

    - BAMonthPeakBAASpecificFRUMeteredDemandAllocatedUncertaintyAmount BQ’m =

If BADayGenOnlyBAAFlag BQ'md = 1

Then

BADayGenOnlyBAAFlag BQ'md * BAAMonthPeakBAASpecificFRUNeutralityMeteredDemandAllocatedAmount Q’m

Else

(BAAMonthPeakBABAASpecificFRUMeteredDemandAllocationQuantity BQ’m / BAAMonthPeakBAASpecificFRUMeteredDemandAllocationQuantity Q’m) * BAAMonthPeakBAASpecificFRUNeutralityMeteredDemandAllocatedAmount Q’m

    - BAMonthOffPeakBAASpecificFRUMeteredDemandAllocatedUncertaintyAmount BQ’m =

If BADayGenOnlyBAAFlag BQ'md = 1

Then

BADayGenOnlyBAAFlag BQ'md * BAAMonthOffPeakBAASpecificFRUNeutralityMeteredDemandAllocatedAmount Q’m

Else

(BAAMonthOffPeakBABAASpecificFRUMeteredDemandAllocationQuantity BQ’m / BAAMonthOffPeakBAASpecificFRUMeteredDemandAllocationQuantity Q’m) * BAAMonthOffPeakBAASpecificFRUNeutralityMeteredDemandAllocatedAmount Q’m

    - EIMAreaMonthPeakPassGroupFRUNeutralityMeteredDemandAllocatedAmount m =

EIMAreaMonthPeakPassGroupFRUUncertaintyAllocationAmount m –

EIMAreaMonthPeakPassGroupFRUAllocatedUncertaintyAmount m

    - EIMAreaMonthOffPeakPassGroupFRUNeutralityMeteredDemandAllocatedAmount m =

EIMAreaMonthOffPeakPassGroupFRUUncertaintyAllocationAmount m –

EIMAreaMonthOffPeakPassGroupFRUAllocatedUncertaintyAmount m

    - BAAMonthPeakBAASpecificFRUNeutralityMeteredDemandAllocatedAmount Q’m =

BAAMonthPeakBAASpecificFRUUncertaintyAllocationAmount Q’m –

BAAMonthPeakBAASpecificFRUAllocatedUncertaintyAmount Q’m

    - BAAMonthOffPeakBAASpecificFRUNeutralityMeteredDemandAllocatedAmount Q’m =

BAAMonthOffPeakBAASpecificFRUUncertaintyAllocationAmount Q’m –

BAAMonthOffPeakBAASpecificFRUAllocatedUncertaintyAmount Q’m

    - EIMAreaMonthPeakPassGroupFRUAllocatedUncertaintyAmount m =

Sum (B,Q’) BAMonthPeakFRUPassGroupCategorySpecificAllocatedUncertaintyAmount BQ’m

    - EIMAreaMonthOffPeakPassGroupFRUAllocatedUncertaintyAmount m =

Sum (B,Q’) BAMonthOffPeakFRUPassGroupCategorySpecificAllocatedUncertaintyAmount BQ’m

    - BAAMonthPeakBAASpecificFRUAllocatedUncertaintyAmount Q’m =

Sum (B) BAMonthPeakFRUBAACategorySpecificAllocatedUncertaintyAmount BQ’m

    - BAAMonthOffPeakBAASpecificFRUAllocatedUncertaintyAmount Q’m =

Sum (B) BAMonthOffPeakFRUBAACategorySpecificAllocatedUncertaintyAmount BQ’m

    - BAMonthPeakFRUPassGroupCategorySpecificAllocatedUncertaintyAmount BQ’m =

Sum (r,t,F’,S’)

BAMonthResourcePeakPassGroupLoadFRUUncertaintyAllocationAmount BrtF’S’Q’m +

BAMonthResourcePeakPassGroupIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’m + BAMonthResourcePeakPassGroupSupplyFRUUncertaintyAllocationAmount BrtF’S’Q’m

    - BAMonthOffPeakFRUPassGroupCategorySpecificAllocatedUncertaintyAmount BQ’m =

Sum (r,t,F’,S’)

BAMonthResourceOffPeakPassGroupLoadFRUUncertaintyAllocationAmount BrtF’S’Q’m +

BAMonthResourceOffPeakPassGroupIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’m + BAMonthResourceOffPeakPassGroupSupplyFRUUncertaintyAllocationAmount BrtF’S’Q’m

    - BAMonthPeakFRUBAACategorySpecificAllocatedUncertaintyAmount BQ’m =

Sum (r,t,F’,S’)

BAMonthResourcePeakBAASpecificLoadFRUUncertaintyAllocationAmount BrtF’S’Q’m +

BAMonthResourcePeakBAASpecificIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’m + BAMonthResourcePeakBAASpecificSupplyFRUUncertaintyAllocationAmount BrtF’S’Q’m

    - BAMonthOffPeakFRUBAACategorySpecificAllocatedUncertaintyAmount BQ’m =

Sum (r,t,F’,S’)

BAMonthResourceOffPeakBAASpecificLoadFRUUncertaintyAllocationAmount BrtF’S’Q’m +

BAMonthResourceOffPeakBAASpecificIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’m + BAMonthResourceOffPeakBAASpecificSupplyFRUUncertaintyAllocationAmount BrtF’S’Q’m

    - BAMonthResourcePeakPassGroupLoadFRUUncertaintyAllocationAmount BrtF’S’Q’m =

IF

EIMAreaMonthPeakTotalPassGroupLoadFRUUncertaintyAllocationQuantity m <> 0

THEN

BAMonthResourcePeakPassGroupLoadFRUUncertaintyAllocationAmount BrtF’S’Q’m =

(BAMonthResourcePeakPassGroupLoadFRUUncertaintyAllocationQuantityBrtF’S’Q’m

/ EIMAreaMonthPeakTotalPassGroupLoadFRUUncertaintyAllocationQuantity m) * EIMAreaMonthPeakPassGroupLoadFRUUncertaintyAllocationAmount m

ELSE

BAMonthResourcePeakPassGroupLoadFRUUncertaintyAllocationAmount BrtF’S’Q’m  = 0

END IF

    - BAMonthResourceOffPeakPassGroupLoadFRUUncertaintyAllocationAmount BrtF’S’Q’m =

IF

EIMAreaMonthOffPeakTotalPassGroupLoadFRUUncertaintyAllocationQuantity m <> 0

THEN

BAMonthResourceOffPeakPassGroupLoadFRUUncertaintyAllocationAmount BrtF’S’Q’m =

(BAMonthResourceOffPeakPassGroupLoadFRUUncertaintyAllocationQuantityBrtF’S’Q’m

/ EIMAreaMonthOffPeakTotalPassGroupLoadFRUUncertaintyAllocationQuantity m) * EIMAreaMonthOffPeakPassGroupLoadFRUUncertaintyAllocationAmount m

ELSE

BAMonthResourceOffPeakPassGroupLoadFRUUncertaintyAllocationAmount BrtF’S’Q’m  = 0

END IF

    - BAMonthResourcePeakBAASpecificLoadFRUUncertaintyAllocationAmount BrtF’S’Q’m =

IF

BAAMonthPeakTotalBAASpecificLoadFRUUncertaintyAllocationQuantity Q’m <> 0

THEN

BAMonthResourcePeakBAASpecificLoadFRUUncertaintyAllocationAmount BrtF’S’Q’m =

(BAMonthResourcePeakBAASpecificLoadFRUUncertaintyAllocationQuantityBrtF’S’Q’m  / BAAMonthPeakTotalBAASpecificLoadFRUUncertaintyAllocationQuantity Q’m) * BAAMonthPeakLoadBAAConstraintFRUUncertaintyAllocationAmount Q’m

ELSE

BAMonthResourcePeakBAASpecificLoadFRUUncertaintyAllocationAmount BrtF’S’Q’m = 0

END IF

    - BAMonthResourceOffPeakBAASpecificLoadFRUUncertaintyAllocationAmount BrtF’S’Q’m =

IF

BAAMonthOffPeakTotalBAASpecificLoadFRUUncertaintyAllocationQuantity Q’m <> 0

THEN

BAMonthResourceOffPeakBAASpecificLoadFRUUncertaintyAllocationAmount BrtF’S’Q’m =

(BAMonthResourceOffPeakBAASpecificLoadFRUUncertaintyAllocationQuantityBrtF’S’Q’m  / BAAMonthOffPeakTotalBAASpecificLoadFRUUncertaintyAllocationQuantity Q’m) * BAAMonthOffPeakLoadBAAConstraintFRUUncertaintyAllocationAmount Q’m

ELSE

BAMonthResourceOffPeakBAASpecificLoadFRUUncertaintyAllocationAmount BrtF’S’Q’m = 0

END IF

    - BAAMonthPeakTotalBAASpecificLoadFRUUncertaintyAllocationQuantity Q’m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) *  BAA5mTotalBAASpecificLoadFRUUncertaintyAllocationQuantity Q’mdhcif

    - BAAMonthOffPeakTotalBAASpecificLoadFRUUncertaintyAllocationQuantity Q’m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) *  BAA5mTotalBAASpecificLoadFRUUncertaintyAllocationQuantity Q’mdhcif

    - BAMonthResourcePeakBAASpecificLoadFRUUncertaintyAllocationQuantityBrtF’S’Q’m  =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) *  BA5mResourceBAASpecificLoadFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - BAMonthResourceOffPeakBAASpecificLoadFRUUncertaintyAllocationQuantityBrtF’S’Q’m  =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) *  BA5mResourceBAASpecificLoadFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - EIMAreaMonthPeakTotalPassGroupLoadFRUUncertaintyAllocationQuantity m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) *  EIMArea5mTotalPassGroupLoadFRUUncertaintyAllocationQuantity mdhcif

    - EIMAreaMonthOffPeakTotalPassGroupLoadFRUUncertaintyAllocationQuantity m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) *  EIMArea5mTotalPassGroupLoadFRUUncertaintyAllocationQuantity mdhcif

    - BAMonthResourcePeakPassGroupLoadFRUUncertaintyAllocationQuantityBrtF’S’Q’m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) *  BA5mResourcePassGroupLoadFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - BAMonthResourceOffPeakPassGroupLoadFRUUncertaintyAllocationQuantityBrtF’S’Q’m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) *  BA5mResourcePassGroupLoadFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - BAMonthResourcePeakPassGroupIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’m =

IF

EIMAreaMonthPeakTotalPassGroupIntertieFRUUncertaintyAllocationQuantity m <> 0

THEN

BAMonthResourcePeakPassGroupIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’m =

(BAMonthResourcePeakPassGroupIntertieFRUUncertaintyAllocationQuantityBrtF’S’Q’m

/ EIMAreaMonthPeakTotalPassGroupIntertieFRUUncertaintyAllocationQuantity m) * EIMAreaMonthPeakPassGroupIntertieFRUUncertaintyAllocationAmount m

ELSE

BAMonthResourcePeakPassGroupIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’m  = 0

END IF

    - BAMonthResourceOffPeakPassGroupIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’m =

IF

EIMAreaMonthOffPeakTotalPassGroupIntertieFRUUncertaintyAllocationQuantity m <> 0

THEN

BAMonthResourceOffPeakPassGroupIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’m =

(BAMonthResourceOffPeakPassGroupIntertieFRUUncertaintyAllocationQuantityBrtF’S’Q’m

/ EIMAreaMonthOffPeakTotalPassGroupIntertieFRUUncertaintyAllocationQuantity m) * EIMAreaMonthOffPeakPassGroupIntertieFRUUncertaintyAllocationAmount m

ELSE

BAMonthResourceOffPeakPassGroupIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’m  = 0

END IF

    - BAMonthResourcePeakBAASpecificIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’m =

IF

BAAMonthPeakTotalBAASpecificIntertieFRUUncertaintyAllocationQuantity Q’m <> 0

THEN

BAMonthResourcePeakBAASpecificIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’m =

(BAMonthResourcePeakBAASpecificIntertieFRUUncertaintyAllocationQuantityBrtF’S’Q’m  / BAAMonthPeakTotalBAASpecificIntertieFRUUncertaintyAllocationQuantity Q’m) * BAAMonthPeakIntertieBAAConstraintFRUUncertaintyAllocationAmount Q’m

ELSE

BAMonthResourcePeakBAASpecificIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’m = 0

END IF

    - BAMonthResourceOffPeakBAASpecificIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’m =

IF

BAAMonthOffPeakTotalBAASpecificIntertieFRUUncertaintyAllocationQuantity Q’m <> 0

THEN

BAMonthResourceOffPeakBAASpecificIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’m =

(BAMonthResourceOffPeakBAASpecificIntertieFRUUncertaintyAllocationQuantityBrtF’S’Q’m  / BAAMonthOffPeakTotalBAASpecificIntertieFRUUncertaintyAllocationQuantity Q’m) * BAAMonthOffPeakIntertieBAAConstraintFRUUncertaintyAllocationAmount Q’m

ELSE

BAMonthResourceOffPeakBAASpecificIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’m = 0

END IF

    - BAAMonthPeakTotalBAASpecificIntertieFRUUncertaintyAllocationQuantity Q’m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) *  BAA5mTotalBAASpecificIntertieFRUUncertaintyAllocationQuantity Q’mdhcif

    - BAAMonthOffPeakTotalBAASpecificIntertieFRUUncertaintyAllocationQuantity Q’m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) *  BAA5mTotalBAASpecificIntertieFRUUncertaintyAllocationQuantity Q’mdhcif

    - BAMonthResourcePeakBAASpecificIntertieFRUUncertaintyAllocationQuantityBrtF’S’Q’m  =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) *  BA5mResourceBAASpecificIntertieFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - BAMonthResourceOffPeakBAASpecificIntertieFRUUncertaintyAllocationQuantityBrtF’S’Q’m  =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) *  BA5mResourceBAASpecificIntertieFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - EIMAreaMonthPeakTotalPassGroupIntertieFRUUncertaintyAllocationQuantity m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) *  EIMArea5mTotalPassGroupIntertieFRUUncertaintyAllocationQuantity mdhcif

    - EIMAreaMonthOffPeakTotalPassGroupIntertieFRUUncertaintyAllocationQuantity m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) *  EIMArea5mTotalPassGroupIntertieFRUUncertaintyAllocationQuantity mdhcif

    - BAMonthResourcePeakPassGroupIntertieFRUUncertaintyAllocationQuantityBrtF’S’Q’m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) *  BA5mResourcePassGroupIntertieFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - BAMonthResourceOffPeakPassGroupIntertieFRUUncertaintyAllocationQuantityBrtF’S’Q’m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) *  BA5mResourcePassGroupIntertieFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - BAMonthResourcePeakPassGroupSupplyFRUUncertaintyAllocationAmount BrtF’S’Q’m =

IF

EIMAreaMonthPeakTotalPassGroupSupplyFRUUncertaintyAllocationQuantity m <> 0

THEN

BAMonthResourcePeakPassGroupSupplyFRUUncertaintyAllocationAmount BrtF’S’Q’m =

(BAMonthResourcePeakPassGroupSupplyFRUUncertaintyAllocationQuantityBrtF’S’Q’m

/ EIMAreaMonthPeakTotalPassGroupSupplyFRUUncertaintyAllocationQuantity m) * EIMAreaMonthPeakPassGroupSupplyFRUUncertaintyAllocationAmount m

ELSE

BAMonthResourcePeakPassGroupSupplyFRUUncertaintyAllocationAmount BrtF’S’Q’m  = 0

END IF

    - BAMonthResourceOffPeakPassGroupSupplyFRUUncertaintyAllocationAmount BrtF’S’Q’m =

IF

EIMAreaMonthOffPeakTotalPassGroupSupplyFRUUncertaintyAllocationQuantity m <> 0

THEN

BAMonthResourceOffPeakPassGroupSupplyFRUUncertaintyAllocationAmount BrtF’S’Q’m =

(BAMonthResourceOffPeakPassGroupSupplyFRUUncertaintyAllocationQuantityBrtF’S’Q’m

/ EIMAreaMonthOffPeakTotalPassGroupSupplyFRUUncertaintyAllocationQuantity m) * EIMAreaMonthOffPeakPassGroupSupplyFRUUncertaintyAllocationAmount m

ELSE

BAMonthResourceOffPeakPassGroupSupplyFRUUncertaintyAllocationAmount BrtF’S’Q’m  = 0

END IF

    - BAMonthResourcePeakBAASpecificSupplyFRUUncertaintyAllocationAmount BrtF’S’Q’m =

IF

BAAMonthPeakTotalBAASpecificSupplyFRUUncertaintyAllocationQuantity Q’m <> 0

THEN

BAMonthResourcePeakBAASpecificSupplyFRUUncertaintyAllocationAmount BrtF’S’Q’m =

(BAMonthResourcePeakBAASpecificSupplyFRUUncertaintyAllocationQuantityBrtF’S’Q’m  / BAAMonthPeakTotalBAASpecificSupplyFRUUncertaintyAllocationQuantity Q’m) * BAAMonthPeakSupplyBAAConstraintFRUUncertaintyAllocationAmount Q’m

ELSE

BAMonthResourcePeakBAASpecificSupplyFRUUncertaintyAllocationAmount BrtF’S’Q’m = 0

END IF

    - BAMonthResourceOffPeakBAASpecificSupplyFRUUncertaintyAllocationAmount BrtF’S’Q’m =

IF

BAAMonthOffPeakTotalBAASpecificSupplyFRUUncertaintyAllocationQuantity Q’m <> 0

THEN

BAMonthResourceOffPeakBAASpecificSupplyFRUUncertaintyAllocationAmount BrtF’S’Q’m =

(BAMonthResourceOffPeakBAASpecificSupplyFRUUncertaintyAllocationQuantityBrtF’S’Q’m  / BAAMonthOffPeakTotalBAASpecificSupplyFRUUncertaintyAllocationQuantity Q’m) * BAAMonthOffPeakSupplyBAAConstraintFRUUncertaintyAllocationAmount Q’m

ELSE

BAMonthResourceOffPeakBAASpecificSupplyFRUUncertaintyAllocationAmount BrtF’S’Q’m = 0

END IF

    - BAAMonthPeakTotalBAASpecificSupplyFRUUncertaintyAllocationQuantity Q’m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) *  BAA5mTotalBAASpecificSupplyFRUUncertaintyAllocationQuantity Q’mdhcif

    - BAAMonthOffPeakTotalBAASpecificSupplyFRUUncertaintyAllocationQuantity Q’m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) *  BAA5mTotalBAASpecificSupplyFRUUncertaintyAllocationQuantity Q’mdhcif

    - BAMonthResourcePeakBAASpecificSupplyFRUUncertaintyAllocationQuantityBrtF’S’Q’m  =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) *  BA5mResourceBAASpecificSupplyFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - BAMonthResourceOffPeakBAASpecificSupplyFRUUncertaintyAllocationQuantityBrtF’S’Q’m  =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) *  BA5mResourceBAASpecificSupplyFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - EIMAreaMonthPeakTotalPassGroupSupplyFRUUncertaintyAllocationQuantity m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) *  EIMArea5mTotalPassGroupSupplyFRUUncertaintyAllocationQuantity mdhcif

    - EIMAreaMonthOffPeakTotalPassGroupSupplyFRUUncertaintyAllocationQuantity m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) *  EIMArea5mTotalPassGroupSupplyFRUUncertaintyAllocationQuantity mdhcif

    - BAMonthResourcePeakPassGroupSupplyFRUUncertaintyAllocationQuantityBrtF’S’Q’m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) *  BA5mResourcePassGroupSupplyFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - BAMonthResourceOffPeakPassGroupSupplyFRUUncertaintyAllocationQuantityBrtF’S’Q’m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) *  BA5mResourcePassGroupSupplyFRUUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - EIMAreaMonthOffPeakPassGroupLoadFRUUncertaintyAllocationAmount m =

Sum (Q’,Q’’)

IF

EIMAreaMonthOffPeakPassGroupAllCategoriesFRUUncertaintyQuantity m <> 0

THEN

EIMAreaMonthOffPeakPassGroupLoadFRUUncertaintyAllocationAmount m =

BAAMonthOffPeakHourFlexibleRampUpUncertaintySettlementAmount Q’Q’’m * (EIMAreaMonthOffPeakPassGroupLoadFRUUncertaintyQuantity m

/ EIMAreaMonthOffPeakPassGroupAllCategoriesFRUUncertaintyQuantity m)

ELSE

EIMAreaMonthOffPeakPassGroupLoadFRUUncertaintyAllocationAmount m = 0

END IF

Where Q’’ = ‘FRU_PASS_GRP’

    - EIMAreaMonthPeakPassGroupLoadFRUUncertaintyAllocationAmount m =

Sum (Q’,Q’’)

IF

EIMAreaMonthPeakPassGroupAllCategoriesFRUUncertaintyQuantity m <> 0

THEN

EIMAreaMonthPeakPassGroupLoadFRUUncertaintyAllocationAmount m =

BAAMonthPeakHourFlexibleRampUpUncertaintySettlementAmount Q’Q’’m * (EIMAreaMonthPeakPassGroupLoadFRUUncertaintyQuantity m

/ EIMAreaMonthPeakPassGroupAllCategoriesFRUUncertaintyQuantity m)

ELSE

EIMAreaMonthPeakPassGroupLoadFRUUncertaintyAllocationAmount m = 0

END IF

Where Q’’ = ‘FRU_PASS_GRP’

    - BAAMonthOffPeakLoadBAAConstraintFRUUncertaintyAllocationAmount Q’m =

Sum (Q’’)

IF

BAAMonthOffPeakBAASpecificAllCategoriesFRUUncertaintyQuantity Q’m

<> 0

THEN

BAAMonthOffPeakLoadBAAConstraintFRUUncertaintyAllocationAmount Q’m =

BAAMonthOffPeakHourFlexibleRampUpUncertaintySettlementAmount Q’Q’’m * (BAAMonthOffPeakBAASpecificLoadFRUUncertaintyQuantity Q’m

/ BAAMonthOffPeakBAASpecificAllCategoriesFRUUncertaintyQuantity Q’m)

ELSE

BAAMonthOffPeakLoadBAAConstraintFRUUncertaintyAllocationAmount Q’m =0

END IF

Where Q’’ <> ‘FRU_PASS_GRP’

    - BAAMonthPeakLoadBAAConstraintFRUUncertaintyAllocationAmount Q’m =

Sum (Q’’)

IF

BAAMonthPeakBAASpecificAllCategoriesFRUUncertaintyQuantity Q’m

<> 0

THEN

BAAMonthPeakLoadBAAConstraintFRUUncertaintyAllocationAmount Q’m =

BAAMonthPeakHourFlexibleRampUpUncertaintySettlementAmount Q’Q’’m * (BAAMonthPeakBAASpecificLoadFRUUncertaintyQuantity Q’m

/ BAAMonthPeakBAASpecificAllCategoriesFRUUncertaintyQuantity Q’m)

ELSE

BAAMonthPeakLoadBAAConstraintFRUUncertaintyAllocationAmount Q’m = 0

END IF

Where Q’’ <> ‘FRU_PASS_GRP’

    - EIMAreaMonthOffPeakPassGroupIntertieFRUUncertaintyAllocationAmount m =

Sum (Q’,Q’’)

IF

EIMAreaMonthOffPeakPassGroupAllCategoriesFRUUncertaintyQuantity m <> 0

THEN

EIMAreaMonthOffPeakPassGroupIntertieFRUUncertaintyAllocationAmount m =

BAAMonthOffPeakHourFlexibleRampUpUncertaintySettlementAmount Q’Q’’m

* (EIMAreaMonthOffPeakPassGroupIntertieFRUUncertaintyQuantity m

/ EIMAreaMonthOffPeakPassGroupAllCategoriesFRUUncertaintyQuantity m)

ELSE

EIMAreaMonthOffPeakPassGroupIntertieFRUUncertaintyAllocationAmount m = 0

END IF

Where Q’’ = ‘FRU_PASS_GRP’

    - EIMAreaMonthPeakPassGroupIntertieFRUUncertaintyAllocationAmount m =

Sum (Q’,Q’’)

IF

EIMAreaMonthPeakPassGroupAllCategoriesFRUUncertaintyQuantity m <> 0

THEN

EIMAreaMonthPeakPassGroupIntertieFRUUncertaintyAllocationAmount m =

BAAMonthPeakHourFlexibleRampUpUncertaintySettlementAmount Q’Q’’m

* (EIMAreaMonthPeakPassGroupIntertieFRUUncertaintyQuantity m

/ EIMAreaMonthPeakPassGroupAllCategoriesFRUUncertaintyQuantity m)

ELSE

EIMAreaMonthPeakPassGroupIntertieFRUUncertaintyAllocationAmount m = 0

END IF

Where Q’’ = ‘FRU_PASS_GRP’

    - BAAMonthOffPeakIntertieBAAConstraintFRUUncertaintyAllocationAmount Q’m =

Sum (Q’’)

IF

BAAMonthOffPeakBAASpecificAllCategoriesFRUUncertaintyQuantity Q’m

<> 0

THEN

BAAMonthOffPeakIntertieBAAConstraintFRUUncertaintyAllocationAmount Q’m =

BAAMonthOffPeakHourFlexibleRampUpUncertaintySettlementAmount Q’Q’’m * (BAAMonthOffPeakBAASpecificIntertieFRUUncertaintyQuantity Q’m

/ BAAMonthOffPeakBAASpecificAllCategoriesFRUUncertaintyQuantity Q’m)

ELSE

BAAMonthOffPeakIntertieBAAConstraintFRUUncertaintyAllocationAmount Q’m =0

END IF

Where Q’’ <> ‘FRU_PASS_GRP’

    - BAAMonthPeakIntertieBAAConstraintFRUUncertaintyAllocationAmount Q’m =

Sum (Q’’)

IF

BAAMonthPeakBAASpecificAllCategoriesFRUUncertaintyQuantity Q’m

<> 0

THEN

BAAMonthPeakIntertieBAAConstraintFRUUncertaintyAllocationAmount Q’m =

BAAMonthPeakHourFlexibleRampUpUncertaintySettlementAmount Q’Q’’m * (BAAMonthPeakBAASpecificIntertieFRUUncertaintyQuantity Q’m

/ BAAMonthPeakBAASpecificAllCategoriesFRUUncertaintyQuantity Q’m)

ELSE

BAAMonthPeakIntertieBAAConstraintFRUUncertaintyAllocationAmount Q’m = 0

END IF

Where Q’’ <> ‘FRU_PASS_GRP’

    - EIMAreaMonthOffPeakPassGroupSupplyFRUUncertaintyAllocationAmount m =

Sum (Q’,Q’’)

IF

EIMAreaMonthOffPeakPassGroupAllCategoriesFRUUncertaintyQuantity m <> 0

THEN

EIMAreaMonthOffPeakPassGroupSupplyFRUUncertaintyAllocationAmount m =

BAAMonthOffPeakHourFlexibleRampUpUncertaintySettlementAmount Q’Q’’m * (EIMAreaMonthOffPeakPassGroupSupplyFRUUncertaintyQuantity m

/ EIMAreaMonthOffPeakPassGroupAllCategoriesFRUUncertaintyQuantity m)

ELSE

EIMAreaMonthOffPeakPassGroupSupplyFRUUncertaintyAllocationAmount m = 0

END IF

Where Q’’ = ‘FRU_PASS_GRP’

    - EIMAreaMonthPeakPassGroupSupplyFRUUncertaintyAllocationAmount m =

Sum (Q’,Q’’)

IF

EIMAreaMonthPeakPassGroupAllCategoriesFRUUncertaintyQuantity m <> 0

THEN

EIMAreaMonthPeakPassGroupSupplyFRUUncertaintyAllocationAmount m =

BAAMonthPeakHourFlexibleRampUpUncertaintySettlementAmount Q’Q’’m

* (EIMAreaMonthPeakPassGroupSupplyFRUUncertaintyQuantity m

/ EIMAreaMonthPeakPassGroupAllCategoriesFRUUncertaintyQuantity m)

ELSE

EIMAreaMonthPeakPassGroupSupplyFRUUncertaintyAllocationAmount m = 0

END IF

Where Q’’ = ‘FRU_PASS_GRP’

    - BAAMonthOffPeakSupplyBAAConstraintFRUUncertaintyAllocationAmount Q’m =

Sum (Q’’)

IF

BAAMonthOffPeakBAASpecificAllCategoriesFRUUncertaintyQuantity Q’m

<> 0

THEN

BAAMonthOffPeakSupplyBAAConstraintFRUUncertaintyAllocationAmount Q’m =

BAAMonthOffPeakHourFlexibleRampUpUncertaintySettlementAmount Q’Q’’m

* (BAAMonthOffPeakBAASpecificSupplyFRUUncertaintyQuantity Q’m

/ BAAMonthOffPeakBAASpecificAllCategoriesFRUUncertaintyQuantity Q’m)

ELSE

BAAMonthOffPeakSupplyBAAConstraintFRUUncertaintyAllocationAmount Q’m =0

END IF

Where Q’’ <> ‘FRU_PASS_GRP’

    - BAAMonthPeakSupplyBAAConstraintFRUUncertaintyAllocationAmount Q’m =

Sum (Q’’)

IF

BAAMonthPeakBAASpecificAllCategoriesFRUUncertaintyQuantity Q’m

<> 0

THEN

BAAMonthPeakSupplyBAAConstraintFRUUncertaintyAllocationAmount Q’m =

BAAMonthPeakHourFlexibleRampUpUncertaintySettlementAmount Q’Q’’m

* (BAAMonthPeakBAASpecificSupplyFRUUncertaintyQuantity Q’m

/ BAAMonthPeakBAASpecificAllCategoriesFRUUncertaintyQuantity Q’m)

ELSE

BAAMonthPeakSupplyBAAConstraintFRUUncertaintyAllocationAmount Q’m = 0

END IF

Where Q’’ <> ‘FRU_PASS_GRP’

    - BAAMonthOffPeakHourFlexibleRampUpUncertaintySettlementAmount Q’Q’’m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) *  (-1) * BAAConstraint5mFlexRampUpUncertaintyAmount Q’Q’’mdhcif

    - BAAMonthPeakHourFlexibleRampUpUncertaintySettlementAmount Q’Q’’m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) *  (-1) * BAAConstraint5mFlexRampUpUncertaintyAmount Q’Q’’mdhcif

    - EIMAreaMonthOffPeakPassGroupFRUUncertaintyAllocationAmount m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) * EIMArea5mPassGroupFRUUncertaintyAllocationAmount mdhcif

    - EIMAreaMonthPeakPassGroupFRUUncertaintyAllocationAmount m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) * EIMArea5mPassGroupFRUUncertaintyAllocationAmount mdhcif

    - BAAMonthOffPeakBAASpecificFRUUncertaintyAllocationAmount Q’m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) * BAA5mBAASpecificFRUUncertaintyAllocationAmount Q’mdhcif

    - BAAMonthPeakBAASpecificFRUUncertaintyAllocationAmount Q’m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) * BAA5mBAASpecificFRUUncertaintyAllocationAmount Q’mdhcif

    - EIMAreaMonthPeakPassGroupAllCategoriesFRUUncertaintyQuantity m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) * EIMArea5mPassGroupAllCategoriesFRUUncertaintyQuantity mdhcif

### EIMAreaMonthOffPeakPassGroupAllCategoriesFRUUncertaintyQuantity m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) * EIMArea5mPassGroupAllCategoriesFRUUncertaintyQuantity mdhcif

    - BAAMonthPeakBAASpecificAllCategoriesFRUUncertaintyQuantity Q’m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) * BAA5mBAASpecificAllCategoriesFRUUncertaintyQuantity Q’mdhcif

    - BAAMonthOffPeakBAASpecificAllCategoriesFRUUncertaintyQuantity Q’m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) * BAA5mBAASpecificAllCategoriesFRUUncertaintyQuantity Q’mdhcif

    - EIMAreaMonthOffPeakPassGroupLoadFRUUncertaintyQuantity m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) * EIMArea5mPassGroupLoadFRUUncertaintyQuantity mdhcif

    - EIMAreaMonthPeakPassGroupLoadFRUUncertaintyQuantity m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) * EIMArea5mPassGroupLoadFRUUncertaintyQuantity mdhcif

    - BAAMonthOffPeakBAASpecificLoadFRUUncertaintyQuantity Q’m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) * BAA5mBAASpecificLoadFRUUncertaintyQuantity Q’mdhcif

    - BAAMonthPeakBAASpecificLoadFRUUncertaintyQuantity Q’m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) * BAA5mBAASpecificLoadFRUUncertaintyQuantity Q’mdhcif

    - EIMAreaMonthOffPeakPassGroupIntertieFRUUncertaintyQuantity m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) * EIMArea5mPassGroupIntertieFRUUncertaintyQuantity mdhcif

    - EIMAreaMonthPeakPassGroupIntertieFRUUncertaintyQuantity m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) * EIMArea5mPassGroupIntertieFRUUncertaintyQuantity mdhcif

    - BAAMonthOffPeakBAASpecificIntertieFRUUncertaintyQuantity Q’m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) * BAA5mBAASpecificIntertieFRUUncertaintyQuantity Q’mdhcif

    - BAAMonthPeakBAASpecificIntertieFRUUncertaintyQuantity Q’m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) * BAA5mBAASpecificIntertieFRUUncertaintyQuantity Q’mdhcif

    - EIMAreaMonthOffPeakPassGroupSupplyFRUUncertaintyQuantity m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) * EIMArea5mPassGroupSupplyFRUUncertaintyQuantity mdhcif

    - EIMAreaMonthPeakPassGroupSupplyFRUUncertaintyQuantity m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) * EIMArea5mPassGroupSupplyFRUUncertaintyQuantity mdhcif

    - BAAMonthOffPeakBAASpecificSupplyFRUUncertaintyQuantity Q’m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) * BAA5mBAASpecificSupplyFRUUncertaintyQuantity Q’mdhcif

    - BAAMonthPeakBAASpecificSupplyFRUUncertaintyQuantity Q’m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) * BAA5mBAASpecificSupplyFRUUncertaintyQuantity Q’mdhcif

## Outputs

| Row # | Name | Description |
| --- | --- | --- |
| -- | In addition to any outputs listed below, all inputs shall be included as outputs. |  |
|  | BAMonthlyCompleteFRUUncertaintyAllocationAmount BQ’m | FRU Uncertainty Allocation Amount (in $) assessed monthly to a BA of the BAA as the difference of the monthly FRU Allocation Amount for the designated |
|  | BAMonthlyTotalDailyFRUUncertaintyAllocationAmount BQ’m | The montly total (in $) of the daily FRU Uncertainty Allocation Amounts assessed to a BA of resources throughout the EIM Area over all Trading Days of the designated Trading Month. |
|  | BAMonthAllHourCompleteFRUUncertaintyAllocationAmount BQ’m | The total (in $) of the FRU Uncertainty Allocation Amount assessed on a monthly basis to a BA of resources throughout the EIM Area for the designated Trading Month. |
|  | BAMonthPeakPassGroupFRUMeteredDemandAllocatedUncertaintyAmount BQ’m | The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is shared over the EIM Area of EIM entities that pass their Flex Ramp Up Balancing Test and apportioned to a BA of the specified BAA based on metered Demand in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthOffPeakPassGroupFRUMeteredDemandAllocatedUncertaintyAmount BQ’m | The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is shared over the EIM Area of EIM entities that pass their Flex Ramp Up Balancing Test and apportioned to a BA of the specified BAA based on metered Demand in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthPeakBAASpecificFRUMeteredDemandAllocatedUncertaintyAmount BQ’m | The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is assigned to the specific BAA who failed their Flex Ramp Up Balancing Test and apportioned to a BA of the specified BAA based on metered Demand in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthOffPeakBAASpecificFRUMeteredDemandAllocatedUncertaintyAmount BQ’m | The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is assigned to the specific BAA who failed their Flex Ramp Up Balancing Test and apportioned to a BA of the specified BAA based on metered Demand in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthPeakPassGroupFRUNeutralityMeteredDemandAllocatedAmount m | The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is shared over the EIM Areaof EIM entities that pass their Flex Ramp Up Balancing Test in association with the Peak Flexible Ramp Hours of the Trading Month for which the allocation is based on metered Demand. |
|  | EIMAreaMonthOffPeakPassGroupFRUNeutralityMeteredDemandAllocatedAmount m | The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is shared over the EIM Areaof EIM entities that pass their Flex Ramp Up Balancing Test in association with the Off Peak Flexible Ramp Hours of the Trading Month for which the allocation is based on metered Demand. |
|  | BAAMonthPeakBAASpecificFRUNeutralityMeteredDemandAllocatedAmount Q’m | The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is assigned to the specific BAA who failed their Flex Ramp Up Balancing Test in association with the Peak Flexible Ramp Hours of the Trading Month for which the allocation is based on metered Demand. |
|  | BAAMonthOffPeakBAASpecificFRUNeutralityMeteredDemandAllocatedAmount Q’m | The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is assigned to the specific BAA who failed their Flex Ramp Up Balancing Test in association with the Off Peak Flexible Ramp Hours of the Trading Month for which the allocation is based on metered Demand. |
|  | EIMAreaMonthPeakPassGroupFRUAllocatedUncertaintyAmount m | The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is assigned to EIM Area resources of EIM entities that pass their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthOffPeakPassGroupFRUAllocatedUncertaintyAmount m | The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is assigned to EIM Area resources of EIM entities that pass their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthPeakBAASpecificFRUAllocatedUncertaintyAmount Q’m | The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is assigned to the specific BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthOffPeakBAASpecificFRUAllocatedUncertaintyAmount Q’m | The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is assigned to the specific BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthPeakFRUPassGroupCategorySpecificAllocatedUncertaintyAmount BQ’m | The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is assigned to resources of a BA of the specified BAA of EIM entities that pass their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthOffPeakFRUPassGroupCategorySpecificAllocatedUncertaintyAmount BQ’m | The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is assigned to resources of a BA of the specified BAA of EIM entities that pass their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthPeakFRUBAACategorySpecificAllocatedUncertaintyAmount BQ’m | The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is assigned to resources of a BA of the specified BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthOffPeakFRUBAACategorySpecificAllocatedUncertaintyAmount BQ’m | The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is assigned to resources of a BA of the specified BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourcePeakPassGroupLoadFRUUncertaintyAllocationAmount BrtF’S’Q’m | The monthly allocated FRU Uncertainty Allocation amount (in $) that is assigned to a Load category resource of the specified BA and BAA of EIM entities that pass their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourceOffPeakPassGroupLoadFRUUncertaintyAllocationAmount BrtF’S’Q’m | The monthly allocated FRU Uncertainty Allocation amount (in $) that is assigned to a Load category resource of the specified BA and BAA of EIM entities that pass their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourcePeakBAASpecificLoadFRUUncertaintyAllocationAmount BrtF’S’Q’m | The monthly allocated FRU Uncertainty Allocation amount (in $) that is assigned to a Load category resource of the specified BA and BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourceOffPeakBAASpecificLoadFRUUncertaintyAllocationAmount BrtF’S’Q’m | The monthly allocated FRU Uncertainty Allocation amount (in $) that is assigned to a Load category resource of the specified BA and BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthPeakTotalBAASpecificLoadFRUUncertaintyAllocationQuantity Q’m | The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to Load category resources of the specified BAA who failed their Flex Ramp Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthOffPeakTotalBAASpecificLoadFRUUncertaintyAllocationQuantity Q’m | The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to Load category resources of the specified BAA who failed their Flex Ramp Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourcePeakBAASpecificLoadFRUUncertaintyAllocationQuantityBrtF’S’Q’m | The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to the specified Load category resource of a BA of a specific BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourceOffPeakBAASpecificLoadFRUUncertaintyAllocationQuantityBrtF’S’Q’m | The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to the specified Load category resource of a BA of a specific BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthPeakTotalPassGroupLoadFRUUncertaintyAllocationQuantity m | The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to Load category resources across the EIM Area of EIM entities that passed theif Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthOffPeakTotalPassGroupLoadFRUUncertaintyAllocationQuantity m | The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to Load category resources across the EIM Area of EIM entities that passed theif Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourcePeakPassGroupLoadFRUUncertaintyAllocationQuantityBrtF’S’Q’m | The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to the specified Load category resource of a BA that belongs to a specific BAA that passed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourceOffPeakPassGroupLoadFRUUncertaintyAllocationQuantityBrtF’S’Q’m | The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to the specified Load category resource of a BA that belongs to a specific BAA that passed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourcePeakPassGroupIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’m | The monthly allocated FRU Uncertainty Allocation amount (in $) that is assigned to a Intertie category resource of the specified BA and BAA of EIM entities that pass their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourceOffPeakPassGroupIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’m | The monthly allocated FRU Uncertainty Allocation amount (in $) that is assigned to a Intertie category resource of the specified BA and BAA of EIM entities that pass their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourcePeakBAASpecificIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’m | The monthly allocated FRU Uncertainty Allocation amount (in $) that is assigned to a Intertie category resource of the specified BA and BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourceOffPeakBAASpecificIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’m | The monthly allocated FRU Uncertainty Allocation amount (in $) that is assigned to a Intertie category resource of the specified BA and BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthPeakTotalBAASpecificIntertieFRUUncertaintyAllocationQuantity Q’m | The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to Intertie category resources of the specified BAA who failed their Flex Ramp Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthOffPeakTotalBAASpecificIntertieFRUUncertaintyAllocationQuantity Q’m | The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to Intertie category resources of the specified BAA who failed their Flex Ramp Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourcePeakBAASpecificIntertieFRUUncertaintyAllocationQuantityBrtF’S’Q’m | The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to the specified Intertie category resource of a BA of a specific BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourceOffPeakBAASpecificIntertieFRUUncertaintyAllocationQuantityBrtF’S’Q’m | The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to the specified Intertie category resource of a BA of a specific BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthPeakTotalPassGroupIntertieFRUUncertaintyAllocationQuantity m | The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to Intertie category resources across the EIM Area of EIM entities that passed theif Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthOffPeakTotalPassGroupIntertieFRUUncertaintyAllocationQuantity m | The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to Intertie category resources across the EIM Area of EIM entities that passed theif Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourcePeakPassGroupIntertieFRUUncertaintyAllocationQuantityBrtF’S’Q’m | The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to the specified Intertie category resource of a BA that belongs to a specific BAA that passed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourceOffPeakPassGroupIntertieFRUUncertaintyAllocationQuantityBrtF’S’Q’m | The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to the specified Intertie category resource of a BA that belongs to a specific BAA that passed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourcePeakPassGroupSupplyFRUUncertaintyAllocationAmount BrtF’S’Q’m | The monthly allocated FRU Uncertainty Allocation amount (in $) that is assigned to a Supply category resource of the specified BA and BAA of EIM entities that pass their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourceOffPeakPassGroupSupplyFRUUncertaintyAllocationAmount BrtF’S’Q’m | The monthly allocated FRU Uncertainty Allocation amount (in $) that is assigned to a Supply category resource of the specified BA and BAA of EIM entities that pass their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourcePeakBAASpecificSupplyFRUUncertaintyAllocationAmount BrtF’S’Q’m | The monthly allocated FRU Uncertainty Allocation amount (in $) that is assigned to a Supply category resource of the specified BA and BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourceOffPeakBAASpecificSupplyFRUUncertaintyAllocationAmount BrtF’S’Q’m | The monthly allocated FRU Uncertainty Allocation amount (in $) that is assigned to a Supply category resource of the specified BA and BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthPeakTotalBAASpecificSupplyFRUUncertaintyAllocationQuantity Q’m | The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to Supply category resources of the specified BAA who failed their Flex Ramp Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthOffPeakTotalBAASpecificSupplyFRUUncertaintyAllocationQuantity Q’m | The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to Supply category resources of the specified BAA who failed their Flex Ramp Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourcePeakBAASpecificSupplyFRUUncertaintyAllocationQuantityBrtF’S’Q’m | The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to the specified Supply category resource of a BA of a specific BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourceOffPeakBAASpecificSupplyFRUUncertaintyAllocationQuantityBrtF’S’Q’m | The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to the specified Supply category resource of a BA of a specific BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthPeakTotalPassGroupSupplyFRUUncertaintyAllocationQuantity m | The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to Supply category resources across the EIM Area of EIM entities that passed theif Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthOffPeakTotalPassGroupSupplyFRUUncertaintyAllocationQuantity m | The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to Supply category resources across the EIM Area of EIM entities that passed theif Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourcePeakPassGroupSupplyFRUUncertaintyAllocationQuantityBrtF’S’Q’m | The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to the specified Supply category resource of a BA that belongs to a specific BAA that passed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourceOffPeakPassGroupSupplyFRUUncertaintyAllocationQuantityBrtF’S’Q’m | The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to the specified Supply category resource of a BA that belongs to a specific BAA that passed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthOffPeakPassGroupLoadFRUUncertaintyAllocationAmount m | The monthly FRU Uncertainty Allocation amount (in $) over the EIM Area of EIM entities that pass their Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU Uncertainty attributed to the Load category that occur during the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthPeakPassGroupLoadFRUUncertaintyAllocationAmount m | The monthly FRU Uncertainty Allocation amount (in $) over the EIM Area of EIM entities that pass their Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU Uncertainty attributed to the Load category that occur during the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthOffPeakLoadBAAConstraintFRUUncertaintyAllocationAmount Q’m | The monthly FRU Uncertainty Allocation amount (in $) for a BAA that failed theif Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU Uncertainty attributed to the Load category in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthPeakLoadBAAConstraintFRUUncertaintyAllocationAmount Q’m | The monthly FRU Uncertainty Allocation amount (in $) for a BAA that failed theif Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU Uncertainty attributed to the Load category in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthOffPeakPassGroupIntertieFRUUncertaintyAllocationAmount m | The monthly FRU Uncertainty Allocation amount (in $) over the EIM Area of EIM entities that pass their Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU Uncertainty attributed to the Intertie category that occur during the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthPeakPassGroupIntertieFRUUncertaintyAllocationAmount m | The monthly FRU Uncertainty Allocation amount (in $) over the EIM Area of EIM entities that pass their Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU Uncertainty attributed to the Intertie category that occur during the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthOffPeakIntertieBAAConstraintFRUUncertaintyAllocationAmount Q’m | The monthly FRU Uncertainty Allocation amount (in $) for a BAA that failed theif Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU Uncertainty attributed to the Intertie category in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthPeakIntertieBAAConstraintFRUUncertaintyAllocationAmount Q’m | The monthly FRU Uncertainty Allocation amount (in $) for a BAA that failed theif Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU Uncertainty attributed to the Intertie category in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthOffPeakPassGroupSupplyFRUUncertaintyAllocationAmount m | The monthly FRU Uncertainty Allocation amount (in $) over the EIM Area of EIM entities that pass their Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU Uncertainty attributed to the Supply category that occur during the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthPeakPassGroupSupplyFRUUncertaintyAllocationAmount m | The monthly FRU Uncertainty Allocation amount (in $) over the EIM Area of EIM entities that pass their Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU Uncertainty attributed to the Supply category that occur during the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthOffPeakSupplyBAAConstraintFRUUncertaintyAllocationAmount Q’m | The monthly FRU Uncertainty Allocation amount (in $) for a BAA that failed theif Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU Uncertainty attributed to the Supply category in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthPeakSupplyBAAConstraintFRUUncertaintyAllocationAmount Q’m | The monthly FRU Uncertainty Allocation amount (in $) for a BAA that failed theif Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU Uncertainty attributed to the Supply category in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthOffPeakHourFlexibleRampUpUncertaintySettlementAmount Q’Q’’m | The monthly FRU Uncertainty Allocation amount (in $) that follows from the settlement (in CC 7071) of FRU Uncertainty in association with the specified FRU Uncertainty Pass Group Q'’ and BAA Q’ for the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthPeakHourFlexibleRampUpUncertaintySettlementAmount Q’Q’’m | The monthly FRU Uncertainty Allocation amount (in $) that follows from the settlement (in CC 7071) of FRU Uncertainty in association with the specified FRU Uncertainty Pass Group Q'’ and BAA Q’ for the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthOffPeakPassGroupFRUUncertaintyAllocationAmount m | The monthly FRU Uncertainty Allocation amount (in $) over the EIM Area of EIM Entities that pass their Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU for the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthPeakPassGroupFRUUncertaintyAllocationAmount m | The monthly FRU Uncertainty Allocation amount (in $) over the EIM Area of EIM Entities that pass their Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU for the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthOffPeakBAASpecificFRUUncertaintyAllocationAmount Q’m | The monthly FRU Uncertainty Allocation amount (in $) for a specific BAA that failed their Flex Ramp Up Balancing test that follows from the settlement (in CC 7071) of FRU Uncertainty for the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthPeakBAASpecificFRUUncertaintyAllocationAmount Q’m | The monthly FRU Uncertainty Allocation amount (in $) for a specific BAA that failed their Flex Ramp Up Balancing test that follows from the settlement (in CC 7071) of FRU Uncertainty for the Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthPeakPassGroupAllCategoriesFRUUncertaintyQuantity m | The monthly FRU Uncertainty quantity (in MWh) for all resource categories (Load, Intertie and Supply) over the EIM Area for BAAs that passed their Flex Ramp Up Balancing Test the Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthOffPeakPassGroupAllCategoriesFRUUncertaintyQuantity m | The monthly FRU Uncertainty quantity (in MWh) for all resource categories (Load, Intertie and Supply) over the EIM Area for BAAs that passed their Flex Ramp Up Balancing Test the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthPeakBAASpecificAllCategoriesFRUUncertaintyQuantity Q’m | The monthly FRU Uncertainty quantity (in MWh) for all resource categories (Load, Intertie and Supply) for a specific BAA that failed their Flex Ramp Up Balancing Test the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthOffPeakBAASpecificAllCategoriesFRUUncertaintyQuantity Q’m | The monthly FRU Uncertainty quantity (in MWh) for all resource categories (Load, Intertie and Supply) for a specific BAA that failed their Flex Ramp Up Balancing Test the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthOffPeakPassGroupLoadFRUUncertaintyQuantity m | The monthly FRU Uncertainty quantity (in MWh) for Load resources over the EIM Area for BAAs that passed their Flex Ramp Up Balancing Test the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthPeakPassGroupLoadFRUUncertaintyQuantity m | The monthly FRU Uncertainty quantity (in MWh) for Load resources over the EIM Area for BAAs that passed their Flex Ramp Up Balancing Test the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthOffPeakBAASpecificLoadFRUUncertaintyQuantity Q’m | The monthly FRU Uncertainty quantity (in MWh) for Load resources of a specific BAA that failed their Flex Ramp Up Balancing Test the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthPeakBAASpecificLoadFRUUncertaintyQuantity Q’m | The monthly FRU Uncertainty quantity (in MWh) for Load resources of a specific BAA that failed their Flex Ramp Up Balancing Test the Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthOffPeakPassGroupIntertieFRUUncertaintyQuantity m | The monthly FRU Uncertainty quantity (in MWh) for Intertie resources over the EIM Area for BAAs that passed their Flex Ramp Up Balancing Test the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthPeakPassGroupIntertieFRUUncertaintyQuantity m | The monthly FRU Uncertainty quantity (in MWh) for Intertie resources over the EIM Area for BAAs that passed their Flex Ramp Up Balancing Test the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthOffPeakBAASpecificIntertieFRUUncertaintyQuantity Q’m | The monthly FRU Uncertainty quantity (in MWh) for Intertie resources of a specific BAA that failed their Flex Ramp Up Balancing Test the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthPeakBAASpecificIntertieFRUUncertaintyQuantity Q’m | The monthly FRU Uncertainty quantity (in MWh) for Intertie resources of a specific BAA that failed their Flex Ramp Up Balancing Test the Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthOffPeakPassGroupSupplyFRUUncertaintyQuantity m | The monthly FRU Uncertainty quantity (in MWh) for Supply resources over the EIM Area for BAAs that passed their Flex Ramp Up Balancing Test the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthPeakPassGroupSupplyFRUUncertaintyQuantity m | The monthly FRU Uncertainty quantity (in MWh) for Supply resources over the EIM Area for BAAs that passed their Flex Ramp Up Balancing Test the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthOffPeakBAASpecificSupplyFRUUncertaintyQuantity Q’m | The monthly FRU Uncertainty quantity (in MWh) for Supply resources of a specific BAA that failed their Flex Ramp Up Balancing Test the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthPeakBAASpecificSupplyFRUUncertaintyQuantity Q’m | The monthly FRU Uncertainty quantity (in MWh) for Supply resources of a specific BAA that failed their Flex Ramp Up Balancing Test the Peak Flexible Ramp Hours of the Trading Month. |

# Charge Code Effective Dates

| Charge Code/ Pre-calc Name | Document Version | Effective Start Date | Effective End Date | Version Update Type |
| --- | --- | --- | --- | --- |
| CC 7077 Daily Flexible Ramp Up Uncertainty Award Allocation | 5.0 | 11/1/2016 | Open | Initial Implementation |