Monthly Flexible Ramp Down Uncertainty Award Allocation

Version 5.1

Table of Contents

1.	Purpose of Document	3

2.	Introduction	3

2.1	Background	3

2.2	Description	3

3.	Charge Code Requirements	3

3.1	Business Rules	4

3.2	Predecessor Charge Codes	8

3.3	Successor Charge Codes	8

3.4	Inputs - External Systems	8

3.5	Inputs - Predecessor Charge Codes or Pre-calculations	8

3.6	CAISO Formula	12

3.7	Outputs	27

4.	Charge Code Effective Dates	44

# Purpose of Document

The purpose of this document is to capture the requirements and design specification for a Charge Code in one document.

# Introduction

## Background

The Flexible Ramping product (FRP) is designed to ensure that there is sufficient ramping capability available in the financially binding five-minute interval to meet the forecasted net load for interval t+5 and cover upwards and downwards forecast error uncertainty.

FRP will help the system to maintain and use dispatchable capacity, as well as provide the market more transparent and less volatile price signals when undergoing forecasted ramp-constrained conditions.  It will be procured and dispatched to meet five minute to five minute net forecast (load forecast minus VER forecast) changes plus uncertainty and will be modeled as a ramping capability constraint.

The ISO will financially settle FRP in the fifteen-minute market and the five-minute market, with rescission of payments applied to resources with UIE (uninstructed imbalance energy) or OA (operational adjustment) amounts, positive or negative, which are imposed on reserved FRP capacity awards. Settlement and allocation of FRP costs will happen on a daily basis for forecasted movement portion and uncertainty award portion.  At the end of the month, the uncertainty award allocation will be reversed and will be re-allocated based on the month’s net UIE or OA values.

As no economic bids are applied to FRP, FRU/FRD awards will be exempt from grid management charges (GMC).  Additionally, dispatchable resources will have their FRP awards and forecasted movement assessments - ignoring rescission settlement - included as part of daily RTM bid cost recovery calculations.

## Description

For each Balancing Authority Area (BAA) and Trading Day this charge code configuration shall allocate the charges associated with the total Flexible Ramp Down Uncertainty capacity award settlement amounts that are paid in association with charge code CC 7081 (Daily Flexible Ramp Down Uncertainty Capacity Settlement) to the BAA. The allocation shall be performed in accord with the business rules and attendant allocation criteria stipulated in Business Rules section 3.1 below. The allocation shall be on a monthly basis and result in the resettlement of the daily charges already calculated with charge code CC 7087 (Daily Flexible Ramp Down Uncertainty Award Allocation), with separate monthly allocation amounts determined for the Peak Flexible Ramp Hours and Off-Peak Flexible Ramp Hours of the Trading Month.

# Charge Code Requirements

## Business Rules

| Bus Req ID | Business Rule |
| --- | --- |
|  | FRU and FRD uncertainty movement payments and charges shall apply daily with monthly resettlement of the FRU and FRD uncertainty movement charges. |
|  | The CAISO will allocate: |
|  | - the cost of the Uncertainty Award within each Balancing Authority Area in the EIM Area and within the EIM Area on a daily basis according to resource categories; |
|  | - the daily amounts to Scheduling Coordinators. |
|  | - Uncertainty awards to the BAA based upon the pass group. Note: For the EIM_area host control area ID, Settlements shall allocate the costs to the BAAs that pass the sufficiency test based on their categories. For the uncertainty award cost associated with the BAA that did not pass the sufficiency test, those costs will be allocated to the BAA based on its categories and any residual unallocated balance to the metered demand of that BAA, however if metered demand is not available and there are BAA specific costs, those will be allocated to the entity. |
|  | For monthly allocation of costs of Flexible Ramping Uncertaintly Movement procured, the CAISO will resettle the costs of the Uncertainty Awards by — |
|  | - reversing the daily allocation; |
|  | - assigning the monthly costs of the Uncertainty Awards to Peak Flexible Ramp Hours and Off-Peak Flexible Ramp Hours; |
|  | - separately allocating the monthly Peak Flexible Ramp Hours amounts and Off-Peak Flexible Ramp Hours amounts to the resource categories within each Balancing Authority Area in the EIM Area and within the EIM Area, as described below in subsequent business rules; and |
|  | - allocating the monthly amounts in each category to Scheduling Coordinators. |
|  | For each interval, the CAISO will calculate the net Uncertainty Movement of each resource as follow: |
|  | - for Supply resources other than non-Dynamic System Resources as the difference between the Dispatch Instruction of the binding interval in the next RTD run and the first advisory RTD interval in the current run. |
|  | - for non-Dynamic System Resource and export schedule as the difference between the schedule used in the RTD (accounting for ramp) for the binding interval in the next RTD run and the scheduled use for the first advisory interval in the current RTD run. |
|  | The CAISO will determine the total net RTD Uncertainty Movement for each category separately for each Balancing Authority Area in the EIM Area and by EIM Area as follows: |
|  | - for the category of Supply resources, which shall not include non-Dynamic System Resources, as the net sum of the five-minute Uncertainty Movement of all the Supply resources in the category; |
|  | - for the category of Intertie resources, which shall comprise non-Dynamic System Resources and exports, as the net sum of the five-minute Uncertainty Movement determined of all the non-Dynamic System resources and export schedules; |
|  | - for the non-Participating Load category, as the difference between- - the CAISO Forecast of CAISO Demand, the CAISO forecast of Balancing Authority Area EIM Demand, or the CAISO forecast of EIM Area EIM Demand, as applicable, of the binding interval in the next RTD run; and - the CAISO Forecast of CAISO Demand, the CAISO forecast of Balancing Authority Area EIM Demand, or the CAISO forecast of EIM Area EIM Demand, as applicable, for the first advisory interval in the current RTD run. |
|  | The CAISO will allocate the total upward Uncertainty Award cost to each of the three (3) resource categories based on — |
|  | - for upward Uncertainty Award cost, the ratio of such category’s positive Uncertainty Movement to the sum of the positive Uncertainty Movements of all categories with positive Uncertainty Movement for - each Balancing Authority Area in the EIM Area that failed their Flex Ramp Up Balancing Test - the EIM Area for EIM entities that passed their Flex Ramp Up Balancing Test. |
|  | - for downward Uncertainty Award cost, the ratio of such category’s positive Uncertainty Movement to the sum of the negative Uncertainty Movements of all categories with negative Uncertainty Movement for - each Balancing Authority Area in the EIM Area that failed their Flex Ramp Up Balancing Test - the EIM Area for EIM entities that passed their Flex Ramp Up Balancing Test.. |
|  | The CAISO will allocate the Uncertainty Awards costs of the non-Participating Load category to Scheduling Coordinators as follows: |
|  | - for upward Uncertainty Award cost in proportion to the Scheduling Coordinator’s negative non-Participating Load UIE, excluding the non-Participating Load of an MSS that has elected to load-follow according to an MSS Agreement, without netting that UIE across Settlement Intervals, to the total of such negative non-Participating Load UIE, without netting that UIE across Settlement Intervals, in the Balancing Authority Area or EIM Area as applicable; |
|  | The allocated upward Uncertainty Award costs shall signify an increase in the Scheduling Coordinator’s Demand (i.e., more Demand than expected). |
|  | - for downward Uncertainty Award cost in proportion to the Scheduling Coordinator’s daily positive non-Participating Load UIE, excluding the non-Participating Load of an MSS that has elected to load-follow according to an MSS Agreement, without netting that UIE across Settlement Intervals, to the total of such positive non-Participating Load UIE, without netting that UIE across Settlement Intervals, in the BAA or EIM Area as applicable. |
|  | The allocated upward Uncertainty Award costs shall signify a decrease in the Scheduling Coordinator’s Demand (i.e., less Demand than expected). |
|  | The CAISO will allocate the Uncertainty Awards costs of the Supply category to Scheduling Coordinators for each resource in the Supply category based on the sum of the resource’s Uncertainty Movement and UIE as follows: |
|  | - for upward Uncertainty Award cost in proportion to the Scheduling Coordinator’s negative sum of the resource’s Uncertainty Movement and UIE, without netting that sum across Settlement Intervals, to the total negative sum of all resources’ Uncertainty Movement and UIE, without netting that sum across Settlement Intervals, in the Balancing Authority Area or EIM Area as applicable, except as stipulated in 3) below; |
|  | The allocated upward Uncertainty Award costs shall signify a decrease in the Scheduling Coordinator’s Generation (i.e., less Generation than expected). |
|  | - for downward Uncertainty Award cost in proportion to the Scheduling Coordinator’s positive sum of the resource’s Uncertainty Movement and UIE, without netting that sum across Settlement Intervals, to the total positive sum of all resources’ Uncertainty Movement and UIE, without netting that sum across Settlement Intervals, in the BAA or EIM Area as applicable, except as stipulated in 3) below; |
|  | The allocated upward Uncertainty Award costs shall signify an increase in the Scheduling Coordinator’s Generation (i.e., more Generation than expected). |
|  | - for the MSS that have elected to load follow pursuant to an MSS Agreement, the CAISO will calculate the positive and negative sums specified above for each Settlement Interval as the sum of MSS non-Participating Load UIE, Supply resources within the MSS UIE, MSS Load Following Energy, MSS Load Following Operational Adjustments, and Uncertainty Movement of resources within the MSS Aggregation. |
|  | The CAISO will allocate the Uncertainty Awards costs of the Intertie category to Scheduling Coordinators for each non-Dynamic System Resource and export based on the sum of the resource’s Uncertainty Movement and Operational Adjustment as follows: |
|  | - for upward Uncertainty Award cost in proportion to the magnitude of the Scheduling Coordinator’s negative Operational Adjustment for non-Dynamic System Resources, or positive Operational Adjustment for export resources (where export Energy is considered positive) relative to the sum of magnitude of such Operational Adjustments in the Balancing Authority Area or EIM Area, as applicable, without netting that sum across Settlement Intervals; |
|  | The allocated upward Uncertainty Award costs shall signify a decrease in the Scheduling Coordinator’s import Generation (i.e., imported Energy less than expected) or an increase in the Scheduling Coordinator’s export Generation (i.e., exported Energy more than expected). |
|  | - for downward Uncertainty Award cost in in proportion to the magnitude of the Scheduling Coordinator’s positive Operational Adjustment for non-Dynamic System Resources, or magnitude of negative Operational Adjustment for export resources (where export Energy is considered positive) relative to the sum of the magnitude of such Operational Adjustments in the Balancing Authority Area or EIM Area, as applicable, without netting that sum across Settlement Intervals; |
|  | The allocated upward Uncertainty Award costs shall signify an increase in the Scheduling Coordinator’s import Generation (i.e., imported Energy more than expected) or a decrease in the Scheduling Coordinator’s export Generation (i.e., exported Energy less than expected). |
|  | - for the purposes of the allocations specified above, the MSS Load Following Operational Adjustment is excluded. |
|  | If the sum of the settlement of Uncertainty Awards and the charges to Scheduling Coordinators for Uncertainty Award costs is nonzero, the CAISO will allocate such amounts to Scheduling Coordinators based on the ratio of its metered CAISO Demand and metered EIM Demand to the total EIM area metered demand. |
|  | For adjustments to the Charge Code that cannot be accomplished by correction of upstream data inputs/recalculation or operator override, Pass Through Bill Charge logic will be applied. |
|  | The categories for the EIM Area Pass Group will be derived as the total quantity of each category for BAAs that belong to the Pass Group. |

## Predecessor Charge Codes

Charge Code/ Pre-calc Name

CC 7081 Daily Flexible Ramp Down Uncertainty Capacity Settlement

CC 7087 Daily Flexible Ramp Down Uncertainty Award Allocation

CC 7078 Monthly Flexible Ramp Up Uncertainty Award Allocation

Pre-calculation Flexible Ramp Product

## Successor Charge Codes

Charge Code/ Pre-calc Name

CC 4999 Monthly Rounding Adjustment Allocation

## Inputs - External Systems

| Row # | Variable Name | Description |
| --- | --- | --- |
|  | PeakHourFlag mdh | A flag (1/0/NULL) that, when equal to 1, identifies an associated Trading Hour as being a Peak Flexible Ramp Hour. When the flag is equal to 0 or is NULL (missing), the Trading Hour is deemed to be an Off Peak Flexible Ramp Hour. Peak Flexible Ramp Hours are defined to be Trading Hours 7 through 22, while Off Peak Flexible Ramp Hours consist of Trading Hours 1 through 6 and 23 through 25. |
|  | PTBBAAMonthFRDUncertaintyAllocationAmount BJm | PTB charge adjustment for FRD Uncertainty Allocation (in $) |
|  | BADayGenOnlyBAAFlag BQ'md | Flag indicating a Gen-Only BAA with 1. |

## Inputs - Predecessor Charge Codes or Pre-calculations

| Row # | Variable Name | Predecessor Charge Code/ Pre-calc Configuration |
| --- | --- | --- |
| 1 | BAAMonthPeakBAASpecificFRDMeteredDemandAllocationQuantity Q’m | PC - Flexible Ramp Product |
| 2 | EIMAreaMonthOffPeakFRDPassGroupMeteredDemandAllocationQuantity m | PC - Flexible Ramp Product |
| 3 | EIMAreaMonthPeakFRDPassGroupMeteredDemandAllocationQuantity m | PC - Flexible Ramp Product |
| 4 | BAAMonthOffPeakBAASpecificFRDMeteredDemandAllocationQuantity Q’m | PC - Flexible Ramp Product |
| 5 | BAAMonthPeakBAPassGroupFRDMeteredDemandAllocationQuantity BQ’m | PC - Flexible Ramp Product |
| 6 | BAAMonthOffPeakBAPassGroupFRDMeteredDemandAllocationQuantity BQ’m | PC - Flexible Ramp Product |
| 7 | BAAMonthPeakBABAASpecificFRDMeteredDemandAllocationQuantity BQ’m | PC - Flexible Ramp Product |
| 8 | BAAMonthOffPeakBABAASpecificFRDMeteredDemandAllocationQuantity BQ’m | PC - Flexible Ramp Product |
| 9 | BAA5mBAASpecificAllCategoriesFRDUncertaintyQuantity Q’mdhcif | CC 7087 Daily Flexible Ramp Down Uncertainty Award Allocation |
| 10 | BAA5mTotalBAASpecificIntertieFRDUncertaintyAllocationQuantity Q’mdhcif | CC 7087 Daily Flexible Ramp Down Uncertainty Award Allocation |
| 11 | BA5mResourcePassGroupSupplyFRDUncertaintyAllocationQuantity BrtF’S’Q’mdhcif | CC 7087 Daily Flexible Ramp Down Uncertainty Award Allocation |
| 12 | BAA5mTotalBAASpecificSupplyFRDUncertaintyAllocationQuantity Q’mdhcif | CC 7087 Daily Flexible Ramp Down Uncertainty Award Allocation |
| 13 | EIMArea5mPassGroupAllCategoriesFRDUncertaintyQuantity mdhcif | CC 7087 Daily Flexible Ramp Down Uncertainty Award Allocation |
| 14 | BAA5mBAASpecificSupplyFRDUncertaintyQuantity Q’mdhcif | CC 7087 Daily Flexible Ramp Down Uncertainty Award Allocation |
| 15 | BA5mResourcePassGroupLoadFRDUncertaintyAllocationQuantity BrtF’S’Q’mdhcif | CC 7087 Daily Flexible Ramp Down Uncertainty Award Allocation |
| 16 | BA5mResourcePassGroupIntertieFRDUncertaintyAllocationQuantity BrtF’S’Q’mdhcif | CC 7087 Daily Flexible Ramp Down Uncertainty Award Allocation |
| 17 | EIMArea5mPassGroupLoadFRDUncertaintyQuantity mdhcif | CC 7087 Daily Flexible Ramp Down Uncertainty Award Allocation |
| 18 | EIMArea5mPassGroupSupplyFRDUncertaintyQuantity mdhcif | CC 7087 Daily Flexible Ramp Down Uncertainty Award Allocation |
| 19 | BAA5mBAASpecificLoadFRDUncertaintyQuantity Q’mdhcif | CC 7087 Daily Flexible Ramp Down Uncertainty Award Allocation |
| 20 | BADailyCompleteFRDUncertaintyAllocationAmount BQ’md | CC 7087 Daily Flexible Ramp Down Uncertainty Award Allocation |
| 21 | BAA5mTotalBAASpecificLoadFRDUncertaintyAllocationQuantity Q’mdhcif | CC 7087 Daily Flexible Ramp Down Uncertainty Award Allocation |
| 22 | BA5mResourceBAASpecificIntertieFRDUncertaintyAllocationQuantity BrtF’S’Q’mdhcif | CC 7087 Daily Flexible Ramp Down Uncertainty Award Allocation |
| 23 | EIMArea5mPassGroupFRDUncertaintyAllocationAmount mdhcif | CC 7087 Daily Flexible Ramp Down Uncertainty Award Allocation |
| 24 | EIMArea5mTotalPassGroupLoadFRDUncertaintyAllocationQuantity mdhcif | CC 7087 Daily Flexible Ramp Down Uncertainty Award Allocation |
| 25 | EIMArea5mTotalPassGroupIntertieFRDUncertaintyAllocationQuantity mdhcif | CC 7087 Daily Flexible Ramp Down Uncertainty Award Allocation |
| 26 | EIMArea5mTotalPassGroupSupplyFRDUncertaintyAllocationQuantity mdhcif | CC 7087 Daily Flexible Ramp Down Uncertainty Award Allocation |
| 27 | EIMArea5mPassGroupIntertieFRDUncertaintyQuantity mdhcif | CC 7087 Daily Flexible Ramp Down Uncertainty Award Allocation |
| 28 | BAA5mBAASpecificFRDUncertaintyAllocationAmount Q’mdhcif | CC 7087 Daily Flexible Ramp Down Uncertainty Award Allocation |
| 29 | BA5mResourceBAASpecificLoadFRDUncertaintyAllocationQuantity BrtF’S’Q’mdhcif | CC 7087 Daily Flexible Ramp Down Uncertainty Award Allocation |
| 30 | BAA5mBAASpecificIntertieFRDUncertaintyQuantity Q’mdhcif | CC 7087 Daily Flexible Ramp Down Uncertainty Award Allocation |
| 31 | BA5mResourceBAASpecificSupplyFRDUncertaintyAllocationQuantity BrtF’S’Q’mdhcif | CC 7087 Daily Flexible Ramp Down Uncertainty Award Allocation |
| 32 | BAAConstraint5mFlexRampDownUncertaintyAmount Q’Q’’mdhcif | CC 7081 Flexible Ramp Down Uncertainty Capacity Settlement |

## CAISO Formula

The CAISO formula for Monthly Flexible Ramp Down Uncertainty charge allocation by BA ID (B) is as follows:

    - BAMonthlyCompleteFRDUncertaintyAllocationAmount BQ’m =

(– BAMonthlyTotalDailyFRDUncertaintyAllocationAmount BQ’m) + BAMonthAllHourCompleteFRDUncertaintyAllocationAmount BQ’m

    - BAMonthlyTotalDailyFRDUncertaintyAllocationAmount BQ’m =

dBADailyCompleteFRDUncertaintyAllocationAmount BQ’md

    - BAMonthAllHourCompleteFRDUncertaintyAllocationAmount BQ’m =

BAMonthPeakPassGroupFRDMeteredDemandAllocatedUncertaintyAmount BQ’m +

BAMonthOffPeakPassGroupFRDMeteredDemandAllocatedUncertaintyAmount BQ’m + BAMonthPeakBAASpecificFRDMeteredDemandAllocatedUncertaintyAmount BQ’m +

BAMonthOffPeakBAASpecificFRDMeteredDemandAllocatedUncertaintyAmount BQ’m +

BAMonthPeakFRDBAACategorySpecificAllocatedUncertaintyAmount BQ’m +

BAMonthOffPeakFRDBAACategorySpecificAllocatedUncertaintyAmount BQ’m +

BAMonthPeakFRDPassGroupCategorySpecificAllocatedUncertaintyAmount BQ’m +

BAMonthOffPeakFRDPassGroupCategorySpecificAllocatedUncertaintyAmount BQ’m +

    - BAMonthPeakPassGroupFRDMeteredDemandAllocatedUncertaintyAmount BQ’m =

(BAAMonthPeakBAPassGroupFRDMeteredDemandAllocationQuantity BQ’m / EIMAreaMonthPeakFRDPassGroupMeteredDemandAllocationQuantity m) * EIMAreaMonthPeakPassGroupFRDNeutralityMeteredDemandAllocatedAmount m

    - BAMonthOffPeakPassGroupFRDMeteredDemandAllocatedUncertaintyAmount BQ’m =

(BAAMonthOffPeakBAPassGroupFRDMeteredDemandAllocationQuantity BQ’m / EIMAreaMonthOffPeakFRDPassGroupMeteredDemandAllocationQuantity m) * EIMAreaMonthOffPeakPassGroupFRDNeutralityMeteredDemandAllocatedAmount m

    - BAMonthPeakBAASpecificFRDMeteredDemandAllocatedUncertaintyAmount BQ’m =

If BADayGenOnlyBAAFlag BQ'md = 1

Then

BADayGenOnlyBAAFlag BQ'md * BAAMonthPeakBAASpecificFRDNeutralityMeteredDemandAllocatedAmount Q’m

Else

(BAAMonthPeakBABAASpecificFRDMeteredDemandAllocationQuantity BQ’m / BAAMonthPeakBAASpecificFRDMeteredDemandAllocationQuantity Q’m) * BAAMonthPeakBAASpecificFRDNeutralityMeteredDemandAllocatedAmount Q’m

    - BAMonthOffPeakBAASpecificFRDMeteredDemandAllocatedUncertaintyAmount BQ’m =

If BADayGenOnlyBAAFlag BQ'md = 1

Then

BADayGenOnlyBAAFlag BQ'md * BAAMonthOffPeakBAASpecificFRDNeutralityMeteredDemandAllocatedAmount Q’m

Else

(BAAMonthOffPeakBABAASpecificFRDMeteredDemandAllocationQuantity BQ’m / BAAMonthOffPeakBAASpecificFRDMeteredDemandAllocationQuantity Q’m) * BAAMonthOffPeakBAASpecificFRDNeutralityMeteredDemandAllocatedAmount Q’m

    - EIMAreaMonthPeakPassGroupFRDNeutralityMeteredDemandAllocatedAmount m =

EIMAreaMonthPeakPassGroupFRDUncertaintyAllocationAmount m –

EIMAreaMonthPeakPassGroupFRDAllocatedUncertaintyAmount m

    - EIMAreaMonthOffPeakPassGroupFRDNeutralityMeteredDemandAllocatedAmount m =

EIMAreaMonthOffPeakPassGroupFRDUncertaintyAllocationAmount m –

EIMAreaMonthOffPeakPassGroupFRDAllocatedUncertaintyAmount m

    - BAAMonthPeakBAASpecificFRDNeutralityMeteredDemandAllocatedAmount Q’m =

BAAMonthPeakBAASpecificFRDUncertaintyAllocationAmount Q’m –

BAAMonthPeakBAASpecificFRDAllocatedUncertaintyAmount Q’m

    - BAAMonthOffPeakBAASpecificFRDNeutralityMeteredDemandAllocatedAmount Q’m =

BAAMonthOffPeakBAASpecificFRDUncertaintyAllocationAmount Q’m –

BAAMonthOffPeakBAASpecificFRDAllocatedUncertaintyAmount Q’m

    - EIMAreaMonthPeakPassGroupFRDAllocatedUncertaintyAmount m =

Sum (B,Q’) BAMonthPeakFRDPassGroupCategorySpecificAllocatedUncertaintyAmount BQ’m

    - EIMAreaMonthOffPeakPassGroupFRDAllocatedUncertaintyAmount m =

Sum (B,Q’) BAMonthOffPeakFRDPassGroupCategorySpecificAllocatedUncertaintyAmount BQ’m

    - BAAMonthPeakBAASpecificFRDAllocatedUncertaintyAmount Q’m =

Sum (B) BAMonthPeakFRDBAACategorySpecificAllocatedUncertaintyAmount BQ’m

    - BAAMonthOffPeakBAASpecificFRDAllocatedUncertaintyAmount Q’m =

Sum (B) BAMonthOffPeakFRDBAACategorySpecificAllocatedUncertaintyAmount BQ’m

    - BAMonthPeakFRDPassGroupCategorySpecificAllocatedUncertaintyAmount BQ’m =

Sum (r,t,F’,S’)

BAMonthResourcePeakPassGroupLoadFRDUncertaintyAllocationAmount BrtF’S’Q’m +

BAMonthResourcePeakPassGroupIntertieFRDUncertaintyAllocationAmount BrtF’S’Q’m + BAMonthResourcePeakPassGroupSupplyFRDUncertaintyAllocationAmount BrtF’S’Q’m

    - BAMonthOffPeakFRDPassGroupCategorySpecificAllocatedUncertaintyAmount BQ’m =

Sum (r,t,F’,S’)

BAMonthResourceOffPeakPassGroupLoadFRDUncertaintyAllocationAmount BrtF’S’Q’m +

BAMonthResourceOffPeakPassGroupIntertieFRDUncertaintyAllocationAmount BrtF’S’Q’m + BAMonthResourceOffPeakPassGroupSupplyFRDUncertaintyAllocationAmount BrtF’S’Q’m

    - BAMonthPeakFRDBAACategorySpecificAllocatedUncertaintyAmount BQ’m =

Sum (r,t,F’,S’)

BAMonthResourcePeakBAASpecificLoadFRDUncertaintyAllocationAmount BrtF’S’Q’m +

BAMonthResourcePeakBAASpecificIntertieFRDUncertaintyAllocationAmount BrtF’S’Q’m + BAMonthResourcePeakBAASpecificSupplyFRDUncertaintyAllocationAmount BrtF’S’Q’m

    - BAMonthOffPeakFRDBAACategorySpecificAllocatedUncertaintyAmount BQ’m =

Sum (r,t,F’,S’)

BAMonthResourceOffPeakBAASpecificLoadFRDUncertaintyAllocationAmount BrtF’S’Q’m +

BAMonthResourceOffPeakBAASpecificIntertieFRDUncertaintyAllocationAmount BrtF’S’Q’m + BAMonthResourceOffPeakBAASpecificSupplyFRDUncertaintyAllocationAmount BrtF’S’Q’m

    - BAMonthResourcePeakPassGroupLoadFRDUncertaintyAllocationAmount BrtF’S’Q’m =

IF

EIMAreaMonthPeakTotalPassGroupLoadFRDUncertaintyAllocationQuantity m <> 0

THEN

BAMonthResourcePeakPassGroupLoadFRDUncertaintyAllocationAmount BrtF’S’Q’m =

(BAMonthResourcePeakPassGroupLoadFRDUncertaintyAllocationQuantityBrtF’S’Q’m

/ EIMAreaMonthPeakTotalPassGroupLoadFRDUncertaintyAllocationQuantity m) * EIMAreaMonthPeakPassGroupLoadFRDUncertaintyAllocationAmount m

ELSE

BAMonthResourcePeakPassGroupLoadFRDUncertaintyAllocationAmount BrtF’S’Q’m  = 0

END IF

    - BAMonthResourceOffPeakPassGroupLoadFRDUncertaintyAllocationAmount BrtF’S’Q’m =

IF

EIMAreaMonthOffPeakTotalPassGroupLoadFRDUncertaintyAllocationQuantity m <> 0

THEN

BAMonthResourceOffPeakPassGroupLoadFRDUncertaintyAllocationAmount BrtF’S’Q’m =

(BAMonthResourceOffPeakPassGroupLoadFRDUncertaintyAllocationQuantityBrtF’S’Q’m

/ EIMAreaMonthOffPeakTotalPassGroupLoadFRDUncertaintyAllocationQuantity m) * EIMAreaMonthOffPeakPassGroupLoadFRDUncertaintyAllocationAmount m

ELSE

BAMonthResourceOffPeakPassGroupLoadFRDUncertaintyAllocationAmount BrtF’S’Q’m  = 0

END IF

    - BAMonthResourcePeakBAASpecificLoadFRDUncertaintyAllocationAmount BrtF’S’Q’m =

IF

BAAMonthPeakTotalBAASpecificLoadFRDUncertaintyAllocationQuantity Q’m <> 0

THEN

BAMonthResourcePeakBAASpecificLoadFRDUncertaintyAllocationAmount BrtF’S’Q’m =

(BAMonthResourcePeakBAASpecificLoadFRDUncertaintyAllocationQuantityBrtF’S’Q’m  / BAAMonthPeakTotalBAASpecificLoadFRDUncertaintyAllocationQuantity Q’m) * BAAMonthPeakLoadBAAConstraintFRDUncertaintyAllocationAmount Q’m

ELSE

BAMonthResourcePeakBAASpecificLoadFRDUncertaintyAllocationAmount BrtF’S’Q’m = 0

END IF

    - BAMonthResourceOffPeakBAASpecificLoadFRDUncertaintyAllocationAmount BrtF’S’Q’m =

IF

BAAMonthOffPeakTotalBAASpecificLoadFRDUncertaintyAllocationQuantity Q’m <> 0

THEN

BAMonthResourceOffPeakBAASpecificLoadFRDUncertaintyAllocationAmount BrtF’S’Q’m =

(BAMonthResourceOffPeakBAASpecificLoadFRDUncertaintyAllocationQuantityBrtF’S’Q’m  / BAAMonthOffPeakTotalBAASpecificLoadFRDUncertaintyAllocationQuantity Q’m) * BAAMonthOffPeakLoadBAAConstraintFRDUncertaintyAllocationAmount Q’m

ELSE

BAMonthResourceOffPeakBAASpecificLoadFRDUncertaintyAllocationAmount BrtF’S’Q’m = 0

END IF

    - BAAMonthPeakTotalBAASpecificLoadFRDUncertaintyAllocationQuantity Q’m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) *  BAA5mTotalBAASpecificLoadFRDUncertaintyAllocationQuantity Q’mdhcif

    - BAAMonthOffPeakTotalBAASpecificLoadFRDUncertaintyAllocationQuantity Q’m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) *  BAA5mTotalBAASpecificLoadFRDUncertaintyAllocationQuantity Q’mdhcif

    - BAMonthResourcePeakBAASpecificLoadFRDUncertaintyAllocationQuantityBrtF’S’Q’m  =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) *  BA5mResourceBAASpecificLoadFRDUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - BAMonthResourceOffPeakBAASpecificLoadFRDUncertaintyAllocationQuantityBrtF’S’Q’m  =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) *  BA5mResourceBAASpecificLoadFRDUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - EIMAreaMonthPeakTotalPassGroupLoadFRDUncertaintyAllocationQuantity m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) *  EIMArea5mTotalPassGroupLoadFRDUncertaintyAllocationQuantity mdhcif

    - EIMAreaMonthOffPeakTotalPassGroupLoadFRDUncertaintyAllocationQuantity m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) *  EIMArea5mTotalPassGroupLoadFRDUncertaintyAllocationQuantity mdhcif

    - BAMonthResourcePeakPassGroupLoadFRDUncertaintyAllocationQuantityBrtF’S’Q’m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) *  BA5mResourcePassGroupLoadFRDUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - BAMonthResourceOffPeakPassGroupLoadFRDUncertaintyAllocationQuantityBrtF’S’Q’m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) *  BA5mResourcePassGroupLoadFRDUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - BAMonthResourcePeakPassGroupIntertieFRDUncertaintyAllocationAmount BrtF’S’Q’m =

IF

EIMAreaMonthPeakTotalPassGroupIntertieFRDUncertaintyAllocationQuantity m <> 0

THEN

BAMonthResourcePeakPassGroupIntertieFRDUncertaintyAllocationAmount BrtF’S’Q’m =

(BAMonthResourcePeakPassGroupIntertieFRDUncertaintyAllocationQuantityBrtF’S’Q’m

/ EIMAreaMonthPeakTotalPassGroupIntertieFRDUncertaintyAllocationQuantity m) * EIMAreaMonthPeakPassGroupIntertieFRDUncertaintyAllocationAmount m

ELSE

BAMonthResourcePeakPassGroupIntertieFRDUncertaintyAllocationAmount BrtF’S’Q’m  = 0

END IF

    - BAMonthResourceOffPeakPassGroupIntertieFRDUncertaintyAllocationAmount BrtF’S’Q’m =

IF

EIMAreaMonthOffPeakTotalPassGroupIntertieFRDUncertaintyAllocationQuantity m <> 0

THEN

BAMonthResourceOffPeakPassGroupIntertieFRDUncertaintyAllocationAmount BrtF’S’Q’m =

(BAMonthResourceOffPeakPassGroupIntertieFRDUncertaintyAllocationQuantityBrtF’S’Q’m

/ EIMAreaMonthOffPeakTotalPassGroupIntertieFRDUncertaintyAllocationQuantity m) * EIMAreaMonthOffPeakPassGroupIntertieFRDUncertaintyAllocationAmount m

ELSE

BAMonthResourceOffPeakPassGroupIntertieFRDUncertaintyAllocationAmount BrtF’S’Q’m  = 0

END IF

    - BAMonthResourcePeakBAASpecificIntertieFRDUncertaintyAllocationAmount BrtF’S’Q’m =

IF

BAAMonthPeakTotalBAASpecificIntertieFRDUncertaintyAllocationQuantity Q’m <> 0

THEN

BAMonthResourcePeakBAASpecificIntertieFRDUncertaintyAllocationAmount BrtF’S’Q’m =

(BAMonthResourcePeakBAASpecificIntertieFRDUncertaintyAllocationQuantityBrtF’S’Q’m  / BAAMonthPeakTotalBAASpecificIntertieFRDUncertaintyAllocationQuantity Q’m) * BAAMonthPeakIntertieBAAConstraintFRDUncertaintyAllocationAmount Q’m

ELSE

BAMonthResourcePeakBAASpecificIntertieFRDUncertaintyAllocationAmount BrtF’S’Q’m = 0

END IF

    - BAMonthResourceOffPeakBAASpecificIntertieFRDUncertaintyAllocationAmount BrtF’S’Q’m =

IF

BAAMonthOffPeakTotalBAASpecificIntertieFRDUncertaintyAllocationQuantity Q’m <> 0

THEN

BAMonthResourceOffPeakBAASpecificIntertieFRDUncertaintyAllocationAmount BrtF’S’Q’m =

(BAMonthResourceOffPeakBAASpecificIntertieFRDUncertaintyAllocationQuantityBrtF’S’Q’m  / BAAMonthOffPeakTotalBAASpecificIntertieFRDUncertaintyAllocationQuantity Q’m) * BAAMonthOffPeakIntertieBAAConstraintFRDUncertaintyAllocationAmount Q’m

ELSE

BAMonthResourceOffPeakBAASpecificIntertieFRDUncertaintyAllocationAmount BrtF’S’Q’m = 0

END IF

    - BAAMonthPeakTotalBAASpecificIntertieFRDUncertaintyAllocationQuantity Q’m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) *  BAA5mTotalBAASpecificIntertieFRDUncertaintyAllocationQuantity Q’mdhcif

    - BAAMonthOffPeakTotalBAASpecificIntertieFRDUncertaintyAllocationQuantity Q’m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) *  BAA5mTotalBAASpecificIntertieFRDUncertaintyAllocationQuantity Q’mdhcif

    - BAMonthResourcePeakBAASpecificIntertieFRDUncertaintyAllocationQuantityBrtF’S’Q’m  =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) *  BA5mResourceBAASpecificIntertieFRDUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - BAMonthResourceOffPeakBAASpecificIntertieFRDUncertaintyAllocationQuantityBrtF’S’Q’m  =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) *  BA5mResourceBAASpecificIntertieFRDUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - EIMAreaMonthPeakTotalPassGroupIntertieFRDUncertaintyAllocationQuantity m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) *  EIMArea5mTotalPassGroupIntertieFRDUncertaintyAllocationQuantity mdhcif

    - EIMAreaMonthOffPeakTotalPassGroupIntertieFRDUncertaintyAllocationQuantity m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) *  EIMArea5mTotalPassGroupIntertieFRDUncertaintyAllocationQuantity mdhcif

    - BAMonthResourcePeakPassGroupIntertieFRDUncertaintyAllocationQuantityBrtF’S’Q’m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) *  BA5mResourcePassGroupIntertieFRDUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - BAMonthResourceOffPeakPassGroupIntertieFRDUncertaintyAllocationQuantityBrtF’S’Q’m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) *  BA5mResourcePassGroupIntertieFRDUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - BAMonthResourcePeakPassGroupSupplyFRDUncertaintyAllocationAmount BrtF’S’Q’m =

IF

EIMAreaMonthPeakTotalPassGroupSupplyFRDUncertaintyAllocationQuantity m <> 0

THEN

BAMonthResourcePeakPassGroupSupplyFRDUncertaintyAllocationAmount BrtF’S’Q’m =

(BAMonthResourcePeakPassGroupSupplyFRDUncertaintyAllocationQuantityBrtF’S’Q’m

/ EIMAreaMonthPeakTotalPassGroupSupplyFRDUncertaintyAllocationQuantity m) * EIMAreaMonthPeakPassGroupSupplyFRDUncertaintyAllocationAmount m

ELSE

BAMonthResourcePeakPassGroupSupplyFRDUncertaintyAllocationAmount BrtF’S’Q’m  = 0

END IF

    - BAMonthResourceOffPeakPassGroupSupplyFRDUncertaintyAllocationAmount BrtF’S’Q’m =

IF

EIMAreaMonthOffPeakTotalPassGroupSupplyFRDUncertaintyAllocationQuantity m <> 0

THEN

BAMonthResourceOffPeakPassGroupSupplyFRDUncertaintyAllocationAmount BrtF’S’Q’m =

(BAMonthResourceOffPeakPassGroupSupplyFRDUncertaintyAllocationQuantityBrtF’S’Q’m

/ EIMAreaMonthOffPeakTotalPassGroupSupplyFRDUncertaintyAllocationQuantity m) * EIMAreaMonthOffPeakPassGroupSupplyFRDUncertaintyAllocationAmount m

ELSE

BAMonthResourceOffPeakPassGroupSupplyFRDUncertaintyAllocationAmount BrtF’S’Q’m  = 0

END IF

    - BAMonthResourcePeakBAASpecificSupplyFRDUncertaintyAllocationAmount BrtF’S’Q’m =

IF

BAAMonthPeakTotalBAASpecificSupplyFRDUncertaintyAllocationQuantity Q’m <> 0

THEN

BAMonthResourcePeakBAASpecificSupplyFRDUncertaintyAllocationAmount BrtF’S’Q’m =

(BAMonthResourcePeakBAASpecificSupplyFRDUncertaintyAllocationQuantityBrtF’S’Q’m  / BAAMonthPeakTotalBAASpecificSupplyFRDUncertaintyAllocationQuantity Q’m) * BAAMonthPeakSupplyBAAConstraintFRDUncertaintyAllocationAmount Q’m

ELSE

BAMonthResourcePeakBAASpecificSupplyFRDUncertaintyAllocationAmount BrtF’S’Q’m = 0

END IF

    - BAMonthResourceOffPeakBAASpecificSupplyFRDUncertaintyAllocationAmount BrtF’S’Q’m =

IF

BAAMonthOffPeakTotalBAASpecificSupplyFRDUncertaintyAllocationQuantity Q’m <> 0

THEN

BAMonthResourceOffPeakBAASpecificSupplyFRDUncertaintyAllocationAmount BrtF’S’Q’m =

(BAMonthResourceOffPeakBAASpecificSupplyFRDUncertaintyAllocationQuantityBrtF’S’Q’m  / BAAMonthOffPeakTotalBAASpecificSupplyFRDUncertaintyAllocationQuantity Q’m) * BAAMonthOffPeakSupplyBAAConstraintFRDUncertaintyAllocationAmount Q’m

ELSE

BAMonthResourceOffPeakBAASpecificSupplyFRDUncertaintyAllocationAmount BrtF’S’Q’m = 0

END IF

    - BAAMonthPeakTotalBAASpecificSupplyFRDUncertaintyAllocationQuantity Q’m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) *  BAA5mTotalBAASpecificSupplyFRDUncertaintyAllocationQuantity Q’mdhcif

    - BAAMonthOffPeakTotalBAASpecificSupplyFRDUncertaintyAllocationQuantity Q’m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) *  BAA5mTotalBAASpecificSupplyFRDUncertaintyAllocationQuantity Q’mdhcif

    - BAMonthResourcePeakBAASpecificSupplyFRDUncertaintyAllocationQuantityBrtF’S’Q’m  =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) *  BA5mResourceBAASpecificSupplyFRDUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - BAMonthResourceOffPeakBAASpecificSupplyFRDUncertaintyAllocationQuantityBrtF’S’Q’m  =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) *  BA5mResourceBAASpecificSupplyFRDUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - EIMAreaMonthPeakTotalPassGroupSupplyFRDUncertaintyAllocationQuantity m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) *  EIMArea5mTotalPassGroupSupplyFRDUncertaintyAllocationQuantity mdhcif

    - EIMAreaMonthOffPeakTotalPassGroupSupplyFRDUncertaintyAllocationQuantity m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) *  EIMArea5mTotalPassGroupSupplyFRDUncertaintyAllocationQuantity mdhcif

    - BAMonthResourcePeakPassGroupSupplyFRDUncertaintyAllocationQuantityBrtF’S’Q’m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) *  BA5mResourcePassGroupSupplyFRDUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - BAMonthResourceOffPeakPassGroupSupplyFRDUncertaintyAllocationQuantityBrtF’S’Q’m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) *  BA5mResourcePassGroupSupplyFRDUncertaintyAllocationQuantity BrtF’S’Q’mdhcif

    - EIMAreaMonthOffPeakPassGroupLoadFRDUncertaintyAllocationAmount m =

Sum (Q’,Q’’)

IF

EIMAreaMonthOffPeakPassGroupAllCategoriesFRDUncertaintyQuantity m <> 0

THEN

EIMAreaMonthOffPeakPassGroupLoadFRDUncertaintyAllocationAmount m =

BAAMonthOffPeakHourFlexibleRampDownUncertaintySettlementAmount Q’Q’’m * (EIMAreaMonthOffPeakPassGroupLoadFRDUncertaintyQuantity m

/ EIMAreaMonthOffPeakPassGroupAllCategoriesFRDUncertaintyQuantity m)

ELSE

EIMAreaMonthOffPeakPassGroupLoadFRDUncertaintyAllocationAmount m = 0

END IF

Where Q’’ = ‘FRD_PASS_GRP’

    - EIMAreaMonthPeakPassGroupLoadFRDUncertaintyAllocationAmount m =

Sum (Q’,Q’’)

IF

EIMAreaMonthPeakPassGroupAllCategoriesFRDUncertaintyQuantity m <> 0

THEN

EIMAreaMonthPeakPassGroupLoadFRDUncertaintyAllocationAmount m =

BAAMonthPeakHourFlexibleRampDownUncertaintySettlementAmount Q’Q’’m * (EIMAreaMonthPeakPassGroupLoadFRDUncertaintyQuantity m

/ EIMAreaMonthPeakPassGroupAllCategoriesFRDUncertaintyQuantity m)

ELSE

EIMAreaMonthPeakPassGroupLoadFRDUncertaintyAllocationAmount m = 0

END IF

Where Q’’ = ‘FRD_PASS_GRP’

    - BAAMonthOffPeakLoadBAAConstraintFRDUncertaintyAllocationAmount Q’m =

Sum (Q’’)

IF

BAAMonthOffPeakBAASpecificAllCategoriesFRDUncertaintyQuantity Q’m

<> 0

THEN

BAAMonthOffPeakLoadBAAConstraintFRDUncertaintyAllocationAmount Q’m =

BAAMonthOffPeakHourFlexibleRampDownUncertaintySettlementAmount Q’Q’’m * (BAAMonthOffPeakBAASpecificLoadFRDUncertaintyQuantity Q’m

/ BAAMonthOffPeakBAASpecificAllCategoriesFRDUncertaintyQuantity Q’m)

ELSE

BAAMonthOffPeakLoadBAAConstraintFRDUncertaintyAllocationAmount Q’m =0

END IF

Where Q’’ <> ‘FRD_PASS_GRP’

    - BAAMonthPeakLoadBAAConstraintFRDUncertaintyAllocationAmount Q’m =

Sum (Q’’)

IF

BAAMonthPeakBAASpecificAllCategoriesFRDUncertaintyQuantity Q’m

<> 0

THEN

BAAMonthPeakLoadBAAConstraintFRDUncertaintyAllocationAmount Q’m =

BAAMonthPeakHourFlexibleRampDownUncertaintySettlementAmount Q’Q’’m * (BAAMonthPeakBAASpecificLoadFRDUncertaintyQuantity Q’m

/ BAAMonthPeakBAASpecificAllCategoriesFRDUncertaintyQuantity Q’m)

ELSE

BAAMonthPeakLoadBAAConstraintFRDUncertaintyAllocationAmount Q’m = 0

END IF

Where Q’’ <> ‘FRD_PASS_GRP’

    - EIMAreaMonthOffPeakPassGroupIntertieFRDUncertaintyAllocationAmount m =

Sum (Q’,Q’’)

IF

EIMAreaMonthOffPeakPassGroupAllCategoriesFRDUncertaintyQuantity m <> 0

THEN

EIMAreaMonthOffPeakPassGroupIntertieFRDUncertaintyAllocationAmount m =

BAAMonthOffPeakHourFlexibleRampDownUncertaintySettlementAmount Q’Q’’m

* (EIMAreaMonthOffPeakPassGroupIntertieFRDUncertaintyQuantity m

/ EIMAreaMonthOffPeakPassGroupAllCategoriesFRDUncertaintyQuantity m)

ELSE

EIMAreaMonthOffPeakPassGroupIntertieFRDUncertaintyAllocationAmount m = 0

END IF

Where Q’’ = ‘FRD_PASS_GRP’

    - EIMAreaMonthPeakPassGroupIntertieFRDUncertaintyAllocationAmount m =

Sum (Q’,Q’’)

IF

EIMAreaMonthPeakPassGroupAllCategoriesFRDUncertaintyQuantity m <> 0

THEN

EIMAreaMonthPeakPassGroupIntertieFRDUncertaintyAllocationAmount m =

BAAMonthPeakHourFlexibleRampDownUncertaintySettlementAmount Q’Q’’m

* (EIMAreaMonthPeakPassGroupIntertieFRDUncertaintyQuantity m

/ EIMAreaMonthPeakPassGroupAllCategoriesFRDUncertaintyQuantity m)

ELSE

EIMAreaMonthPeakPassGroupIntertieFRDUncertaintyAllocationAmount m = 0

END IF

Where Q’’ = ‘FRD_PASS_GRP’

    - BAAMonthOffPeakIntertieBAAConstraintFRDUncertaintyAllocationAmount Q’m =

Sum (Q’’)

IF

BAAMonthOffPeakBAASpecificAllCategoriesFRDUncertaintyQuantity Q’m

<> 0

THEN

BAAMonthOffPeakIntertieBAAConstraintFRDUncertaintyAllocationAmount Q’m =

BAAMonthOffPeakHourFlexibleRampDownUncertaintySettlementAmount Q’Q’’m * (BAAMonthOffPeakBAASpecificIntertieFRDUncertaintyQuantity Q’m

/ BAAMonthOffPeakBAASpecificAllCategoriesFRDUncertaintyQuantity Q’m)

ELSE

BAAMonthOffPeakIntertieBAAConstraintFRDUncertaintyAllocationAmount Q’m =0

END IF

Where Q’’ <> ‘FRD_PASS_GRP’

    - BAAMonthPeakIntertieBAAConstraintFRDUncertaintyAllocationAmount Q’m =

Sum (Q’’)

IF

BAAMonthPeakBAASpecificAllCategoriesFRDUncertaintyQuantity Q’m

<> 0

THEN

BAAMonthPeakIntertieBAAConstraintFRDUncertaintyAllocationAmount Q’m =

BAAMonthPeakHourFlexibleRampDownUncertaintySettlementAmount Q’Q’’m * (BAAMonthPeakBAASpecificIntertieFRDUncertaintyQuantity Q’m

/ BAAMonthPeakBAASpecificAllCategoriesFRDUncertaintyQuantity Q’m)

ELSE

BAAMonthPeakIntertieBAAConstraintFRDUncertaintyAllocationAmount Q’m = 0

END IF

Where Q’’ <> ‘FRD_PASS_GRP’

    - EIMAreaMonthOffPeakPassGroupSupplyFRDUncertaintyAllocationAmount m =

Sum (Q’,Q’’)

IF

EIMAreaMonthOffPeakPassGroupAllCategoriesFRDUncertaintyQuantity m <> 0

THEN

EIMAreaMonthOffPeakPassGroupSupplyFRDUncertaintyAllocationAmount m =

BAAMonthOffPeakHourFlexibleRampDownUncertaintySettlementAmount Q’Q’’m * (EIMAreaMonthOffPeakPassGroupSupplyFRDUncertaintyQuantity m

/ EIMAreaMonthOffPeakPassGroupAllCategoriesFRDUncertaintyQuantity m)

ELSE

EIMAreaMonthOffPeakPassGroupSupplyFRDUncertaintyAllocationAmount m = 0

END IF

Where Q’’ = ‘FRD_PASS_GRP’

    - EIMAreaMonthPeakPassGroupSupplyFRDUncertaintyAllocationAmount m =

Sum (Q’,Q’’)

IF

EIMAreaMonthPeakPassGroupAllCategoriesFRDUncertaintyQuantity m <> 0

THEN

EIMAreaMonthPeakPassGroupSupplyFRDUncertaintyAllocationAmount m =

BAAMonthPeakHourFlexibleRampDownUncertaintySettlementAmount Q’Q’’m

* (EIMAreaMonthPeakPassGroupSupplyFRDUncertaintyQuantity m

/ EIMAreaMonthPeakPassGroupAllCategoriesFRDUncertaintyQuantity m)

ELSE

EIMAreaMonthPeakPassGroupSupplyFRDUncertaintyAllocationAmount m = 0

END IF

Where Q’’ = ‘FRD_PASS_GRP’

    - BAAMonthOffPeakSupplyBAAConstraintFRDUncertaintyAllocationAmount Q’m =

Sum (Q’’)

IF

BAAMonthOffPeakBAASpecificAllCategoriesFRDUncertaintyQuantity Q’m

<> 0

THEN

BAAMonthOffPeakSupplyBAAConstraintFRDUncertaintyAllocationAmount Q’m =

BAAMonthOffPeakHourFlexibleRampDownUncertaintySettlementAmount Q’Q’’m

* (BAAMonthOffPeakBAASpecificSupplyFRDUncertaintyQuantity Q’m

/ BAAMonthOffPeakBAASpecificAllCategoriesFRDUncertaintyQuantity Q’m)

ELSE

BAAMonthOffPeakSupplyBAAConstraintFRDUncertaintyAllocationAmount Q’m =0

END IF

Where Q’’ <> ‘FRD_PASS_GRP’

    - BAAMonthPeakSupplyBAAConstraintFRDUncertaintyAllocationAmount Q’m =

Sum (Q’’)

IF

BAAMonthPeakBAASpecificAllCategoriesFRDUncertaintyQuantity Q’m

<> 0

THEN

BAAMonthPeakSupplyBAAConstraintFRDUncertaintyAllocationAmount Q’m =

BAAMonthPeakHourFlexibleRampDownUncertaintySettlementAmount Q’Q’’m

* (BAAMonthPeakBAASpecificSupplyFRDUncertaintyQuantity Q’m

/ BAAMonthPeakBAASpecificAllCategoriesFRDUncertaintyQuantity Q’m)

ELSE

BAAMonthPeakSupplyBAAConstraintFRDUncertaintyAllocationAmount Q’m = 0

END IF

Where Q’’ <> ‘FRD_PASS_GRP’

    - BAAMonthOffPeakHourFlexibleRampDownUncertaintySettlementAmount Q’Q’’m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) *  (-1) * BAAConstraint5mFlexRampDownUncertaintyAmount Q’Q’’mdhcif

    - BAAMonthPeakHourFlexibleRampDownUncertaintySettlementAmount Q’Q’’m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) *  (-1) * BAAConstraint5mFlexRampDownUncertaintyAmount Q’Q’’mdhcif

    - EIMAreaMonthOffPeakPassGroupFRDUncertaintyAllocationAmount m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) * EIMArea5mPassGroupFRDUncertaintyAllocationAmount mdhcif

    - EIMAreaMonthPeakPassGroupFRDUncertaintyAllocationAmount m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) * EIMArea5mPassGroupFRDUncertaintyAllocationAmount mdhcif

    - BAAMonthOffPeakBAASpecificFRDUncertaintyAllocationAmount Q’m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) * BAA5mBAASpecificFRDUncertaintyAllocationAmount Q’mdhcif

    - BAAMonthPeakBAASpecificFRDUncertaintyAllocationAmount Q’m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) * BAA5mBAASpecificFRDUncertaintyAllocationAmount Q’mdhcif

### EIMAreaMonthPeakPassGroupAllCategoriesFRDUncertaintyQuantity m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) * EIMArea5mPassGroupAllCategoriesFRDUncertaintyQuantity mdhcif

### EIMAreaMonthOffPeakPassGroupAllCategoriesFRDUncertaintyQuantity m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) * EIMArea5mPassGroupAllCategoriesFRDUncertaintyQuantity mdhcif

    - BAAMonthPeakBAASpecificAllCategoriesFRDUncertaintyQuantity Q’m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) * BAA5mBAASpecificAllCategoriesFRDUncertaintyQuantity Q’mdhcif

    - BAAMonthOffPeakBAASpecificAllCategoriesFRDUncertaintyQuantity Q’m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) * BAA5mBAASpecificAllCategoriesFRDUncertaintyQuantity Q’mdhcif

    - EIMAreaMonthOffPeakPassGroupLoadFRDUncertaintyQuantity m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) * EIMArea5mPassGroupLoadFRDUncertaintyQuantity mdhcif

    - EIMAreaMonthPeakPassGroupLoadFRDUncertaintyQuantity m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) * EIMArea5mPassGroupLoadFRDUncertaintyQuantity mdhcif

    - BAAMonthOffPeakBAASpecificLoadFRDUncertaintyQuantity Q’m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) * BAA5mBAASpecificLoadFRDUncertaintyQuantity Q’mdhcif

    - BAAMonthPeakBAASpecificLoadFRDUncertaintyQuantity Q’m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) * BAA5mBAASpecificLoadFRDUncertaintyQuantity Q’mdhcif

    - EIMAreaMonthOffPeakPassGroupIntertieFRDUncertaintyQuantity m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) * EIMArea5mPassGroupIntertieFRDUncertaintyQuantity mdhcif

    - EIMAreaMonthPeakPassGroupIntertieFRDUncertaintyQuantity m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) * EIMArea5mPassGroupIntertieFRDUncertaintyQuantity mdhcif

    - BAAMonthOffPeakBAASpecificIntertieFRDUncertaintyQuantity Q’m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) * BAA5mBAASpecificIntertieFRDUncertaintyQuantity Q’mdhcif

    - BAAMonthPeakBAASpecificIntertieFRDUncertaintyQuantity Q’m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) * BAA5mBAASpecificIntertieFRDUncertaintyQuantity Q’mdhcif

    - EIMAreaMonthOffPeakPassGroupSupplyFRDUncertaintyQuantity m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) * EIMArea5mPassGroupSupplyFRDUncertaintyQuantity mdhcif

    - EIMAreaMonthPeakPassGroupSupplyFRDUncertaintyQuantity m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) * EIMArea5mPassGroupSupplyFRDUncertaintyQuantity mdhcif

    - BAAMonthOffPeakBAASpecificSupplyFRDUncertaintyQuantity Q’m =

Sum(d,h,c,i,f) (1 – PeakHourFlag mdh ) * BAA5mBAASpecificSupplyFRDUncertaintyQuantity Q’mdhcif

    - BAAMonthPeakBAASpecificSupplyFRDUncertaintyQuantity Q’m =

Sum(d,h,c,i,f) (PeakHourFlag mdh ) * BAA5mBAASpecificSupplyFRDUncertaintyQuantity Q’mdhcif

## Outputs

| - Row # Name; Description. --; In addition to any outputs listed below, all inputs shall be included as outputs.. BAMonthlyCompleteFRUUncertaintyAllocationAmount BQ’m; FRU Uncertainty Allocation Amount (in $) assessed monthly to a BA of the BAA as the difference of the monthly FRU Allocation Amount for the designated. BAMonthlyTotalDailyFRUUncertaintyAllocationAmount BQ’m; The montly total (in $) of the daily FRU Uncertainty Allocation Amounts assessed to a BA of resources throughout the EIM Area over all Trading Days of the designated Trading Month.. BAMonthAllHourCompleteFRUUncertaintyAllocationAmount BQ’m; The total (in $) of the FRU Uncertainty Allocation Amount assessed on a monthly basis to a BA of resources throughout the EIM Area for the designated Trading Month.. BAMonthPeakPassGroupFRUMeteredDemandAllocatedUncertaintyAmount BQ’m; The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is shared over the EIM Area of EIM entities that pass their Flex Ramp Up Balancing Test and apportioned to a BA of the specified BAA based on metered Demand in association with the Peak Flexible Ramp Hours of the Trading Month.. BAMonthOffPeakPassGroupFRUMeteredDemandAllocatedUncertaintyAmount BQ’m; The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is shared over the EIM Area of EIM entities that pass their Flex Ramp Up Balancing Test and apportioned to a BA of the specified BAA based on metered Demand in association with the Off Peak Flexible Ramp Hours of the Trading Month.. BAMonthPeakBAASpecificFRUMeteredDemandAllocatedUncertaintyAmount BQ’m; The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is assigned to the specific BAA who failed their Flex Ramp Up Balancing Test and apportioned to a BA of the specified BAA based on metered Demand in association with the Peak Flexible Ramp Hours of the Trading Month.. BAMonthOffPeakBAASpecificFRUMeteredDemandAllocatedUncertaintyAmount BQ’m; The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is assigned to the specific BAA who failed their Flex Ramp Up Balancing Test and apportioned to a BA of the specified BAA based on metered Demand in association with the Off Peak Flexible Ramp Hours of the Trading Month.. EIMAreaMonthPeakPassGroupFRUNeutralityMeteredDemandAllocatedAmount m; The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is shared over the EIM Areaof EIM entities that pass their Flex Ramp Up Balancing Test in association with the Peak Flexible Ramp Hours of the Trading Month for which the allocation is based on metered Demand.. EIMAreaMonthOffPeakPassGroupFRUNeutralityMeteredDemandAllocatedAmount m; The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is shared over the EIM Areaof EIM entities that pass their Flex Ramp Up Balancing Test in association with the Off Peak Flexible Ramp Hours of the Trading Month for which the allocation is based on metered Demand.. BAAMonthPeakBAASpecificFRUNeutralityMeteredDemandAllocatedAmount Q’m; The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is assigned to the specific BAA who failed their Flex Ramp Up Balancing Test in association with the Peak Flexible Ramp Hours of the Trading Month for which the allocation is based on metered Demand.. BAAMonthOffPeakBAASpecificFRUNeutralityMeteredDemandAllocatedAmount Q’m; The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is assigned to the specific BAA who failed their Flex Ramp Up Balancing Test in association with the Off Peak Flexible Ramp Hours of the Trading Month for which the allocation is based on metered Demand.. EIMAreaMonthPeakPassGroupFRUAllocatedUncertaintyAmount m; The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is assigned to EIM Area resources of EIM entities that pass their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month.. EIMAreaMonthOffPeakPassGroupFRUAllocatedUncertaintyAmount m; The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is assigned to EIM Area resources of EIM entities that pass their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month.. BAAMonthPeakBAASpecificFRUAllocatedUncertaintyAmount Q’m; The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is assigned to the specific BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month.. BAAMonthOffPeakBAASpecificFRUAllocatedUncertaintyAmount Q’m; The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is assigned to the specific BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month.. BAMonthPeakFRUPassGroupCategorySpecificAllocatedUncertaintyAmount BQ’m; The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is assigned to resources of a BA of the specified BAA of EIM entities that pass their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month.. BAMonthOffPeakFRUPassGroupCategorySpecificAllocatedUncertaintyAmount BQ’m; The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is assigned to resources of a BA of the specified BAA of EIM entities that pass their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month.. BAMonthPeakFRUBAACategorySpecificAllocatedUncertaintyAmount BQ’m; The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is assigned to resources of a BA of the specified BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month.. BAMonthOffPeakFRUBAACategorySpecificAllocatedUncertaintyAmount BQ’m; The portion (in $) of the monthly allocated FRU Uncertainty Allocation amount that is assigned to resources of a BA of the specified BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month.. BAMonthResourcePeakPassGroupLoadFRUUncertaintyAllocationAmount BrtF’S’Q’m; The monthly allocated FRU Uncertainty Allocation amount (in $) that is assigned to a Load category resource of the specified BA and BAA of EIM entities that pass their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month.. BAMonthResourceOffPeakPassGroupLoadFRUUncertaintyAllocationAmount BrtF’S’Q’m; The monthly allocated FRU Uncertainty Allocation amount (in $) that is assigned to a Load category resource of the specified BA and BAA of EIM entities that pass their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month.. BAMonthResourcePeakBAASpecificLoadFRUUncertaintyAllocationAmount BrtF’S’Q’m; The monthly allocated FRU Uncertainty Allocation amount (in $) that is assigned to a Load category resource of the specified BA and BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month.. BAMonthResourceOffPeakBAASpecificLoadFRUUncertaintyAllocationAmount BrtF’S’Q’m; The monthly allocated FRU Uncertainty Allocation amount (in $) that is assigned to a Load category resource of the specified BA and BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month.. BAAMonthPeakTotalBAASpecificLoadFRUUncertaintyAllocationQuantity Q’m; The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to Load category resources of the specified BAA who failed their Flex Ramp Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month.. BAAMonthOffPeakTotalBAASpecificLoadFRUUncertaintyAllocationQuantity Q’m; The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to Load category resources of the specified BAA who failed their Flex Ramp Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month.. BAMonthResourcePeakBAASpecificLoadFRUUncertaintyAllocationQuantityBrtF’S’Q’m; The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to the specified Load category resource of a BA of a specific BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month.. BAMonthResourceOffPeakBAASpecificLoadFRUUncertaintyAllocationQuantityBrtF’S’Q’m; The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to the specified Load category resource of a BA of a specific BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month.. EIMAreaMonthPeakTotalPassGroupLoadFRUUncertaintyAllocationQuantity m; The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to Load category resources across the EIM Area of EIM entities that passed theif Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month.. EIMAreaMonthOffPeakTotalPassGroupLoadFRUUncertaintyAllocationQuantity m; The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to Load category resources across the EIM Area of EIM entities that passed theif Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month.. BAMonthResourcePeakPassGroupLoadFRUUncertaintyAllocationQuantityBrtF’S’Q’m; The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to the specified Load category resource of a BA that belongs to a specific BAA that passed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month.. BAMonthResourceOffPeakPassGroupLoadFRUUncertaintyAllocationQuantityBrtF’S’Q’m; The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to the specified Load category resource of a BA that belongs to a specific BAA that passed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month.. BAMonthResourcePeakPassGroupIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’m; The monthly allocated FRU Uncertainty Allocation amount (in $) that is assigned to a Intertie category resource of the specified BA and BAA of EIM entities that pass their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month.. BAMonthResourceOffPeakPassGroupIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’m; The monthly allocated FRU Uncertainty Allocation amount (in $) that is assigned to a Intertie category resource of the specified BA and BAA of EIM entities that pass their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month.. BAMonthResourcePeakBAASpecificIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’m; The monthly allocated FRU Uncertainty Allocation amount (in $) that is assigned to a Intertie category resource of the specified BA and BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month.. BAMonthResourceOffPeakBAASpecificIntertieFRUUncertaintyAllocationAmount BrtF’S’Q’m; The monthly allocated FRU Uncertainty Allocation amount (in $) that is assigned to a Intertie category resource of the specified BA and BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month.. BAAMonthPeakTotalBAASpecificIntertieFRUUncertaintyAllocationQuantity Q’m; The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to Intertie category resources of the specified BAA who failed their Flex Ramp Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month.. BAAMonthOffPeakTotalBAASpecificIntertieFRUUncertaintyAllocationQuantity Q’m; The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to Intertie category resources of the specified BAA who failed their Flex Ramp Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month.. BAMonthResourcePeakBAASpecificIntertieFRUUncertaintyAllocationQuantityBrtF’S’Q’m; The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to the specified Intertie category resource of a BA of a specific BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month.. BAMonthResourceOffPeakBAASpecificIntertieFRUUncertaintyAllocationQuantityBrtF’S’Q’m; The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to the specified Intertie category resource of a BA of a specific BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month.. EIMAreaMonthPeakTotalPassGroupIntertieFRUUncertaintyAllocationQuantity m; The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to Intertie category resources across the EIM Area of EIM entities that passed theif Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month.. EIMAreaMonthOffPeakTotalPassGroupIntertieFRUUncertaintyAllocationQuantity m; The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to Intertie category resources across the EIM Area of EIM entities that passed theif Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month.. BAMonthResourcePeakPassGroupIntertieFRUUncertaintyAllocationQuantityBrtF’S’Q’m; The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to the specified Intertie category resource of a BA that belongs to a specific BAA that passed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month.. BAMonthResourceOffPeakPassGroupIntertieFRUUncertaintyAllocationQuantityBrtF’S’Q’m; The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to the specified Intertie category resource of a BA that belongs to a specific BAA that passed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month.. BAMonthResourcePeakPassGroupSupplyFRUUncertaintyAllocationAmount BrtF’S’Q’m; The monthly allocated FRU Uncertainty Allocation amount (in $) that is assigned to a Supply category resource of the specified BA and BAA of EIM entities that pass their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month.. BAMonthResourceOffPeakPassGroupSupplyFRUUncertaintyAllocationAmount BrtF’S’Q’m; The monthly allocated FRU Uncertainty Allocation amount (in $) that is assigned to a Supply category resource of the specified BA and BAA of EIM entities that pass their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month.. BAMonthResourcePeakBAASpecificSupplyFRUUncertaintyAllocationAmount BrtF’S’Q’m; The monthly allocated FRU Uncertainty Allocation amount (in $) that is assigned to a Supply category resource of the specified BA and BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month.. BAMonthResourceOffPeakBAASpecificSupplyFRUUncertaintyAllocationAmount BrtF’S’Q’m; The monthly allocated FRU Uncertainty Allocation amount (in $) that is assigned to a Supply category resource of the specified BA and BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month.. BAAMonthPeakTotalBAASpecificSupplyFRUUncertaintyAllocationQuantity Q’m; The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to Supply category resources of the specified BAA who failed their Flex Ramp Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month.. BAAMonthOffPeakTotalBAASpecificSupplyFRUUncertaintyAllocationQuantity Q’m; The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to Supply category resources of the specified BAA who failed their Flex Ramp Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month.. BAMonthResourcePeakBAASpecificSupplyFRUUncertaintyAllocationQuantityBrtF’S’Q’m; The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to the specified Supply category resource of a BA of a specific BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month.. BAMonthResourceOffPeakBAASpecificSupplyFRUUncertaintyAllocationQuantityBrtF’S’Q’m; The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to the specified Supply category resource of a BA of a specific BAA who failed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month.. EIMAreaMonthPeakTotalPassGroupSupplyFRUUncertaintyAllocationQuantity m; The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to Supply category resources across the EIM Area of EIM entities that passed theif Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month.. EIMAreaMonthOffPeakTotalPassGroupSupplyFRUUncertaintyAllocationQuantity m; The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to Supply category resources across the EIM Area of EIM entities that passed theif Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month.. BAMonthResourcePeakPassGroupSupplyFRUUncertaintyAllocationQuantityBrtF’S’Q’m; The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to the specified Supply category resource of a BA that belongs to a specific BAA that passed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month.. BAMonthResourceOffPeakPassGroupSupplyFRUUncertaintyAllocationQuantityBrtF’S’Q’m; The monthly FRU Uncertainty Allocation quantity (in MWh) that is attributable to the specified Supply category resource of a BA that belongs to a specific BAA that passed their Flex Ramp Up Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month.. EIMAreaMonthOffPeakPassGroupLoadFRUUncertaintyAllocationAmount m; The monthly FRU Uncertainty Allocation amount (in $) over the EIM Area of EIM entities that pass their Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU Uncertainty attributed to the Load category that occur during the Off Peak Flexible Ramp Hours of the Trading Month.. EIMAreaMonthPeakPassGroupLoadFRUUncertaintyAllocationAmount m; The monthly FRU Uncertainty Allocation amount (in $) over the EIM Area of EIM entities that pass their Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU Uncertainty attributed to the Load category that occur during the Peak Flexible Ramp Hours of the Trading Month.. BAAMonthOffPeakLoadBAAConstraintFRUUncertaintyAllocationAmount Q’m; The monthly FRU Uncertainty Allocation amount (in $) for a BAA that failed theif Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU Uncertainty attributed to the Load category in association with the Off Peak Flexible Ramp Hours of the Trading Month.. BAAMonthPeakLoadBAAConstraintFRUUncertaintyAllocationAmount Q’m; The monthly FRU Uncertainty Allocation amount (in $) for a BAA that failed theif Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU Uncertainty attributed to the Load category in association with the Peak Flexible Ramp Hours of the Trading Month.. EIMAreaMonthOffPeakPassGroupIntertieFRUUncertaintyAllocationAmount m; The monthly FRU Uncertainty Allocation amount (in $) over the EIM Area of EIM entities that pass their Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU Uncertainty attributed to the Intertie category that occur during the Off Peak Flexible Ramp Hours of the Trading Month.. EIMAreaMonthPeakPassGroupIntertieFRUUncertaintyAllocationAmount m; The monthly FRU Uncertainty Allocation amount (in $) over the EIM Area of EIM entities that pass their Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU Uncertainty attributed to the Intertie category that occur during the Peak Flexible Ramp Hours of the Trading Month.. BAAMonthOffPeakIntertieBAAConstraintFRUUncertaintyAllocationAmount Q’m; The monthly FRU Uncertainty Allocation amount (in $) for a BAA that failed theif Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU Uncertainty attributed to the Intertie category in association with the Off Peak Flexible Ramp Hours of the Trading Month.. BAAMonthPeakIntertieBAAConstraintFRUUncertaintyAllocationAmount Q’m; The monthly FRU Uncertainty Allocation amount (in $) for a BAA that failed theif Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU Uncertainty attributed to the Intertie category in association with the Peak Flexible Ramp Hours of the Trading Month.. EIMAreaMonthOffPeakPassGroupSupplyFRUUncertaintyAllocationAmount m; The monthly FRU Uncertainty Allocation amount (in $) over the EIM Area of EIM entities that pass their Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU Uncertainty attributed to the Supply category that occur during the Off Peak Flexible Ramp Hours of the Trading Month.. EIMAreaMonthPeakPassGroupSupplyFRUUncertaintyAllocationAmount m; The monthly FRU Uncertainty Allocation amount (in $) over the EIM Area of EIM entities that pass their Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU Uncertainty attributed to the Supply category that occur during the Peak Flexible Ramp Hours of the Trading Month.. BAAMonthOffPeakSupplyBAAConstraintFRUUncertaintyAllocationAmount Q’m; The monthly FRU Uncertainty Allocation amount (in $) for a BAA that failed theif Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU Uncertainty attributed to the Supply category in association with the Off Peak Flexible Ramp Hours of the Trading Month.. BAAMonthPeakSupplyBAAConstraintFRUUncertaintyAllocationAmount Q’m; The monthly FRU Uncertainty Allocation amount (in $) for a BAA that failed theif Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU Uncertainty attributed to the Supply category in association with the Peak Flexible Ramp Hours of the Trading Month.. BAAMonthOffPeakHourFlexibleRampUpUncertaintySettlementAmount Q’Q’’m; The monthly FRU Uncertainty Allocation amount (in $) that follows from the settlement (in CC 7071) of FRU Uncertainty in association with the specified FRU Uncertainty Pass Group Q'’ and BAA Q’ for the Peak Flexible Ramp Hours of the Trading Month.. BAAMonthPeakHourFlexibleRampUpUncertaintySettlementAmount Q’Q’’m; The monthly FRU Uncertainty Allocation amount (in $) that follows from the settlement (in CC 7071) of FRU Uncertainty in association with the specified FRU Uncertainty Pass Group Q'’ and BAA Q’ for the Off Peak Flexible Ramp Hours of the Trading Month.. EIMAreaMonthOffPeakPassGroupFRUUncertaintyAllocationAmount m; The monthly FRU Uncertainty Allocation amount (in $) over the EIM Area of EIM Entities that pass their Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU for the Off Peak Flexible Ramp Hours of the Trading Month.. EIMAreaMonthPeakPassGroupFRUUncertaintyAllocationAmount m; The monthly FRU Uncertainty Allocation amount (in $) over the EIM Area of EIM Entities that pass their Flex Ramp Up Balancing Test that follows from the settlement (in CC 7071) of FRU for the Peak Flexible Ramp Hours of the Trading Month.. BAAMonthOffPeakBAASpecificFRUUncertaintyAllocationAmount Q’m; The monthly FRU Uncertainty Allocation amount (in $) for a specific BAA that failed their Flex Ramp Up Balancing test that follows from the settlement (in CC 7071) of FRU Uncertainty for the Off Peak Flexible Ramp Hours of the Trading Month.. BAAMonthPeakBAASpecificFRUUncertaintyAllocationAmount Q’m; The monthly FRU Uncertainty Allocation amount (in $) for a specific BAA that failed their Flex Ramp Up Balancing test that follows from the settlement (in CC 7071) of FRU Uncertainty for the Peak Flexible Ramp Hours of the Trading Month.. EIMAreaMonthPeakPassGroupAllCategoriesFRUUncertaintyQuantity m; The monthly FRU Uncertainty quantity (in MWh) for all resource categories (Load, Intertie and Supply) over the EIM Area for BAAs that passed their Flex Ramp Up Balancing Test the Peak Flexible Ramp Hours of the Trading Month.. EIMAreaMonthOffPeakPassGroupAllCategoriesFRUUncertaintyQuantity m; The monthly FRU Uncertainty quantity (in MWh) for all resource categories (Load, Intertie and Supply) over the EIM Area for BAAs that passed their Flex Ramp Up Balancing Test the Off Peak Flexible Ramp Hours of the Trading Month.. BAAMonthPeakBAASpecificAllCategoriesFRUUncertaintyQuantity Q’m; The monthly FRU Uncertainty quantity (in MWh) for all resource categories (Load, Intertie and Supply) for a specific BAA that failed their Flex Ramp Up Balancing Test the Peak Flexible Ramp Hours of the Trading Month.. BAAMonthOffPeakBAASpecificAllCategoriesFRUUncertaintyQuantity Q’m; The monthly FRU Uncertainty quantity (in MWh) for all resource categories (Load, Intertie and Supply) for a specific BAA that failed their Flex Ramp Up Balancing Test the Off Peak Flexible Ramp Hours of the Trading Month.. EIMAreaMonthOffPeakPassGroupLoadFRUUncertaintyQuantity m; The monthly FRU Uncertainty quantity (in MWh) for Load resources over the EIM Area for BAAs that passed their Flex Ramp Up Balancing Test the Off Peak Flexible Ramp Hours of the Trading Month.. EIMAreaMonthPeakPassGroupLoadFRUUncertaintyQuantity m; The monthly FRU Uncertainty quantity (in MWh) for Load resources over the EIM Area for BAAs that passed their Flex Ramp Up Balancing Test the Peak Flexible Ramp Hours of the Trading Month.. BAAMonthOffPeakBAASpecificLoadFRUUncertaintyQuantity Q’m; The monthly FRU Uncertainty quantity (in MWh) for Load resources of a specific BAA that failed their Flex Ramp Up Balancing Test the Off Peak Flexible Ramp Hours of the Trading Month.. BAAMonthPeakBAASpecificLoadFRUUncertaintyQuantity Q’m; The monthly FRU Uncertainty quantity (in MWh) for Load resources of a specific BAA that failed their Flex Ramp Up Balancing Test the Peak Flexible Ramp Hours of the Trading Month.. EIMAreaMonthOffPeakPassGroupIntertieFRUUncertaintyQuantity m; The monthly FRU Uncertainty quantity (in MWh) for Intertie resources over the EIM Area for BAAs that passed their Flex Ramp Up Balancing Test the Off Peak Flexible Ramp Hours of the Trading Month.. EIMAreaMonthPeakPassGroupIntertieFRUUncertaintyQuantity m; The monthly FRU Uncertainty quantity (in MWh) for Intertie resources over the EIM Area for BAAs that passed their Flex Ramp Up Balancing Test the Peak Flexible Ramp Hours of the Trading Month.. BAAMonthOffPeakBAASpecificIntertieFRUUncertaintyQuantity Q’m; The monthly FRU Uncertainty quantity (in MWh) for Intertie resources of a specific BAA that failed their Flex Ramp Up Balancing Test the Off Peak Flexible Ramp Hours of the Trading Month.. BAAMonthPeakBAASpecificIntertieFRUUncertaintyQuantity Q’m; The monthly FRU Uncertainty quantity (in MWh) for Intertie resources of a specific BAA that failed their Flex Ramp Up Balancing Test the Peak Flexible Ramp Hours of the Trading Month.. EIMAreaMonthOffPeakPassGroupSupplyFRUUncertaintyQuantity m; The monthly FRU Uncertainty quantity (in MWh) for Supply resources over the EIM Area for BAAs that passed their Flex Ramp Up Balancing Test the Off Peak Flexible Ramp Hours of the Trading Month.. EIMAreaMonthPeakPassGroupSupplyFRUUncertaintyQuantity m; The monthly FRU Uncertainty quantity (in MWh) for Supply resources over the EIM Area for BAAs that passed their Flex Ramp Up Balancing Test the Peak Flexible Ramp Hours of the Trading Month.. BAAMonthOffPeakBAASpecificSupplyFRUUncertaintyQuantity Q’m; The monthly FRU Uncertainty quantity (in MWh) for Supply resources of a specific BAA that failed their Flex Ramp Up Balancing Test the Off Peak Flexible Ramp Hours of the Trading Month.. BAAMonthPeakBAASpecificSupplyFRUUncertaintyQuantity Q’m; The monthly FRU Uncertainty quantity (in MWh) for Supply resources of a specific BAA that failed their Flex Ramp Up Balancing Test the Peak Flexible Ramp Hours of the Trading Month.. |  |  |
| --- | --- | --- |
| Row # | Name | Description |
| -- | In addition to any outputs listed below, all inputs shall be included as outputs. |  |
|  | BAMonthlyCompleteFRDUncertaintyAllocationAmount BQ’m | FRD Uncertainty Allocation Amount (in $) assessed monthly to a BA of the BAA as the difference of the monthly FRD Allocation Amount for the designated |
|  | BAMonthlyTotalDailyFRDUncertaintyAllocationAmount BQ’m | The montly total (in $) of the daily FRD Uncertainty Allocation Amounts assessed to a BA of resources throughout the EIM Area over all Trading Days of the designated Trading Month. |
|  | BAMonthAllHourCompleteFRDUncertaintyAllocationAmount BQ’m | The total (in $) of the FRD Uncertainty Allocation Amount assessed on a monthly basis to a BA of resources throughout the EIM Area for the designated Trading Month. |
|  | BAMonthPeakPassGroupFRDMeteredDemandAllocatedUncertaintyAmount BQ’m | The portion (in $) of the monthly allocated FRD Uncertainty Allocation amount that is shared over the EIM Area of EIM entities that pass their Flex Ramp Down Balancing Test and apportioned to a BA of the specified BAA based on metered Demand in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthOffPeakPassGroupFRDMeteredDemandAllocatedUncertaintyAmount BQ’m | The portion (in $) of the monthly allocated FRD Uncertainty Allocation amount that is shared over the EIM Area of EIM entities that pass their Flex Ramp Down Balancing Test and apportioned to a BA of the specified BAA based on metered Demand in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthPeakBAASpecificFRDMeteredDemandAllocatedUncertaintyAmount BQ’m | The portion (in $) of the monthly allocated FRD Uncertainty Allocation amount that is assigned to the specific BAA who failed their Flex Ramp Down Balancing Test and apportioned to a BA of the specified BAA based on metered Demand in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthOffPeakBAASpecificFRDMeteredDemandAllocatedUncertaintyAmount BQ’m | The portion (in $) of the monthly allocated FRD Uncertainty Allocation amount that is assigned to the specific BAA who failed their Flex Ramp Down Balancing Test and apportioned to a BA of the specified BAA based on metered Demand in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthPeakPassGroupFRDNeutralityMeteredDemandAllocatedAmount m | The portion (in $) of the monthly allocated FRD Uncertainty Allocation amount that is shared over the EIM Areaof EIM entities that pass their Flex Ramp Down Balancing Test in association with the Peak Flexible Ramp Hours of the Trading Month for which the allocation is based on metered Demand. |
|  | EIMAreaMonthOffPeakPassGroupFRDNeutralityMeteredDemandAllocatedAmount m | The portion (in $) of the monthly allocated FRD Uncertainty Allocation amount that is shared over the EIM Areaof EIM entities that pass their Flex Ramp Down Balancing Test in association with the Off Peak Flexible Ramp Hours of the Trading Month for which the allocation is based on metered Demand. |
|  | BAAMonthPeakBAASpecificFRDNeutralityMeteredDemandAllocatedAmount Q’m | The portion (in $) of the monthly allocated FRD Uncertainty Allocation amount that is assigned to the specific BAA who failed their Flex Ramp Down Balancing Test in association with the Peak Flexible Ramp Hours of the Trading Month for which the allocation is based on metered Demand. |
|  | BAAMonthOffPeakBAASpecificFRDNeutralityMeteredDemandAllocatedAmount Q’m | The portion (in $) of the monthly allocated FRD Uncertainty Allocation amount that is assigned to the specific BAA who failed their Flex Ramp Down Balancing Test in association with the Off Peak Flexible Ramp Hours of the Trading Month for which the allocation is based on metered Demand. |
|  | EIMAreaMonthPeakPassGroupFRDAllocatedUncertaintyAmount m | The portion (in $) of the monthly allocated FRD Uncertainty Allocation amount that is assigned to EIM Area resources of EIM entities that pass their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthOffPeakPassGroupFRDAllocatedUncertaintyAmount m | The portion (in $) of the monthly allocated FRD Uncertainty Allocation amount that is assigned to EIM Area resources of EIM entities that pass their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthPeakBAASpecificFRDAllocatedUncertaintyAmount Q’m | The portion (in $) of the monthly allocated FRD Uncertainty Allocation amount that is assigned to the specific BAA who failed their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthOffPeakBAASpecificFRDAllocatedUncertaintyAmount Q’m | The portion (in $) of the monthly allocated FRD Uncertainty Allocation amount that is assigned to the specific BAA who failed their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthPeakFRDPassGroupCategorySpecificAllocatedUncertaintyAmount BQ’m | The portion (in $) of the monthly allocated FRD Uncertainty Allocation amount that is assigned to resources of a BA of the specified BAA of EIM entities that pass their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthOffPeakFRDPassGroupCategorySpecificAllocatedUncertaintyAmount BQ’m | The portion (in $) of the monthly allocated FRD Uncertainty Allocation amount that is assigned to resources of a BA of the specified BAA of EIM entities that pass their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthPeakFRDBAACategorySpecificAllocatedUncertaintyAmount BQ’m | The portion (in $) of the monthly allocated FRD Uncertainty Allocation amount that is assigned to resources of a BA of the specified BAA who failed their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthOffPeakFRDBAACategorySpecificAllocatedUncertaintyAmount BQ’m | The portion (in $) of the monthly allocated FRD Uncertainty Allocation amount that is assigned to resources of a BA of the specified BAA who failed their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourcePeakPassGroupLoadFRDUncertaintyAllocationAmount BrtF’S’Q’m | The monthly allocated FRD Uncertainty Allocation amount (in $) that is assigned to a Load category resource of the specified BA and BAA of EIM entities that pass their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourceOffPeakPassGroupLoadFRDUncertaintyAllocationAmount BrtF’S’Q’m | The monthly allocated FRD Uncertainty Allocation amount (in $) that is assigned to a Load category resource of the specified BA and BAA of EIM entities that pass their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourcePeakBAASpecificLoadFRDUncertaintyAllocationAmount BrtF’S’Q’m | The monthly allocated FRD Uncertainty Allocation amount (in $) that is assigned to a Load category resource of the specified BA and BAA who failed their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourceOffPeakBAASpecificLoadFRDUncertaintyAllocationAmount BrtF’S’Q’m | The monthly allocated FRD Uncertainty Allocation amount (in $) that is assigned to a Load category resource of the specified BA and BAA who failed their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthPeakTotalBAASpecificLoadFRDUncertaintyAllocationQuantity Q’m | The monthly FRD Uncertainty Allocation quantity (in MWh) that is attributable to Load category resources of the specified BAA who failed their Flex Ramp Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthOffPeakTotalBAASpecificLoadFRDUncertaintyAllocationQuantity Q’m | The monthly FRD Uncertainty Allocation quantity (in MWh) that is attributable to Load category resources of the specified BAA who failed their Flex Ramp Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourcePeakBAASpecificLoadFRDUncertaintyAllocationQuantityBrtF’S’Q’m | The monthly FRD Uncertainty Allocation quantity (in MWh) that is attributable to the specified Load category resource of a BA of a specific BAA who failed their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourceOffPeakBAASpecificLoadFRDUncertaintyAllocationQuantityBrtF’S’Q’m | The monthly FRD Uncertainty Allocation quantity (in MWh) that is attributable to the specified Load category resource of a BA of a specific BAA who failed their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthPeakTotalPassGroupLoadFRDUncertaintyAllocationQuantity m | The monthly FRD Uncertainty Allocation quantity (in MWh) that is attributable to Load category resources across the EIM Area of EIM entities that passed theif Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthOffPeakTotalPassGroupLoadFRDUncertaintyAllocationQuantity m | The monthly FRD Uncertainty Allocation quantity (in MWh) that is attributable to Load category resources across the EIM Area of EIM entities that passed theif Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourcePeakPassGroupLoadFRDUncertaintyAllocationQuantityBrtF’S’Q’m | The monthly FRD Uncertainty Allocation quantity (in MWh) that is attributable to the specified Load category resource of a BA that belongs to a specific BAA that passed their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourceOffPeakPassGroupLoadFRDUncertaintyAllocationQuantityBrtF’S’Q’m | The monthly FRD Uncertainty Allocation quantity (in MWh) that is attributable to the specified Load category resource of a BA that belongs to a specific BAA that passed their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourcePeakPassGroupIntertieFRDUncertaintyAllocationAmount BrtF’S’Q’m | The monthly allocated FRD Uncertainty Allocation amount (in $) that is assigned to a Intertie category resource of the specified BA and BAA of EIM entities that pass their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourceOffPeakPassGroupIntertieFRDUncertaintyAllocationAmount BrtF’S’Q’m | The monthly allocated FRD Uncertainty Allocation amount (in $) that is assigned to a Intertie category resource of the specified BA and BAA of EIM entities that pass their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourcePeakBAASpecificIntertieFRDUncertaintyAllocationAmount BrtF’S’Q’m | The monthly allocated FRD Uncertainty Allocation amount (in $) that is assigned to a Intertie category resource of the specified BA and BAA who failed their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourceOffPeakBAASpecificIntertieFRDUncertaintyAllocationAmount BrtF’S’Q’m | The monthly allocated FRD Uncertainty Allocation amount (in $) that is assigned to a Intertie category resource of the specified BA and BAA who failed their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthPeakTotalBAASpecificIntertieFRDUncertaintyAllocationQuantity Q’m | The monthly FRD Uncertainty Allocation quantity (in MWh) that is attributable to Intertie category resources of the specified BAA who failed their Flex Ramp Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthOffPeakTotalBAASpecificIntertieFRDUncertaintyAllocationQuantity Q’m | The monthly FRD Uncertainty Allocation quantity (in MWh) that is attributable to Intertie category resources of the specified BAA who failed their Flex Ramp Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourcePeakBAASpecificIntertieFRDUncertaintyAllocationQuantityBrtF’S’Q’m | The monthly FRD Uncertainty Allocation quantity (in MWh) that is attributable to the specified Intertie category resource of a BA of a specific BAA who failed their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourceOffPeakBAASpecificIntertieFRDUncertaintyAllocationQuantityBrtF’S’Q’m | The monthly FRD Uncertainty Allocation quantity (in MWh) that is attributable to the specified Intertie category resource of a BA of a specific BAA who failed their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthPeakTotalPassGroupIntertieFRDUncertaintyAllocationQuantity m | The monthly FRD Uncertainty Allocation quantity (in MWh) that is attributable to Intertie category resources across the EIM Area of EIM entities that passed theif Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthOffPeakTotalPassGroupIntertieFRDUncertaintyAllocationQuantity m | The monthly FRD Uncertainty Allocation quantity (in MWh) that is attributable to Intertie category resources across the EIM Area of EIM entities that passed theif Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourcePeakPassGroupIntertieFRDUncertaintyAllocationQuantityBrtF’S’Q’m | The monthly FRD Uncertainty Allocation quantity (in MWh) that is attributable to the specified Intertie category resource of a BA that belongs to a specific BAA that passed their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourceOffPeakPassGroupIntertieFRDUncertaintyAllocationQuantityBrtF’S’Q’m | The monthly FRD Uncertainty Allocation quantity (in MWh) that is attributable to the specified Intertie category resource of a BA that belongs to a specific BAA that passed their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourcePeakPassGroupSupplyFRDUncertaintyAllocationAmount BrtF’S’Q’m | The monthly allocated FRD Uncertainty Allocation amount (in $) that is assigned to a Supply category resource of the specified BA and BAA of EIM entities that pass their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourceOffPeakPassGroupSupplyFRDUncertaintyAllocationAmount BrtF’S’Q’m | The monthly allocated FRD Uncertainty Allocation amount (in $) that is assigned to a Supply category resource of the specified BA and BAA of EIM entities that pass their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourcePeakBAASpecificSupplyFRDUncertaintyAllocationAmount BrtF’S’Q’m | The monthly allocated FRD Uncertainty Allocation amount (in $) that is assigned to a Supply category resource of the specified BA and BAA who failed their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourceOffPeakBAASpecificSupplyFRDUncertaintyAllocationAmount BrtF’S’Q’m | The monthly allocated FRD Uncertainty Allocation amount (in $) that is assigned to a Supply category resource of the specified BA and BAA who failed their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthPeakTotalBAASpecificSupplyFRDUncertaintyAllocationQuantity Q’m | The monthly FRD Uncertainty Allocation quantity (in MWh) that is attributable to Supply category resources of the specified BAA who failed their Flex Ramp Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthOffPeakTotalBAASpecificSupplyFRDUncertaintyAllocationQuantity Q’m | The monthly FRD Uncertainty Allocation quantity (in MWh) that is attributable to Supply category resources of the specified BAA who failed their Flex Ramp Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourcePeakBAASpecificSupplyFRDUncertaintyAllocationQuantityBrtF’S’Q’m | The monthly FRD Uncertainty Allocation quantity (in MWh) that is attributable to the specified Supply category resource of a BA of a specific BAA who failed their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourceOffPeakBAASpecificSupplyFRDUncertaintyAllocationQuantityBrtF’S’Q’m | The monthly FRD Uncertainty Allocation quantity (in MWh) that is attributable to the specified Supply category resource of a BA of a specific BAA who failed their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthPeakTotalPassGroupSupplyFRDUncertaintyAllocationQuantity m | The monthly FRD Uncertainty Allocation quantity (in MWh) that is attributable to Supply category resources across the EIM Area of EIM entities that passed theif Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthOffPeakTotalPassGroupSupplyFRDUncertaintyAllocationQuantity m | The monthly FRD Uncertainty Allocation quantity (in MWh) that is attributable to Supply category resources across the EIM Area of EIM entities that passed theif Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourcePeakPassGroupSupplyFRDUncertaintyAllocationQuantityBrtF’S’Q’m | The monthly FRD Uncertainty Allocation quantity (in MWh) that is attributable to the specified Supply category resource of a BA that belongs to a specific BAA that passed their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAMonthResourceOffPeakPassGroupSupplyFRDUncertaintyAllocationQuantityBrtF’S’Q’m | The monthly FRD Uncertainty Allocation quantity (in MWh) that is attributable to the specified Supply category resource of a BA that belongs to a specific BAA that passed their Flex Ramp Down Balancing Test based on various uncertainty criteria in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthOffPeakPassGroupLoadFRDUncertaintyAllocationAmount m | The monthly FRD Uncertainty Allocation amount (in $) over the EIM Area of EIM entities that pass their Flex Ramp Down Balancing Test that follows from the settlement (in CC 7071) of FRD Uncertainty attributed to the Load category that occur during the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthPeakPassGroupLoadFRDUncertaintyAllocationAmount m | The monthly FRD Uncertainty Allocation amount (in $) over the EIM Area of EIM entities that pass their Flex Ramp Down Balancing Test that follows from the settlement (in CC 7071) of FRD Uncertainty attributed to the Load category that occur during the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthOffPeakLoadBAAConstraintFRDUncertaintyAllocationAmount Q’m | The monthly FRD Uncertainty Allocation amount (in $) for a BAA that failed theif Flex Ramp Down Balancing Test that follows from the settlement (in CC 7071) of FRD Uncertainty attributed to the Load category in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthPeakLoadBAAConstraintFRDUncertaintyAllocationAmount Q’m | The monthly FRD Uncertainty Allocation amount (in $) for a BAA that failed theif Flex Ramp Down Balancing Test that follows from the settlement (in CC 7071) of FRD Uncertainty attributed to the Load category in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthOffPeakPassGroupIntertieFRDUncertaintyAllocationAmount m | The monthly FRD Uncertainty Allocation amount (in $) over the EIM Area of EIM entities that pass their Flex Ramp Down Balancing Test that follows from the settlement (in CC 7071) of FRD Uncertainty attributed to the Intertie category that occur during the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthPeakPassGroupIntertieFRDUncertaintyAllocationAmount m | The monthly FRD Uncertainty Allocation amount (in $) over the EIM Area of EIM entities that pass their Flex Ramp Down Balancing Test that follows from the settlement (in CC 7071) of FRD Uncertainty attributed to the Intertie category that occur during the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthOffPeakIntertieBAAConstraintFRDUncertaintyAllocationAmount Q’m | The monthly FRD Uncertainty Allocation amount (in $) for a BAA that failed theif Flex Ramp Down Balancing Test that follows from the settlement (in CC 7071) of FRD Uncertainty attributed to the Intertie category in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthPeakIntertieBAAConstraintFRDUncertaintyAllocationAmount Q’m | The monthly FRD Uncertainty Allocation amount (in $) for a BAA that failed theif Flex Ramp Down Balancing Test that follows from the settlement (in CC 7071) of FRD Uncertainty attributed to the Intertie category in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthOffPeakPassGroupSupplyFRDUncertaintyAllocationAmount m | The monthly FRD Uncertainty Allocation amount (in $) over the EIM Area of EIM entities that pass their Flex Ramp Down Balancing Test that follows from the settlement (in CC 7071) of FRD Uncertainty attributed to the Supply category that occur during the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthPeakPassGroupSupplyFRDUncertaintyAllocationAmount m | The monthly FRD Uncertainty Allocation amount (in $) over the EIM Area of EIM entities that pass their Flex Ramp Down Balancing Test that follows from the settlement (in CC 7071) of FRD Uncertainty attributed to the Supply category that occur during the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthOffPeakSupplyBAAConstraintFRDUncertaintyAllocationAmount Q’m | The monthly FRD Uncertainty Allocation amount (in $) for a BAA that failed theif Flex Ramp Down Balancing Test that follows from the settlement (in CC 7071) of FRD Uncertainty attributed to the Supply category in association with the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthPeakSupplyBAAConstraintFRDUncertaintyAllocationAmount Q’m | The monthly FRD Uncertainty Allocation amount (in $) for a BAA that failed theif Flex Ramp Down Balancing Test that follows from the settlement (in CC 7071) of FRD Uncertainty attributed to the Supply category in association with the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthOffPeakHourFlexibleRampDownUncertaintySettlementAmount Q’Q’’m | The monthly FRD Uncertainty Allocation amount (in $) that follows from the settlement (in CC 7071) of FRD Uncertainty in association with the specified FRD Uncertainty Pass Group Q'’ and BAA Q’ for the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthPeakHourFlexibleRampDownUncertaintySettlementAmount Q’Q’’m | The monthly FRD Uncertainty Allocation amount (in $) that follows from the settlement (in CC 7071) of FRD Uncertainty in association with the specified FRD Uncertainty Pass Group Q'’ and BAA Q’ for the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthOffPeakPassGroupFRDUncertaintyAllocationAmount m | The monthly FRD Uncertainty Allocation amount (in $) over the EIM Area of EIM Entities that pass their Flex Ramp Down Balancing Test that follows from the settlement (in CC 7071) of FRD for the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthPeakPassGroupFRDUncertaintyAllocationAmount m | The monthly FRD Uncertainty Allocation amount (in $) over the EIM Area of EIM Entities that pass their Flex Ramp Down Balancing Test that follows from the settlement (in CC 7071) of FRD for the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthOffPeakBAASpecificFRDUncertaintyAllocationAmount Q’m | The monthly FRD Uncertainty Allocation amount (in $) for a specific BAA that failed their Flex Ramp Down Balancing test that follows from the settlement (in CC 7071) of FRD Uncertainty for the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthPeakBAASpecificFRDUncertaintyAllocationAmount Q’m | The monthly FRD Uncertainty Allocation amount (in $) for a specific BAA that failed their Flex Ramp Down Balancing test that follows from the settlement (in CC 7071) of FRD Uncertainty for the Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthPeakPassGroupAllCategoriesFRDUncertaintyQuantity m | The monthly FRD Uncertainty quantity (in MWh) for all resource categories (Load, Intertie and Supply) over the EIM Area for BAAs that passed their Flex Ramp Down Balancing Test the Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthOffPeakPassGroupAllCategoriesFRDUncertaintyQuantity m | The monthly FRD Uncertainty quantity (in MWh) for all resource categories (Load, Intertie and Supply) over the EIM Area for BAAs that passed their Flex Ramp Down Balancing Test the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthPeakBAASpecificAllCategoriesFRDUncertaintyQuantity Q’m | The monthly FRD Uncertainty quantity (in MWh) for all resource categories (Load, Intertie and Supply) for a specific BAA that failed their Flex Ramp Down Balancing Test the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthOffPeakBAASpecificAllCategoriesFRDUncertaintyQuantity Q’m | The monthly FRD Uncertainty quantity (in MWh) for all resource categories (Load, Intertie and Supply) for a specific BAA that failed their Flex Ramp Down Balancing Test the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthOffPeakPassGroupLoadFRDUncertaintyQuantity m | The monthly FRD Uncertainty quantity (in MWh) for Load resources over the EIM Area for BAAs that passed their Flex Ramp Down Balancing Test the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthPeakPassGroupLoadFRDUncertaintyQuantity m | The monthly FRD Uncertainty quantity (in MWh) for Load resources over the EIM Area for BAAs that passed their Flex Ramp Down Balancing Test the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthOffPeakBAASpecificLoadFRDUncertaintyQuantity Q’m | The monthly FRD Uncertainty quantity (in MWh) for Load resources of a specific BAA that failed their Flex Ramp Down Balancing Test the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthPeakBAASpecificLoadFRDUncertaintyQuantity Q’m | The monthly FRD Uncertainty quantity (in MWh) for Load resources of a specific BAA that failed their Flex Ramp Down Balancing Test the Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthOffPeakPassGroupIntertieFRDUncertaintyQuantity m | The monthly FRD Uncertainty quantity (in MWh) for Intertie resources over the EIM Area for BAAs that passed their Flex Ramp Down Balancing Test the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthPeakPassGroupIntertieFRDUncertaintyQuantity m | The monthly FRD Uncertainty quantity (in MWh) for Intertie resources over the EIM Area for BAAs that passed their Flex Ramp Down Balancing Test the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthOffPeakBAASpecificIntertieFRDUncertaintyQuantity Q’m | The monthly FRD Uncertainty quantity (in MWh) for Intertie resources of a specific BAA that failed their Flex Ramp Down Balancing Test the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthPeakBAASpecificIntertieFRDUncertaintyQuantity Q’m | The monthly FRD Uncertainty quantity (in MWh) for Intertie resources of a specific BAA that failed their Flex Ramp Down Balancing Test the Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthOffPeakPassGroupSupplyFRDUncertaintyQuantity m | The monthly FRD Uncertainty quantity (in MWh) for Supply resources over the EIM Area for BAAs that passed their Flex Ramp Down Balancing Test the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | EIMAreaMonthPeakPassGroupSupplyFRDUncertaintyQuantity m | The monthly FRD Uncertainty quantity (in MWh) for Supply resources over the EIM Area for BAAs that passed their Flex Ramp Down Balancing Test the Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthOffPeakBAASpecificSupplyFRDUncertaintyQuantity Q’m | The monthly FRD Uncertainty quantity (in MWh) for Supply resources of a specific BAA that failed their Flex Ramp Down Balancing Test the Off Peak Flexible Ramp Hours of the Trading Month. |
|  | BAAMonthPeakBAASpecificSupplyFRDUncertaintyQuantity Q’m | The monthly FRD Uncertainty quantity (in MWh) for Supply resources of a specific BAA that failed their Flex Ramp Down Balancing Test the Peak Flexible Ramp Hours of the Trading Month. |

# Charge Code Effective Dates

| Charge Code/ Pre-calc Name | Document Version | Effective Start Date | Effective End Date | Version Update Type |
| --- | --- | --- | --- | --- |
| CC 7088 Monthly Flexible Ramp Down Uncertainty Award Allocation | 5.0 | 11/1/2016 | Open | Initial Implementation |