Settlements and Billing

Configuration Guide: CG PC Measured Demand Over Control Area Excl MSS Energy v5.4

Pre-calculation

Version 5.4

Table of Contents

1.	Purpose of Document	3

2.	Introduction	3

2.1	Background	3

2.2	Description	8

3.	Charge Code Requirements	9

3.1	Business Rules	9

3.2	Predecessor Charge Codes	11

3.3	Successor Charge Codes	11

3.4	Inputs - External Systems	11

3.5	Inputs - Predecessor Charge Codes or Pre-calculations	12

3.6	CAISO Formula	14

3.7	Outputs	18

4.	Charge Code Effective Dates	21

# Purpose of Document

The purpose of this document is to capture the requirements and design specification for a SaMC Charge Code in one document.

# Introduction

## Background

In the allocation of various payment and charges, the amounts are divided among Business Associates (BAs) in proportion to their relative contribution to the overall Measured Demand quantity (in MWh) over which the allocation is performed.  Thus, an allocation of payment or charges requires the following two Measured Demand calculations that are conducted for the particular time period over which the allocation is performed:

- the overall metered CAISO Demand plus Real-Time Interchange export schedules, excluding that portion of Demand of Non-Generator Resources dispatched as Regulation through Regulation Energy Management, in the grid sector or area over which the allocated payments and charges apply and

- the contribution of each Business Associate to the overall CAISO Demand plus Real-Time Interchange export schedules, excluding that portion of Demand of Non-Generator Resources dispatched as Regulation through Regulation Energy Management.

To provide the Measured Demand quantities that are used in multiple Charge Code calculations that allocate various payments or charges, the CAISO implements a set of Pre-calculation processes, referred to as Measured Demand Pre-calculations, to calculate particular Measured Demand quantities.  There are five (5) Measured Demand Pre-calculation configurations in which an overall Measured Demand quantity is calculated.  The five Pre-calculations differ by the composition of the Demand resources and exports that are included in their Measured Demand outputs.  The Measured Demand Pre-calculation process configurations, the Demand resources and export composition of their outputs, and the associated output frequency (10-minute, hourly, etc.) are identified as follows:

| Measured Demand Pre-calculation Configurations |  |  |
| --- | --- | --- |
| Name | Output Composition and Frequency | Dependent Allocation Configurations |
| MD over Control Area Pre-calculation | Composition: Measured Demand: The sum (as a negative value) of all - metered CAISO Demand* - Real-Time Interchange export schedules Net MSS Measured Demand: The lesser of - zero (0) or the net value defined as - the sum of Meter Data (as a negative value) for Demand of all Load and other non-export Demand resources internal to an MSS plus (+) the sum of - Meter Data representing Generation of all Generating Units for the MSS to which lesser value is added - the sum of all of the MSS Real-Time Interchange export schedules (as a negative value). Frequency: 5-minute,10-minute, hourly, daily | CC 6474 – Real Time Unaccounted for Energy Settlement |
|  |  | CC 6475 – Real Time Uninstructed Imbalance Energy Settlement |
|  |  | CC 6477 – Real Time Imbalance Energy Offset |
|  |  | CC 6480 – Excess Cost Neutrality Allocation |
|  |  | CC 6636 – IFM Bid Cost Recovery Tier 1 Allocation |
|  |  | CC 6637 – IFM Bid Cost Recovery Tier 2 Allocation |
|  |  | CC 6678 – Real Time Bid Cost Recovery Allocation |
|  |  | CC 6774 – RT Congestion Offset |
|  |  | CC 6790 – CRR Balancing Account |
|  |  | CC 6806 – Day Ahead Residual Unit Commitment (RUC) Tier 1 Allocation |
|  |  | CC 6807 – Day Ahead Residual Unit Commitment (RUC) Tier 2 Allocation |
|  |  | CC 6947 – IFM Marginal Losses Surplus Credit Allocation |
|  |  | CC 550 – Monthly FERC Fee Settlement |
|  |  | CC 551 – Annual FERC Fee Settlement |
|  |  | CC 4989 – Daily Rounding Adjustment Allocation |
|  |  | CC 4999 – Monthly Rounding Adjustment |
|  |  | CC 6457 – Declined Hourly Pre-Dispatch Penalty Allocation |
|  |  | CC 7056 – Flexible Ramp Cost Allocation |
|  |  | CC 8999 – Neutrality Adjustment |
|  |  | PC – Metered Demand Over TAC Area And CPM |
|  |  | PC – Standard Capacity Product |
| MD over Control Area Excl MSS Energy Pre-calculation | Composition: Measured Demand: The sum (as a negative value) of all - metered CAISO Demand - Real-Time Interchange export schedules The overall Measured Demand output quantity excludes Energy from MSS entities. Frequency: 5-minute, 10-minute and monthly | CC 1302 – Long-Term Voltage Support Allocation |
|  |  | CC 1303 – Supplemental Reactive Energy Allocation |
| MD Emissions over Control Area Excl External Exports Pre-calculation | Composition: Measured Demand (in-state): The sum (as a negative value) of all - metered CAISO Demand - in-state Real-Time Interchange export schedules The sum excludes CAISO-external exports and any external contractual Transmission Loss adjustments. Net MSS Measured Demand (in-state): The lesser of - zero (0) or the net value defined as - the sum of Meter Data (as a negative value) for Demand of all Load and other non-export Demand resources internal to an MSS plus (+) the sum of - Meter Data representing Generation of all Generating Units for the MSS to which lesser value is added - the sum of all of the MSS in-state Real-Time Interchange export schedules (as a negative value). The sum excludes CAISO-external exports and any external contractual Transmission Loss adjustments. Frequency: Monthly | CC 591 – Emissions Cost Recovery |
| MD Black Start Excl Exports Pre-calculation | Composition: Metered CAISO Demand: the sum of all metered CAISO Demand (as a negative value). Net MSS Demand: The greater of - zero (0) or the net value defined as - the sum of Meter Data (as a negative value) for Demand of all Load and other non-export Demand resources internal to an MSS, plus (+) the sum of - Meter Data representing Generation of all Generating Units for the MSS. Frequency: Hourly | CC 1353- Black Start Energy Allocation |
| MD over Control Area Excl Transmission Loss Adjustment Pre-calculation | Composition: Measured Demand: The sum (as a negative value) of all - metered CAISO Demand - Real-Time Interchange export schedules The calculated value excludes contractual Transmission Loss adjustments. Net MSS Measured Demand: The greater of - zero (0) or the net value defined as - the sum of Meter Data for Demand (as a negative value) of all Load and other non-export Demand resources internal to an MSS plus (+) the sum of - Meter Data representing Generation of all Generating Units for the MSS to which lesser value is added - the sum of all of the MSS Real-Time Interchange export schedules (as a negative value). The calculated value excludes contractual Transmission Loss adjustments. Frequency: 5-minute, 10-minute and Hourly | CC 6977 – Allocation of Transmission Loss Obligation Charge for Real Time Schedules under Control Agreement |

Note:

* 	Metered CAISO Demand excludes that portion of Demand of Non-Generator Resources dispatched as Regulation through Regulation Energy Management.

Some of the Measured Demand configurations listed above recognize exemptions to the calculation for various Business Associates and resources.  In some instances a Measured Demand configuration may produce multiple outputs, each one recognizing a particular exemption or set of exemptions.  For each Measured Demand calculation, its exemptions are determined and defined by the requirements of the allocations in which the Measured Demand Pre-calculation outputs are employed.

All of the dependent allocation charge codes listed in the above Measured Demand Pre-calculation configuration summary table do not necessarily use the entire set of Measured Demand output components that the table associates with the charge codes. Some of the outputs used in the allocations are adjusted for exceptions as explained in the prior paragraph. Moreover some outputs used in various allocations are adjusted for various transmission-related contracts in order to exclude Measured Demand that is associated with contract rights from being used in a calculation. Additionally, based on the MRTU Tariff, special rules apply for the application of Measured Demand components for MSS entities that Load Follow, or for the allocation of Unaccounted-For-Energy (UFE). Some allocations may be based on metered CAISO Demand, not the full Measured Demand quantity. Thus, the configuration summary table above is quite simplistic and is intended to present only a top-level view of the output components of the Measured Demand Pre-calculations.

## Description

Generically, Measured Demand calculations represent the sum of metered CAISO Demand (or net MSS Demand) quantities plus Real-Time Interchange export schedule quantities, excluding that portion of Demand of Non-Generator Resources dispatched as Regulation through Regulation Energy Management.  Depending upon the exact Measured Demand calculation, various components are included or excluded in a summed Measured Demand output value.  The “MD over Control Area Excl MSS Energy” output calculations do not include MSS Energy.  The “Measured Demand Black Start Excl Exports” output calculations do not include exports.  Likewise, the calculations for the Pre-calculations “Measured Demand Black Start Excl Exports” and “Measured Demand over Control Area Excl Transmission Loss Adjustment” do not employ quantities for Transmission Losses under operating agreements (contractual Transmission Loss adjustments that serve as a form of Energy Export).  The “CG PC Measured Demand Over Control Area Excl MSS Energy v5.4” process provides for BA and resource-specific exemptions for particular Business Associates and combinations of BA and resource.  The Energy for exempted entities or resources is not included in the Measured Demand outputs.

The “Measured Demand over Control Area Excl MSS Energy” process derives its Measured Demand output quantities from the metered CAISO Demand quantities as well as the Real-Time Interchange export schedule quantities in the CAISO Control Area that are of the Energy types firm, non-firm, dynamic, unit-contingent, and wheeling, while excluding that portion of Demand of Non-Generator Resources dispatched as Regulation through Regulation Energy Management.  The outputs are expected to serve the “CC 1302 - Long-Term Voltage Support Allocation” and “CC 1303 - Short-Term Voltage Support Allocation” Charge Codes, as indicated in the table of the prior section.  As required for the two latter Voltage Support Energy allocations, the “Measured Demand over Control Area Excl MSS Energy” Measured Demand calculation excludes MSS Energy.  Measured Demand quantities are output both on a Settlement Interval and a monthly basis.  The calculation produces an output at two levels of summation for use in allocating charges and payments.  A Measured Demand output presents Non-MSS Energy for each BA as the sum (as a negative value) of the BA’s Metered CAISO Demand, scheduled Energy Export quantities and contractually-determined Transmission Losses, excluding that portion of Demand of Non-Generator Resources dispatched as Regulation through Regulation Energy Management.  Besides being computed on a BA basis, the Measured Demand quantity is summed over all BAs to formulate an overall Measured Demand value for the CAISO Control Area.

# Charge Code Requirements

## Business Rules

| Bus Req ID | Business Rule |
| --- | --- |
|  | The CG PC Measured Demand Over Control Area Excl MSS Energy v5.4 pre-calculation shall perform a set of calculations to determine and output Measured Demand (metered CAISO Demand plus Real-Time Interchange export schedules, excluding that portion of Demand of Non-Generator Resources dispatched as Regulation through Regulation Energy Management) over the CAISO Control Area. |
|  | The Measured Demand over Control Area Excl MSS Energy pre-calculation shall apply only to a non-MSS entity. |
|  | Measured Demand shall be calculated and output as a total quantity for the overall CAISO Control Area. |
|  | Measured Demand shall be calculated and output as a total quantity for each Business Associate. |
|  | Contractual Transmission Losses, representing Transmission Losses that are covered by agreements between transmission line operators and the CAISO, shall be included in a Measured Demand calculation as Real-Time Interchange export schedules. |
|  | The CG PC Measured Demand Over Control Area Excl MSS Energy v5.4 shall provide Measured Demand outputs that exclude particular combinations of Business Associate (BA) and resource values from the calculated output values. |
|  | Exceptions shall be defined in a standing data table. |
|  | Exemptions shall be organized in the standing data table as “exception sets”. |
|  | A set of distinct Measured Demand outputs shall be provided for each exemption set. |
|  | An exception flag input shall be provided for each exception set in order to indicate when an exception condition within the exception set is valid. |
|  | The current implementation of the CG PC Measured Demand Over Control Area Excl MSS Energy v5.4 shall provide for two (2) exception sets, referred to as “Exceptions #1” and “Exceptions #2”. |
|  | Each exception set shall be effective-dated (with a start data and end date) to indicate the period over which the exception set is in effect. |
|  | The CG PC Measured Demand Over Control Area Excl MSS Energy v5.4 shall provide Measured Demand outputs that exclude the transmission rights (in MWh) associated with particular Business Associate (BA) values. |
|  | The Measured Demand outputs shall be calculated on a 10-minute, hourly and monthly interval basis. Although currently not required by other SaMC processes, the hourly outputs shall be included in the current configuration for possible future use. |
|  | PTB Charge Adjustment does not apply to the CG PC Measured Demand Over Control Area Excl MSS Energy v5.4 process. (Fact) |
|  | Gross Load shall be submitted through MRI-s under Measurement type ‘LOAD’ |

## Predecessor Charge Codes

Charge Code/ Pre-calc Name

Allocation of Transmission Losses Under Control Agreements Pre-calculation

ETC, TOR, CVR Quantity Pre-calculation

MSS Netting Pre-calculation

System Resource Deemed Delivered Energy Pre-calculation

## Successor Charge Codes

Charge Code/ Pre-calc Name

CC 1302 - Long-Term Voltage Support Allocation

CC 1303 - Short-Term Voltage Support Allocation

## Inputs - External Systems

Subscripts (attributes) that are listed for a variable name in the table below, but are not described in the associated description are incidental to the calculations herein.

| Row # | Variable Name | Description |
| --- | --- | --- |
| 1 | BAResMeasuredDemandControlAreaExclMSSExceptions1_Flag Brt | Flag value, provided as a standing data input, that indicates whether a condition from Exemptions #1 is met (true) for a given combination of a Business Associate and resource values. When an Exemptions #1 condition is true, the “CG PC Measured Demand Over Control Area Excl MSS Energy v5.4” process shall exclude any metered CAISO Demand, export Energy, (or net MSS Measured Demand) for the resource in the calculation of Measured Demand outputs. |
| 2 | BAMeasuredDemandExclMSSExceptionsFlag B | Flag value, provided as a standing data input, that indicates whether a condition from Exemptions #1 is met (true) for a given Business Associate value. When an Exemptions #1 condition is true, the “CG PC Measured Demand Over Control Area Excl MSS Energy v5.4” process shall exclude any metered CAISO Demand, export Energy, (or net MSS Measured Demand) for the resource in the calculation of Measured Demand outputs. |
| 3 | BAMeasuredDemandControlAreaExclMSSExceptions2_Flag B | Flag value, derived by the data mapping process from the Exceptions #2 standing data input, that indicates whether a condition from Exceptions #2 is met (true) for a given Business Associate. When an Exceptions #2 condition is true, the CG PC Measured Demand Over Control Area Excl MSS Energy v5.4 shall exclude any associated transmission rights Energy from the calculation of the Measured Demand outputs CAISOTotalMonthlyMeasuredDemandControlAreaExclMSSEx1Qtym, CAISOTotalSettlementIntervalMeasuredDemandControlAreaExclMSSEx1Qtymdhcif, BAMonthlyMeasuredDemandControlAreaExclMSSEx1QtyBm and BASettlementIntervalMeasuredDemandControlAreaExclMSSEx1QtyBmdhcif. |

## Inputs - Predecessor Charge Codes or Pre-calculations

Subscripts (attributes) that are listed for a variable name in the table below, but are not described in the associated description are incidental to the calculations herein.

| Row # | Variable Name | Predecessor Charge Code/ Pre-calc Configuration |
| --- | --- | --- |
| 1 | Op_Agreement_Export_Loss_Allocation_Quantity BrtEuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’OVvHn’L’mdhcif | This input originates in the “Pre-Calc Allocation of Transmission Losses under Operating Agreements” process. The input (in MWh as a negative value) represents operating agreement export losses quantity (billable) in MWh, calculated for a Business Associate for a settlement interval. In the CG PC Measured Demand Over Control Area Excl MSS Energy v5.4, resource type t is restricted to the value ‘ETIE’. |
| 2 | SettlementIntervalDeemedDeliveredInterchangeEnergyQuantity BrtEuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’OVvHn’L’mdhcif | This input originates in the System Resource Deemed Delivered Energy Pre-calculation and applies to all Energy types. The derived input (in MWh as a negative value) represents Real-Time Interchange export schedule Energy as Deemed-Delivered Energy quantity in MWh, calculated for a Business associate and settlement interval. In the CG PC Measured Demand Over Control Area Excl MSS Energy v5.4 pre-calculation output derivation, the resource type t is restricted to the value ‘ETIE’. |
| 3 | BAResSettlementIntervalMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif | Received from the MSS Netting Pre-calculation, the this input presents five -minute CAISO Metered Demand (in MWh as a negative value) that is attributable to a resource for a settlement interval. |
| 4 | SettlementIntervalResourceContractMD Brtz’mdhcif | Received from the ETC/TOR/CVR Quantity Pre-calculation, this input presents the transmission right-exempted Measured Demand quantity (in MWh as a negative value) for a resource for a settlement interval. |
| 5 | BAResSettlementIntervalGrossMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif | Received from MSS Netting Pre-calculation. This input represents the settlement interval resource meter for the calculation of CAISO Metered Demand (in MWh as a negative value) where MSS load is gross. |

## CAISO Formula

    - Measured Demand Quantity over CAISO Control Area Excl MSS (by Settlement Month)

CAISOTotalMonthlyMeasuredDemandControlAreaExclMSSQty m = 
dh CAISOTotalHourlyMeasuredDemandControlAreaExclMSSQty mdh

      - Where

CAISOTotalHourlyMeasuredDemandControlAreaExclMSSQty mdh =
cf∈c CAISOTotalSettlementIntervalMeasuredDemandControlAreaExclMSSQty mdhcif

    - Measured Demand Quantity over CAISO Control Area Excl MSS (by Settlement Interval)

Where

CAISOTotalSettlementIntervalMeasuredDemandControlAreaExclMSSQty mdhcif = BBASettlementIntervalMeasuredDemandControlAreaExclMSSQty Bmdhcif

    - Measured Demand Quantity over CAISO Control Area Excl MSS (by BA and Settlement Month)

BAMonthlyMeasuredDemandControlAreaExclMSSQty Bm = dhBAHourlyMeasuredDemandControlAreaExclMSSQty Bmdh

      - Where

BAHourlyMeasuredDemandControlAreaExclMSSQty Bmdh =cf∈cBASettlementIntervalMeasuredDemandControlAreaExclMSSQty Bmdhcif

    - Measured Demand Quantity over CAISO Control Area Excl MSS (by BA and Settlement Interval)

Where

BASettlementIntervalMeasuredDemandControlAreaExclMSSQty Bmdhcif = BASettlementIntervalTotalMeteredCAISODemandQuantity_MDOverCAExclMSSEnergy Bmdhcif + BASettlementIntervalExportQuantity_MDOverCAExclMSSEnergy Bmdhcif

      - Where

BASettlementIntervalTotalMeteredCAISODemandQuantity_MDOverCAExclMSSEnergy Bmdhcif = rtuT'I'Q'M'AA'R'pPW'Qd'Nz'VvHn'L'BAResSettlementIntervalMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif

        - Where

T’ <> ‘MSS’

      - And Where

BASettlementIntervalExportQuantity_MDOverCAExclMSSEnergy Bmdhcif = 
rtEuT'I'Q'M'AA'F'R'pPW'QS'd'Nz'vOVHn'L'(SettlementIntervalDeemedDeliveredInterchangeEnergyQuantity BrtEuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’OVvHn’L’mdhcif + Op_Agreement_Export_Loss_Allocation_Quantity BrtEuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’OVvHn’L’mdhcif )

        - Where

t = ‘ETIE’

        - And Where

E In (‘FIRM’, ‘NFRM’, ‘WHEEL’, ‘DYN’, ‘UCTG’)

        - And Where

T’ <> ‘MSS’

        - And Where

Q’ = ‘CISO’

    - Measured Demand Quantity over CAISO Control Area Excl MSS w/Exemptions #1 (by Settlement Month)

CAISOTotalMonthlyMeasuredDemandControlAreaExclMSSEx1Qty m = 
dh CAISOTotalHourlyMeasuredDemandControlAreaExclMSSEx1Qty mdh

      - Where

CAISOTotalHourlyMeasuredDemandControlAreaExclMSSEx1Qty mdh =
cf∈c CAISOTotalSettlementIntervalMeasuredDemandControlAreaExclMSSEx1Qty mdhcif

    - Measured Demand Quantity over CAISO Control Area Excl MSS w/Exemptions #1 (by Settlement Interval)

Where

CAISOTotalSettlementIntervalMeasuredDemandControlAreaExclMSSEx1Qty mdhcif = BBASettlementIntervalMeasuredDemandControlAreaExclMSSEx1Qty Bmdhcif

    - Measured Demand Quantity over CAISO Control Area Excl MSS w/Exemptions #1 (by BA and Settlement Month)

BAMonthlyMeasuredDemandControlAreaExclMSSEx1Qty Bm = dhBAHourlyMeasuredDemandControlAreaExclMSSEx1Qty Bmdh

      - Where

BAHourlyMeasuredDemandControlAreaExclMSSEx1Qty Bmdh = cf∈cBASettlementIntervalMeasuredDemandControlAreaExclMSSEx1Qty Bmdhcif

    - Measured Demand Quantity over CAISO Control Area Excl MSS w/Exemptions #1 (by BA and Settlement Interval)

Where

BASettlementIntervalMeasuredDemandControlAreaExclMSSEx1Qty Bmdhcif = Min(BASettlementIntervalTotalMeteredCAISODemandQuantityEx1_MDOverCAExclMSSEnergy Bmdhcif+ BASettlementIntervalExportQuantityEx1_MDOverCAExclMSSEnergy Bmdhcif- BASettlementIntervalRights_ControlAreaExclMSSQty Bmdhcif , 0)

      - Where

If BAMeasuredDemandControlAreaExclMSSExceptions2_Flag B = ‘True’

Then
(

        - BASettlementIntervalRights_ControlAreaExclMSSQty Bmdhcif =  rtz'SettlementIntervalResourceContractMD Brtz’mdhcif

)

Else

(

        - BASettlementIntervalRights_ControlAreaExclMSSQty Bmdhcif =  0

)

      - And Where

BASettlementIntervalTotalMeteredCAISODemandQuantityEx1_MDOverCAExclMSSEnergy Bmdhcif =rtuT'I'Q'M'AA'R'pPW'Qd'Nz'VvHn'L' BAResSettlementIntervalGrossMeteredCAISODemandQuantity BrtuT’I’Q’M’AA’R’pPW’Qd’Nz’VvHn’L’mdhcif

        - Where

T’ <> ‘MSS’

        - And Where

BAResMeasuredDemandControlAreaExclMSSExceptions1_Flag Brt <> True

        - And Where

BAMeasuredDemandExclMSSExceptionsFlag B <> True

      - And Where

BASettlementIntervalExportQuantityEx1_MDOverCAExclMSSEnergy Bmdhcif = 
rtEuT'I'Q'M'AA'F'R'pPW'QS'd'Nz'vOVHn'L'(SettlementIntervalDeemedDeliveredInterchangeEnergyQuantity BrtEuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’OVvHn’L’mdhcif + Op_Agreement_Export_Loss_Allocation_Quantity BrtEuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’OVvHn’L’mdhcif )

        - Where

t = ‘ETIE’

        - And Where

E In (‘FIRM’, ‘NFRM’, ‘WHEEL’, ‘DYN’, ‘UCTG’)

        - And Where

T’ <> ‘MSS’

        - And Where

BAResMeasuredDemandControlAreaExclMSSExceptions1_Flag Brt <> True

        - And Where

BAMeasuredDemandExclMSSExceptionsFlag B <> True

        - And Where

Q’ = ‘CISO’

## Outputs

| Row # | Name | Description |
| --- | --- | --- |
| 1 | In addition to any outputs listed below, all inputs shall be included as outputs. | All inputs. Refer to section 3.2above for input descriptions. |
| 2 | CAISOTotalMonthlyMeasuredDemandControlAreaExclMSSQty m | Total monthly Measured Demand quantity (in MWh as a negative value) calculated over the CAISO Control. |
| 3 | CAISOTotalHourlyMeasuredDemandControlAreaExclMSSQty mdh | Total hourly Measured Demand quantity (in MWh as a negative value) calculated over the CAISO Control Area. |
| 4 | CAISOTotalSettlementIntervalMeasuredDemandControlAreaExclMSSQty mdhcif | Total Measured Demand quantity (in MWh as a negative value) calculated over the CAISO Control Area for a settlement interval. |
| 5 | BAMonthlyMeasuredDemandControlAreaExclMSSQty Bm | Monthly Measured Demand quantity (in MWh as a negative value) calculated for a Business Associate over the CAISO Control Area. |
| 6 | BAHourlyMeasuredDemandControlAreaExclMSSQty Bmdh | Hourly Measured Demand quantity (in MWh as a negative value) calculated for a Business Associate over the CAISO Control Area. |
| 7 | BASettlementIntervalMeasuredDemandControlAreaExclMSSQty Bmdhcif | Measured Demand quantity (in MWh as a negative value) calculated for a Business Associate over the CAISO Control Area for a settlement interval. |
| 8 | BASettlementIntervalTotalMeteredCAISODemandQuantity_MDOverCAExclMSSEnergy Bmdhcif | Sum of metered CAISO Demand input values over all Load and other non-export Demand resources that were scheduled by a Business Associate for a settlement interval. The summed quantity (in MWh as a negative value) is restricted to Energy from non-MSS entities. |
| 9 | BASettlementIntervalExportQuantity_MDOverCAExclMSSEnergy Bmdhcif | Sum of Real-Time Deemed-Delivered export quantities plus export losses settled under contractual operating agreements. The summed quantity (in MWh as a negative value) is calculated for a Business Associate and for a settlement interval. |
| 10 | CAISOTotalMonthlyMeasuredDemandControlAreaExclMSSEx1Qty m | Total monthly Measured Demand quantity (in MWh as a negative value) calculated over the CAISO Control Area, with the exclusion of Energy provided by Business Associates and resources that comprise Exemptions #1. For BAs of exception set “Exceptions #2” that have TOR contract-established transmission rights, the Demand Energy represented by the output will be reduced by an applicable contract-designated quantity. |
| 11 | CAISOTotalHourlyMeasuredDemandControlAreaExclMSSEx1Qty mdh | Total hourly Measured Demand quantity (in MWh as a negative value) calculated over the CAISO Control Area, with the exclusion of Energy provided by Business Associates and resources that comprise Exemptions #1. For BAs of exception set “Exceptions #2” that have TOR contract-established transmission rights, the Demand Energy represented by the output will be reduced by an applicable contract-designated quantity. |
| 12 | CAISOTotalSettlementIntervalMeasuredDemandControlAreaExclMSSEx1Qty mdhcif | Total Measured Demand quantity (in MWh as a negative value) calculated over the CAISO Control Area for a settlement interval, with the exclusion of Energy provided by Business Associates and resources that comprise Exemptions #1. For BAs of exception set “Exceptions #2” that have TOR contract-established transmission rights, the Demand Energy represented by the output will be reduced by an applicable contract-designated quantity. |
| 13 | BAMonthlyMeasuredDemandControlAreaExclMSSEx1Qty Bm | Monthly Measured Demand quantity (in MWh as a negative value) calculated for a Business Associate over the CAISO Control Area, with the exclusion of Energy provided by Business Associates and resources that comprise Exemptions #1. For BAs of exception set “Exceptions #2” that have TOR contract-established transmission rights, the Demand Energy represented by the output will be reduced by an applicable contract-designated quantity. |
| 14 | BAHourlyMeasuredDemandControlAreaExclMSSEx1Qty Bmdh | Hourly Measured Demand quantity (in MWh as a negative value) calculated for a Business Associate over the CAISO Control Area, with the exclusion of Energy provided by Business Associates and resources that comprise Exemptions #1. For BAs of exception set “Exceptions #2” that have TOR contract-established transmission rights, the Demand Energy represented by the output will be reduced by an applicable contract-designated quantity. |
| 15 | BASettlementIntervalMeasuredDemandControlAreaExclMSSEx1Qty Bmdhcif | Measured Demand quantity (in MWh as a negative value) calculated for Business Associate B over the CAISO Control Area for a settlement interval, with exclusion of Energy provided by Business Associates and resources that comprise Exemptions #1. For BAs of exception set “Exceptions #2” that have TOR contract-established transmission rights, the Demand Energy represented by the output will be reduced by an applicable contract-designated quantity. |
| 16 | BASettlementIntervalRights_ControlAreaExclMSSQty Bmdhcif | Transmission Rights quantity (in MWh as a negative value) that apply over the CAISO Control Area for a Business Associate for a settlement interval. |
| 17 | BASettlementIntervalTotalMeteredCAISODemandQuantityEx1_MDOverCAExclMSSEnergy Bmdhcif | Sum of metered CAISO Demand input values over all Load and other non-export Demand resources that were scheduled by a Business Associate for a settlement interval, with the exclusion of input values for Business Associates and resources comprising Exemptions #1. The summed quantity (in MWh as a negative value) is restricted to Energy from non-MSS entities. |
| 18 | BASettlementIntervalExportQuantityEx1_MDOverCAExclMSSEnergy Bmdhcif | Sum of Real-Time Deemed-Delivered export quantities plus export losses settled under contractual operating agreements. The summed quantity (in MWh as a negative value) is calculated for a Business Associate and for a settlement interval, with exclusion of input values for Business Associates and resources comprising Exemptions #1. The calculated quantity contains only export Energy from non-MSS entities. |

# Charge Code Effective Dates

| Charge Code/ Pre-calc Name | Document Version | Effective Start Date | Effective End Date | Version Update Type |
| --- | --- | --- | --- | --- |
| CG PC Measured Demand Over Control Area Excl MSS Energy v5.4 | 5.0 | 04/01/09 | 05/23/10 | Documentation Edits Only |
| CG PC Measured Demand Over Control Area Excl MSS Energy v5.4 | 5.1 | 05/24/10 | 01/31/11 | Documentation Edits and Configuration Impacted |
| CG PC Measured Demand Over Control Area Excl MSS Energy v5.4 | 5.1a | 02/01/11 | 12/12/11 | Documentation Edits Only |
| CG PC Measured Demand Over Control Area Excl MSS Energy v5.4 | 5.1b | 12/13/11 | 11/30/12 | Documentation Edits Only |
| CG PC Measured Demand Over Control Area Excl MSS Energy v5.4 | 5.1c | 12/01/12 | 04/30/13 | Documentation Edits Only |
| CG PC Measured Demand Over Control Area Excl MSS Energy v5.4 | 5.2 | 05/01/14 | 10/31/14 | Documentation Edits and Configuration Impacted |
| CG PC Measured Demand Over Control Area Excl MSS Energy v5.4 | 5.3 | 11/01/14 | 12/31/20 | Documentation Edits and Configuration Impacted |
| CG PC Measured Demand Over Control Area Excl MSS Energy v5.4 | 5.4 | 1/1/21 | Open | Documentation Edits and Configuration Impacted |