Settlements & Billing

Configuration Guide: Resource Adequacy Availability Incentive Mechanism (RAAIM)

Version 5.5

Table of Contents

1.	Purpose of Document	3

2.	Introduction	3

2.1	Background	3

2.2	Description	3

3.	Charge Code Requirements	3

3.1	Business Rules	3

3.2	Predecessor Charge Codes	7

3.3	Successor Charge Codes	7

3.4	Inputs – External Systems	7

3.5	Inputs - Predecessor Charge Codes or Pre-calculations	13

3.6	CAISO Formula	15

3.7	Outputs	41

4.	Charge Code Effective Date	54

# Purpose of Document

The purpose of this document is to capture the requirements and design specification for a Charge Code in one document.

# Introduction

## Background

The Resource Adequacy Availability Incentive Mechanism (RAAIM) was part of the Reliability Services Initiative, a suite of policies to address rules and process surrounding RA resources.

RAAIM enhances grid reliability and market efficiency by providing an incentive for RA resources that meet their bidding obligation and provide energy bids to the market. The new incentive mechanism replaces the previously existing Standard Capacity Product and utilizes a resource’s economic and self schedule bids to evaluate Resource Adequacy and Capacity Procurement Capacity availability. Similar to SCP, RAAIM establishes a monthly standard that RA resources are expected to perform. RA resources that fail to meet the threshold are subject to a penalty, while resources that exceed the threshold may receive a payment.

Under SCP, a large number of resources were exempted due to the limitation of evaluating availability based on forced outages. A resource’s availability was used in the evaluation of RA and CPM Capacity, but calculated separately under SCP and CPM settlement. In reality, RA and CPM capacity share the same must offer rules and adhere to inter-related rules. Additionally, the Flexible Resource Adequacy Criteria and Must Offer Obligation (FRAC-MOO) initiative created economic bidding requirements that were impossible to measure using an outage availability value. A new incentive mechanism was thus necessary in order to expand, integrate, and evaluate more resources on the grid. and accommodate the new bidding obligations introduced with FRAC-MOO.

## Description

The Resource Adequacy Availability Incentive Mechanism pre-calculation is the pre-calculation which executes daily the hourly availability and obligation quantities that will be used in the monthly calculation of the availability standard and incentive mechanism.

# Charge Code Requirements

## Business Rules

| Bus Req ID | Business Rule |
| --- | --- |
| 1.0 | General |
| 1.1 | A resource that designates generic/flexible RA capacity in a supply plan, or is party to replacement, substitution, or CPM designation shall be subject to the Resource Adequacy Availability Incentive Mechanism (RAAIM). |
| 1.2 | The monthly performance of an RA resource shall be measured by the Availability of the resource in the CAISO market. Availability is calculated based on a resource’s submission of bids to meet their RA bidding obligation on designated assessment hours. |
| 1.3 | Each day the system shall calculate a Daily Availability Percentage for the day ahead and real time market. Hourly availability values and its associated RA data from the the market with the lower daily availability percentage shall become the inputs to the monthly performance calculation. |
| 1.4 | The Daily Availability Percentage shall be calculated as the sum of each assessment hour Hourly Availability divided by the sum of the corresponding assessment hour Hourly Total RA obligation. - All values of target trade day for each market: ∑Hourly availability / ∑Hourly Total RA obligation |
| 1.5 | Each RA category has the following assessment hour profile: Generic RA 5 hours per non-holiday weekdays Seasonal Peak hours 1pm – 6pm (HE14 – HE18) April to October 4pm – 9pm (HE17 – HE21) November to March Flexible RA (Category1) 17 hours per day 5am – 10pm (HE6 – HE22) Flexible RA (Category 2) 5 hours per day, seven days a week Seasonal Peak hours The five-hour period is determined by the CAISO’s Flexible Capacity Needs Assessment for the applicable year Flexible RA (Category 3) 5 hours per non-holiday weekdays Seasonal Peak hours The five-hour period is determined by the CAISO’s Flexible Capacity Needs Assessment for the applicable year |
| 2.0 | Obligation |
| 2.1 | A resource’s Hourly RAAIM obligation shall be no greater than the maximum of the resource’s Generic and Flexible RA obligation. RA bidding obligation is dependent on the amount of generic RA capacity, generic CPM capacity, flexible RA capacity, and/or flexible CPM capacity. |
| 2.2 | A resource with generic RA capacity or generic CPM capacity has an obligation to submit bids up to the committed capacity each hour. |
| 2.3 | A resource with flexible RA capacity or flexible CPM capacity has an obligation to submit economic bids up to the committed capacity on designated hours dependent on the category of flexible capacity. |
| 2.4 | Where a VER is shown on a monthly flexible RA plan for less than its Effective Flexible Capacity (EFC), its flexible bid obligation will be based on its forecast and adjusted downward/upward in proportion to the percent of its EFC that was shown on the monthly flexible RA plan. |
| 2.5 | Long Start resources not committed in the Day Ahead Market or RUC for a given trade hour, will be released from any RAAIM obligation in the Real Time Market for the given trade hour |
| 2.6 | Extremely Long Start resources not committed by Day Ahead Market under ELC process for a given trade hour, will not be subject to RAAIM obligation in Real Time Market for the given trade hour |
| 3.0 | Exemptions |
| 3.1 | RA exempt outages that qualify may exempt all or a portion of a resource’s generic and/or flexible RA obligation. |
| 3.2 | A use limited outage shall not qualify as a RA exemption if the resource has not reached its use limitation. |
| 3.3 | Eligible Generic RA Outage shall be equal to the Generic RA bid obligation that exceeds the RA Exempt Outage Threshold. |
| 3.4 | Eligible Flexible RA Outage shall be equal to the Flexible RA bid obligation that exceeds the RA Exempt Outage Threshold. |
| 3.5 | For a generator or tie-generator resource, the RA Exempt Outage Threshold is the resource’s Pmax less RA Exempt Outage MW. |
| 3.6 | For a non-Resource-Specific System Resource, the RA Exempt Outage Threshold is the RA Exempt Outage MW |
| 3.7 | The following resources are exempt from the RAAIM: - Resource’s with Pmax < 1.0 - Contracts for Energy delivered to CAISO BAA from non-specified resources. - Load following MSS resources that are on the monthly supply plan of a load-following MSS. - QF resources - CHP (Generic RA only) - Participating Load that is pumping load - Acquired resources under specific conditions. - VER resources (Generic RA only) - RMR Units - Combined Flexible RA Capacity Resource (Flexible only) |
| 3.8 | RDRR resources shall not be subject to RAAIM obligation in the Day Ahead Market |
| 3.9 | Load-following MSS resources – RA capacity from load-following MSS resources that are not on the monthly supply plan of a load-following MSS shall be subject to RAAIM. |
| 4.0 | Total and economic bids |
| 4.1 | A resource’s bids shall be no greater than its Outage Availability MW. |
| 4.2 | Non-Generator Resource-Regulation Energy Management (NGR-REM) resources shall include regulation bid data. |
| 5.0 | Hourly Availability |
| 5.1 | A resource’s Hourly Availability MW shall use the following data in its calculation. The data is needed at an hourly granularity and for each market. Generic RA obligation Flexible RA obligation EFC Pmin Economic bid Total bid |
| 5.2 | A resource’s Hourly Availability shall be equal to the sum of generic and flexible availability. |
| 5.3 | A resource’s flexible availability is equal to the sum of eligible EFC Pmin and submitted economic bids, but no greater than its flexible RA obligation. |
| 5.4 | A resource’s generic availability is equal to the resource’s total submitted bid less the portion of: (economic bid plus eligible EFC pmin used for flexible availability), but no greater than the positive value of generic RA obligation minus flexible RA obligation. |
| 5.5 | Day Ahead and Real Time Must Offer rules for resources that only need to be assessed an hourly availability for a single market |
| 5.6 | Non-Generator Resource-Regulation Energy Management (NGR-REM) resources’ Real Time availability shall include regulation awarded in the Day Ahead in addition to Real Time regulation bid data. |

## Predecessor Charge Codes

Charge Code/ Pre-calc Name

Pre-calculation Measured Demand Over Control Area

Pre-calculation Measured Demand Over TAC Area and CPM

Real-Time Energy Quantity Pre-calculation

Use-Limited Resource Utilization Pre-calculation

## Successor Charge Codes

Charge Code/ Pre-calc Name

CC 8830 – Monthly Resource Adequacy Availability Incentive Mechanism Settlement

CC 8831 – Monthly Resource Adequacy Availability Incentive Mechanism Allocation

CC 8835 – Annual Resource Adequacy Availability Incentive Mechanism Neutrality

## Inputs – External Systems

| Row # | Variable Name | Description |
| --- | --- | --- |
|  | MinOperMW BrtF'S'md | This input represents a resource’s minimum energy output. Pmin Mapped to the single interval value that exists for the whole day. This variable is also used in No Pay RUC Pre Calculation. |
|  | Resource90MinStartUpFlag Brtmd | Indicates if a resource’s cold startup time is equal to or less than 90 minutes. Startup equal to or less than 90Minutes = 1 Startup greater than 90 minutes = 0 |
|  | GenericAssessmentFlag mdh | Indicates the hour that a generic RA obligation shall be assessed for RAAIM performance. |
|  | ResourceFlexibleAssessmentFlag j’mdh | Indicates the hour that a Flexible RA obligation shall be assessed for RAAIM performance. Note that the flag is different based on the flexible RA category defined by attribute (j’). |
|  | ResourceEFCQty rmd | A resource’s Eligible Flexible Capacity quantity. |
|  | DayAheadGenericCapacityQty BrtF’S’mdh | Day Ahead generic RA capacity. |
|  | DayAheadGenericCPMCapacityQty BrtF’S’mdh | Day Ahead generic CPM capacity. |
|  | DayAheadFlexibleCapacityQty BrtF’S’j’mdh | Day Ahead flexible RA capacity. |
|  | DayAheadFlexibleCPMCapacityQty BrtF’S’j’mdh | Day Ahead flexible CPM capacity. |
|  | RealTimeGenericCapacityQty BrtF’S’mdh | Real time generic RA capacity. |
|  | RealTimeGenericCPMCapacityQty BrtF’S’mdh | Real time generic CPM capacity. |
|  | RealTimeFlexibleCapacityQty BrtF’S’j’mdh | Real time flexible RA capacity. |
|  | RealTimeFlexibleCPMCapacityQty BrtF’S’j’mdh | Real time flexible CPM capacity. |
|  | DayAheadFlexibleExemptOutageQty BrtF’S’mdh | Sum of flexible RA exempt outage curtailment each hour from the day ahead market (excluding use limited exempt outages) |
|  | DayAheadGenericExemptOutageQty BrtF’S’mdh | Sum of generic RA exempt outage curtailment each hour from the day ahead market (excluding use limited exempt outages) |
|  | DayAheadUseLimitedExemptOutageQty BrtF’S’mdh | Sum of RA use limited exempt outage curtailment each hour from the day ahead market. |
|  | RealTimeFlexibleExemptOutageQty BrtF’S’mdh | Sum of flexible RA exempt outage curtailment each hour from the real time market (excluding use limited exempt outages) |
|  | RealTimeGenericExemptOutageQty BrtF’S’mdh | Sum of generic RA exempt outage curtailment each hour from the real time market (excluding use limited exempt outages) |
|  | RealTimeUseLimitedExemptOutageQty BrtF’S’mdh | Sum of RA use limited exempt outage curtailment each hour from the real time market. |
|  | MaxOperMW BrtF'S'md | Maximum Operating MW. The resource’s maximum energy output (Pmax) registered in masterfile. Mapped to the single interval value that exists for the whole day. |
|  | ResourceAcquiredRightsFlag rmd | Indicates that a resource has met the requirements of acquired rights and is exempt from the Resource Adequacy Availability Incentive Mechnism. |
|  | ResourcePMaxExemptionFlag rmd | Resource’s PMax is less than one MW, and is therefore exempt from RAAIM. |
|  | ResourceQFExemptionFlag rmd | Resource is identified as QF, and is therefore exempt from RAAIM |
|  | ResourceParticipatingLoadExemptionFlag rmd | Resource is identified as participating load, and is therefore exempt from RAAIM. |
|  | ResourceVERExemptionFlag rmd | Resource is identified as a Variable Energy Resource, and is therefore exempt from generic RAAIM. |
|  | ResourceCHPExemptionFlag rmd | Resource is identified as a Combined Heat Power Resource, and is therefore exempt from generic RAAIM. |
|  | ResourceRDRRFlag rmd | Resource identified as RDRR resource |
|  | ResourceRMRFlag rmd | Resource identified as RMR resource |
|  | ResourceLongStartFlag rmd | Resource identified as Long Start Resource |
|  | ResourceExtremelyLongStartFlag rmd | Resource identified as Extremely Long Start Resource |
|  | ResourceCombinedFlexibleRAExemptionFlag rmd | A RA resource that is designated in the RA showings to be combined with another RA resource to meet a singular flexible RA obligation. These resources are deemed “Combined Flexible RA Capacity Resources”, and their flexible capacity is exempt from RAAIM. |
|  | DayAheadUpperOperatingLimit BrtF’S’mdh | Resource’s effective upper operating limit as reported in OMS as of the day ahead market. |
|  | DayAheadLowerOperatingLimit BrtF’S’mdh | Resource’s effective lower operating limit as reported in OMS as of the day ahead market. |
|  | RealTimeUpperOperatingLimit BrtF’S’mdhcif | Resource’s effective upper operating limit as reported in OMS as of the RTM market. |
|  | RealTimeLowerOperatingLimit BrtF’S’mdhcif | Resource’s effective lower operating limit as reported in OMS as of the RTM market. |
|  | BAHourlyLowerRegulationLimitQty Brtmdh | Registered lower limit range that a resource can respond and deliver regulation energy. For an MSG resource, this reflects lower regulation limit of configuration in which MSG resource was awarded Day-Ahead Regulation Up or Down. |
|  | DARegDownAwardedBidQuantity BrtT’uI’M’R’W’F’S’NVL'mdh | IFM Awarded Regulation Down Bid Capacity for a given resource and Trading Hour. |
|  | DARegUpAwardedBidQuantity BrtT’uI’M’R’W’F’S’NVL'mdh | IFM Awarded Regulation Up Bid Capacity for a given resource and Trading Hour. |
|  | BAHourlyResDAMEnergySelfScheduleBidQty BrtubAA’pF’S’amdh | The input represents the Hourly Day Ahead Market Self Schedule Bid Quantity by Business Associate. The bid segment number for Self Schedule Qty will be set to “0”. This variable is also used in CC4515. |
|  | BAHourlyResDAMEnergyBidQty BrtubAA’pF’S’mdh | The input represents the Day Ahead Market Energy Bid quantity (in MWh) for bid segment, as submitted by Business Associate. This variable is also used in CC4515. |
|  | BAHourlyResRTMEnergySelfScheduleBidQty BrtuQ’bAA’pF’S’amdh | The input represents the Hourly Real Time Market Energy Self Schedule Bid Quantity by Business Associate. The bid segment number for Self Schedule Qty will be set to “0”. This variable is also used in CC4515. |
|  | BAHourlyResRTMEnergyBidQty BrtuQ’bAA’pF’S’mdh | The input represents the Real Time Market Energy Bid quantity (in MWh) for bid segment, as submitted by Business Associate This variable is also used in CC4515. |
|  | BAHourlyResDAMRegUpSelfProvisionBidQty BrtbF’S’mdh | The input represents the Hourly Day Ahead Regulation Up Capacity Self Provision Bid quantity (MW) as submitted by Business Associate. The bid segment number for Self Provision Qty will be set to “0”. This variable is also used in CC4515. |
|  | BAHourlyResDAMRegUpBidQty BrtbF’S’mdh | The input represents the Day Ahead Market Regulation Up Capacity Bid quantity (MW) as submitted by Business Associate. This variable is also used in CC4515. |
|  | BAHourlyResRTMRegUpSelfProvisionBidQty BrtbF’S’mdh | The input represents the Hourly Real Time Market Regulation Up Capacity Self Provision Bid Quantity (MW) as submitted by Business Associate. The bid segment number for Self Provision Qty will be set to “0” This variable is also used in CC4515. |
|  | BAHourlyResRTMRegUpBidQty BrtbF’S’mdh | The input represents the Hourly Real Time Market Regulation Up Capacity Bid quantity (in MW) as submitted by Business Associate. This variable is also used in CC4515. |
|  | BAHourlyResDAMRegDownSelfProvisionBidQty BrtbF’S’mdh | The input represents the Hourly Day Ahead Energy Regulation Down Capacity Self Provision Bid Quantity (MW) as submitted by Business Associate. The bid segment number for Self Provision Qty will be set to “0” This variable is also used in CC4515. |
|  | BAHourlyResDAMRegDownBidQty BrtbF’S’mdh | The input represents the Day Ahead Market Regulation Down Capacity Bid quantity (MW) as submitted by Business Associate. This variable is also used in CC4515. |
|  | BAHourlyResRTMRegDownSelfProvisionBidQty BrtbF’S’mdh | The input represents the Hourly Real Time Market Regulation Down Self Provision Capacity Bid Quantity (MW) as submitted by Business Associate. The bid segment number for Self Provision Qty will be set to “0” This variable is also used in CC4515. |
|  | BAHourlyResRTMRegDownBidQty BrtbF’S’mdh | The input represents the Hourly Real Time Market Regulation Down Capacity Bid quantity (in MW) as submitted by Business Associate. This variable is also used in CC4515. |
|  | BA15MinResourceIntertieDeclinePenaltyFMMFinalForecastEnergy BrtF’S’mdhc | The input represents the 15 minute forecast of a VER resource in the FMM market, as provided for a given VER resource and FMM Interval. |
|  | BAResourceHourlyRUCAwardedBidCapacity BrtuT'I'M'VL'W'R'F'S'mdh | Hourly RUC Awarded Bid Capacity |
|  | CAISOMonthlyCPMSoftOfferCapPrice m | The CPM Soft Offer Cap (in $ / kW-month) to which the price of capacity offered into a Competitive Solicitation Process is subject. This variable is also used in MDoTAC_CPM_PC. |
|  | CountFlag m | Dummy flag used for counting calculations. |

## Inputs - Predecessor Charge Codes or Pre-calculations

| Row # | Name | Predecessor Charge Code/ Pre-calc Configuration |
| --- | --- | --- |
|  | BAMonthlyResourceCPMCapacityPaymentPrice Brto’k’m | Pre-calculation Measured Demand Over TAC Area and CPM Monthly CPM Capacity Payment Price (in $ / MW-month) by resource and CPM Transaction ID |
|  | BAHourlyNodalMeteredCAISODemandQuantity_MDOverCA BAA’mdh | Pre-calculation Measured Demand Over Control Area Sum of meter input values over the Aggregated Pricing Node A of Aggregated Pricing Node Type A’ for Business Associate B, Trading Hour h, Trading Day d and Trading Month m. The summed quantity includes the individual meter readings for Loads of non-MSS entities and gross-settled MSS entities, and also includes the calculated net MSS Demand value for each net-settled MSS entity. |
|  | SettlementIntervalResouceDayAheadEnergy BrtuT’I’Q’M’F’S’mdhcif | Real-Time Energy Quantity Pre-calculation Settlement Interval Resource Day Ahead Energy |
|  | ResourceUseLimitationReachedFlag Brtmdh | Use-Limited Resource Utilization Pre-calculation Indicates for a given trade hour if a use limited resource has reached its use limitation. 1 = Use limitation reached 0 = Use limitation not reached |

## CAISO Formula

### Assessment Bid Obligation & Availability

#### HourlyAssessmentBidObligationQuantity

IF

DailyRAAIMDAorRTAssessmentFlag BrtF’S’md = 1

THEN

HourlyAssessmentBidObligationQuantity BrtF’S’mdh = DayAheadAssessmentBidObligationQuantity BrtF’S’mdh

ELSE

HourlyAssessmentBidObligationQuantity BrtF’S’mdh = RealTimeAssessmentBidObligationQuantity BrtF’S’mdh

END IF

#### HourlyAssessmentAvailabilityQuantity

IF

DailyRAAIMDAorRTAssessmentFlag BrtF’S’md = 1

THEN

HourlyAssessmentAvailabilityQuantity BrtF’S’mdh = DayAheadAvailabilityQuantity BrtF’S’mdh

ELSE

HourlyAssessmentAvailabilityQuantity BrtF’S’mdh = RealTimeAvailabilityQuantity BrtF’S’mdh

END IF

#### DailyRAAIMDAorRTAssessmentFlag

IF

((DailyDayAheadPerformance BrtF’S’md < DailyRealTimePerformance BrtF’S’md

AND

hDayAheadAssessmentBidObligationQuantity BrtF’S’mdh > 0)

OR

(hDayAheadAssessmentBidObligationQuantity BrtF’S’mdh > 0

AND

hRealTimeAssessmentBidObligationQuantity BrtF’S’mdh = 0))

AND

(DayAheadResourceGenericRAAIMExemptionFlag rmd = 0

OR

DayAheadResourceFlexibleRAAIMExemptionFlag rmd = 0)

THEN

DailyRAAIMDAorRTAssessmentFlag BrtF’S’md = 1

ELSE

DailyRAAIMDAorRTAssessmentFlag BrtF’S’md = 0

END IF

Note: DayAheadAssessmentBidObligationQuantity BrtF’S’mdh and RealTimeAssessmentBidObligationQuantity BrtF’S’mdh shall be used as the business drivers for this equation.

#### DailyDayAheadPerformance

DailyDayAheadPerformance BrtF’S’md =  ( BAResourceDailyDayAheadAvailabilityQuantity BrtF’S’md / BAResourceDailyDayAheadAssessmentBidObligationQuantity BrtF’S’md)

#### DailyRealTimePerformance

DailyRealTimePerformance BrtF’S’md =  ( BAResourceDailyRealTimeAvailabilityQuantity BrtF’S’md / BAResourceDailyRealTimeAssessmentBidObligationQuantity BrtF’S’md)

#### BAResourceDailyDayAheadAssessmentBidObligationQuantity

BAResourceDailyDayAheadAssessmentBidObligationQuantity BrtF’S’md = hDayAheadAssessmentBidObligationQuantity BrtF’S’mdh

#### DayAheadAssessmentBidObligationQuantity

DayAheadAssessmentBidObligationQuantity BrtF’S’mdh = Max(DayAheadAssessmentGenericObligationQuantity BrtF’S’mdh, DayAheadAssessmentFlexibleObligationQuantity BrtF’S’mdh)

#### BAResourceDailyRealTimeAssessmentBidObligationQuantity

BAResourceDailyRealTimeAssessmentBidObligationQuantity BrtF’S’md = hRealTimeAssessmentBidObligationQuantity BrtF’S’mdh

#### RealTimeAssessmentBidObligationQuantity

RealTimeAssessmentBidObligationQuantity BrtF’S’mdh = Max(RealTimeAssessmentGenericObligationQuantity BrtF’S’mdh, RealTimeAssessmentFlexibleObligationQuantity BrtF’S’mdh)

### RAAIM & CPM Obligation

#### DayAheadOverallFlexibleObligationQuantity

DayAheadOverallFlexibleObligationQuantity BrtF’S’mdh  =

j'DayAheadFlexibleObligationQuantity BrtF’S’j’mdh

#### RealTimeOverallFlexibleObligationQuantity

RealTimeOverallFlexibleObligationQuantity BrtF’S’mdh  =

IF

ResourceVERExemptionFlag rmd = 1

THEN

RealTimeOverallFlexibleObligationQuantity BrtF’S’mdh =

j'RealTimeVERFlexibleObligationQuantity BrtF’S’j’mdh * (RealTimeFlexibleObligationQuantity BrtF’S’j’mdh / (RealTimeFlexibleObligationQuantity BrtF’S’j’mdh + RealTimeFlexibleCPMObligationQuantity BrtF’S’j’mdh))

ELSE

RealTimeOverallFlexibleObligationQuantity BrtF’S’mdh =

j'RealTimeFlexibleObligationQuantity BrtF’S’j’mdh

END IF

#### DailyRAAIMObligationQuantity

IF

DailyRAAIMObligationCount BrtF’S’md = 0

THEN

DailyRAAIMObligationQuantity BrtF’S’md = 0

ELSE

DailyRAAIMObligationQuantity BrtF’S’md =

h (HourlyRAAIMObligationQuantity BrtF’S’mdh / DailyRAAIMObligationCount BrtF’S’md)

END IF

#### HourlyRAAIMObligationQuantity

IF

DailyRAAIMDAorRTAssessmentFlag BrtF’S’md = 1

THEN

HourlyRAAIMObligationQuantity BrtF’S’mdh = Max(DayAheadGenericObligationQuantity BrtF’S’mdh, DayAheadOverallFlexibleObligationQuantity BrtF’S’mdh)

ELSE

HourlyRAAIMObligationQuantity BrtF’S’mdh = Max(RealTimeGenericObligationQuantity BrtF’S’mdh, RealTimeOverallFlexibleObligationQuantity BrtF’S’mdh)

END IF

Note: The above calculation is at an hourly granularity. DayAheadGenericObligationQuantity, DayAheadOverallFlexibleObligationQuantity, RealTimeGenericObligationQuantity, and RealTimeOverallFlexibleObligationQuantity shall be used as the business drivers for this equation.

#### DailyRAAIMGenericCPMQuantity

IF

DailyRAAIMGenericCPMCount BrtF’S’md = 0

THEN

DailyRAAIMGenericCPMQuantity BrtF’S’md = 0

ELSE

DailyRAAIMGenericCPMQuantity BrtF’S’md =

h (HourlyRAAIMGenericCPMQuantity BrtF’S’mdh / DailyRAAIMGenericCPMCount BrtF’S’md)

END IF

#### HourlyRAAIMGenericCPMQuantity

IF

DailyRAAIMDAorRTAssessmentFlag BrtF’S’md = 1

THEN

HourlyRAAIMGenericCPMQuantity BrtF’S’mdh = DayAheadGenericCPMObligationQuantity BrtF’S’mdh

ELSE

HourlyRAAIMGenericCPMQuantity BrtF’S’mdh = RealTimeGenericCPMObligationQuantity BrtF’S’mdh

END IF

Note: The above calculation is at an hourly granularity. DayAheadGenericCPMObligationQuantity and RealTimeGenericCPMObligationQuantity shall be used as the business drivers for this equation.

#### DayAheadOverallFlexibleCPMObligationQuantity

DayAheadOverallFlexibleCPMObligationQuantity BrtF’S’mdh  =

j'DayAheadFlexibleCPMObligationQuantity BrtF’S’j’mdh

#### RealTimeOverallFlexibleCPMObligationQuantity

IF

ResourceVERExemptionFlag rmd = 1

THEN

RealTimeOverallFlexibleCPMObligationQuantity BrtF’S’mdh =

j'RealTimeVERFlexibleObligationQuantity BrtF’S’j’mdh * (RealTimeFlexibleCPMObligationQuantity BrtF’S’j’mdh / (RealTimeFlexibleObligationQuantity BrtF’S’j’mdh + RealTimeFlexibleCPMObligationQuantity BrtF’S’j’mdh))

ELSE

RealTimeOverallFlexibleCPMObligationQuantity BrtF’S’mdh =

j'RealTimeFlexibleCPMObligationQuantity BrtF’S’j’mdh

END IF

#### DailyRAAIMFlexibleCPMQuantity

IF

DailyRAAIMFlexibleCPMCount BrtF’S’md = 0

THEN

DailyRAAIMFlexibleCPMQuantity BrtF’S’md = 0

ELSE

DailyRAAIMFlexibleCPMQuantity BrtF’S’md =

h (HourlyRAAIMFlexibleCPMQuantity BrtF’S’mdh / DailyRAAIMFlexibleCPMCount BrtF’S’md)

END IF

#### HourlyRAAIMFlexibleCPMQuantity

IF

DailyRAAIMDAorRTAssessmentFlag BrtF’S’md = 1

THEN

HourlyRAAIMFlexibleCPMQuantity BrtF’S’mdh = DayAheadOverallFlexibleCPMObligationQuantity BrtF’S’mdh

ELSE

HourlyRAAIMFlexibleCPMQuantity BrtF’S’mdh =  RealTimeOverallFlexibleCPMObligationQuantity BrtF’S’mdh

END IF

Note: The above calculation is at an hourly granularity. DayAheadFlexibleCPMObligationQuantity and RealTimeFlexibleCPMObligationQuantity shall be used as the business drivers for this equation.

### Day Ahead Availability

#### BAResourceDailyDayAheadAvailabilityQuantity

BAResourceDailyDayAheadAvailabilityQuantity BrtF’S’md = h DayAheadAvailabilityQuantity BrtF’S’mdh

#### DayAheadAvailabilityQuantity

DayAheadAvailabilityQuantity BrtF’S’mdh = DayAheadGenericAvailabilityQuantity BrtF’S’mdh + DayAheadFlexibleAvailabilityQuantity BrtF’S’mdh

#### DayAheadGenericAvailabilityQuantity

DayAheadGenericAvailabilityQuantity BrtF’S’mdh =

Min(DayAheadMaxGenericAvailabilityQuantity BrtF’S’mdh, Max(0, DayAheadAvailableTotalBidQuantity BrtF’S’mdh – DayAheadFlexibleAvailabilityQuantity BrtF’S’mdh))

#### DayAheadMaxGenericAvailabilityQuantity

DayAheadMaxGenericAvailabilityQuantity BrtF’S’mdh =

Max(0, DayAheadAssessmentGenericObligationQuantity BrtF’S’mdh – DayAheadAssessmentFlexibleObligationQuantity BrtF’S’mdh)

#### DayAheadFlexibleAvailabilityQuantity

DayAheadFlexibleAvailabilityQuantity BrtF’S’mdh = Min (DayAheadAvailableEconomicBidQuantity BrtF’S’mdh + DayAheadEligibleEFCPminQuantity BrtF’S’mdh, DayAheadAssessmentFlexibleObligationQuantity BrtF’S’mdh)

#### DayAheadEligibleEFCPminQuantity

DayAheadEligibleEFCPminQuantity BrtF’S’mdh = Max(0, Min (DayAheadUpperOperatingLimit BrtF’S’mdh , DayAheadNoSelfScheduleFlag BrtF’S’mdh * Resource90MinStartUpFlag Brtmd  * MinOperMW BrtF'S'md ))

Where F’ <> LESR

Note: In the above calculation the MinOperMW daily value will be duplicated each hour.

#### DayAheadNoSelfScheduleFlag

IF

(DayAheadUpperSelfScheduleEnergyBidQuantity BrtF’S’mdh = 0

AND DayAheadUpperEnergyBidQuantity BrtF’S’mdh > 0)

THEN

DayAheadNoSelfScheduleFlag BrtF’S’mdh = 1

ELSE

DayAheadNoSelfScheduleFlag BrtF’S’mdh = 0

END IF

Note: The above calculation shall use DayAheadAssessmentFlexibleObligationQuantity as a business driver.

### Real Time Availability

#### BAResourceDailyRealTimeAvailabilityQuantity

BAResourceDailyRealTimeAvailabilityQuantity BrtF’S’md = hRealTimeAvailabilityQuantity BrtF’S’mdh

#### RealTimeAvailabilityQuantity

RealTimeAvailabilityQuantity BrtF’S’mdh = RealTimeGenericAvailabilityQuantity BrtF’S’mdh + RealTimeFlexibleAvailabilityQuantity BrtF’S’mdh

#### RealTimeGenericAvailabilityQuantity

RealTimeGenericAvailabilityQuantity BrtF’S’mdh = Min(RealTimeMaxGenericAvailabilityQuantity BrtF’S’mdh, Max(0, RealTimeAvailableTotalBidQuantity BrtF’S’mdh – RealTimeFlexibleAvailabilityQuantity BrtF’S’mdh))

#### RealTimeMaxGenericAvailabilityQuantity

RealTimeMaxGenericAvailabilityQuantity BrtF’S’mdh = Max(0, RealTimeAssessmentGenericObligationQuantity BrtF’S’mdh – RealTimeAssessmentFlexibleObligationQuantity BrtF’S’mdh)

#### RealTimeFlexibleAvailabilityQuantity

IF

BAHourlyResDAMRegulationAward Brtmdh > 0

THEN

RealTimeFlexibleAvailabilityQuantity BrtF’S’mdh  = Min ((BAHourlyResRegulationSlackCapacity BrtF’S’mdh + RealTimeAvailableEconomicBidQuantity BrtF’S’mdh + RealTimeEligibleEFCPminQuantity BrtF’S’mdh), RealTimeAssessmentFlexibleObligationQuantity BrtF’S’mdh, RealTimeHourlyUpperOperatingLimit BrtF’S’mdh)

ELSE

RealTimeFlexibleAvailabilityQuantity BrtF’S’mdh  = Min (RealTimeAvailableEconomicBidQuantity BrtF’S’mdh + RealTimeEligibleEFCPminQuantity BrtF’S’mdh), RealTimeAssessmentFlexibleObligationQuantity BrtF’S’mdh)

END IF

#### BAHourlyResRegulationSlackCapacity

IF

BAHourlyResDAMRegulationAward Brtmdh > 0

THEN

BAHourlyResRegulationSlackCapacity BrtF’S’mdh = Min(RealTimeUpperSelfScheduleEnergyBidQuantity BrtF’S’mdh , BAHourlyLowerRegulationLimitQty Brtmdh + BAHourlyResDAMRegDownAward BrtF’S’mdh) - MinOperMW BrtF'S'md

ELSE

BAHourlyResRegulationSlackCapacity BrtF’S’mdh = 0

END IF

Note: This will be calculated only when RealTimeUpperSelfScheduleEnergyBidQuantity BrtF’S’mdh exists. The MinOperMW daily value will be duplicated each hour.

#### BAHourlyResDAMRegDownAward

BAHourlyResDAMRegDownAward BrtF’S’mdh = uT'I'M'VL'W'R'DARegDownAwardedBidQuantity BrtT’uI’M’R’W’F’S’VL'mdh

#### BAHourlyResDAMRegUpAward

BAHourlyResDAMRegUpAward BrtF’S’mdh = uT'I'M'VL'W'R' DARegUpAwardedBidQuantity BrtT’uI’M’R’W’F’S’VL'mdh

#### BAHourlyResDAMRegulationAward

BAHourlyResDAMRegulationAward Brtmdh = uT'I'M'VL'W'R'F'S'( DARegUpAwardedBidQuantity BrtT’uI’M’R’W’F’S’VL'mdh + DARegDownAwardedBidQuantity BrtT’uI’M’R’W’F’S’VL'mdh )

#### RealTimeEligibleEFCPminQuantity

IF

BAHourlyResDAMRegulationAward Brtmdh > 0

THEN

RealTimeEligibleEFCPminQuantity BrtF’S’mdh = Max(0, Min (RealTimeHourlyUpperOperatingLimit BrtF’S’mdh , Resource90MinStartUpFlag Brtmd  * MinOperMW BrtF'S'md ))

ELSE

RealTimeEligibleEFCPminQuantity BrtF’S’mdh = Max(0, Min (RealTimeHourlyUpperOperatingLimit BrtF’S’mdh ,RealTimeNoSelfScheduleFlag BrtF’S’mdh * Resource90MinStartUpFlag Brtmd  * MinOperMW BrtF'S'md ))

END IF

Where F’ <> LESR

Note: In the above calculation the MinOperMW daily value will be duplicated each hour.

#### RealTimeNoSelfScheduleFlag

IF

(RealTimeUpperSelfScheduleEnergyBidQuantity BrtF’S’mdh = 0

AND

RealTimeUpperEnergyBidQuantity BrtF’S’mdh > 0)

THEN

RealTimeNoSelfScheduleFlag BrtF’S’mdh = 1

ELSE

RealTimeNoSelfScheduleFlag BrtF’S’mdh = 0

END IF

Note: The above calculation shall use RealTimeAssessmentFlexibleObligationQuantity as a business driver.

### Day Ahead Generic Obligation

#### DayAheadAssessmentGenericObligationQuantity

DayAheadAssessmentGenericObligationQuantity BrtF’S’mdh = DayAheadGenericObligationQuantity BrtF’S’mdh + DayAheadGenericCPMObligationQuantity BrtF’S’mdh

#### DayAheadGenericObligationQuantity

IF

DayAheadResourceGenericRAAIMExemptionFlag rmd = 1

THEN

DayAheadGenericObligationQuantity BrtF’S’mdh = 0

ELSE

DayAheadGenericObligationQuantity BrtF’S’mdh = MAX(0, DayAheadGenericCapacityQty BrtF’S’mdh – DayAheadGenericRAOutageExemptQuantity BrtF’S’mdh ) * GenericAssessmentFlag mdh

END IF

Note: This equation shall utilize DayAheadGenericCapacityQty as the business driver.

#### DayAheadGenericCPMObligationQuantity

IF

DayAheadResourceGenericRAAIMExemptionFlag rmd = 1

THEN

DayAheadGenericCPMObligationQuantity BrtF’S’mdh =0

ELSE

DayAheadGenericCPMObligationQuantity BrtF’S’mdh = MAX(0, DayAheadGenericCPMCapacityQty BrtF’S’mdh – DayAheadGenericCPMOutageExemptQuantity BrtF’S’mdh ) * GenericAssessmentFlag mdh

END IF

Note: This equation shall utilize DayAheadGenericCPMCapacityQty as the business driver.

### Day Ahead Flexible Obligation

#### DayAheadAssessmentFlexibleObligationQuantity

DayAheadAssessmentFlexibleObligationQuantity BrtF’S’mdh = j' (DayAheadFlexibleObligationQuantity BrtF’S’j’mdh + DayAheadFlexibleCPMObligationQuantity BrtF’S’j’mdh)

#### DayAheadFlexibleObligationQuantity

IF

DayAheadResourceFlexibleRAAIMExemptionFlag rmd = 1

THEN

DayAheadFlexibleObligationQuantity BrtF’S’j’mdh = 0

ELSE

DayAheadFlexibleObligationQuantity BrtF’S’j’mdh = MAX(0, DayAheadFlexibleCapacityQty BrtF’S’j’mdh – DayAheadFlexibleRAOutageExemptQuantity BrtF’S’j’mdh ) * ResourceFlexibleAssessmentFlag j’mdh

END IF

Note: This equation shall utilize DayAheadFlexibleCapacityQty as the business driver.

#### DayAheadFlexibleCPMObligationQuantity

IF

DayAheadResourceFlexibleRAAIMExemptionFlag rmd = 1

THEN

DayAheadFlexibleCPMObligationQuantity BrtF’S’j’mdh = 0

ELSE

DayAheadFlexibleCPMObligationQuantity BrtF’S’j’mdh = MAX(0, DayAheadFlexibleCPMCapacityQty BrtF’S’j’mdh – DayAheadFlexibleCPMOutageExemptQuantity BrtF’S’j’mdh ) * ResourceFlexibleAssessmentFlag j’mdh

END IF

Note: This equation shall utilize DayAheadFlexibleCPMCapacityQty as the business driver.

### Real Time Generic Obligation

#### RealTimeAssessmentGenericObligationQuantity

RealTimeAssessmentGenericObligationQuantity BrtF’S’mdh = RealTimeGenericObligationQuantity BrtF’S’mdh + RealTimeGenericCPMObligationQuantity BrtF’S’mdh

#### RealTimeGenericObligationQuantity

IF

INTDUPLICATE(RealTimeResourceGenericRAAIMExemptionFlag rmd) + ResourceLongStartRealTimeRAAIMExemptionFlag rmdh + ResourceExtremelyLongStartRealTimeRAAIMExemptionFlag rmdh > 0

THEN

RealTimeGenericObligationQuantity BrtF’S’mdh = 0

ELSE

RealTimeGenericObligationQuantity BrtF’S’mdh = MAX(0, RealTimeGenericCapacityQty BrtF’S’mdh – RealTimeGenericRAOutageExemptQuantity BrtF’S’mdh ) * GenericAssessmentFlag mdh

END IF

Note: This equation shall utilize RealTimeGenericCapacityQty as the business driver.

#### RealTimeGenericCPMObligationQuantity

IF

INTDUPLICATE(RealTimeResourceGenericRAAIMExemptionFlag rmd) + ResourceLongStartRealTimeRAAIMExemptionFlag rmdh + ResourceExtremelyLongStartRealTimeRAAIMExemptionFlag rmdh > 0

THEN

RealTimeGenericCPMObligationQuantity BrtF’S’mdh = 0

ELSE

RealTimeGenericCPMObligationQuantity BrtF’S’mdh = MAX(0, RealTimeGenericCPMCapacityQty BrtF’S’mdh – RealTimeGenericCPMOutageExemptQuantity BrtF’S’mdh ) * GenericAssessmentFlag mdh

END IF

Note: This equation shall utilize RealTimeGenericCPMCapacityQty as the business driver.

### Real Time Flexible Obligation

#### RealTimeAssessmentFlexibleObligationQuantity

RealTimeAssessmentFlexibleObligationQuantity BrtF’S’mdh = j' (RealTimeNonVERFlexibleObligationQuantity BrtF’S’j’mdh + RealTimeVERFlexibleObligationQuantity BrtF’S’j’mdh)

#### RealTimeNonVERFlexibleObligationQuantity

IF

ResourceVERExemptionFlag Brtmd = 0

THEN

RealTimeNonVERFlexibleObligationQuantity BrtF’S’j’mdh = RealTimeFlexibleObligationQuantity BrtF’S’j’mdh + RealTimeFlexibleCPMObligationQuantity BrtF’S’j’mdh

ELSE

RealtimeNonVERFlexibleObligationQuantity BrtF’S’j’mdh = 0

END IF

#### RealTimeVERFlexibleObligationQuantity

IF

ResourceVERExemptionFlag rmd = 1

THEN

RealTimeVERFlexibleObligationQuantity BrtF’S’j’mdh = RealTimeHourlyVERForecastQuantity BrtF’S’mdh * ((RealTimeFlexibleObligationQuantity BrtF’S’j’mdh + RealTimeFlexibleCPMObligationQuantity BrtF’S’j’mdh) / ResourceEFCQty rmd)

ELSE

RealTimeVERFlexibleObligationQuantity BrtF’S’j’mdh = 0

END IF

#### RealTimeHourlyVERForecastQuantity

RealTimeHourlyVERForecastQuantity BrtF’S’mdh =

c BA15MinResourceIntertieDeclinePenaltyFMMFinalForecastEnergy BrtF’S’mdhc / 4

#### RealTimeFlexibleObligationQuantity

IF

INTDUPLICATE(RealTimeResourceFlexibleRAAIMExemptionFlag rmd) + ResourceLongStartRealTimeRAAIMExemptionFlag rmdh + ResourceExtremelyLongStartRealTimeRAAIMExemptionFlag rmdh > 0

THEN

RealTimeFlexibleObligationQuantity BrtF’S’j’mdh = 0

ELSE

RealTimeFlexibleObligationQuantity BrtF’S’j’mdh = MAX(0, RealTimeFlexibleCapacityQty BrtF’S’j’mdh – RealTimeFlexibleRAOutageExemptQuantity BrtF’S’j’mdh ) * ResourceFlexibleAssessmentFlag j’mdh

END IF

Note: This equation shall utilize RealTimeFlexibleCapacityQty as the business driver.

#### RealTimeFlexibleCPMObligationQuantity

IF

INTDUPLICATE(RealTimeResourceFlexibleRAAIMExemptionFlag rmd) + ResourceLongStartRealTimeRAAIMExemptionFlag rmdh + ResourceExtremelyLongStartRealTimeRAAIMExemptionFlag rmdh > 0

THEN

RealTimeFlexibleCPMObligationQuantity BrtF’S’j’mdh = 0

ELSE

RealTimeFlexibleCPMObligationQuantity BrtF’S’j’mdh = MAX(0, RealTimeFlexibleCPMCapacityQty BrtF’S’j’mdh – RealTimeFlexibleCPMOutageExemptQuantity BrtF’S’j’mdh ) * ResourceFlexibleAssessmentFlag j’mdh

END IF

Note: This equation shall utilize RealTimeFlexibleCPMCapacityQty as the business driver.

### RAAIM Exemption

#### DayAheadResourceGenericRAAIMExemptionFlag

IF

ResourceAcquiredRightsFlag rmd + ResourcePMaxExemptionFlag rmd + ResourceQFExemptionFlag rmd + ResourceParticipatingLoadExemptionFlag rmd + ResourceVERExemptionFlag rmd + ResourceCHPExemptionFlag rmd + ResourceRDRRFlag rmd + ResourceRMRFlag rmd >= 1

THEN

DayAheadResourceGenericRAAIMExemptionFlag rmd = 1

ELSE

DayAheadResourceGenericRAAIMExemptionFlag rmd = 0

END IF

#### RealTimeResourceGenericRAAIMExemptionFlag

IF

ResourceAcquiredRightsFlag rmd + ResourcePMaxExemptionFlag rmd + ResourceQFExemptionFlag rmd + ResourceParticipatingLoadExemptionFlag rmd + ResourceVERExemptionFlag rmd + ResourceCHPExemptionFlag rmd + ResourceRMRFlag rmd >= 1

THEN

RealTimeResourceGenericRAAIMExemptionFlag rmd = 1

ELSE

RealTimeResourceGenericRAAIMExemptionFlag rmd = 0

END IF

#### DayAheadResourceFlexibleRAAIMExemptionFlag

IF

ResourceAcquiredRightsFlag rmd + ResourcePMaxExemptionFlag rmd + ResourceQFExemptionFlag rmd + ResourceCombinedFlexibleRAExemptionFlag rmd + ResourceVERExemptionFlag rmd + ResourceRDRRFlag rmd + ResourceParticipatingLoadExemptionFlag rmd + ResourceRMRFlag rmd >= 1

THEN

DayAheadResourceFlexibleRAAIMExemptionFlag rmd = 1

ELSE

DayAheadResourceFlexibleRAAIMExemptionFlag rmd = 0

END IF

#### RealTimeResourceFlexibleRAAIMExemptionFlag

IF

ResourceAcquiredRightsFlag rmd + ResourcePMaxExemptionFlag rmd + ResourceQFExemptionFlag rmd + ResourceCombinedFlexibleRAExemptionFlag rmd + ResourceParticipatingLoadExemptionFlag rmd + ResourceRMRFlag rmd >= 1

THEN

RealTimeResourceFlexibleRAAIMExemptionFlag rmd = 1

ELSE

RealTimeResourceFlexibleRAAIMExemptionFlag rmd = 0

END IF

#### DayAheadGenericRAOutageExemptQuantity

DayAheadGenericRAOutageExemptQuantity BrtF’S’mdh =

DayAheadEligibleGenericExemptOutageQuantity BrtF’S’mdh * (DayAheadGenericCapacityQty BrtF’S’mdh / (DayAheadGenericCapacityQty BrtF’S’mdh + DayAheadGenericCPMCapacityQty BrtF’S’mdh ))

#### DayAheadGenericCPMOutageExemptQuantity

DayAheadGenericCPMOutageExemptQuantity BrtF’S’mdh =

DayAheadEligibleGenericExemptOutageQuantity BrtF’S’mdh * (DayAheadGenericCPMCapacityQty BrtF’S’mdh / (DayAheadGenericCapacityQty BrtF’S’mdh + DayAheadGenericCPMCapacityQty BrtF’S’mdh ))

#### DayAheadEligibleGenericExemptOutageQuantity

DayAheadEligibleGenericExemptOutageQuantity BrtF’S’mdh = MAX(0, (DayAheadGenericCapacityQty BrtF’S’mdh + DayAheadGenericCPMCapacityQty BrtF’S’mdh ) – (DayAheadResourceGenericExemptOutageThreshold BrtF’S’mdh + DayAheadNRSSResourceGenericExemptOutageThreshold BrtF’S’mdh ))

#### DayAheadResourceGenericExemptOutageThreshold

DayAheadResourceGenericExemptOutageThreshold BrtF’S’mdh = MaxOperMW BrtF'S'md – (DayAheadGenericExemptOutageQty BrtF’S’mdh + (ResourceUseLimitationReachedFlag Brtmdh * DayAheadUseLimitedExemptOutageQty BrtF’S’mdh ))

#### Where Entity Component Type (F’) <> INTERTIE

#### DayAheadNRSSResourceGenericExemptOutageThreshold

DayAheadNRSSResourceGenericExemptOutageThreshold BrtF’S’mdh = DayAheadGenericExemptOutageQty BrtF’S’mdh

#### Where Resource Type (t) = ITIE and Entity Component Type (F’) = INTERTIE

#### DayAheadFlexibleRAOutageExemptQuantity

DayAheadFlexibleRAOutageExemptQuantity BrtF’S’j’mdh = DayAheadEligibleFlexibleExemptOutageQuantity BrtF’S’mdh * (DayAheadFlexibleCapacityQty BrtF’S’j’mdh / (DayAheadFlexibleCapacityQty BrtF’S’j’mdh + DayAheadFlexibleCPMCapacityQty BrtF’S’j’mdh ))

#### DayAheadFlexibleCPMOutageExemptQuantity

DayAheadFlexibleCPMOutageExemptQuantity BrtF’S’j’mdh = DayAheadEligibleFlexibleExemptOutageQuantity BrtF’S’j’mdh * (DayAheadFlexibleCPMCapacityQty BrtF’S’j’mdh / (DayAheadFlexibleCapacityQty BrtF’S’j’mdh + DayAheadFlexibleCPMCapacityQty BrtF’S’j’mdh ))

#### DayAheadEligibleFlexibleExemptOutageQuantity

DayAheadEligibleFlexibleExemptOutageQuantity BrtF’S’j’mdh = MAX(0, (DayAheadFlexibleCapacityQty BrtF’S’j’mdh + DayAheadFlexibleCPMCapacityQty BrtF’S’j’mdh + ((1- Resource90MinStartUpFlag Brtmd ) * MinOperMW BrtF'S'md )) – (DayAheadResourceFlexibleExemptOutageThreshold BrtF’S’mdh + DayAheadNRSSResourceFlexibleExemptOutageThreshold BrtF’S’mdh) )

#### DayAheadResourceFlexibleExemptOutageThreshold

DayAheadResourceFlexibleExemptOutageThreshold BrtF’S’mdh = MaxOperMW BrtF'S'md – (DayAheadFlexibleExemptOutageQty BrtF’S’mdh + (ResourceUseLimitationReachedFlag Brtmdh * DayAheadUseLimitedExemptOutageQty BrtF’S’mdh ))

#### Where Entity Component Type (F’) <> INTERTIE

#### DayAheadNRSSResourceFlexibleExemptOutageThreshold

DayAheadNRSSResourceFlexibleExemptOutageThreshold BrtF’S’mdh = DayAheadFlexibleExemptOutageQty BrtF’S’mdh

#### Where Resource Type (t) = ITIE and Entity Component Type (F’) = INTERTIE

#### RealTimeGenericRAOutageExemptQuantity

RealTimeGenericRAOutageExemptQuantity BrtF’S’mdh =

RealTimeEligibleGenericExemptOutageQuantity BrtF’S’mdh * (RealTimeGenericCapacityQty BrtF’S’mdh / (RealTimeGenericCapacityQty BrtF’S’mdh + RealTimeGenericCPMCapacityQty BrtF’S’mdh ))

#### RealTimeGenericCPMOutageExemptQuantity

RealTimeGenericCPMOutageExemptQuantity BrtF’S’mdh =

RealTimeEligibleGenericExemptOutageQuantity BrtF’S’mdh * (RealTimeGenericCPMCapacityQty BrtF’S’mdh / (RealTimeGenericCapacityQty BrtF’S’mdh + RealTimeGenericCPMCapacityQty BrtF’S’mdh ))

#### RealTimeEligibleGenericExemptOutageQuantity

RealTimeEligibleGenericExemptOutageQuantity BrtF’S’mdh = MAX(0, (RealTimeGenericCapacityQty BrtF’S’mdh + RealTimeGenericCPMCapacityQty BrtF’S’mdh ) – (RealTimeResourceGenericExemptOutageThreshold BrtF’S’mdh + RealTimeNRSSResourceGenericExemptOutageThreshold BrtF’S’mdh ))

#### RealTimeResourceGenericExemptOutageThreshold

RealTimeResourceGenericExemptOutageThreshold BrtF’S’mdh = MaxOperMW BrtF'S'md – (RealTimeGenericExemptOutageQty BrtF’S’mdh + (ResourceUseLimitationReachedFlag Brtmdh * RealTimeUseLimitedExemptOutageQty BrtF’S’mdh ))

#### Where Entity Component Type (F’) <> INTERTIE

#### RealTimeNRSSResourceGenericExemptOutageThreshold

RealTimeNRSSResourceGenericExemptOutageThreshold BrtF’S’mdh = RealTimeGenericExemptOutageQty BrtF’S’mdh

#### Where Resource Type (t) = ITIE and Entity Component Type (F’) = INTERTIE

#### RealTimeFlexibleRAOutageExemptQuantity

RealTimeFlexibleRAOutageExemptQuantity BrtF’S’j’mdh = RealTimeEligibleFlexibleExemptOutageQuantity BrtF’S’mdh * (RealTimeFlexibleCapacityQty BrtF’S’j’mdh / (RealTimeFlexibleCapacityQty BrtF’S’j’mdh + RealTImeFlexibleCPMCapacityQty BrtF’S’j’mdh ))

#### RealTimeFlexibleCPMOutageExemptQuantity

RealTimeFlexibleCPMOutageExemptQuantity BrtF’S’j’mdh = RealTimeEligibleFlexibleExemptOutageQuantity BrtF’S’j’mdh * (RealTimeFlexibleCPMCapacityQty BrtF’S’j’mdh / (RealTimeFlexibleCapacityQty BrtF’S’j’mdh + RealTimeFlexibleCPMCapacityQty BrtF’S’j’mdh ))

#### RealTimeEligibleFlexibleExemptOutageQuantity

RealTimeEligibleFlexibleExemptOutageQuantity BrtF’S’j’mdh = MAX(0, (RealTimeFlexibleCapacityQty BrtF’S’j’mdh + RealTimeFlexibleCPMCapacityQty BrtF’S’j’mdh + ((1- Resource90MinStartUpFlag Brtmd ) * MinOperMW BrtF'S'md )) – (RealTimeResourceFlexibleExemptOutageThreshold BrtF’S’mdh + RealTimeNRSSResourceFlexibleExemptOutageThreshold BrtF’S’mdh) )

#### RealTimeResourceFlexibleExemptOutageThreshold

RealTimeResourceFlexibleExemptOutageThreshold BrtF’S’mdh = MaxOperMW BrtF'S'md – (RealTimeFlexibleExemptOutageQty BrtF’S’mdh + (ResourceUseLimitationReachedFlag Brtmdh * RealTimeUseLimitedExemptOutageQty BrtF’S’mdh ))

#### Where Entity Component Type (F’) <> INTERTIE

#### RealTimeNRSSResourceFlexibleExemptOutageThreshold

RealTimeNRSSResourceFlexibleExemptOutageThreshold BrtF’S’mdh = RealTimeFlexibleExemptOutageQty BrtF’S’mdh

#### Where Resource Type (t) = ITIE and Entity Component Type (F’) = INTERTIE

#### ResourceExtremelyLongStartRealTimeRAAIMExemptionFlag

IF

ResourceHourlyDayAheadEnergy rmdh = 0

AND

ResourceExtremelyLongStartFlag rmd = 1

THEN

ResourceExtremelyLongStartRealTimeRAAIMExemptionFlag rmdh = 1

ELSE

ResourceExtremelyLongStartRealTimeRAAIMExemptionFlag rmdh = 0

END IF

#### ResourceLongStartRealTimeRAAIMExemptionFlag

IF

(ResourceHourlyRUCAwardQuantity rmdh + ResourceHourlyDayAheadEnergy rmdh) = 0

AND

ResourceLongStartFlag rmd = 1

THEN

ResourceLongStartRealTimeRAAIMExemptionFlag rmdh = 1

ELSE

ResourceLongStartRealTimeRAAIMExemptionFlag rmdh = 0

END IF

#### ResourceHourlyRUCAwardQuantity

ResourceHourlyRUCAwardQuantity rmdh = BtuT'I'M'VLW'R'F'S'cf∈ c (BAResourceHourlyRUCAwardedBidCapacity BrtuT'I'M'VL'W'R'F'S'mdh

#### ResourceHourlyDayAheadEnergy

ResourceHourlyDayAheadEnergy rmdh = BtuT'I'Q'M'F'S'cf∈ cSettlementIntervalResouceDayAheadEnergy BrtuT’I’Q’M’F’S’mdhcif

### Day Ahead Total & Economic Bid

#### DayAheadAvailableTotalBidQuantity

DayAheadAvailableTotalBidQuantity BrtF’S’mdh = DayAheadTotalEnergyBidQuantity BrtF’S’mdh + DayAheadNGRTotalBidQuantity BrtF’S’mdh

#### DayAheadAvailableEconomicBidQuantity

DayAheadAvailableEconomicBidQuantity BrtF’S’mdh = DayAheadEconomicEnergyBidQuantity BrtF’S’mdh + DayAheadNGREconomicBidQuantity BrtF’S’mdh

#### DayAheadTotalEnergyBidQuantity

DayAheadTotalEnergyBidQuantity BrtF’S’mdh =

Min(DayAheadOutageAvailabilityQuantity BrtF’S’mdh , Max(0,DayAheadUpperSelfScheduleEnergyBidQuantity BrtF’S’mdh , DayAheadUpperEnergyBidQuantity BrtF’S’mdh))

Where S’ <> REM

#### DayAheadUpperSelfScheduleEnergyBidQuantity

DayAheadUpperSelfScheduleEnergyBidQuantity BrtF’S’mdh = ubAA'paBAHourlyResDAMEnergySelfScheduleBidQty BrtubAA’pF’S’amdh

Where S’ <> REM

#### DayAheadEconomicEnergyBidQuantity

DayAheadEconomicEnergyBidQuantity BrtF’S’mdh = Max(0, Min(DayAheadOutageAvailabilityQuantity BrtF’S’mdh , DayAheadUpperEnergyBidQuantity BrtF’S’mdh ) - DayAheadLowerEnergyBidQuantity BrtF’S’mdh )

Where S’ <> REM

#### DayAheadUpperEnergyBidQuantity

DayAheadUpperEnergyBidQuantity BrtF’S’mdh = Max over u, b, A, A’, p (BAHourlyResDAMEnergyBidQty BrtubAA’pF’S’mdh)

#### DayAheadLowerEnergyBidQuantity

DayAheadLowerEnergyBidQuantity BrtF’S’mdh = ubAA'pBAHourlyResDAMEnergyBidQty BrtubAA’pF’S’mdh

Where Bid Segment Number (b) = 1

#### DayAheadNGRTotalBidQuantity

DayAheadNGRTotalBidQuantity BrtF’S’mdh = Min(DayAheadOutageAvailabilityQuantity BrtF’S’mdh , DayAheadNGRTotalRegUpBidQuantity BrtF’S’mdh , DayAheadNGRTotalRegDownBidQuantity BrtF’S’mdh )

#### DayAheadNGRTotalRegUpBidQuantity

DayAheadNGRTotalRegUpBidQuantity BrtF’S’mdh = b (BAHourlyResDAMRegUpSelfProvisionBidQty BrtbF’S’mdh + BAHourlyResDAMRegUpBidQty BrtbF’S’mdh)

Where S’ = REM

#### DayAheadNGRTotalRegDownBidQuantity

DayAheadNGRTotalRegDownBidQuantity BrtF’S’mdh = b (BAHourlyResDAMRegDownSelfProvisionBidQty BrtbF’S’mdh + BAHourlyResDAMRegDownBidQty BrtbF’S’mdh)

Where S’ = REM

#### DayAheadNGREconomicBidQuantity

DayAheadNGREconomicBidQuantity BrtF’S’mdh = Min(DayAheadOutageAvailabilityQuantity BrtF’S’mdh , DayAheadNGREconomicRegUpBidQuantity BrtF’S’mdh , DayAheadNGREconomicRegDownBidQuantity BrtF’S’mdh )

#### DayAheadNGREconomicRegUpBidQuantity

DayAheadNGREconomicRegUpBidQuantity BrtF’S’mdh =

b BAHourlyResDAMRegUpBidQty BrtbF’S’mdh

Where S’ = REM

#### DayAheadNGREconomicRegDownBidQuantity

DayAheadNGREconomicRegDownBidQuantity BrtF’S’mdh =

b  BAHourlyResDAMRegDownBidQty BrtbF’S’mdh

Where S’ = REM

#### DayAheadOutageAvailabilityQuantity

DayAheadOutageAvailabilityQuantity BrtF’S’mdh = Max(0, DayAheadUpperOperatingLimit BrtF’S’mdh – MIN(0, DayAheadLowerOperatingLimit BrtF’S’mdh ))

### Real Time Total & Economic Bid

#### RealTimeAvailableTotalBidQuantity

RealTimeAvailableTotalBidQuantity BrtF’S’mdh = RealTimeTotalEnergyBidQuantity BrtF’S’mdh + RealTimeNGRTotalBidQuantity BrtF’S’mdh

#### RealTimeAvailableEconomicBidQuantity

RealTimeAvailableEconomicBidQuantity BrtF’S’mdh = RealTimeEconomicEnergyBidQuantity BrtF’S’mdh + RealTimeNGREconomicBidQuantity BrtF’S’mdh

#### RealTimeTotalEnergyBidQuantity

RealTimeTotalEnergyBidQuantity BrtF’S’mdh =

Min(RealTimeOutageAvailabilityQuantity BrtF’S’mdh , Max(0,RealTimeUpperSelfScheduleEnergyBidQuantity BrtF’S’mdh , RealTimeUpperEnergyBidQuantity BrtF’S’mdh))

Where S’ <> REM

#### RealTimeUpperSelfScheduleEnergyBidQuantity

RealTimeUpperSelfScheduleEnergyBidQuantity BrtF’S’mdh = uQ'bAA'paBAHourlyResRTMEnergySelfScheduleBidQty BrtuQ’bAA’pF’S’amdh

Where S’ <> REM

#### RealTimeEconomicEnergyBidQuantity

RealTimeEconomicEnergyBidQuantity BrtF’S’mdh = Max(0, Min(RealTimeOutageAvailabilityQuantity BrtF’S’mdh , RealTimeUpperEnergyBidQuantity BrtF’S’mdh ) - RealTimeLowerEnergyBidQuantity BrtF’S’mdh)

Where S’ <> REM

#### RealTimeUpperEnergyBidQuantity

RealTimeUpperEnergyBidQuantity BrtF’S’mdh = Max over u, Q’, b, A, A’, p (BAHourlyResRTMEnergyBidQty BrtuQ’bAA’pF’S’mdh )

#### RealTimeLowerEnergyBidQuantity

RealTimeLowerEnergyBidQuantity BrtF’S’mdh = uQ'bAA'pBAHourlyResRTMEnergyBidQty BrtuQ’bAA’pF’S’mdh

Where Bid Segment Number (b) = 1

#### RealTimeNGRTotalBidQuantity

RealTimeNGRTotalBidQuantity BrtF’S’mdh = Min(RealTimeOutageAvailabilityQuantity BrtF’S’mdh , (RealTimeNGRTotalRegUpBidQuantity BrtF’S’mdh + BAHourlyResDAMRegUpAward BrtF’S’mdh , RealTimeNGRTotalRegDownBidQuantity BrtF’S’mdh + BAHourlyResDAMRegDownAward BrtF’S’mdh ))

Where S’ = REM

#### RealTimeNGRTotalRegUpBidQuantity

RealTimeNGRTotalRegUpBidQuantity BrtF’S’mdh = b (BAHourlyResRTMRegUpSelfProvisionBidQty BrtbF’S’mdh + BAHourlyResRTMRegUpBidQty BrtbF’S’mdh)

Where S’ = REM

#### RealTimeNGRTotalRegDownBidQuantity

RealTimeNGRTotalRegDownBidQuantity BrtF’S’mdh = b (BAHourlyResRTMRegDownSelfProvisionBidQty BrtbF’S’mdh + BAHourlyResRTMRegDownBidQty BrtbF’S’mdh)

Where S’ = REM

#### RealTimeNGREconomicBidQuantity

RealTimeNGREconomicBidQuantity BrtF’S’mdh = Min(RealTimeOutageAvailabilityQuantity BrtF’S’mdh , (RealTimeNGREconomicRegUpBidQuantity BrtF’S’mdh + BAHourlyResDAMRegUpAward BrtF’S’mdh , RealTimeNGREconomicRegDownBidQuantity BrtF’S’mdh + BAHourlyResDAMRegDownAward BrtF’S’mdh))

Where S’ = REM

#### RealTimeNGREconomicRegUpBidQuantity

RealTimeNGREconomicRegUpBidQuantity BrtF’S’mdh =

b BAHourlyResRTMRegUpBidQty BrtbF’S’mdh

Where S’ = REM

#### RealTimeNGREconomicRegDownBidQuantity

RealTimeNGREconomicRegDownBidQuantity BrtF’S’mdh =

b BAHourlyResRTMRegDownBidQty BrtbF’S’mdh

Where S’ = REM

#### RealTimeOutageAvailabilityQuantity

RealTimeOutageAvailabilityQuantity BrtF’S’mdh = Max(0, RealTimeHourlyUpperOperatingLimit BrtF’S’mdh – MIN(0, RealTimeHourlyLowerOperatingLimit BrtF’S’mdh ))

#### RealTimeHourlyUpperOperatingLimit

RealTimeHourlyUpperOperatingLimit BrtF’S’mdh = cc∈f INTMAX(RealTimeUpperOperatingLimit BrtF’S’mdhcif )

Note: The above equation does not perform a frequency conversion of the 5-minute variable.

#### RealTimeHourlyLowerOperatingLimit

RealTimeHourlyLowerOperatingLimit BrtF’S’mdh = cc∈f INTMAX(RealTimeLowerOperatingLimit  BrtF’S’mdhcif )

Note: The above equation does not perform a frequency conversion of the 5-minute variable.

### Prices for RAAIM Settlement

#### RAAIMNonAvailabiltyChargePrice

RAAIMNonAvailabiltyChargePrice m = 0.6*1000*CAISOMonthlyCPMSoftOfferCapPrice m

Note:

In the above formula the 1,000 multiplier serves to convert the price units from $ / kW–month to $ / MW–month

#### MonthlyResourceRAAIMCPMPrice

MonthlyResourceRAAIMCPMPrice Brtm = Max over o’ and k’ (BAMonthlyResourceCPMCapacityPaymentPrice Brto’k’m)

WHERE o’ <> FRDEF

#### MonthlyResourceRAAIMFlexibleCPMPrice

MonthlyResourceRAAIMFlexibleCPMPrice Brtm = Max over o’ and k’ (BAMonthlyResourceCPMCapacityPaymentPrice Brto’k’m)

WHERE o’ = FRDEF

### Determining Number of Obligation Hours for RAAIM Settlement

#### DailyResourceRAAIMObligationProrateFactor

DailyResourceRAAIMObligationProrateFactor BrtF’S’md = DailyRAAIMObligationFlag BrtF’S’md / MAX(DailyResourceGenericObligationProrateCount BrtF’S’md , DailyResourceFlexibleObligationProrateCount BrtF’S’md)

#### DailyResourceGenericObligationProrateCount

DailyResourceGenericObligationProrateCount BrtF’S’md = DailyRAAIMGenericObligationFlag BrtF’S’md * INTDUPLICATE(MonthlyAssessDaysGenericObligationCount m)

#### DailyRAAIMGenericObligationFlag

IF

h (DayAheadGenericObligationQuantity BrtF’S’mdh + RealTimeGenericObligationQuantity BrtF’S’mdh) > 0

THEN

DailyRAAIMGenericObligationFlag BrtF’S’md = 1

ELSE

DailyRAAIMGenericObligationFlag BrtF’S’md = 0

END IF

Note: The INTMAX function will be utilized such that if any hour within a day has DayAheadGenericObligationQuantity BrtF’S’mdh + RealTimeGenericObligationQuantity BrtF’S’mdh > 0

Then DailyRAAIMGenericObligationFlag BrtF’S’md = 1 for that day

#### DailyResourceFlexibleObligationProrateCount

DailyResourceFlexibleObligationProrateCount BrtF’S’md = Max over j’ (DailyRAAIMFlexibleObligationFlag BrtF’S’j’md * INTDUPLICATE(MonthlyAssessDaysFlexibleObligationCount j’m))

#### DailyRAAIMFlexibleObligationFlag

IF

h (DayAheadFlexibleObligationQuantity BrtF’S’j’mdh + RealTimeFlexibleObligationQuantity BrtF’S’j’mdh) > 0

THEN

DailyRAAIMFlexibleObligationFlag BrtF’S’j’md = 1

ELSE

DailyRAAIMFlexibleObligationFlag BrtF’S’j’md = 0

END IF

Note: The INTMAX function will be utilized such that if any hour within a day has DayAheadFlexibleObligationQuantity BrtF’S’j’mdh + RealTimeFlexibleObligationQuantity BrtF’S’j’mdh > 0

Then DailyRAAIMFlexibleObligationFlag BrtF’S’j’md = 1 for that day

#### DailyResourceGenericCPMObligationProrateFactor

DailyResourceGenericCPMObligationProrateFactor BrtF’S’md = DailyRAAIMGenericCPMFlag BrtF’S’md / MonthlyAssessDaysGenericObligationCount m

#### DailyResourceFlexibleCPMObligationProrateFactor

IF

hDayAheadFlexibleCPMObligationQuantity BrtF’S’j’mdh > 0

OR

hRealTimeFlexibleCPMObligationQuantity BrtF’S’j’mdh > 0

THEN

DailyResourceFlexibleCPMObligationProrateFactor BrtF’S’md =

j'(DailyRAAIMFlexibleCPMFlag BrtF’S’md / MonthlyAssessDaysFlexibleObligationCount j’m)

ELSE

DailyResourceFlexibleCPMObligationProrateFactor BrtF’S’md = 0

END IF

#### DailyRAAIMGenericCPMFlag

IF

DailyRAAIMGenericCPMCount BrtF’S’md > 0

THEN

DailyRAAIMGenericCPMFlag BrtF’S’md = 1

ELSE

DailyRAAIMGenericCPMFlag BrtF’S’md = 0

END IF

#### DailyRAAIMGenericCPMCount

DailyRAAIMGenericCPMCount BrtF’S’md = hHourlyRAAIMGenericCPMFlag BrtF’S’mdh

#### HourlyRAAIMGenericCPMFlag

IF

HourlyRAAIMGenericCPMQuantity BrtF’S’mdh > 0

THEN

HourlyRAAIMGenericCPMFlag BrtF’S’mdh = 1

ELSE

HourlyRAAIMGenericCPMFlag BrtF’S’mdh = 0

END IF

#### DailyRAAIMFlexibleCPMFlag

IF

DailyRAAIMFlexibleCPMCount BrtF’S’md > 0

THEN

DailyRAAIMFlexibleCPMFlag BrtF’S’md = 1

ELSE

DailyRAAIMFlexibleCPMFlag BrtF’S’md = 0

END IF

#### DailyRAAIMFlexibleCPMCount

#### DailyRAAIMFlexibleCPMCount BrtF’S’md = h HourlyRAAIMFlexibleCPMFlag BrtF’S’mdh

#### HourlyRAAIMFlexibleCPMFlag

IF

HourlyRAAIMFlexibleCPMQuantity BrtF’S’mdh > 0

THEN

HourlyRAAIMFlexibleCPMFlag BrtF’S’mdh = 1

ELSE

HourlyRAAIMFlexibleCPMFlag BrtF’S’mdh = 0

END IF

#### DailyRAAIMObligationFlag

IF

DailyRAAIMObligationCount BrtF’S’md > 0

THEN

DailyRAAIMObligationFlag BrtF’S’md = 1

ELSE

DailyRAAIMObligationFlag BrtF’S’md = 0

END IF

#### DailyRAAIMObligationCount

#### DailyRAAIMObligationCount BrtF’S’md = h HourlyRAAIMObligationFlag BrtF’S’mdh

#### HourlyRAAIMObligationFlag

IF

HourlyRAAIMObligationQuantity BrtF’S’mdh > 0

THEN

HourlyRAAIMObligationFlag BrtF’S’mdh = 1

ELSE

HourlyRAAIMObligationFlag BrtF’S’mdh = 0

END IF

#### MonthlyAssessDaysGenericObligationCount

MonthlyAssessDaysGenericObligationCount m = dDailyAssessGenericObligationFlag md

#### DailyAssessGenericObligationFlag

#### DailyAssessGenericObligationFlag md =  hINTMAX(GenericAssessmentFlag mdh)

#### MonthlyAssessDaysFlexibleObligationCount

#### MonthlyAssessDaysFlexibleObligationCount j’m = d DailyAssessFlexibleObligationFlag j’md

#### DailyAssessFlexibleObligationFlag

#### DailyAssessFlexibleObligationFlag j’md = hINTMAX(ResourceFlexibleAssessmentFlag j’mdh)

#### MonthlyObligationHoursQuantity

MonthlyObligationHoursQuantity m = dhDailyTradeHoursCount mdh

#### DailyTradeHoursCount

DailyTradeHoursCount mdh = INTDUPLICATE(CountFlag m)

### Metered Demand for RAAIM Neutrality Settlement

#### BusinessAssociateMonthlyRAAIMNodalMeteredCAISODemandQty

BusinessAssociateMonthlyRAAIMNodalMeteredCAISODemandQty Bm =

#### AA'BAHourlyNodalMeteredCAISODemandQuantity_MDOverCA BAA’mdh

## Outputs

| Output Req ID | Name | Description |
| --- | --- | --- |
|  | In addition to any outputs listed below, all inputs shall be included as outputs. |  |
|  | HourlyAssessmentBidObligationQuantity BrtF’S’mdh | An RA resource’s hourly RA bid obligation for each RAAIM assessment hour where a bidding obligation existed. |
|  | HourlyAssessmentAvailabilityQuantity BrtF’S’mdh | An RA resource’s hourly availability for each RAAIM assessment hour where a bidding obligation existed. |
|  | DailyDayAheadPerformance BrtF’S’md | Represents an RA resources overall performance as a percentage using day ahead availability and obligations values. |
|  | DailyRealTimePerformance BrtF’S’md | Represents an RA resources overall performance as a percentage using real time availability and obligations values. |
|  | DailyRAAIMDAorRTAssessmentFlag BrtF’S’md | Flag indicating which market to assess RAAIM 1 = RAAIM assessment using Day Ahead Market 0 = RAAIM assessment using Real Time Market |
|  | BAResourceDailyDayAheadAssessmentBidObligationQuantity BrtF’S’md | Sum of a RA Resource’s day ahead assessment hour obligation for the entire day. |
|  | DayAheadAssessmentBidObligationQuantity BrtF’S’mdh | RA resource’s hourly day ahead bid obligation made up of generic and flexible RA capacity. |
|  | BAResourceDailyRealTimeAssessmentBidObligationQuantity BrtF’S’md | Sum of a RA Resource’s real time assessment hour obligation for the entire day. |
|  | RealTimeAssessmentBidObligationQuantity BrtF’S’mdh | RA resource’s hourly real time bid obligation made up of generic and flexible RA capacity. |
|  | DayAheadOverallFlexibleObligationQuantity BrtF’S’mdh | The sum of the Day-ahead RAAIM obligation (capacity without CPM) over all Flexible RA categories for a resource and Trading Hour |
|  | RealTimeOverallFlexibleObligationQuantity BrtF’S’mdh | The sum of the Real-Time RAAIM obligation (capacity without CPM) over all Flexible RA categories for a resource and Trading Hour |
|  | DailyRAAIMObligationQuantity BrtF’S’md | The day average of an RA resource’s hourly RAAIM obligation (capacity without generic CPM or flexible CPM), used to calculate the resource’s monthly average RAAIM obligation (capacity without generic CPM or flexible CPM) quantity |
|  | HourlyRAAIMObligationQuantity BrtF’S’mdh | This value represents the RA resource’s hourly RAAIM obligation (capacity without CPM) |
|  | DailyRAAIMGenericCPMQuantity BrtF’S’md | The day average of the hourly generic CPM capacity, used to calculate the resource’s monthly average generic CPM quantity for RAAIM settlement |
|  | HourlyRAAIMGenericCPMQuantity BrtF’S’mdh | This value represents the hourly generic CPM capacity |
|  | DayAheadOverallFlexibleCPMObligationQuantity BrtF’S’mdh | The sum of the Day-ahead RAAIM obligation CPM capacity over all Flexible RA categories for a resource and Trading Hour |
|  | RealTimeOverallFlexibleCPMObligationQuantity BrtF’S’mdh | The sum of the Real-Time RAAIM obligation CPM capacity over all Flexible RA categories for a resource and Trading Hour |
|  | DailyRAAIMFlexibleCPMQuantity BrtF’S’md | The day average of the hourly flexible CPM capacity, used to calculate the resource’s monthly average flexible CPM quanitity for RAAIM settlement |
|  | HourlyRAAIMFlexibleCPMQuantity BrtF’S’mdh | This value represents the hourly flexible CPM capacity |
|  | BAResourceDailyDayAheadAvailabilityQuantity BrtF’S’md | Sum of a RA Resource’s day ahead assessment hour availability for the entire day. |
|  | DayAheadAvailabilityQuantity BrtF’S’mdh | RA resource’s day ahead availability determined by bids and RA bidding obligations. |
|  | DayAheadGenericAvailabilityQuantity BrtF’S’mdh | The quantity of bids that meet the day ahead generic RA capacity bidding obligation under RAAIM. |
|  | DayAheadMaxGenericAvailabilityQuantity BrtF’S’mdh | The share of generic RA bidding obligation in day ahead subject to RAAIM after taking into consideration flexible RA capacity. |
|  | DayAheadFlexibleAvailabilityQuantity BrtF’S’mdh | The quantity of economic bids that meet the day ahead flexible RA capacity bidding obligation under RAAIM. |
|  | DayAheadEligibleEFCPminQuantity BrtF’S’mdh | The portion of an RA resource’s Pmin that count towards its day ahead hourly flexible availability. |
|  | DayAheadNoSelfScheduleFlag BrtF’S’mdh | Indicates if a resource did not submit a self schedule into the day ahead market. |
|  | BAResourceDailyRealTimeAvailabilityQuantity BrtF’S’md | Sum of a RA Resource’s real time assessment hour availability for the entire day. |
|  | RealTimeAvailabilityQuantity BrtF’S’mdh | RA resource’s real time availability determined by bids and RA bidding obligations. |
|  | RealTimeGenericAvailabilityQuantity BrtF’S’mdh | The quantity of bids that meet the real time generic RA capacity bidding obligation under RAAIM. |
|  | RealTimeMaxGenericAvailabilityQuantity BrtF’S’mdh | The share of generic RA bidding obligation in real time subject to RAAIM after taking into consideration flexible RA capacity. |
|  | RealTimeFlexibleAvailabilityQuantity BrtF’S’mdh | The quantity of economic bids that meet the real time flexible RA capacity bidding obligation under RAAIM. |
|  | BAHourlyResDAMRegDownAward BrtF’S’md | Day-Ahead Regulation Down Award summed over non-used attributes. |
|  | BAHourlyResDAMRegUpAward BrtF’S’md | Day-Ahead Regulation Up Award summed over non-used attributes. |
|  | BAHourlyResDAMRegulationAward Brtmd | Sum of Day-ahead regulation up or down award, with unused attributes removed. |
|  | BAHourlyResRegulationSlackCapacity BrtF’S’mdh | Capacity between Pmin and RT self- schedule that is eligible towards RT flexible RA availability. This eligibility is due to a resource’s required self-scheduling requirements in RT due to DA up/down regulation awards. |
|  | RealTimeEligibleEFCPminQuantity BrtF’S’mdh | The portion of an RA resource’s Pmin that count towards its real time hourly flexible availability. |
|  | RealTimeNoSelfScheduleFlag BrtF’S’mdh | Indicates if a resource did not submit a self schedule into the real time market. |
|  | DayAheadAssessmentGenericObligationQuantity BrtF’S’mdh | RA resource’s hourly day ahead bidding obligation assessed under RAAIM. This value is the sum of day ahead generic and CPM capacity bidding obligation. |
|  | DayAheadGenericObligationQuantity BrtF’S’mdh | Day ahead generic capacity subject to RAAIM after applying outage and resource specific exemptions. |
|  | DayAheadGenericCPMObligationQuantity BrtF’S’mdh | Day ahead generic CPM capacity subject to RAAIM after applying outage and resource specific exemptions. |
|  | DayAheadAssessmentFlexibleObligationQuantity BrtF’S’mdh | RA resource’s hourly day ahead flexible bidding obligation assessed under RAAIM. This value is the sum of day ahead flexible and flexible CPM bidding obligation. |
|  | DayAheadFlexibleObligationQuantity BrtF’S’j’mdh | Day Ahead flexible capacity subject to RAAIM after applying outage and resource specific exemptions. |
|  | DayAheadFlexibleCPMObligationQuantity BrtF’S’j’mdh | Day Ahead flexible CPM capacity subject to RAAIM after applying outage and resource specific exemptions. |
|  | RealTimeAssessmentGenericObligationQuantity BrtF’S’mdh | RA resource’s hourly real time bidding obligation assessed under RAAIM. This value is the sum of real time generic and CPM capacity bidding obligation. |
|  | RealTimeGenericObligationQuantity BrtF’S’mdh | Real time generic capacity subject to RAAIM after applying outage and resource specific exemptions. |
|  | RealTimeGenericCPMObligationQuantity BrtF’S’mdh | Real time generic CPM capacity subject to RAAIM after applying outage and resource specific exemptions. |
|  | RealTimeAssessmentFlexibleObligationQuantity BrtF’S’mdh | RA resource’s hourly real time flexible bidding obligation assessed under RAAIM. This value is the sum of Non VER and VER real time flexible bidding obligation. |
|  | RealTimeNonVERFlexibleObligationQuantity BrtF’S’j’mdh | Non Variable Energy Resource’s (VER) real time flexible bidding obligation. |
|  | RealTimeVERFlexibleObligationQuantity BrtF’S’j’mdh | Variable Energy Resource’s (VER’s) real time flexible bidding obligation after adjusting for the resource’s forecast. |
|  | RealTimeHourlyVERForecastQuantity BrtF’S’mdh | The hourly average of the 15 minute VER forecasts from the FMM market. |
|  | RealTimeFlexibleObligationQuantity BrtF’S’j’mdh | Real time flexible capacity subject to RAAIM after applying outage and resource specific exemptions. |
|  | RealTimeFlexibleCPMObligationQuantity BrtF’S’j’mdh | Real time flexible CPM capacity subject to RAAIM after applying outage and resource specific exemptions. |
|  | DayAheadResourceGenericRAAIMExemptionFlag rmd | Identifies if a resource’s generic capacity/CPM is exempt in day ahead from RAAIM. |
|  | RealTimeResourceGenericRAAIMExemptionFlag rmd | Identifies if a resource’s generic capacity/CPM is exempt in real time from RAAIM. |
|  | DayAheadResourceFlexibleRAAIMExemptionFlag rmd | Identifies if a resource’s flexible capacity/CPM is exempt in day ahead from RAAIM. |
|  | RealTimeResourceFlexibleRAAIMExemptionFlag rmd | Identifies if a resource’s flexible capacity/CPM is exempt in real time from RAAIM. |
|  | DayAheadGenericRAOutageExemptQuantity BrtF’S’mdh | Portion of generic RA capacity in the day ahead market that is exempt from RAAIM. |
|  | DayAheadGenericCPMOutageExemptQuantity BrtF’S’mdh | Portion of generic CPM capacity in the day ahead market that is exempt from RAAIM. |
|  | DayAheadEligibleGenericExemptOutageQuantity BrtF’S’mdh | Quantity of generic RAAIM eligible to be exempt in the day ahead by comparing exempt outages and generic obligation of the resource. |
|  | DayAheadResourceGenericExemptOutageThreshold BrtF’S’mdh | MW value that qualifies day ahead generic RAAIM obligation to be exempt for generators and tie-generators. This value is calculated by subtracting the hourly max curtailment value of eligible outages from a resource’s p-max. |
|  | DayAheadNRSSResourceGenericExemptOutageThreshold BrtF’S’mdh | MW value that qualifies day ahead generic RAAIM obligation to be exempt for NRSS resources. This value is calculated from the hourly max curtailment value of eligible outages. |
|  | DayAheadFlexibleRAOutageExemptQuantity BrtF’S’j’mdh | Portion of flexible RA capacity in the day ahead market that is exempt from RAAIM. |
|  | DayAheadFlexibleCPMOutageExemptQuantity BrtF’S’j’mdh | Portion of flexible CPM capacity in the day ahead market that is exempt from RAAIM. |
|  | DayAheadEligibleFlexibleExemptOutageQuantity BrtF’S’j’mdh | Quantity of flexible RAAIM eligible to be exempt in the day ahead by comparing exempt outages and flexible obligation of the resource. |
|  | DayAheadResourceFlexibleExemptOutageThreshold BrtF’S’mdh | MW value that qualifies day ahead flexible RAAIM obligation to be exempt for generators and tie-generators. This value is calculated by subtracting the hourly max curtailment value of eligible outages from a resource’s p-max. |
|  | DayAheadNRSSResourceFlexibleExemptOutageThreshold BrtF’S’mdh | MW value that qualifies day ahead flexible RAAIM obligation to be exempt for NRSS resources. This value is calculated from the hourly max curtailment value of eligible outages. |
|  | DayAheadNRSSResourceExemptOutageThreshold BrtF’S’mdh | MW value that qualifies day ahead RAAIM obligation to be exempt for a non-Resource Specfic System Resource. This value is the hourly max curtailment value of eligible outages. |
|  | RealTimeGenericRAOutageExemptQuantity BrtF’S’mdh | Portion of generic RA capacity in the real time market that is exempt from RAAIM. |
|  | RealTimeGenericCPMOutageExemptQuantity BrtF’S’mdh | Portion of generic CPM capacity in the real time market that is exempt from RAAIM. |
|  | RealTimeEligibleGenericExemptOutageQuantity BrtF’S’mdh | Quantity of generic RAAIM eligible to be exempt in the real time market by comparing exempt outages and generic obligation of the resource. |
|  | RealTimeResourceGenericExemptOutageThreshold BrtF’S’mdh | MW value that qualifies real time generic RAAIM obligation to be exempt for generators and tie-generators. This value is calculated by subtracting the hourly max curtailment value of eligible outages from a resource’s p-max. |
|  | RealTimeNRSSResourceGenericExemptOutageThreshold BrtF’S’mdh | MW value that qualifies real time generic RAAIM obligation to be exempt for NRSS resources. This value is calculated from the hourly max curtailment value of eligible outages. |
|  | RealTimeFlexibleRAOutageExemptQuantity BrtF’S’j’mdh | Portion of flexible RA capacity in the real time market that is exempt from RAAIM. |
|  | RealTimeFlexibleCPMOutageExemptQuantity BrtF’S’j’mdh | Portion of flexible CPM capacity in the real time market that is exempt from RAAIM. |
|  | RealTimeEligibleFlexibleExemptOutageQuantity BrtF’S’j’mdh | Quantity of flexible RAAIM eligible to be exempt in the real time market by comparing exempt outages and flexible obligation of the resource. |
|  | RealTimeResourceFlexibleExemptOutageThreshold BrtF’S’mdh | MW value that qualifies real time flexible RAAIM obligation to be exempt for generators and tie-generators. This value is calculated by subtracting the hourly max curtailment value of eligible outages from a resource’s p-max. |
|  | RealTimeNRSSResourceFlexibleExemptOutageThreshold BrtF’S’mdh | MW value that qualifies real time flexible RAAIM obligation to be exempt for NRSS resources. This value is calculated from the hourly max curtailment value of eligible outages. |
|  | RealTimeNRSSResourceExemptOutageThreshold BrtF’S’mdh | MW value that qualifies RAAIM obligation to be exempt for a non-Resource Specfic System Resource. This value is the hourly max curtailment value of eligible outages. |
|  | ResourceLongStartRealTimeRAAIMExemptionFlag rmdh | Flag exempting Long Start Resource from RAAIM obligation in Real Time Market for trade hour h |
|  | ResourceExtremelyLongStartRealTimeRAAIMExemptionFlag rmdh | Flag exempting Extremely Long Start Resource from RAAIM obligation in Real Time Market for trade hour h |
|  | ResourceHourlyRUCAwardQuantity rmdh | Hourly RUC Award Quantity |
|  | ResourceHourlyDayAheadEnergy rmdh | Hourly Day Ahead Energy |
|  | DayAheadAvailableTotalBidQuantity BrtF’S’mdh | Represents the portion of a resource’s day ahead total bids after outages and use limitations that will be used in the RAAIM availability calculation. |
|  | DayAheadAvailableEconomicBidQuantity BrtF’S’mdh | Represents the portion of a resource’s day ahead economic bids after outages and use limitations that will be used in the RAAIM availability calculation. |
|  | DayAheadTotalEnergyBidQuantity BrtF’S’mdh | The MW value of all bids/schedules for a resource that submitted into the day ahead market. |
|  | DayAheadUpperSelfScheduleEnergyBidQuantity BrtF’S’mdh | The MW quantity of self-scheduled bids that a resource submitted for the day ahead market. |
|  | DayAheadEconomicEnergyBidQuantity BrtF’S’mdh | The MW quantity of economic bids that a resource submitted for the day ahead market. |
|  | DayAheadUpperEnergyBidQuantity BrtF’S’mdh | The MW value at the top of a resource’s day ahead energy bid curve. |
|  | DayAheadLowerEnergyBidQuantity BrtF’S’mdh | The MW value at the bottom of a resource’s day ahead energy bid curve. |
|  | DayAheadNGRTotalBidQuantity BrtF’S’mdh | Day ahead bid quantity used for day ahead generic RA availability for an NGR-REM resource |
|  | DayAheadNGRTotalRegUpBidQuantity BrtF’S’mdh | Total of regulation up economic bids and self provision submitted by a NGR-REM resource in day ahead. |
|  | DayAheadNGRTotalRegDownBidQuantity BrtF’S’mdh | Total of regulation down economic bids and self provision submitted by a NGR-REM resource in day ahead. |
|  | DayAheadNGREconomicBidQuantity BrtF’S’mdh | Day ahead bid quantity used for day ahead flexible RA availability for an NGR-REM resource |
|  | DayAheadNGREconomicRegUpBidQuantity BrtF’S’mdh | Total of regulation up economic bids submitted by a NGR-REM resource in day ahead. |
|  | DayAheadNGREconomicRegDownBidQuantity BrtF’S’mdh | Total of regulation down economic bids submitted by a NGR-REM resource in day ahead. |
|  | DayAheadOutageAvailabilityQuantity BrtF’S’mdh | A resource’s operational availability based on outages. |
|  | RealTimeAvailableTotalBidQuantity BrtF’S’mdh | Represents the portion of a resource’s real time total bids after outages and use limitations that will be used in the RAAIM availability calculation. |
|  | RealTimeAvailableEconomicBidQuantity BrtF’S’mdh | Represents the portion of a resource’s real time economic bids after outages and use limitations that will be used in the RAAIM availability calculation. |
|  | RealTimeTotalEnergyBidQuantity BrtF’S’mdh | The MW value of all bids/schedules for a resource that submitted into the real time market. |
|  | RealTimeUpperSelfScheduleEnergyBidQuantity BrtF’S’mdh | The MW quantity of self-scheduled bids that a resource submitted for the real time market. |
|  | RealTimeEconomicEnergyBidQuantity BrtF’S’mdh | The MW quantity of economic bids that a resource submitted for the real time market. |
|  | RealTimeUpperEnergyBidQuantity BrtF’S’mdh | The MW value at the top of a resource’s real time energy bid curve. |
|  | RealTimeLowerEnergyBidQuantity BrtF’S’mdh | The MW value at the bottom of a resource’s real time energy bid curve. |
|  | RealTimeNGRTotalBidQuantity BrtF’S’mdh | Day ahead bid quantity used for day ahead generic RA availability for an NGR-REM resource |
|  | RealTimeNGRTotalRegUpBidQuantity BrtF’S’mdh | Total regulation up economic bids and self provision submitted by a NGR-REM resource in real time. |
|  | RealTimeNGRTotalRegDownBidQuantity BrtF’S’mdh | Total regulation down economic bids and self provision submitted by a NGR-REM resource in real time. |
|  | RealTimeNGREconomicBidQuantity BrtF’S’mdh | Real time bid quantity used for real time flexible RA availability for an NGR-REM resource |
|  | RealTimeNGREconomicRegUpBidQuantity BrtF’S’mdh | Total of regulation up economic bids submitted by a NGR-REM resource in real time. |
|  | RealTimeNGREconomicRegDownBidQuantity BrtF’S’mdh | Total of regulation down economic bids submitted by a NGR-REM resource in real time. |
|  | RealTimeOutageAvailabilityQuantity BrtF’S’mdh | A resource’s operational availability based on outages. |
|  | RealTimeHourlyUpperOperatingLimit BrtF’S’mdh | Represents a resource’s maximum upper operating limit for the hour. |
|  | RealTimeHourlyLowerOperatingLimit BrtF’S’mdh | Represents a resource’s minimum lower operating limit for the hour. |
|  | RAAIMNonAvailabiltyChargePrice m | The "price" value of the Non-Availability Charge is 60% of the CSP soft offer cap. |
|  | MonthlyResourceRAAIMCPMPrice Brtm | The “price” value of the Non-Availability Charge for generic CPM |
|  | MonthlyResourceRAAIMFlexibleCPMPrice Brtm | The “price” value of the Non-Availability Charge for flexible CPM |
|  | DailyResourceRAAIMObligationProrateFactor BrtF’S’md | Ratio defined as DailyRAAIMObligationFlag BrtF’S’md over the total number of days with RAAIM obligation within the trade month |
|  | DailyResourceGenericObligationProrateCount BrtF’S’md | Count of assessment days of generic RA obligation within trade month m, used to calculate RAAIM Obligation proration for resource r |
|  | DailyRAAIMGenericObligationFlag BrtF’S’md | Indicator that for trade date d, the resource has generic RA oblgiation |
|  | DailyResourceFlexibleObligationProrateCount BrtF’S’md | Count of assessment days of flexible RA obligation within trade month m Total number of assessment days of generic obligation within trade month m |
|  | DailyRAAIMFlexibleObligationFlag BrtF’S’j’md | Indicator that for trade date d, the resource has flexible RA oblgiation |
|  | DailyResourceGenericCPMObligationProrateFactor BrtF’S’md | Ratio defined as DailyRAAIMGenericCPMFlag BrtF’S’md over the total number of days with generic CPM capacity within the trade month |
|  | DailyResourceFlexibleCPMObligationProrateFactor BrtF’S’md | Ratio defined as DailyRAAIMFlexibleCPMFlag BrtF’S’md over the total number of days with flexible CPM capacity within the trade month |
|  | DailyRAAIMGenericCPMFlag BrtF’S’md | Indicator for when a resource is assessed RAAIM for generic CPM capacity for trade date d, used to calculate the resource’s monthly average generic CPM quanitity |
|  | DailyRAAIMGenericCPMCount BrtF’S’md | The day count of a resource’s number of hours where it is assessed RAAIM for generic CPM capacity, used to calculate the resource’s day average generic CPM quanitity |
|  | HourlyRAAIMGenericCPMFlag BrtF’S’mdh | Indicator if a resource is assessed RAAIM for generic CPM capacity for given trade hour h |
|  | DailyRAAIMFlexibleCPMFlag BrtF’S’md | Indicator for when a resource is assessed RAAIM for flexible CPM capacity for trade date d, used to calculate the resource’s monthly average flexible CPM quanitity |
|  | DailyRAAIMFlexibleCPMCount BrtF’S’md | The day count of a resource’s number of hours where it is assessed RAAIM for flexible CPM capacity, used to calculate the resource’s day average flexible CPM quanitity |
|  | HourlyRAAIMFlexibleCPMFlag BrtF’S’mdh | Indicator if a resource is assessed RAAIM for flexible CPM capacity for given trade hour h |
|  | DailyRAAIMObligationFlag BrtF’S’md | Indicator for when a resource is assessed RAAIM (excluding generic CPM or flexible CPM) for trade date d, used to calculate the resource’s monthly average RAAIM obligation (capacity without generic CPM or flexible CPM) quantity |
|  | DailyRAAIMObligationCount BrtF’S’md | The day count of the RA resource’s hourly RAAIM obligation (capacity without CPM) hours, used to calculate the resource’s day average RAAIM obligation (capacity without generic CPM or flexible CPM) quantity |
|  | HourlyRAAIMObligationFlag BrtF’S’mdh | Indicator if an RA resource has RAAIM obligation (capacity without generic CPM or flexible CPM) for given trade hour h |
|  | MonthlyAssessDaysGenericObligationCount m | Count of assessment days of generic obligation within trade month m |
|  | DailyAssessGenericObligationFlag md | Indicator for assessment of generic obligation for trade date d |
|  | MonthlyAssessDaysFlexibleObligationCount j’m | Count of assessment days of flexible obligation for flex category j’ within trade month m |
|  | DailyAssessFlexibleObligationFlag j’md | Indicator for assessment of flexible obligation for trade date d |
|  | MonthlyObligationHoursQuantity m | Total number of hours within a given trade month. |
|  | DailyTradeHoursCount mdh | Total number of hours within a given trade day. |
|  | BusinessAssociateMonthlyRAAIMNodalMeteredCAISODemandQty Bm | Monthly Nodal RAAIM CAISO Metered Demand by Business Associate B |

# Charge Code Effective Date

| Charge Code/ Pre-calc Name | Document Version | Effective Start Date | Effective End Date | Version Update Type |
| --- | --- | --- | --- | --- |
| Resource Adequacy Availability Incentive Mechanism Pre-Calculation | 5.0 | 11/1/16 | 10/31/16 | Configuration Impacted |
| Resource Adequacy Availability Incentive Mechanism Pre-Calculation | 5.1 | 11/1/16 | 10/31/16 | Configuration Impacted |
| Resource Adequacy Availability Incentive Mechanism Pre-Calculation | 5.1a | 11/1/16 | 10/31/16 | Documentation Only Update |
| Resource Adequacy Availability Incentive Mechanism Pre-Calculation | 5.2 | 11/1/16 | 10/31/16 | Configuration Impacted |
| Resource Adequacy Availability Incentive Mechanism Pre-Calculation | 5.3 | 11/1/16 | 10/31/17 | Configuration Impacted |
| Resource Adequacy Availability Incentive Mechanism Pre-Calculation | 5.4 | 11/1/17 | 3/31/2018 | Configuration Impacted |
| Resource Adequacy Availability Incentive Mechanism Pre-Calculation | 5.5 | 4/1/18 | Open | Configuration Impacted |