Settlements & Billing

Configuration Guide: CG PC Wheel Export Quantity 5.6

Version 5.6

Table of Contents

1.	Purpose of Document	3

2.	Introduction	3

2.1	Background	3

2.2	Description	3

3.	Charge Code Requirements	3

3.1	Business Rules	3

3.2	Predecessor Charge Codes	5

3.3	Successor Charge Codes	5

3.4	Inputs - External Systems	5

3.5	Inputs - Predecessor Charge Codes or Pre-calculations	6

3.6	CAISO Formula	6

3.7	Outputs	9

4.	Charge Code Effective Dates	12

# Purpose of Document

The purpose of this document is to capture the requirements and design specification for a SaMC Charge Code in one document.

# Introduction

## Background

The Wheeling export quantity pre-calculation calculates quantity for each low or high voltage export transaction at an Intertie in order to collect High Voltage Wheeling Access Charge allocation (CC 382) and Low Voltage Wheeling Access Charge allocation (CC 383).  SCs that schedule Wheeling Out or Wheeling Through transactions to a Scheduling Point or Take-Out Point between the CAISO Controlled Grid and the transmission system of a Non-Participating Transmission Owner (Non-PTO), that is located within the CAISO Balancing Authority Area or another Balancing Authority Area provides the CAISO details of such transactions scheduled by them (other than transactions scheduled pursuant to Existing Contracts).  The CAISO uses such information, which may be subject to review by the CAISO, to settle Wheeling Access Charges (WAC) and payments.

## Description

This calculation outputs the Wheeling export quantity for use in High Voltage Wheeling Access Charge allocation (CC 382) and Low Voltage Wheeling Access Charge  allocation (CC 383).  It considers both scheduled and Real-Time gross exports (excluding amounts exempted due to Existing Transmission Contracts (ETC) or resource specific exemptions) at an Intertie and is calculated daily based on hourly volumes, but aggregated, charged and allocated monthly in successor Charge Codes CC 382 and CC 383.  All properly scheduled ETC, using the appropriate Contract Reference Number (CRN), are exempted from the Wheeling quantity based on exported volumes as are resource specific exemptions.

# Charge Code Requirements

## Business Rules

| Bus Req ID | Business Rule |
| --- | --- |
| 1.0 | WheelExportQuantity is calculated Daily by Business Associate ID B, Intertie ID Q, Resource Type t, PTO ID P, Trading Month m, Trading Day d, and Trading Hour h |
| 2.0 | Non PWT Schedules: WheelExportQuantity is defined as the maximum of either zero (0) or the difference between the BusinessAssociateSettlementIntervalResourceDeemedDeliveredSwapQuantity and NormalizedETCPrecalcSettlementIntervalValueByContractReferenceNumberQuantity. Settlement for Priority-Wheeling-Through (PWT): Entities obtaining PWT pay the Wheeling Access Charges (WAC) for period across which ATC is reserved (i.e. allocated). - If the entity’s actual schedule (aka “usage”) is less than the allocated (i.e. awarded) PWT capacity, then the entity would pay WAC on the allocated capacity, rather than its actual schedule. - If the entity’s actual schedule (aka “usage”) is greater than the allocated (i.e. awarded) PWT capacity, then the entity would pay WAC on actual schedule. |
| 3.0 | ResourceLayoffWheelExportQuantityExceptionFlag rt flags will only be created by RSRC_ID and RSRC_TYPE where exceptions exist and will be populated with an interval value of 1. It is further assumed that RSRC_TYPE (t) will always be ‘ETIE’ |
| 4.0 | NonPTOMeteredLoadExceptionFlag Brt flags will only be created for BA/RSRC_ID combinations where exceptions exist and will be populated with an interval value of 1. It is further assumed that RSRC_TYPE (t) will always be ‘LOAD’ |
| 5.0 | Wheeling quantities for Take-Out Points are either: - Submitted directly to Settlements from the SC via an interface utilizing the Pass Through Bill – calculation methodology. SCs submit values as one aggregated value for the month, per Take-Out Point. Settlements takes the aggregated monthly MW value and allocates it out equally to each Trading Day within the same Trading Month by creating daily PTB bill determinants - Metered and assessed a Wheeling charge against bill determinant BADispatchIntervalResourceNonPTOMeterLoadSubjectToWheelingQuantity |
| 6.0 | The CAISO accommodates daily submission of Wheeling Out and Wheeling Through data at the option of each Non-Participating TO. For the Non-Participating TOs that elect to participate in the early submission of this data, the CAISO will accept the submission of their Load data in the same manner as currently submitted, except on a daily rather than a monthly basis. Once received, the CAISO will manually process the data for Settlement purposes. SCs will need to submit data in accordance with the posted Non-PTO data submission calendar. Any data received after the expected deadline will go into the next scheduled statement. For those Scheduling Coordinators that do not want to submit the data daily, the current monthly process will remain in effect. CAISO Tariff Section 26.1.4.4 sets the deadline for submitting the data at five days after the end of the Trading Month. These submittal deadlines are also specified in the Non-PTO submission calendar. The existing tariff language allows the data to be submitted earlier, but not later than the deadline. |
| 7.0 | Settlements receives both the original purchaser and the Wheeling Priority Quantity as well as the MW resold to purchaser. Each of these MWs will be consumed by settlement but used differently. The Original Wheeling Priority SC should be charged WAC for the Maximum of Wheeling Priority Quantity and scheduled export. While the purchaser of the resale Wheeling Priority Quantity should be assessed WAC on the maximum of schedule export less purchased Wheeling priority MW and zero. |

## Predecessor Charge Codes

Charge Code/ Pre-calc Name

Pre-Calculation / ETC Pre-Calc

System Resource Deemed Delivered Energy Quantity

## Successor Charge Codes

Charge Code/ Pre-calc Name

CC 383 – Low Voltage Wheeling Allocation

CC 382– High Voltage Wheeling Allocation

## Inputs - External Systems

| Row # | Variable Name | Description |
| --- | --- | --- |
| 1 | VoltageLevelIndicator Q | Voltage level indicator by Intertie ID Q |
| 2 | TakeOutPointWheelExportQty BQPJmd | Take Out Point Wheeling export quantity by Business Associate B, Intertie ID Q, PTO ID P, PTB ID J, Trading Month m, Trading Day d |
| 3 | ResourceLayoffWheelExportQuantityExceptionFlag rt | Resource layoff Wheeling export quantity exception flag for Resource ID r, Resource Type t |
| 4 | BADispatchIntervalResourceNonPTOMeterLoadSubjectToWheelingQuantity BrtQPNmdhcif | Metered Non-PTO Load quantity subject to Wheeling charges |
| 5 | NonPTOMeteredLoadExceptionFlag Brt | Non-PTO metered Load exception flag |
| 6 | BAHourlyATCReservationIntertieQty BrtEuT’I’Q’F’M’AA’R’pPW’QS’d’Nz’OVvHn’L’mdh | Required for Settlement of Priority-Wheeling-Through (PWT). The ATC allocated (i.e awarded) at a specific intertie (-) |
| 7 | BAHourlyATCReservationResaleIntertieQty BrtEuT’I’Q’F’M’AA’R’pPW’QS’d’Nz’OVvHn’L’mdh | Reflects the resale of Priority-Wheeling-Through (PWT) MW resold to purchaser. BA ID reflects the purchaser of the resale Wheeling Priority Quantity (-) |

## Inputs - Predecessor Charge Codes or Pre-calculations

| Row # | Variable Name | Predecessor Charge Code/ Pre-calc Configuration |
| --- | --- | --- |
| 1 | BASettlementIntervalFinalBalancedContractAtScheduleQuantity BrtNmdhcif | Pre-Calculation / ETC Pre-Calc |
| 2 | SettlementIntervalDeemedDeliveredInterchangeEnergyQuantity BrtEuT’I’Q’M’AA’F’R’pPW’QS’d’Nz’OVvHn’L’mdhcif | System Resource Deemed Delivered Energy Quantity |
| 3 | BASettlementIntervalFinalBalancedContractHVACMeterQuantity BrtNmdhcif | System Resource Deemed Delivered Energy Quantity |

## CAISO Formula

The ISO formula for Wheeling export quantity by Business Associate ID B, Intertie ID Q, Resource type t, Trading Month m, Trading Day d, and Trading Hour h is as follows:

      - IF

- VoltageLevelIndicator Q = 0

- THEN

- BusinessAssociateDailyIntertieLowVoltageWheelExportQuantity BQmd = tP h WheelExportQuantity BtQPmdh

- ELSE

- BusinessAssociateDailyIntertieLowVoltageWheelExportQuantity BQmd = 0

      - IF

- VoltageLevelIndicator Q = 0

- THEN

BusinessAssociateDailyTakeOutPointLowVoltageWheelExportQuantity BQmd = P (BADayIntertieTOPWheelExportNormalizedPTBQuantity BPQmd + BADayNonPTOTakeOutPointMarketDataExportQtyLessETCQuantity BPQmd )

- ELSE

- BusinessAssociateDailyTakeOutPointLowVoltageWheelExportQuantity BQmd = 0

BusinessAssociateDailyIntertieLowOrHighVoltageWheelExportQuantity BQmd = tPh  WheelExportQuantity BtQPmdh

BusinessAssociateDailyTakeOutPointLowOrHighVoltageWheelExportQuantity BQmd =  P (BADayIntertieTOPWheelExportNormalizedPTBQuantity BPQmd + BADayNonPTOTakeOutPointMarketDataExportQtyLessETCQuantity BPQmd )

#### WheelExportQuantity BtQPmdh = WheelExportExcludingPWTResaleQuantity BtQPmdh + WheelExportPWTResaleQuantity BtQPmdh

#### WheelExportExcludingPWTResaleQuantity BtQPmdh = c∈hf∈crEuT'I'Q'F'M'AA'R'pW'S'd'Nz'OVvHn'L'

min(0,BAHourlyATCReservationIntertieQty BrtEuT’I’Q’F’M’AA’R’pPW’QS’d’Nz’OVvHn’L’mdh,(min (0, (BusinessAssociateSettlementIntervalResourceDeemedDeliveredSwapQuantity BrtEuT’I’Q’F’M’AA’R’pPW’S’Qd’Nz’OVvHn’L’mdhcif - NormalizedETCPrecalcSettlementIntervalValueByContractReferenceNumberQuantity BrtEuT’I’Q’F’M’AA’R’pPW’S’Qd’Nz’OVvHn’L’mdhcif))))

Where Resource Type (t) =”ETIE”

- AND

- ResourceLayoffWheelExportQuantityExceptionFlag rt <> 1

Note1: Exception Flag will only be created for instances where resource specific exemptions exist.

Note2: The following BD will be utilized as an exclusionary business driver to ensure that purchaser of Priority-Wheeling-Through (PWT) MW are not included in the output matrix:

BAHourlyATCReservationResaleIntertieQty BrtEuT’I’Q’F’M’AA’R’pPW’QS’d’Nz’OVvHn’L’mdh

#### WheelExportPWTResaleQuantity BtQPmdh = c∈hf∈crEuT'I'Q'F'M'AA'R'pW'S'd'Nz'OVvHn'L'

min (0, (BusinessAssociateSettlementIntervalResourceDeemedDeliveredSwapQuantity BrtEuT’I’Q’F’M’AA’R’pPW’S’Qd’Nz’OVvHn’L’mdhcif - BAHourlyATCReservationResaleIntertieQty BrtEuT’I’Q’ F’M’AA’R’pPW’QS’d’Nz’OVvHn’L’mdh))

Where Resource Type (t) =”ETIE”

Note: The following BD will be utilized as the sole business driver to ensure that this equation is derived only for the purchaser of the resale Wheeling Priority Quantity:

BAHourlyATCReservationResaleIntertieQty BrtEuT’I’Q’F’M’AA’R’pPW’QS’d’Nz’OVvHn’L’mdh

      - NormalizedETCPrecalcSettlementIntervalValueByContractReferenceNumberQuantity BrtEuT’I’Q’F’M’AA’R’pPW’S’Qd’Nz’OVvHn’L’mdhcif =

- BASettlementIntervalFinalBalancedContractAtScheduleQuantity BrtNmdhcif

- (BusinessAssociateSettlementIntervalResourceDeemedDeliveredSwapQuantity BrtEuT’I’Q’F’M’AA’R’pPW’S’Qd’Nz’OVvHn’L’mdhcif will not appear in the formula but will be used to build the matrix with required attributes)

- Where Resource Type (t) =”ETIE”

      - BusinessAssociateSettlementIntervalResourceDeemedDeliveredSwapQuantity BrtEuT’I’Q’F’M’AA’R’pPW’S’Qd’Nz’OVvHn’L’mdhcif = ATTRIBUTESWAP(SettlementIntervalDeemedDeliveredInterchangeEnergyQuantity BrtEuT’I’Q’F’M’AA’R’pPW’S’Qd’Nz’OVvHn’L’mdhcif)

- Where Q’ =”CISO”

- NOTE: The attributeswap function will be utilized in order to swap the PTO_ID attribute position on the input over to the PTO_ID attribute position on the output.

      - BADayIntertieTOPWheelExportNormalizedPTBQuantity BPQmd = J TakeOutPointWheelExportQty BQPJmd

      - BADayNonPTOTakeOutPointMarketDataExportQtyLessETCQuantity BPQmd = hc∈h f∈c BASettlementIntervalNonPTOTakeOutPointMarketDataExportQtyLessETCQuantity BPQmdhcif

      - BASettlementIntervalNonPTOTakeOutPointMarketDataExportQtyLessETCQuantity BPQmdhcif = rtNMin (0, BADispatchIntervalResourceNonPTOMeterLoadSubjectToWheelingQuantity BrtQPNmdhcif - BASettlementIntervalFinalBalancedContractHVACMeterQuantity BrtNmdhcif )

- Where

- NonPTOMeteredLoadExceptionFlag Brt <> 1

## Outputs

| Output ID | Name | Description |
| --- | --- | --- |
| 1 | In addition to any outputs listed below, all inputs shall be included as outputs. |  |
| 2 | NormalizedETCPrecalcSettlementIntervalValueByContractReferenceNumberQuantity BrtEuT’I’Q’F’M’AA’R’pPW’S’Qd’Nz’OVvHn’L’mdhcif | Normalized ETC pre-calculation By utilizing the deemed delivered quantity input as business driver, the ETC PC input quantity will be associated with all of the deemed delivered quantity attributes that it does not already carry.This way, when the ETC quantity is later subtracted from the deemed delivered quantity, they will share the same attribute set |
| 3 | BusinessAssociateSettlementIntervalResourceDeemedDeliveredSwapQuantity BrtEuT’I’Q’F’M’AA’R’pPW’S’Qd’Nz’OVvHn’L’mdhcif | Business Associate Dispatch Interval resource deemed delivered swap quantity This formula exists to swap the PTO_ID which the input from the System Resource PC carries at attribute position 28 to attribute position 20 where the Charge Group associated with this PC carries PTO_ID |
| 4 | WheelExportQuantity BtQPmdh | Summation of the Wheeling quantities that will be assessed a Wheeling charge at the Intertie (not Take-Out Point) |
| 5 | BusinessAssociateDailyIntertieLowVoltageWheelExportQuantity BQmd | The Wheeling quantity that will be assessed charges at a specific low voltage Intertie. Per BA |
| 6 | BusinessAssociateDailyTakeOutPointLowVoltageWheelExportQuantity BQmd | The Wheeling quantity that will be assessed charges at a specific low voltage Take-Out Point. Per BA |
| 7 | BusinessAssociateDailyIntertieLowOrHighVoltageWheelExportQuantity BQmd | Total Wheeling quantity per BA that will be assessed a Wheeling charge at either high or low voltage Intertie (not Take-Out Point) |
| 8 | BusinessAssociateDailyTakeOutPointLowOrHighVoltageWheelExportQuantity BQmd | Total Wheeling quantity per BA that will be assessed a Wheeling charge at either high or low voltage Take-Out Point |
| 9 | BADayIntertieTOPWheelExportNormalizedPTBQuantity BPQmd | This formula processes the manually submitted PTB Non-PTO Wheeling quantity. This formula exists to sum over the PTB_ID as this attribute will conflict with the attribute set of the metered Non-PTO Wheeling quantity which does not carry PTB_ID. These two values will be summed in a later calculation. They actually never do 'sum' as they are both mutually exclusive. The 'sum' exists as a means to pass one or the other out to the output |
| 10 | BADayNonPTOTakeOutPointMarketDataExportQtyLessETCQuantity BPQmd | This formula exists to roll up the 5 minute Non-PTO Wheeling export quantity for the day |
| 11 | BASettlementIntervalNonPTOTakeOutPointMarketDataExportQtyLessETCQuantity BPQmdhcif | This formula subtracts Non-PTO ETC quantities from metered Non-PTO Load quantities. Since the metered Non-PTO Load quantity bill determinant is selected as the business driver, this ensures that the matrix will only be built for Non PTO Load quantities and not Interties. As a result, only ETCs tied to these same resources will be evaluated in the equation.Since both inputs are negative, the 'Min' function is used to ensure that a 'positive' value is never passed to the output.The exception flag excludes Non-PTO Loads with qualified exemptions. |
| 12 | WheelExportPWTResaleQuantity BtQPmdh | The purchaser of the resale Wheeling Priority Quantity will be assessed WAC on the maximum of schedule export less purchased Wheeling priority MW and zero. However, since all inputs are (-), the Min function will achieve the results required of the requirement. |
| 13 | WheelExportExcludingPWTResaleQuantity BtQPmdh | The maximum of the ATC Reservation Quantity or the Deemed delivered quantity minus ETC quantity results in the Wheeling quantity that will be assessed a Wheeling charge at the Intertie (not Take-Out Point) However, since all inputs are (-), the Min function will achieve the results required of the requirement. |

# Charge Code Effective Dates

| Charge Code/ Pre-calc Name | Document Version | Effective Start Date | Effective End Date | Version Update Type |
| --- | --- | --- | --- | --- |
| CG PC Wheel Export Quantity | 5.0 | 04/01/09 | 4/30/10 | Documentation Edits Only |
| CG PC Wheel Export Quantity | 5.1 | 05/01/2010 | 03/31/09 | Configuration and Documentation Edits |
| CG PC Wheel Export Quantity | 5.2 | 04/01/09 | 12/31/10 | Configuration and Documentation Edits |
| CG PC Wheel Export Quantity | 5.3 | 1/1/2011 | 4/30/2014 | Configuration and Documentation Edits |
| CG PC Wheel Export Quantity | 5.4 | 5/1/2014 | 10/31/2014 | Configuration and Documentation Edits |
| CG PC Wheel Export Quantity | 5.5 | 11/1/2014 | 06/30/2024 | Configuration Impacted |
| CG PC Wheel Export Quantity | 5.6 | 07/01/2024 | Open | Configuration Impacted |