Settlements & Billing

GMC Bid Transaction Fee

CC 4515

Version 5.0

Table of Contents

1.	Purpose of Document	3

2.	Introduction	3

2.1	Background	3

2.2	Description	3

3.	Charge Code Requirements	4

3.1	Business Rules	4

3.2	Predecessor Charge Codes	4

3.3	Successor Charge Codes	5

3.4	Inputs - External Systems	5

3.5	Inputs - Predecessor Charge Codes or Pre-calculations	9

3.6	CAISO Formula	9

3.7	Outputs	14

4.	Charge Code Effective Dates	18

# Purpose of Document

The purpose of this document is to capture the requirements and design specification for a Charge Code in one document.

# Introduction

## Background

The Grid Management Charge Codes (GMC) are comprised of daily and monthly charges which are assessed to participating Scheduling Coordinators (SC) for the purpose of recovering all of the CAISO’s direct and indirect operating costs.  The costs are comprised of CAISO Operating Costs, CAISO Other Costs and Revenues, CAISO Financial Costs, CAISO Operating Reserve Credit, and CAISO Out-of-Pocket Capital and Project Costs.

Through the GMC stakeholder process, the CAISO has identified three cost service based Charge Codes, a fixed Transmission Ownership Rights Charge Code, as well as four transactional and administrative Charge Codes.  The cost service Charge Codes consist of (1) Market Services Charge; (2) System Operations Charge; and (3) CRR Services Charge.  The four transactional fee consist of (1) Bid Segment Fee; (2) CRR Transaction Fee; (3) Inter-Scheduling Coordinator Trade Transaction Fee; and (4) Scheduling Coordinator ID Charge.

## Description

In order to participate in ISO Energy Markets, Scheduling Coordinators (SCs) must submit Energy (Physical or Virtual), or Ancillary Services Bids.  The Energy Bids comprise of Economic supply or Demand Bids and Self-Schedules.  Virtual Bids comprise Economic Supply or Demand Bids only.  Ancillary Service Bids comprise Economic Bids or Self-Provisions.  All these Bids, Self-Schedules, and Self-Provisions are submitted to the ISO through the Scheduling Infrastructure and Business Rule system (SIBR).  The SIBR system provides the Scheduling Coordinators an interface for submitting Bids, Self-Schedules, and Self-Provisions.  SIBR also enables ISO systems to process and validate these Bids for correctness based upon business rules.  The ISO then utilizes the resulting final Clean Bids, Self-Schedules, and Self-Provisions to manage the source/Demand balance of the electric system within reliability constraints for the Day-Ahead Market (DAM) and Real-Time Market (RTM).  During the execution of DAM and RTM, the ISO processes a significant number of bids which do not clear the spot markets and receive Energy (Physical or Virtual), or Ancillary Service Awards. Processing these spot market bids has an impact on the ISO costs and system performance that is directly proportional to the total volume of bid segments submitted, and system performance can be severely degraded as bid segment volumes approach the system limits.  The Bid Segment Transaction Fee is designed to recover a portion of the costs the ISO incurs during the execution of the Day Ahead and Real Time markets. As such, this charge code will apply a $0.005 fee per bid segment to each scheduling coordinator resource’s final Clean Bids, Self-Schedules and Self-Provisions.  In addition, the credit amounts recovered through this charge code will be applied against the revenue requirements for Market Service Charges (CC 4560).

# Charge Code Requirements

## Business Rules

| Bus Req ID | Business Rule |
| --- | --- |
|  | The Bid Segment Transaction Fee will apply a $0.005 fee per bid segment to each SC resource’s final Clean Bids, Self -Schedules and Self-Provisions. |
|  | The Bid Segment Transaction Fee will apply a $0.005 fee per bid segment to each an SC resource’s Economic Energy Bids and Self-Schedule |
|  | The Bid Segment Transaction Fee will apply a $0.005 fee per bid segment to each an SC resource’s Economic Virtual Bids. |
|  | The Bid Segment Transaction Fee will apply a $0.005 fee per bid segment to each an SC resource’s Economic Ancillary Services Bids and Self Provision per Ancillary Services product type (Regulation Up, Regulation Down, Spin, and Non-Spin). |
|  | The Bid Segment Transaction Fee will NOT apply a $0.005 fee per bid segment to each an SC resource’s Residual Unit Commitment Capacity Bids. |
|  | The Bid Segment Transaction quantities shall be calculated based upon total bid segments of submitted bids per relevant product type and relevant market. |
|  | The Bid Segment Transaction quantities shall be based solely upon the final Clean Bids product type and relevant market. |
|  | Each component rate of the Grid Management Charge will be adjusted automatically on a quarterly basis, up or down, so that rates reflect the annual revenue requirement as posted on the CAISO Website. (Fact) |
| 4.0 | This Charge Code shall provide an output on a daily basis. |
| 5.0 | This Charge Code shall be billed in accordance to CAISO Payments Calendar. |
| 6.0 | A settlement details file shall provide details for each daily settlement amount. |
| 7.0 | For adjustments to the Charge Code that cannot be accomplished by correction of upstream data inputs, recalculation or operator override, Pass Through Bill Charge adjustment shall be applied. |

## Predecessor Charge Codes

Charge Code/ Pre-Calc Name

None

## Successor Charge Codes

Charge Code/ Pre-calc Name

None

## Inputs - External Systems

| Row # | A | Description |
| --- | --- | --- |
| 1 | CAISOGMCBidSegmentFee md | FERC-approved Bid Segment GMC Transaction Fee. The input data is available from the SaMC standing data based on applicable start and end dates. |
| 2 | BAHourlyDAVirtualBidSegSizeQuantity BbA’aj’mdh | The input represents the quantity of Energy offered in association with Business Associate B, bid segment b, Aggregated PNode type A’, bid type a, Eligible PNode/Aggregated PNode j’, and Trading Hour h. (MW) |
| 3 | PTBChargeAdjustmentGMCBidSegmentSettlementAmount BJmd | PTB adjustment variable for this Charge Code, with amount per SC (in $). The input applies to Business Associate B, Trading Month m and Trading Day d. |
| 4 | GMCBidSegmentExclusionFlag B | Flag indicating CRR Services exception from Charge Code, where exception is represented by “1”. |
| 5 | BAHourlyResDAMEnergySelfScheduleBidQty BrtubAA’pamdh | The input represents the Hourly Day Ahead Market Self Schedule Bid Quantity by Business Associate B for Resource ID r of Resource Type t, UDC ID u, bid segment b, APN_ID A, APN Type A’, Price Node ID p, bid type a of Trading Month m, Trading Day d, and Trading Hour h. The bid segment number for Self Schedule Qty will be set to “0” |
| 6 | BAHourlyResDAMEnergyBidQty BrtubAA’pmdh | The input represents the Day Ahead Market Energy Bid quantity (in MWh) for bid segment b, as submitted by Business Associate B for Resource ID r of Resource Type t, UDC ID u, APN_ID A, APN Type A’, Price Node ID p of Trading Month m, Trading Day d, and Trading Hour h. |
| 7 | BAHourlyResRTMEnergySelfScheduleBidQty BrtubAA’pamdh | The input represents the Hourly Real Time Market Energy Self Schedule Bid Quantity by Business Associate B for Resource ID r of Resource Type t, UDC ID u, bid segment b, APN_ID A, APN Type A’, Price Node ID p, bid type a of Trading Month m, Trading Day d, and Trading Hour h. The bid segment number for Self Schedule Qty will be set to “0” |
| 8 | BAHourlyResRTMEnergyBidQty BrtubAA’pmdh | The input represents the Real Time Market Energy Bid quantity (in MWh) for bid segment b, as submitted by Business Associate B for Resource ID r of Resource Type t, UDC ID u, APN_ID A, APN Type A’, Price Node ID p of Trading Month m, Trading Day d, and Trading Hour h. |
| 9 | BAHourlyResDAMSpinSelfProvisionBidQty Brtbmdh | The input represents the Hourly Day Ahead Energy Spin Self Provision Bid Quantity (MW) as submitted by Business Associate B for Resource ID r, Resource Type t, bid segment b of Trading Month m, Trading Day d, and Trading Hour h. The bid segment number for Self Provision Qty will be set to “0” |
| 10 | BAHourlyResDAMSpinBidQty Brtbmdh | The input represents the Day Ahead Market Spin Capacity Bid quantity (MW) as submitted by Business Associate B for Resource ID r, Resource Type t, bid segment b of Trading Month m, Trading Day d, and Trading Hour h. |
| 11 | BAHourlyResRTMSpinSelfProvisionBidQty Brtbmdh | The input represents the Hourly Real Time Market Spin Self Provision Bid Quantity as submitted by Business Associate B for Resource ID r, Resource Type t, bid segment b of Trading Month m, Trading Day d, and Trading Hour h. The bid segment number for Self Provision Qty will be set to “0” |
| 12 | BAHourlyResRTMSpinBidQty Brtbmdh | The input represents the Hourly Real Time Market Spin Capacity Bid quantity (in MW) as submitted by Business Associate B for Resource ID r, Resource Type t, bid segment b of Trading Month m, Trading Day d, and Trading Hour h. |
| 13 | BAHourlyResDAMNonSpinSelfProvisionBidQty Brtbmdh | The input represents the Hourly Day Ahead Energy Non-Spin Self Provision Bid Quantity (MW) as submitted by Business Associate B for Resource ID r, Resource Type t, bid segment b of Trading Month m, Trading Day d, and Trading Hour h. The bid segment number for Self Provision Qty will be set to “0” |
| 14 | BAHourlyResDAMNonSpinBidQty Brtbmdh | The input represents the Day Ahead Market Non-Spin Capacity Bid quantity (MW) as submitted by Business Associate B for Resource ID r, Resource Type t, bid segment b of Trading Month m, Trading Day d, and Trading Hour h. |
| 15 | BAHourlyResRTMNonSpinSelfProvisionBidQty Brtbmdh | The input represents the Hourly Real Time Market Non-Spin Self Provision Bid Quantity as submitted by Business Associate B for Resource ID r, Resource Type t, bid segment b of Trading Month m, Trading Day d, and Trading Hour h. The bid segment number for Self Provision Qty will be set to “0” |
| 16 | BAHourlyResRTMNonSpinBidQty Brtbmdh | The input represents the Hourly Real Time Market Non-Spin Capacity Bid quantity (in MW) as submitted by Business Associate B for Resource ID r, Resource Type t, bid segment b of Trading Month m, Trading Day d, and Trading Hour h. |
| 17 | BAHourlyResDAMRegUpSelfProvisionBidQty Brtbmdh | The input represents the Hourly Day Ahead Regulation Up Capacity Self Provision Bid quantity (MW) as submitted by Business Associate B for Resource ID r, Resource Type t, bid segment b of Trading Month m, Trading Day d, and Trading Hour h. The bid segment number for Self Provision Qty will be set to “0” |
| 18 | BAHourlyResDAMRegUpBidQty Brtbmdh | The input represents the Day Ahead Market Regulation Up Capacity Bid quantity (MW) as submitted by Business Associate B for Resource ID r, Resource Type t, bid segment b of Trading Month m, Trading Day d, and Trading Hour h. |
| 19 | BAHourlyResRTMRegUpSelfProvisionBidQty Brtbmdh | The input represents the Hourly Real Time Market Regulation Up Capacity Self Provision Bid Quantity (MW) as submitted by Business Associate B for Resource ID r, Resource Type t, bid segment b of Trading Month m, Trading Day d, and Trading Hour h. The bid segment number for Self Provision Qty will be set to “0” |
| 20 | BAHourlyResRTMRegUpBidQty Brtbmdh | The input represents the Hourly Real Time Market Regulation Up Capacity Bid quantity (in MW) as submitted by Business Associate B for Resource ID r, Resource Type t, bid segment b of Trading Month m, Trading Day d, and Trading Hour h. |
| 21 | BAHourlyResDAMRegDownSelfProvisionBidQty Brtbmdh | The input represents the Hourly Day Ahead Energy Regulation Down Capacity Self Provision Bid Quantity (MW) as submitted by Business Associate B for Resource ID r, Resource Type t, bid segment b of Trading Month m, Trading Day d, and Trading Hour h. The bid segment number for Self Provision Qty will be set to “0” |
| 22 | BAHourlyResDAMRegDownBidQty Brtbmdh | The input represents the Day Ahead Market Regulation Down Capacity Bid quantity (MW) as submitted by Business Associate B for Resource ID r, Resource Type t, bid segment b of Trading Month m, Trading Day d, and Trading Hour h. |
| 23 | BAHourlyResRTMRegDownSelfProvisionBidQty Brtbmdh | The input represents the Hourly Real Time Market Regulation Down Self Provision Capacity Bid Quantity (MW) as submitted by Business Associate B for Resource ID r, Resource Type t, bid segment b of Trading Month m, Trading Day d, and Trading Hour h. The bid segment number for Self Provision Qty will be set to “0” |
| 24 | BAHourlyResRTMRegDownBidQty Brtbmdh | The input represents the Hourly Real Time Market Regulation Down Capacity Bid quantity (in MW) as submitted by Business Associate B for Resource ID r, Resource Type t, bid segment b of Trading Month m, Trading Day d, and Trading Hour h. |

## Inputs - Predecessor Charge Codes or Pre-calculations

| Row # | Variable Name | Predecessor Charge Code/ Pre-calc Configuration / Description |
| --- | --- | --- |
| 1 | None |  |

## CAISO Formula

The daily settlement System Operations Grid Management Charge by each Business Associate is derived according to the formulation below.

### BADailyBidSegmentFeeAmount Bmd = BADailyBidSegmentFeeCount Bmd * CAISOGMCBidSegmentFee md

### IF

### GMCBidSegmentExclusionFlag B =  1

### THEN

### BADailyBidSegmentFeeCount Bmd = 0

### ELSE

### BADailyBidSegmentFeeCount Bmd = h(BAHourlyTotalEnergyBidCount Bmdh + BAHourlyAncillaryServicesBidCount Bmdh + BAHourlyVirtualBidCount Bmdh)

### BAHourlyTotalEnergyBidCount Bmdh = rtubAA'p (BAHourlyResDAMEnergyBidCount BrtubAA’pmdh + BAHourlyTotalResDAMEnergySelfScheduleBidCount BrtubAA’pmdh + BAHourlyResRTMEnergyBidCount BrtubAA’pmdh + BAHourlyTotalResRTMEnergySelfScheduleBidCount BrtubAA’pmdh)

### BAHourlyTotalResDAMEnergySelfScheduleBidCount BrtubAA’pmdh = aBAHourlyResDAMEnergySelfScheduleBidCount BrtubAA’pamdh

### IF

### BAHourlyResDAMEnergyBidQty BrtubAA’pmdh = 0

### THEN

### BAHourlyResDAMEnergyBidCount BrtubAA’pmdh = 0

### ELSE

### BAHourlyResDAMEnergyBidCount BrtubAA’pmdh = 1

### IF

### BAHourlyResDAMEnergySelfScheduleBidQty BrtubAA’pamdh = 0

### THEN

### BAHourlyResDAMEnergySelfScheduleBidCount BrtubAA’pamdh = 0

### ELSE

### BAHourlyResDAMEnergySelfScheduleBidCount BrtubAA’pamdh = 1

### IF

### BAHourlyResRTMEnergyBidQty BrtubAA’pmdh = 0

### THEN

### BAHourlyResRTMEnergyBidCount BrtubAA’pmdh = 0

### ELSE

### BAHourlyResRTMEnergyBidCount BrtubAA’pmdh = 1

### BAHourlyTotalResRTMEnergySelfScheduleBidCount BrtubAA’pmdh = a BAHourlyResRTMEnergySelfScheduleBidCount BrtubAA’pamdh

### IF

### BAHourlyResRTMEnergySelfScheduleBidQty BrtubAA’pamdh = 0

### THEN

### BAHourlyResRTMEnergySelfScheduleBidCount BrtubAA’pamdh = 0

### ELSE

### BAHourlyResRTMEnergySelfScheduleBidCount BrtubAA’pamdh = 1

### BAHourlyAncillaryServicesBidCount Bmdh = rtb (BAHourlyResDAMSpinSelfProvisionCount Brtbmd + BAHourlyResDAMSpinBidCount Brtbmdh + BAHourlyResRTMSpinSelfProvisionCount Brtbmdh + BAHourlyResRTMSpinBidCount Brtbmdh + BAHourlyResDAMNonSpinSelfProvisionCount Brtbmd + BAHourlyResDAMNonSpinBidCount Brtbmdh + BAHourlyResRTMNonSpinSelfProvisionCount Brtbmdh + BAHourlyResRTMNonSpinBidCount Brtbmdh + BAHourlyResDAMRegUpSelfProvisionCount Brtbmd + BAHourlyResDAMRegUpBidCount Brtbmdh + BAHourlyResRTMRegUpSelfProvisionCount Brtbmdh + BAHourlyResRTMRegUpBidCount Brtbmdh + BAHourlyResDAMRegDownSelfProvisionCount Brtbmd + BAHourlyResDAMRegDownBidCount Brtbmdh + BAHourlyResRTMRegDownSelfProvisionCount Brtbmdh + BAHourlyResRTMRegDownBidCount Brtbmdh )

### IF

### BAHourlyResDAMSpinSelfProvisionBidQty Brtbmdh = 0

### THEN

### BAHourlyResDAMSpinSelfProvisionCount Brtbmdh = 0

### ELSE

### BAHourlyResDAMSpinSelfProvisionCount Brtbmdh = 1

### IF

### BAHourlyResDAMSpinBidQty Brtbmdh = 0

### THEN

### BAHourlyResDAMSpinBidCount Brtbmdh = 0

### ELSE

### BAHourlyResDAMSpinBidCount Brtbmdh = 1

### IF

### BAHourlyResRTMSpinSelfProvisionBidQty Brtbmdh = 0

### THEN

### BAHourlyResRTMSpinSelfProvisionCount Brtbmdh = 0

### ELSE

### BAHourlyResRTMSpinSelfProvisionCount Brtbmdh = 1

### IF

### BAHourlyResRTMSpinBidQty Brtbmdh = 0

### THEN

### BAHourlyResRTMSpinBidCount Brtbmdh = 0

### ELSE

### BAHourlyResRTMSpinBidCount Brtbmdh = 1

### IF

### BAHourlyResDAMNonSpinSelfProvisionBidQty Brtbmdh = 0

### THEN

### BAHourlyResDAMNonSpinSelfProvisionCount Brtbmdh = 0

### ELSE

### BAHourlyResDAMNonSpinSelfProvisionCount Brtbmdh = 1

### IF

### BAHourlyResDAMNonSpinBidQty Brtbmdh = 0

### THEN

### BAHourlyResDAMNonSpinBidCount Brtbmdh = 0

### ELSE

### BAHourlyResDAMNonSpinBidCount Brtbmdh = 1

### IF

### BAHourlyResRTMNonSpinSelfProvisionBidQty Brtbmdh = 0

### THEN

### BAHourlyResRTMNonSpinSelfProvisionCount Brtbmdh = 0

### ELSE

### BAHourlyResRTMNonSpinSelfProvisionCount Brtbmdh = 1

### IF

### BAHourlyResRTMNonSpinBidQty Brtbmdh = 0

### THEN

### BAHourlyResRTMNonSpinBidCount Brtbmdh = 0

### ELSE

### BAHourlyResRTMNonSpinBidCount Brtbmdh = 1

### IF

### BAHourlyResDAMRegUpSelfProvisionBidQty Brtbmdh = 0

### THEN

### BAHourlyResDAMRegUpSelfProvisionCount Brtbmdh = 0

### ELSE

### BAHourlyResDAMRegUpSelfProvisionCount Brtbmdh = 1

### IF

### BAHourlyResDAMRegUpBidQty Brtbmdh = 0

### THEN

### BAHourlyResDAMRegUpBidCount Brtbmdh = 0

### ELSE

### BAHourlyResDAMRegUpBidCount Brtbmdh = 1

### IF

### BAHourlyResRTMRegUpSelfProvisionBidQty Brtbmdh = 0

### THEN

### BAHourlyResRTMRegUpSelfProvisionCount Brtbmdh = 0

### ELSE

### BAHourlyResRTMRegUpSelfProvisionCount Brtbmdh = 1

### IF

### BAHourlyResRTMRegUpBidQty Brtbmdh = 0

### THEN

### BAHourlyResRTMRegUpBidCount Brtbmdh = 0

### ELSE

### BAHourlyResRTMRegUpBidCount Brtbmdh = 1

### IF

### BAHourlyResDAMRegDownSelfProvisionBidQty Brtbmdh = 0

### THEN

### BAHourlyResDAMRegDownSelfProvisionCount Brtbmdh = 0

### ELSE

### BAHourlyResDAMRegDownSelfProvisionCount Brtbmdh = 1

### IF

### BAHourlyResDAMRegDownBidQty Brtbmdh = 0

### THEN

### BAHourlyResDAMRegDownBidCount Brtbmdh = 0

### ELSE

### BAHourlyResDAMRegDownBidCount Brtbmdh = 1

### IF

### BAHourlyResRTMRegDownSelfProvisionBidQty Brtbmdh = 0

### THEN

### BAHourlyResRTMRegDownSelfProvisionCount Brtbmdh = 0

### ELSE

### BAHourlyResRTMRegDownSelfProvisionCount Brtbmdh = 1

### IF

### BAHourlyResRTMRegDownBidQty Brtbmdh = 0

### THEN

### BAHourlyResRTMRegDownBidCount Brtbmdh = 0

### ELSE

### BAHourlyResRTMRegDownBidCount Brtbmdh = 1

### BAHourlyVirtualBidCount Bmdh = bA'aj'BAHourlyDAVirtualBidSegSizeQuantityCount BbA’aj’mdh

### IF

### BAHourlyDAVirtualBidSegSizeQty BbA’aj’mdh = 0

### THEN

### BAHourlyDAVirtualBidSegSizeQuantityCount BbA’aj’mdh = 0

### ELSE

### BAHourlyDAVirtualBidSegSizeQuantityCount BbA’aj’mdh = 1

## Outputs

| Output Req ID | Name | Description |
| --- | --- | --- |
| 1 | In addition to any outputs listed below, all inputs shall be included as outputs. | All inputs |
| 2 | BAHourlyDAVirtualBidSegSizeQuantityCount BbA’aj’mdh | Hourly Virtual Bid Segment Count by Business Associate B, Bid Segment b, Aggregated PNode Type A’, Bid Type a, Financial Node j’ of Trading Month m, Trading Day d, and Trade Hour h. |
| 3 | BAHourlyVirtualBidCount Bmdh | The Total hourly Virtual Bid Count for Business Associate B, of Trading Month m, Trading Day d, and Trade Hour h. |
| 4 | BAHourlyResRTMRegDownBidCount Brtbmdh | Hourly Real Time Market Regulation Down Bid Count of Business Associate B, Resource Id r, Resource Type t, Bid Segment b of Trading Month m, Trading Day d, and Trade Hour h. |
| 5 | BAHourlyResRTMRegDownSelfProvisionCount Brtbmdh | Hourly Real Time Market Regulation Down Self Provision Count of Business Associate B, Resource Id r, Resource Type t, Bid Segment b of Trading Month m, Trading Day d, and Trade Hour h. |
| 6 | BAHourlyResDAMRegDownBidCount Brtbmdh | Hourly Day Ahead Market Regulation Down Bid Count of Business Associate B, Resource Id r, Resource Type t, Bid Segment b of Trading Month m, Trading Day d, and Trade Hour h. |
| 7 | BAHourlyResDAMRegDownSelfProvisionCount Brtbmdh | Hourly Day Ahead Market Regulation Down Self Provision Count of Business Associate B, Resource Id r, Resource Type t, Bid Segment b of Trading Month m, Trading Day d, and Trade Hour h. |
| 8 | BAHourlyResRTMRegUpBidCount Brtbmdh | Hourly Real Time Market Regulation Up Bid Count of Business Associate B, Resource Id r, Resource Type t, Bid Segment b of Trading Month m, Trading Day d, and Trade Hour h. |
| 9 | BAHourlyResRTMRegUpSelfProvisionCount Brtbmdh | Hourly Real Time Market Regulation Up Self Provision Count of Business Associate B, Resource Id r, Resource Type t, Bid Segment b of Trading Month m, Trading Day d, and Trade Hour h. |
| 10 | BAHourlyResDAMRegUpBidCount Brtbmdh | Hourly Day Ahead Market Regulation Up Bid Count of Business Associate B, Resource Id r, Resource Type t, Bid Segment b of Trading Month m, Trading Day d, and Trade Hour h. |
| 11 | BAHourlyResDAMRegUpSelfProvisionCount Brtbmdh | Hourly Day Ahead Market Regulation Up Self Provision Count of Business Associate B, Resource Id r, Resource Type t, Bid Segment b of Trading Month m, Trading Day d, and Trade Hour h. |
| 12 | BAHourlyResRTMNonSpinBidCount Brtbmdh | Hourly Real Time Market Non-Spin Bid Count of Business Associate B, Resource Id r, Resource Type t, Bid Segment b of Trading Month m, Trading Day d, and Trade Hour h. |
| 13 | BAHourlyResRTMNonSpinSelfProvisionCount Brtbmdh | Hourly Real Time Market Non-Spin Self Provision Count of Business Associate B, Resource Id r, Resource Type t, Bid Segment b of Trading Month m, Trading Day d, and Trade Hour h. |
| 14 | BAHourlyResDAMNonSpinBidCount Brtbmdh | Hourly Day Ahead Market Non-Spin Bid Count of Business Associate B, Resource Id r, Resource Type t, Bid Segment b of Trading Month m, Trading Day d, and Trade Hour h. |
| 15 | BAHourlyResDAMNonSpinSelfProvisionCount Brtbmdh | Hourly Day Ahead Market Non-Spin Self Provision Count of Business Associate B, Resource Id r, Resource Type t, Bid Segment b of Trading Month m, Trading Day d, and Trade Hour h. |
| 16 | BAHourlyResRTMSpinBidCount Brtbmdh | Hourly Real Time Market Spin Bid Count of Business Associate B, Resource Id r, Resource Type t, Bid Segment b of Trading Month m, Trading Day d, and Trade Hour h. |
| 17 | BAHourlyResRTMSpinSelfProvisionCount Brtbmdh | Hourly Real Time Market Spin Self Provision Count of Business Associate B, Resource Id r, Resource Type t, Bid Segment b of Trading Month m, Trading Day d, and Trade Hour h. |
| 18 | BAHourlyResDAMSpinBidCount Brtbmdh | Hourly Day Ahead Market Spin Bid Count of Business Associate B, Resource Id r, Resource Type t, Bid Segment b of Trading Month m, Trading Day d, and Trade Hour h. |
| 19 | BAHourlyResDAMSpinSelfProvisionCount Brtbmdh | Hourly Day Ahead Market Spin Self Provision Count of Business Associate B, Resource Id r, Resource Type t, Bid Segment b of Trading Month m, Trading Day d, and Trade Hour h. |
| 20 | BAHourlyAncillaryServicesBidCount Bmdh | Total Hourly Ancillary Services Bid Count by Business Associate B, of Trading Month m, Trading Day d, and Trade Hour h. |
| 21 | BAHourlyResRTMEnergySelfScheduleBidCount BrtubAA’pamdh | The Hourly Real Time Market Energy Self Schedule Count by Business Associate B, Resource ID r, Resource Type t, UDC ID u, Bid Segment b, APN_ID A, APN Type A’, Price Node ID p, Bid type a of Trading Month m, Trading Day d, and Trading Hour h. |
| 22 | BAHourlyTotalResRTMEnergySelfScheduleBidCount BrtubAA’pmdh | The Total Hourly Real Time Market Energy Self Schedule Count by Business Associate B, Resource ID r, Resource Type t, UDC ID u, Bid Segment b, APN_ID A, APN Type A’, Price Node ID p of Trading Month m, Trading Day d, and Trading Hour h. |
| 23 | BAHourlyResRTMEnergyBidCount BrtubAA’pmdh | The Hourly Real Time Market Energy Bid Count by Business Associate B, Resource ID r, Resource Type t, UDC ID u, Bid Segment b, APN_ID A, APN Type A’, Price Node ID p of Trading Month m, Trading Day d, and Trading Hour h. |
| 24 | BAHourlyResDAMEnergySelfScheduleBidCount BrtubAA’pamdh | The Hourly Day Ahead Market Energy Self Schedule Count by Business Associate B, Resource ID r, Resource Type t, UDC ID u, Bid Segment b, APN_ID A, APN Type A’, Price Node ID p, Bid type a of Trading Month m, Trading Day d, and Trading Hour h. |
| 25 | BAHourlyTotalResDAMEnergySelfScheduleBidCount BrtubAA’pmdh | The Total Hourly Day Ahead Market Energy Self Schedule Count by Business Associate B, Resource ID r, Resource Type t, UDC ID u, Bid Segment b, APN_ID A, APN Type A’, Price Node ID p of Trading Month m, Trading Day d, and Trading Hour h. |
| 26 | BAHourlyResDAMEnergyBidCount BrtubAA’pmdh | The Hourly Day Ahead Market Energy Bid Count by Business Associate B, Resource ID r, Resource Type t, UDC ID u, Bid Segment b, APN_ID A, APN Type A’, Price Node ID p of Trading Month m, Trading Day d, and Trading Hour h. |
| 27 | BAHourlyTotalEnergyBidCount Bmdh | The Total Energy Bid Count by Business Associate B of Trading Month m, Trading Day d, and Trading Hour h. |
| 28 | BADailyBidSegmentFeeCount Bmd | The Total Daily Bid Segment Count by Business Associate B of Trading Month m, Trading Day d. |
| 29 | BADailyBidSegmentFeeAmount Bmd | The Daily Bid Segment Fee Amount by Business Associate B of Trading Month m, Trading Day d |

# Charge Code Effective Dates

| Charge Code/ Pre-calc Name | Document Version | Effective Start Date | Effective End Date | Version Update Type |
| --- | --- | --- | --- | --- |
| CC 4515 GMC Bid Transaction Fee | 5.0 | 01/01/12 | Open | Configuration Impacted |